3GPP TSG-RAN WG2 Meeting #127	R2-2406420

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 	ZTE Corporation

Title:            Discussion on measurement event evaluation

Agenda item:	8.6.3

Document for:	Discussion and decision

Introduction

At last meeting, we discussed event-triggered L1 measurement reporting for LTM and made the following agreement:

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this contribution, we discussed some open issues for measurement event evaluation.

Discussions

Event definition

At last meeting, we discussed the event definition and agreed to introduce Event LTM2/3/4/5. However, there is no consensus on whether Event LTM1 is needed or not. Event LTM1 is defined as beam of serving cell becomes better than absolute threshold. The corresponding RRM Event A1 is usually used to cancel handover procedure. For LTM, we think Event LTM1 can also work for cancellation of LTM related procedure, e.g. release or deactivation of L1 measurement or event configuration for candidate cells, cancellation of LTM cell switch.

Event LTM1 can be used for cancellation of LTM related procedure, e.g. release or deactivation of L1 measurement or event configuration for candidate cells, cancellation of LTM cell switch.

Besides, considering that Event LTM2/3/4/5 have been introduced, there is no much additional work to support Event LTM1 as well. Thus, we prefer to introduce Event LTM1.

- RAN2 to support Event LTM1: Beam of serving cell becomes better than absolute threshold.

It’s agreed that the beam level measurement result for event evaluation is baseline, but it’s unclear what beam(s) of the serving cell and neighboring cell is used for event evaluation. For the beam of the serving cell, there are two options on the table:

- Option 1: the beam corresponding to the indicated TCI state;

- Option 2: the best beam.

Option 1 follows the beam definition for the UE initiated beam reporting in RAN1 MIMO. The main purpose of UE initiated beam reporting is used for beam switch, so it’s natural to use the current serving beam for the event evaluation. However, the main objective of LTM event is used for cell-level mobility, so it would be simple and straightforward to use the best beam of the serving cell for the event evaluation. This is also aligned with the beam used for the RRM event evaluation. The best beam evaluated by the UE can reflect the quality change of the measured beam(s) for the serving cell in real time. Although the beam corresponding to the TCI state indicated by the NW is usually the best beam, it may have some lag in reflecting the change of the best beam, e.g. the NW does not switch the beam until receiving the next L1 measurement report.

The best beam evaluated by the UE can reflect the quality change of the measured beam(s) for the serving cell in real time. But the beam corresponding to the TCI state indicated by the NW may have some lag in reflecting the change of the best beam, e.g. the NW does not switch the beam until receiving the next L1 measurement report.

Besides, it’s assumed that the same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. If we use the beam corresponding to the indicated TCI state, it’s possible that the current serving beam corresponding to the indicated TCI state (e.g. CSI-RS beam) is not the same type of configured beams for candidate cells (e.g. SSB beam). In this case, the UE needs to search the same type of beam associated with QCL RS provided in the indicated TCI state for the LTM event evaluation. It is not such straightforward, compared to using the best beam with the same type of the candidate beam.

It’s possible that the current serving beam corresponding to the indicated TCI state (e.g. CSI-RS beam) is not the same type of configured beams for candidate cells (e.g. SSB beam).

Thus, we prefer to use the best beam among the configured beams of the serving cell for the event evaluation. In the same reason, the beam of candidate cell can also be defined as the best beam among the configured beams for simplicity.

- The best beam among the configured beams of the serving cell and candidate cell is used for the LTM event evaluation.

Event evaluation

Although we agreed that the beam level measurement result for event evaluation is baseline, the cell level measurement result should not be excluded. Considering that the reporting triggered by the LTM event is mainly used for the mobility decision, cell level measurement results can further help the NW to select the target cell and improve the mobility robustness. In some cases, the quality of the best beam may not fully reflect the quality of the cell, e.g. only one strongest beam much above a threshold but other beams far below the threshold. In this case, due to the poor quality of most beams in the target cell, another cell switch may be triggered quickly after the UE switches to this target cell, which may cause frequent ping-pong cell switch or too-short stay. Besides, the L1 measurement result may suffer large fluctuations. In order to mitigate the ping-pong effect caused by the single beam-level measurement result, the cell level measurement results can also be considered as the input for the LTM event evaluation.

The L1 measurement result may suffer large fluctuations, so the mobility decision just based on the single beam level measurement result may not ensure the mobility robustness, e.g. causing frequent ping-pong cell switch or too-short stay.

Thus, we think cell level measurement results can also be considered for LTM event evaluation. Regarding how to define the cell level measurement result for event evaluation, there are two options:

- Option 1: several beam level measurement results of the cell meet the entering condition of LTM event, i.e. to consider the best N (N>1) beams of the cell.

- Option 2: the cell level measurement result is consolidated from several beam level measurement results above an absolute threshold, i.e. similar to the cell level consolidation for RRM measurement;

Option 1 is a simpler way to just consider multiple beams of a cell for the event evaluation, i.e. no need to perform the consolidation of beam level measurement results. Option 2 follows the similar handling on the derivation of cell level measurement results in the current RRM measurement. Both options can help to mitigate the ping-pong effect caused by the signal fluctuation of the single beam level measurement result.

- The cell level measurement result is supported for the LTM event evaluation. RAN2 to discuss how to define the cell level measurement result with the following options:

- Option 1: several beam level measurement results of the cell meet the entering condition of LTM event, i.e. to consider the best N (N>1) beams of the cell；

- Option 2: the cell level measurement result is consolidated from several beam level measurement results above an absolute threshold, i.e. similar to the cell level consolidation for RRM measurement.

For LTM event evaluation, TTT mechanism can be applied. It requires the beam level measurement result of the cell continuously satisfies the entering condition of the LTM event to trigger the L1 measurement reporting. But it’s unclear whether the TTT criteria is applicable to a specific beam (i.e. beam level) or any beam of a cell (i.e. cell level). In case that the TTT is applied for a specific beam, if the evaluated beam’s quality falls below the entering condition during the TTT is running, the UE shall stop the event evaluation (e.g. reset the TTT) and shall not trigger the L1 measurement reporting. In this case, the change of the best beam of the same serving/candidate cell may reset the LTM event evaluation frequently. Considering that the L1 measurement result of a specific beam may fluctuate large due to the lack of filtering, the TTT criteria may be not easily satisfied. Thus, the triggering of the L1 report and LTM cell switch may be delayed due to the beam level measurement result fluctuation.

Considering that the L1 measurement result of a specific beam may fluctuate large due to the lack of filtering, the TTT criteria may not be easily satisfied for the specific beam. The triggering of the L1 measurement report and LTM cell switch may be delayed due to the beam level measurement result fluctuation, e.g. the change of the best beam of the cell.

An alternative way is to apply the TTT per cell. The UE shall continue the event evaluation if at least one beam of the cell meets the entering condition during the TTT is running, i.e. the change of the best beam shall not impact the event evaluation for a cell. In this way, the triggering of L1 measurement reporting shall not be delayed as long as there is at least one good enough beam of the cell. Besides, this solution can work with the LTM event evaluation based on the cell level measurement result as discussed in proposal 3, to enable the timely triggering of L1 measurement reporting while ensure the robustness of measurement results. Thus, we prefer to apply the TTT for the cell level.

- For the LTM event evaluation, TTT is evaluated per cell, i.e. not per beam. The change of the best beam of the same serving/candidate cell does not reset the TTT.

For LTM event evaluation, beam specific offset can be applied, but it’s FFS whether cell specific offset is applied as well. In the current RRM event evaluation, both cell specific offset and MO specific offset can be applied to adjust the threshold for serving cell and/or neighbour cell. Correspondingly, both cell specific offset and beam specific offset can be considered for LTM event evaluation. Especially, considering that beams from multiple cells may be grouped in one measurement resource set for the LTM event, the cell specific offset is beneficial for the more flexible adjustment of the event threshold applicable to different cells.

- For LTM event evaluation, cell specific offset can be applied.

Another issue is which layer to perform the event evaluation (i.e. determine whether the LTM event is met and trigger the measurement report) by the UE. There are two alternatives:

- Alt. 1: the UE’s physical layer;

- Alt. 2: the UE’s MAC layer.

Considering that TTT mechanism can be applied for LTM event evaluation, the evaluation layer should be able to maintain a timer to evaluate whether the entering condition is met continuously. Usually, there is no timer defined in the physical layer. It would be more straightforward to maintain a timer in the MAC layer, so it’s preferred to perform the event evaluation in the MAC layer.

- LTM event evaluation is performed in UE’s MAC layer.

Measurement configuration

In Rel-18 LTM, the LTM CSI resource configuration (i.e. LTM-CSI-ResourceConfig) has been introduced for L1 measurement. And each resource set configuration is defined to include one or more SSB resources from one or more LTM candidate cells, wherein the SSB resource configuration is provided in each LTM candidate configuration (i.e. LTM-Candidate).

LTM-CSI-ResourceConfig-r18 ::= SEQUENCE { ltm-CSI-ResourceConfigId-r18 LTM-CSI-ResourceConfigId-r18, ltm-CSI-SSB-ResourceSet-r18 LTM-CSI-SSB-ResourceSet-r18, ... } LTM-CSI-SSB-ResourceSet-r18 ::= SEQUENCE { ltm-CSI-SSB-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index, ltm-CandidateIdList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18, ... }

LTM-Candidate-r18 ::= SEQUENCE { ltm-CandidateId-r18 LTM-CandidateId-r18, ltm-CandidatePCI-r18 PhysCellId OPTIONAL, -- Need M ltm-SSB-Config-r18 LTM-SSB-Config-r18 OPTIONAL, -- Need M ... } LTM-SSB-Config-r18 ::= SEQUENCE { ssb-Frequency-r18 ARFCN-ValueNR, subcarrierSpacing-r18 SubcarrierSpacing, ssb-Periodicity-r18 ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL, -- Need R ssb-PositionsInBurst-r18 CHOICE { shortBitmap BIT STRING (SIZE (4)), mediumBitmap BIT STRING (SIZE (8)), longBitmap BIT STRING (SIZE (64)) } OPTIONAL, -- Need R ss-PBCH-BlockPower-r18 INTEGER (-60..50) OPTIONAL, -- Need R ... }

Before discussing how to provide the L1 measurement resource configuration for LTM event, we think it’s important to firstly clarify the granularity of the event application, i.e. the granularity of measurement resource associated with one event configuration. The following options can be considered:

- Option 1: the event is applied per RS or beam;

- Option 2: the event is applied per cell;

- Option 3: the event is applied per frequency;

- Option 4: the event is applied per CSI resource set.

RAN2 can firstly clarify the granularity of the event application (i.e. the granularity of measurement resource associated with one event configuration), e.g. per beam, per cell, per frequency or per CSI resource set.

The option 1 is the finest granularity of event application. In this option, a beam specific resource configuration list can be defined, e.g. a list of RS measurement resource/object. And each RS measurement resource/object can be associated with an event/report configuration. In option 2, one event configuration is applied for all configured beams in a cell. We can introduce a list of cell specific resource configuration or simply reuse the SSB/CSI-RS resource configuration within the LTM candidate configuration, e.g. LTM-SSB-Config. In option 3, one event configuration is applied for all configured beams and cells in the same frequency. The L3 MO similar configuration can be introduced. In option 4, one event configuration is applied for all beams and cells within one CSI resource set (e.g. LTM-CSI-ResourceConfig). In this way, the NW can flexibly adjust the granularity of event application by including one or more RS(s) from one or more cell(s) in one CSI resource set, e.g. including one RS from one candidate cell (i.e. per RS), all measured RSs from one candidate cell (i.e. per cell), or all measured RSs from candidate cells in the same frequency (i.e. per frequency) in one CSI resource set.

The NW can flexibly adjust the granularity of event application by including one or more RS(s) from one or more cell(s) in one CSI resource set, e.g. including one RS from one candidate cell (i.e. per RS), all measured RSs from one candidate cell (i.e. per cell), or all measured RSs from candidate cells in the same frequency (i.e. per frequency) in one CSI resource set.

Thus, option 4 is preferred to allow flexible granularity of event application. And the similar signaling structure for R18 LTM CSI resource configuration can be considered. In the existing CSI resource configuration signaling, the SSB resource set is mandatory present. And it’s agreed that both SSB and CSI-RS can be taken as the measurement resource for event-triggered measurement. If we directly extend the current signaling to include CSI-RS resources, there must be a SSB resource set associated with the CSI-RS resource even if the NW just wants to configure CSI-RS resources for the LTM event. This seems not a desired signaling design.

The SSB resource set is mandatory present in the current LTM CSI resource configuration signaling. If the current signaling is directly extended to include CSI-RS resources, there must be a SSB resource set associated with the CSI-RS resource even if the NW just wants to configure CSI-RS resources for the LTM event.

Thus, we can further discuss the detailed ASN.1 design for the measurement resource once CSI-RS resource configuration is introduced.

- The similar R18 LTM CSI resource configuration structure is reused as the measurement resource configuration for LTM event, but the detailed ASN.1 design can be discussed later once CSI-RS resource configuration is introduced.

For Rel-18 LTM L1 measurement, one LTM CSI resource configuration is associated with one LTM report configuration by including the LTM-CSI-ResourceConfigId into the LTM-CSI-ReportConfig. In order to associate the measurement resource configuration and the report configuration for LTM event, there are two options that can be considered:

- Option 1: including the LTM event configuration into the LTM-CSI-ReportConfig and reusing the LTM-CSI-ResourceConfigId to associate the measurement resource configuration with the report configuration;

- Option 2: introducing an additional LTM event report configuration (e.g. parallel to LTM-CSI-ReportConfig) and a measurement ID to associate the measurement resource configuration and the LTM event report configuration.

The current LTM-CSI-ReportConfig includes multiple mandatory IEs for UCI based measurement reporting, e.g. ltm-ReportConfigType, ltm-ReportContent. But such IEs shall not be required if MAC CE based reporting is used for event-triggered L1 measurement reporting.

LTM-CSI-ReportConfig-r18 ::= SEQUENCE { ltm-CSI-ReportConfigId-r18 LTM-CSI-ReportConfigId-r18, ltm-ResourcesForChannelMeasurement-r18 LTM-CSI-ResourceConfigId-r18, ltm-ReportConfigType-r18 CHOICE { periodic-r18 SEQUENCE { reportSlotConfig-r18 CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUCCH-r18 SEQUENCE { reportSlotConfig-r18 CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUSCH-r18 SEQUENCE { reportSlotConfig-r18 CSI-ReportPeriodicityAndOffset, reportSlotOffsetList-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-2-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-1-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), p0alpha P0-PUSCH-AlphaSetId }, aperiodic-r18 SEQUENCE { reportSlotOffsetList-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-2-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-1-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128) }, ... }, ltm-ReportContent-r18 LTM-ReportContent-r18, ... } LTM-ReportContent-r18 ::= SEQUENCE { nrOfReportedCells-r18 ENUMERATED {n1,n2,n3,n4}, nrOfReportedRS-PerCell-r18 ENUMERATED {n1,n2,n3,n4}, spCellInclusion-r18 ENUMERATED {true} OPTIONAL -- Need R }

Besides, the current LTM-CSI-ResourceConfig only includes SSB based measurement resources, which is also mandatory present. We agreed that CSI-RS based measurement resource is also supported for event-triggered L1 measurement reporting. Thus, the additional LTM CSI resource configuration ID may also be required in the LTM-CSI-ReportConfig to enable the event-triggered L1 measurement reporting based on CSI-RS resources only.

The current LTM-CSI-ReportConfig includes multiple mandatory IEs for UCI based measurement reporting, e.g. ltm-ReportConfigType, ltm-ReportContent. Such IEs shall not be required if MAC CE based reporting is used for event-triggered L1 measurement reporting.

Since we prefer to use MAC CE carry the event-triggered measurement report, it would be cleaner to introduce an additional LTM event report configuration, i.e. option 2.

- An additional report configuration is introduced for LTM event configuration, e.g. parallel to LTM-CSI-ReportConfig.

In addition, one measurement resource configuration can be associated with different event configuration, e.g. different event type. And one report configuration can be applicable to different measurement resource configurations as well. In order to associate the report configuration with the measurement resource configuration flexibly, a measurement ID can be introduced to associate one measurement resource configuration with one report configuration, similar to MeasId in L3 measurement configuration.

- A measurement ID is introduced to associate one measurement resource configuration with one report configuration for LTM event, e.g. similar to MeasId.

Another issue is whether to have a global report configuration for LTM event (i.e. like the common LTM CSI resource configuration for all candidate cells) or cell-specific report configuration (i.e. like the LTM CSI report configuration within ServingCellConfig). In subsequent LTM, with the serving cell change, the event configuration parameters for the same measurement resource used in the current serving cell may need to be different from that used in the previous serving cell (e.g. different threshold or offset), considering that the radio propagation may be various between different cell pairs. Thus, the cell-specific report configuration is more reasonable. Namely, the report configuration for LTM event and the measurement ID to associate the measurement resource configuration and the report configuration are included in the serving/candidate cell configuration, i.e. similar to LTM-CSI-ReportConfig within ServingCellConfig.

- The report configuration for LTM event and the measurement ID is included in the serving/candidate cell configuration, e.g. similar to LTM-CSI-ReportConfig within ServingCellConfig.

An example of RRC signalling on the measurement configuration for LTM event is provided in the Annex.

- RAN2 to consider the RRC signalling on the measurement configuration for LTM event in the Annex as baseline.

Conclusion and Proposals

In this contribution, we discussed the event-triggered L1 measurement reporting with the following observations and proposals:

Observation 1: Event LTM1 can be used for cancellation of LTM related procedure, e.g. release or deactivation of L1 measurement or event configuration for candidate cells, cancellation of LTM cell switch.

Observation 2: The best beam evaluated by the UE can reflect the quality change of the measured beam(s) for the serving cell in real time. But the beam corresponding to the TCI state indicated by the NW may have some lag in reflecting the change of the best beam, e.g. the NW does not switch the beam until receiving the next L1 measurement report.

Observation 3: It’s possible that the current serving beam corresponding to the indicated TCI state (e.g. CSI-RS beam) is not the same type of configured beams for candidate cells (e.g. SSB beam).

Observation 4: The L1 measurement result may suffer large fluctuations, so the mobility decision just based on the single beam level measurement result may not ensure the mobility robustness, e.g. causing frequent ping-pong cell switch or too-short stay.

Observation 5: Considering that the L1 measurement result of a specific beam may fluctuate large due to the lack of filtering, the TTT criteria may not be easily satisfied for the specific beam. The triggering of the L1 measurement report and LTM cell switch may be delayed due to the beam level measurement result fluctuation, e.g. the change of the best beam of the cell.

Observation 6: RAN2 can firstly clarify the granularity of the event application (i.e. the granularity of measurement resource associated with one event configuration), e.g. per beam, per cell, per frequency or per CSI resource set.

Observation 7: The NW can flexibly adjust the granularity of event application by including one or more RS(s) from one or more cell(s) in one CSI resource set, e.g. including one RS from one candidate cell (i.e. per RS), all measured RSs from one candidate cell (i.e. per cell), or all measured RSs from candidate cells in the same frequency (i.e. per frequency) in one CSI resource set.

Observation 8: The SSB resource set is mandatory present in the current LTM CSI resource configuration signaling. If the current signaling is directly extended to include CSI-RS resources, there must be a SSB resource set associated with the CSI-RS resource even if the NW just wants to configure CSI-RS resources for the LTM event.

Observation 9: The current LTM-CSI-ReportConfig includes multiple mandatory IEs for UCI based measurement reporting, e.g. ltm-ReportConfigType, ltm-ReportContent. Such IEs shall not be required if MAC CE based reporting is used for event-triggered L1 measurement reporting.

Event definition

Proposal 1: RAN2 to support Event LTM1: Beam of serving cell becomes better than absolute threshold.

Proposal 2: The best beam among the configured beams of the serving cell and candidate cell is used for the LTM event evaluation.

Event evaluation

Proposal 3: The cell level measurement result is supported for the LTM event evaluation. RAN2 to discuss how to define the cell level measurement result with the following options:

• Option 1: several beam level measurement results of the cell meet the entering condition of LTM event, i.e. to consider the best N (N>1) beams of the cell；

• Option 2: the cell level measurement result is consolidated from several beam level measurement results above an absolute threshold, i.e. similar to the cell level consolidation for RRM measurement.

Proposal 4: For the LTM event evaluation, TTT is evaluated per cell, i.e. not per beam. The change of the best beam of the same serving/candidate cell does not reset the TTT.

Proposal 5: For LTM event evaluation, cell specific offset can be applied.

Proposal 6: LTM event evaluation is performed in UE’s MAC layer.

Measurement configuration

Proposal 7: The similar R18 LTM CSI resource configuration structure is reused as the measurement resource configuration for LTM event, but the detailed ASN.1 design can be discussed later once CSI-RS resource configuration is introduced.

Proposal 8: An additional report configuration is introduced for LTM event configuration, e.g. parallel to LTM-CSI-ReportConfig.

Proposal 9: A measurement ID is introduced to associate one measurement resource configuration with one report configuration for LTM event, e.g. similar to MeasId.

Proposal 10: The report configuration for LTM event and the measurement ID is included in the serving/candidate cell configuration, e.g. similar to LTM-CSI-ReportConfig within ServingCellConfig.

Proposal 11: RAN2 to consider the RRC signalling on the measurement configuration for LTM event in the Annex as baseline.

References

R2-2405701  Report from session on V2X/SL, R19 NES and MOB  Vice Chairman (Samsung)

- Annex

- –	CSI-MeasConfig

The IE CSI-MeasConfig is used to configure CSI-RS (reference signals) belonging to the serving cell in which CSI-MeasConfig is included, channel state information reports to be transmitted on PUCCH on the serving cell in which CSI-MeasConfig is included and channel state information reports on PUSCH triggered by DCI received on the serving cell in which CSI-MeasConfig is included. See also TS 38.214 [19], clause 5.2.

CSI-MeasConfig information element

-- ASN1START

-- TAG-CSI-MEASCONFIG-START

CSI-MeasConfig ::=                  SEQUENCE {

nzp-CSI-RS-ResourceToAddModList     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource   OPTIONAL, -- Need N

nzp-CSI-RS-ResourceToReleaseList    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToAddModList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToReleaseList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need N

csi-IM-ResourceToAddModList         SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource           OPTIONAL, -- Need N

csi-IM-ResourceToReleaseList        SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId         OPTIONAL, -- Need N

csi-IM-ResourceSetToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet     OPTIONAL, -- Need N

csi-IM-ResourceSetToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId   OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddModList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet   OPTIONAL, -- Need N

csi-SSB-ResourceSetToReleaseList    SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSetId OPTIONAL, -- Need N

csi-ResourceConfigToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig

OPTIONAL, -- Need N

csi-ResourceConfigToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfigId

OPTIONAL, -- Need N

csi-ReportConfigToAddModList        SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig  OPTIONAL, -- Need N

csi-ReportConfigToReleaseList       SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId

OPTIONAL, -- Need N

reportTriggerSize                   INTEGER (0..6)                                                            OPTIONAL, -- Need M

aperiodicTriggerStateList           SetupRelease { CSI-AperiodicTriggerStateList }                            OPTIONAL, -- Need M

semiPersistentOnPUSCH-TriggerStateList    SetupRelease { CSI-SemiPersistentOnPUSCH-TriggerStateList }         OPTIONAL, -- Need M

...,

[[

reportTriggerSizeDCI-0-2-r16        INTEGER (0..6)                                                            OPTIONAL -- Need R

]],

[[

sCellActivationRS-ConfigToAddModList-r17  SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-Config-r17   OPTIONAL, -- Need N

sCellActivationRS-ConfigToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-ConfigId-r17 OPTIONAL  -- Need N

]],

[[

ltm-CSI-ReportConfigToAddModList-r18   SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfig-r18

OPTIONAL, -- Need N

ltm-CSI-ReportConfigToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfigId-r18

OPTIONAL  -- Need N

]],

[[

ltm-Event-ReportConfigToAddModList-r19   SEQUENCE (SIZE (1..maxNrofLTM-Event-ReportConfigurations-r19)) OF LTM-Event-ReportConfig-r19

OPTIONAL, -- Need N

ltm-Event-ReportConfigToReleaseList-r19  SEQUENCE (SIZE (1..maxNrofLTM-Event-ReportConfigurations-r19)) OF LTM-Event-ReportConfigId-r19

OPTIONAL,  -- Need N

ltm-MeasIdToAddModList-r19   SEQUENCE (SIZE (1..maxNrofLTM-MeasIds-r19)) OF LTM-MeasIdConfig-r19

OPTIONAL, -- Need N

ltm-MeasIdToReleaseList-r19  SEQUENCE (SIZE (1..maxNrofLTM-MeasIds-r19)) OF LTM-MeasId-r19

OPTIONAL  -- Need N

]]

}

-- TAG-CSI-MEASCONFIG-STOP

-- ASN1STOP

- –	LTM-Event-ReportConfig

The IE LTM-Event-ReportConfig is used to configure event-triggered L1 measurement report on the cell in which the LTM-Event-ReportConfig is included.

LTM-Event-ReportConfig information element

-- ASN1START

-- TAG-LTM-Event-REPORTCONFIG-START

LTM-Event-ReportConfig-r19 ::=      SEQUENCE {

ltm-Event-ReportConfigId-r19                     LTM-Event-ReportConfigId-r19,

ltm-Event-TriggerConfig-r19                     LTM-Event-TriggerConfig-r19

...

}

LTM-EventTriggerConfig-r19 ::=                      SEQUENCE {

LTM-eventId-r19                                     CHOICE {

LTM-eventA2-r19                                     SEQUENCE {

a2-Threshold-r19                                MeasTriggerQuantity,

hysteresis-r19                                  Hysteresis,

timeToTrigger-r19                               TimeToTrigger

},

LTM-eventA3-r19                                     SEQUENCE {

a3-Offset-r19                                   MeasTriggerQuantityOffset,

hysteresis-r19                                  Hysteresis,

timeToTrigger-r19                               TimeToTrigger,

},

LTM-eventA4-r19                                     SEQUENCE {

a4-Threshold-r19                                MeasTriggerQuantity,

hysteresis-r19                                  Hysteresis,

timeToTrigger-r19                               TimeToTrigger,

},

LTM-eventA5-r19                                     SEQUENCE {

a5-Threshold1-r19                               MeasTriggerQuantity,

a5-Threshold2-r19                               MeasTriggerQuantity,

hysteresis-r19                                  Hysteresis,

timeToTrigger-r19                               TimeToTrigger,

},

...,

},

...

}

-- TAG-LTM-EVENT-REPORTCONFIG-STOP

-- ASN1STOP

- –	LTM-Event-ReportConfigId

The IE LTM-Event-ReportConfigId is used to identify an LTM-Event-ReportConfig.

LTM-Event-ReportConfigId information element

-- ASN1START

-- TAG-LTM-EVENT-REPORTCONFIGID-START

LTM-Event-ReportConfigId-r19 ::=            INTEGER (0..maxNrofLTM-Event-ReportConfigurations-1-r19)

-- TAG-LTM-EVENT-REPORTCONFIGID-STOP

-- ASN1STOP

- –	LTM-MeasId

The IE LTM-MeasId is used to identify an LTM measurement configuration, i.e., linking of a measurement resource and a reporting configuration.

LTM-MeasId information element

-- ASN1START

-- TAG-LTM-MEASID-START

LTM-MeasId-r19 ::=                          INTEGER (1.. maxNrofLTM-MeasIds-r19)

-- TAG-LTM-MEASID-STOP

-- ASN1STOP

- –	LTM-MeasIdConfig

The IE LTM-MeasIdConfig is used to configure the association between a measurement resource and a reporting configuration.

LTM-MeasIdConfig information element

-- ASN1START

-- TAG-LTM-MEASIDCONFIG-START

LTM-MeasIdConfig ::=                  SEQUENCE {

ltm-measId-r19                              LTM-MeasId-r19,

ltm-measObjectId-r19                        LTM-CSI-ResourceConfigId-r18,

ltm-reportConfigId-r19                      LTM-Event-ReportConfigId-r19,

...

}

-- TAG-LTM-MEASIDCONFIG-STOP

-- ASN1STOP