3GPP TSG-RAN2 Meeting #127	R2-2406422

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 			ZTE Corporation, Sanechips

Title: 	Evaluation on RRM measurement prediction

Agenda item:	8.3.2.1

Document for: 	Discussion and Decision

- Introduction

In the RAN2 125bis and RAN2 126 meeting, the simulation assumption, evaluation methodology and metric for RRM measurement prediction were discussed, and many fruitful agreements were achieved [1][2]. Particularly, we aligned most of the simulation assumptions and agreed that average RSRP difference can be used as one of the KPI for performance evaluation. In this contribution, we will provide our initial simulation results for the RRM measurement prediction and share our views on them.

- Discussion

RRM measurement prediction aims to obtain the measurement results without actual measurement in order to reduce measurement overhead and/or improve handover performance. The agreed use cases include temporal domain measurement prediction, frequency domain measurement prediction and spatial domain measurement prediction. In the last meeting, we have discussed the priority of evaluation scenario. The evaluation scenario with higher priority include FR1 to FR1 intra-frequency temporal domain prediction case B, FR2 to FR2 intra-freq temporal domain prediction case A, FR1 to FR1 frequency domain prediction and FR2 to FR2 intra-frequency spatial domain prediction. In this section, we will provide our initial simulation results of these scenarios.

## 2.1 General description

To evaluate performance of RRM measurement prediction, we generate the datasets for model training, validation and inference according to agreed simulation assumptions. In our simulation, 100000 samples are generated through multiple drops, and used for training, validity and test after randomly sorting. Depending on the use cases, the meaning of the samples are different. Specifically, each sample of temporal domain measurement prediction includes the measurement results within observation window and prediction window. Each sample of frequency domain measurement prediction includes the measurement results of frequency for measurement and frequency for prediction. Each sample of spatial domain measurement prediction include the measurement results of all BS beams, including beam for measurement and beam for prediction.

Based on the RAN2’s agreement, companies are expected to report what L1 filtering they use in their simulation. In our simulation, for the measurement model in FR1 scenario, the UE physical layer will take 5 samples during L1 measurement period with 40ms sampling period, and derive L1 filtered results by averaging the results of 5 samples. The layer 1 filtered results are reported to the RRC layer every 200ms. Then the RRC layer will perform beam consolidation and L3 filtering to get L3 filtered cell results according to the agreed RRC configurations. The measurement model in FR1 scenario is shown in Fig. 2.1.1:

Fig. 2.1.1: Measurement model in FR1 scenario

For the measurement model in FR2 scenario, we adopt 4 UE Rx beams in the simulation. The UE physical layer takes one sample per Rx beam and select the best result as the result for one round sampling. This will repeat 5 times. The layer 1 filtered results are derived via averaging results of 5 rounds. The physical layer will report the layer 1 filtered results to the RRC layer every 400ms, and the RRC layer will perform beam consolidation and layer 3 filtering to get layer 3 filtered cell results. The measurement model in FR2 scenario is shown in Fig.2.1.2:

Fig. 2.1.2: Measurement model in FR2 scenario

Regarding to the metric for prediction accuracy, besides the agreed average L3 cell level RSRP difference, we also provide the CDF curve of RSRP difference, which is more intuitive and can provide more information, e.g. the worst prediction performance.

## 2.2 Simulation results for FR1 to FR1 intra-frequency temporal domain case B

FR1 to FR1 intra frequency temporal domain case B is done by predicting sub set measurement instances in temporal domain of the same cell, and the detailed skipping pattern i.e., which time instances are skipped and hence predicted in temporal domain, is not defined. The temporal domain prediction case B model adopted in our simulation is shown in Fig. 2.2.1. The UE performs actual measurement at a few time instances (observation window) first, then based on these results, to predict the results at a few future time instances (prediction window). The motivation of case B is to reduce measurement overhead, the time instances for prediction will not be actual measured. Take Fig.2.2.1 for example, the cell results at time instance  and  are predicted based on the actual measurement results at time instances ~, and the UE will not perform actual measurement at predicted time instance  and . The length of observation window and prediction window are up to different simulation conditions, and will be reported with the corresponding simulation results. As can be seen in Fig. 2.2.1, all results are predicted in advance. This is because: taking L3 filtered results as model input, it is too late if the prediction is performed after the corresponding skipped time instance.

Fig. 2.2.1: Temporal domain measurement prediction case B

The follows shows the simulation results in various simulation conditions:

Table 2.2.1: Simulation temple table for FR1 to FR1 temporal domain measurement prediction case B

| Report parameters |  | ZTE |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 3 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | circle-bouncing model |
|  | UE speed (30,60,90,120 Km/h) |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | \ |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | \ |
|  | Whether LOSsoft is modeled or not | not |
|  | spatial consistency option (A or B) |  |
|  | Number of TX beams | 4 |
|  | Number of RX beams | 1 |
|  | Measurement reduction rate(50%,…Note2) |  |
|  | Observation window(Note3) | \ |
|  | Prediction window (Note3) | \ |
|  | Any other parameters (Note 4) | BS antenna configuration: -16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ UE antenna configuration: (1,1,2,1,1,1,1) |
| Data Size (Number of Samples) | Training/validity | (90000,5000) |
|  | Testing | 5000 |
| AI/ML model input/output | Model input (Note 5) | RRM sub use 2, layer 3 filtered cell result |
|  | Model output(Note 6) | RRM sub use 2, layer 3 filtered cell result |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | See the table 2.2.2 |
|  | Model complexity in model size (e.g. Mbyte) |  |
|  | Computational complexity [FLOPs] |  |
| Metrics | Average L3 cell level RSRP difference (dB) | See the table 2.2.3 and table 2.2.4 for the best cell and sub-optimal cell |

Table 2.2.2 Model complexity for temporal domain measurement prediction Case A

| Measurement reduction rate | [observation window length, prediction window length] | Model complexity in a number of parameters (M) | Model complexity in model size | Computational complexity [FLOPs] |
| --- | --- | --- | --- | --- |
| 33.3% | [400ms, 200ms] | 791.811 K | 3.021 M | 1.592 M |
| 50% | [400ms, 400ms] | 792.580 K | 3.023 M | 1.592 M |
|  | [600ms, 600ms] | 793.861K | 3.028 M | 2.389 M |
| 60% | [400ms, 600ms] | 793.861K | 3.028 M | 2.389M |

Table 2.2.3 Simulation results for temporal domain measurement prediction case B on the best cell

| Measurement reduction rate | [Observation window , predication window] | spatial consistency option | UE speed (km/h) | Average RSRP difference (dB) |
| --- | --- | --- | --- | --- |
| 33.3% | [400ms, 200ms] | A | 30 | 0.27 |
|  |  |  | 90 | 0.39 |
|  |  | B | 30 | 0.56 |
|  |  |  | 90 | 0.72 |
| 50% | [400ms, 400ms] | A | 30 | [0.36, 0.42] |
|  |  |  | 90 | [0.51, 0.64] |
|  |  | B | 30 | [0.87, 0.97] |
|  |  |  | 90 | [1.03, 1.27] |
|  | [600ms, 600ms] | A | 30 | [0.41, 0.46, 0.52] |
|  |  |  | 90 | [0.57, 0.69, 0.84] |
|  |  | B | 30 | [0.93, 1.11, 1.26] |
|  |  |  | 90 | [1.21, 1.56, 1.86] |
| 60% | [400ms, 600ms] | A | 30 | [0.44, 0.49, 0.56] |
|  |  |  | 90 | [0.64, 0.76, 0.91] |
|  |  | B | 30 | [0.99, 1.17, 1.36] |
|  |  |  | 90 | [1.33, 1.62, 1.90] |
|  |  |  |  | Note: xn in the [x1,x2,...xn] represents the average L3 cell level RSRP difference at future time instance 200ms*xn |

| - (a)CDF of RSRP difference in the case of 30km/h UE speed and spatial consistency option A | (b)CDF of RSRP difference in the case of 90km/h UE speed and spatial consistency option A |
| --- | --- |
| (c)CDF of RSRP difference in the case of 30km/h UE speed and spatial consistency option B | (a) CDF of RSRP difference in the case of 90km/h UE speed and spatial consistency option B |

Fig 2.2.1: CDF of temporal domain measurement prediction case B on the best cell (note: CDF curve is for RSRP difference at the last predicted time instance within prediction window)

Table 2.2.4 Simulation results for temporal domain measurement prediction case B on the sub-best cell

| Measurement reduction | [Observation window , predication window] | spatial consistency option | UE speed (km/h) | Average RSRP difference (dB) |
| --- | --- | --- | --- | --- |
| 33.3% | [400ms, 200ms] | A | 30 | 0.29 |
|  |  |  | 90 | 0.37 |
|  |  | B | 30 | 0.59 |
|  |  |  | 90 | 0.71 |
| 50% | [400ms, 400ms] | A | 30 | [0.42, 0.45] |
|  |  |  | 90 | [0.55, 0.61] |
|  |  | B | 30 | [0.84, 0.95] |
|  |  |  | 90 | [1.07, 1.32] |
|  | [600ms, 600ms] | A | 30 | [0.47, 0.53, 0.58] |
|  |  |  | 90 | [0.62,0.74,0.78] |
|  |  | B | 30 | [0.98, 1.20, 1.35] |
|  |  |  | 90 | [1.35, 1.63, 1.92] |
| 60% | [400ms, 600ms] | A | 30 | [0.54, 0.57, 0.60] |
|  |  |  | 90 | [0.72,0.80,0.86] |
|  |  | B | 30 | [1.13, 1.21, 1.39] |
|  |  |  | 90 | [1.43, 1.67, 1.98] |
|  |  |  |  | Note: xn in the [x1,x2,...xn] represents the average L3 cell level RSRP difference at future time instance 200ms*xn |

| - CDF of RSRP difference in the case of 30km/h UE speed and spatial consistency option A | - CDF of RSRP difference in the case of 90km/h UE speed and spatial consistency option A |
| --- | --- |
| - CDF of RSRP difference in the case of 30km/h UE speed and spatial consistency option B | - CDF of RSRP difference in the case of 30km/h UE speed and spatial consistency option B |

Fig. 2.2.1: CDF of temporal domain measurement prediction case B on the sub-best cell (note: CDF curve is for RSRP difference at last predicted time instance within prediction window)

The measurement reduction rate in temporal domain (MRRT) are adopted as metric to evaluate the measurement reduction, which is calculated by skipped measurement time instances / total measurement time instances. We agreed to align various measurement reduction rate among companies, however, the detailed value has not been determined yet. In our simulation, we consider 33.7%, 50% and 60% MRRT. Comparing the simulation results, we can find that the prediction accuracy is decreased with the increase of MRRT. Increasing MRRT from 33.3% to 60%, the average RSRP difference will increase at most 1.27dB, and the maximum prediction error will increase 0.8 dB. This is because the AI model input in higher MRRT scenario can provide less meaningful information for the AI model, which will degrade prediction accuracy.

Observation 1: With the increase of MRRT from 33.3% to 60%, the average RSRP difference increases at most 1.27dB, and the prediction accuracy for temporal domain measurement prediction case B decreases.

In order to study the impact of skipping pattern on the prediction accuracy, we study the case with the same MRRT but different skipping patterns. Specifically, two skipping patterns are studied in the case of 50% MRRT, one is to predict the measurement results of future 400ms based on the measurement results of historical 400ms (i.e. [400ms,400ms]); the other is to predict the measurement results of future 600ms based on the measurement results of historical 600ms (i.e. [600ms,600ms]). According to the simulation results, we can see that even with the same MRRT, the prediction accuracy is different with different skipping pattern. In our simulation, the prediction accuracy of [400ms, 400ms] skipping pattern is much better than that of [600ms, 600ms], and the difference for average prediction error with two skipping pattern can achieve up to 0.6dB.

Observation 2: Regarding temporal domain measurement prediction B, even with the same MRRT, skipping pattern (e.g. [400ms,400ms] or [600ms,600ms]) can also impact the prediction accuracy, their average prediction error difference achieves up to 0.6dB.

Based on the previous agreements, it’s up to companies to select the spatial consistency option A or option B to simulate the spatial consistency. We are wondering whether the prediction accuracy is different with different spatial consistency option. So, we perform simulation with the two options respectively, and compare their simulation results. Then we find that the prediction accuracy of temporal domain measurement prediction with spatial consistency option A is better than that with option B under the same condition. Compared with option B, the maximum RSRP differencewith case A reduces at most 40%.

Observation 3: Regarding temporal domain measurement prediction B, the prediction accuracy of temporal domain measurement prediction with spatial consistency option A is better than that with option B.

As can be seen, in the same condition, the temporal domain prediction with 30 km/h UE speed can achieve better prediction accuracy than that with 90km/h. Compared with 90km/h UE speed, the average RSRP difference with 30km/h UE speed decreases 32.2%, and the maximum RSRP difference decreases 33.3%. This is consistent with our intuition, the channel condition in high speed scenario are changed more rapidly, which bring difficulty for measurement results prediction.

Observation 4: As UE speed decreases from 90km/h to 30km/h, the prediction accuracy of temporal domain measurement prediction Case B improves. The average and maximum RSRP difference decrease at most 32.2% and 33.3%, respectively.

We also note that in some cases, even with the same observation window length and to predict the cell results at the same future time instance, the prediction accuracy can be different. For example, predicted cell result in [400ms,200ms] case, the first predicted result in the [400ms, 400ms] case and the first predicted result in the [400ms, 600ms] case are all to predict the measurement results of future 200ms based on the measurement results of historical 400ms, however, their prediction accuracy are different. The [400ms, 200ms] case can provide the best prediction accuracy, and the [400ms, 600ms] provides the worst prediction accuracy. And the same tendency can be seen in the second cell result in [400ms, 400ms] case and [400ms, 600ms] case. Since the AI model is a black box, it’s difficult to explain the reason precisely. But our understanding is that from the perspective of optimization procedure, their optimization goals are different. Specifically, model training for [400ms, 200ms] case aims to achieve optimal prediction accuracy for predicted cell results at future 200ms. But for [400ms, 400ms] and [400ms,600ms] cases, besides that, their model training also need to optimize the prediction accuracy for cell results at other future time instances, .e.g the cell results at future 400ms.

Observation 5: For temporal domain measurement prediction case B, even under the same condition, different AI model can bring different prediction accuracy.

Besides, we perform prediction on different cells in the simulation to study whether the cell that perform temporal domain prediction on will impact prediction accuracy. Considering the motivation of prediction is for handover, we adopt the best cell and sub-best cell based on the signal quality (i.e. RSRP value). Comparing simulation results, we find that the prediction accuracy of temporal domain measurement prediction Case B on the best cell and sub-best cell are similar. The difference between the average RSRP difference for best cell and for sub-best cell are within 0.14dB, and the tendency of the RSRP difference CDF curve on two cells are almost the same.

Observation 6: The prediction accuracy of temporal domain measurement prediction case B on the best cell is similar as that on the sub-best cell. And the difference between the average RSRP difference for the best cell and for the sub-best cell keeps within 0.14dB.

## 2.3 Simulation results for FR2 to FR2 intra frequency temporal domain case A

According to previous agreements, FR2 to FR2 intra frequency temporal domain case A is done by predicting measurement results in prediction window based on measurement results in observation window of the same cell. The model for intra-freq temporal domain case A is shown in Figure 2.3.1. The difference between temporal domain prediction case A and case B are whether the measurement are reduced or not. For case A, although the cell results at time instance t are predicted, the UE also needs to perform actual measurement on the time instance t, i.e., no measurement reduction. Considering we do not need to perform system level simulation at this stage, there is no obvious difference between temporal domain prediction case A and case B in the simulation.

Fig. 2.3.1: Temporal domain measurement prediction case A

The follows are the simulation results shown in form of simulation template table:

Table 2.3.1: Simulation temple table for FR2 to FR2 temporal domain measurement prediction case A

| Report parameters |  | ZTE |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 3 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | circle-bouncing model |
|  | UE speed (30,60,90,120 Km/h) |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | \ |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | \ |
|  | Whether LOSsoft is modeled or not | not |
|  | spatial consistency option (A or B) | A |
|  | Number of TX beams | 32 |
|  | Number of RX beams | 4 |
|  | Measurement reduction rate(50%,…Note2) | \ |
|  | Observation window(Note3) |  |
|  | Prediction window (Note3) |  |
|  | Any other parameters (Note 4) | BS antenna configuration: Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) |
| Data Size (Number of Samples) | Training/validity | (90000,5000) |
|  | Testing | 5000 |
| AI/ML model input/output | Model input (Note 5) | RRM sub use 1, layer 1 filtered beam result RRM sub use 2, layer 3 filtered cell result |
|  | Model output (Note 6) | RRM sub use 1, layer 1 filtered beam result RRM sub use 2, layer 3 filtered cell result |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | See the table 2.3.2 |
|  | Model complexity in model size (e.g. Mbyte) |  |
|  | Computational complexity [FLOPs] |  |
| Metrics | Average L3 cell level RSRP difference (dB) | See the table 2.3.3 and table 2.3.4 |

Table 2.3.2 Model complexity for temporal domain measurement prediction Case A

| RRM sub use case | Speed (km/h) | [observation window length, prediction window length] | Model complexity in model size | Model complexity in a number of parameters | Computational complexity [FLOPs] |
| --- | --- | --- | --- | --- | --- |
| RRM sub use case 1 | 120 | [800ms, 400ms] | 3.176 M | 832.576 K | 1.665 M |
|  |  | [800ms, 800ms] | 3.270 M | 857.184 K | 1.690 M |
|  |  | [1200ms, 1200ms] | 3.426 M | 898.176 K | 2.559 M |
| RRM sub use case 2 | 120 | [800ms, 400ms] | 3.021 M | 791.811 K | 1.592 M |
|  |  | [800ms, 800ms] | 3.023 M | 792.580K | 1.592M |
|  |  | [1200ms, 1200ms] | 3.028 M | 793.861K | 2.389M |

Table 2.3.3 simulation results for FR2 to FR2 temporal domain measurement prediction case A for the best cell

| RRM sub use case | UE Speed (km/h) | [observation window length, prediction window length] | Average L3 cell level RSRP difference (dB) |
| --- | --- | --- | --- |
| RRM sub use case 1 | 60 | [800ms, 400ms] | 0.70 |
|  |  | [800ms, 800ms] | [0.74, 1.22] |
|  |  | [1200ms, 1200ms] | [0.80, 1.22, 1.69] |
|  | 120 | [800ms, 400ms] | 0.83 |
|  |  | [800ms, 800ms] | [0.89 , 1.54] |
|  |  | [1200ms, 1200ms] | [1.02, 1.63 , 2.28] |
| RRM sub use case 2 | 60 | [800ms, 400ms] | 0.64 |
|  |  | [800ms, 800ms] | [0.72, 1.16] |
|  |  | [1200ms, 1200ms] | [0.80, 1.21, 1.67] |
|  | 120 | [800ms, 400ms] | 0.74 |
|  |  | [800ms, 800ms] | [0.88, 1.48] |
|  |  | [1200ms, 1200ms] | [0.96, 1.53, 2.18] |
|  |  |  | Note: xn in the [x1,x2,...xn] represents the average L3 cell level RSRP difference at future time instance 400ms*xn |

| - (a)CDF of RSRP difference in the case of 60km/h UE speed and RRM sub use case 1 | (b)CDF of RSRP difference in the case of 120km/h UE speed and RRM sub use case 1 |
| --- | --- |
| (c)CDF of RSRP difference in the case of 60km/h UE speed and RRM sub use case 2 | (d)CDF of RSRP difference in the case of 120km/h UE speed and RRM sub use case 2 |

Figure 2.3.2: CDF of of RSRP difference for temporal domain measurement prediction case A on best cell  (note: CDF curve is for RSRP difference at the last predicted time instance within prediction window)

Table 2.3.4 simulation results for FR2 to FR2 temporal domain measurement prediction case A for the sub-best cell

| RRM sub use case | Speed (km/h) | [observation window length, prediction window length] | Average L3 cell level RSRP difference (dB) |
| --- | --- | --- | --- |
| RRM sub use case 1 | 60 | [800ms, 400ms] | 0.76 |
|  |  | [800ms, 800ms] | [0.88, 1.29] |
|  |  | [1200ms, 1200ms] | [0.93, 1.44, 1.95] |
|  | 120 | [800ms, 400ms] | 0.96 |
|  |  | [800ms, 800ms] | [1.07, 1.6] |
|  |  | [1200ms, 1200ms] | [1.22, 1.80, 2.42] |
| RRM sub use case 2 | 60 | [800ms, 400ms] | 0.57 |
|  |  | [800ms, 800ms] | [0.73, 1.01] |
|  |  | [1200ms, 1200ms] | [0.84, 1.15, 1.50] |
|  | 120 | [800ms, 400ms] | 0.74 |
|  |  | [800ms, 800ms] | [0.91, 1.35] |
|  |  | [1200ms, 1200ms] | [1.03, 1.55, 2.05] |
|  |  |  | Note: xn in the [x1,x2,...xn] represents the average L3 cell level RSRP difference at future time instance 400ms*xn |

| - CDF of RSRP difference in the case of 60km/h UE speed with RRM sub use case 1 | (b) CDF of RSRP difference in the case 120km/h UE speed and RRM sub use case 1 |
| --- | --- |
| (c) CDF of RSRP difference in the case of 60km/h UE speed and RRM sub use case 2 | (d) CDF of RSRP difference in the case of 90km/h UE speed and RRM sub use case 2 |

Figure 2.3.3: CDF of RSRP difference for temporal domain measurement prediction case A on sub-best cell  (note: CDF curve is for RSRP difference at the last predicted time instance within prediction window)

Based on the simulation results above, we find that with the same observation window length, the prediction accuracy is decreased with the increase of predicted time. For example, for the case with 60km/h UE speed, RRM sub use case 1 and 1200ms observation window length, the average RSRP difference for cell results at future time instance 400ms, 800ms and 1200ms are 0.80dB, 1.22dB and 1.69dB, respectively. This is because as time goes by, the the correlation between the historical channel and future channel becomes weaker, and the UE is more likely to change its trajectory. This type of uncertainty will degrade e prediction performance.

Observation 7: With the same observation window length, the prediction accuracy is decreased with the increase of predicted time, and the average RSRP difference increases at most 127.1%.

Comparing the simulation results of RRM sub use case 1 and RRM sub use case 2, we can find that the prediction performance of RRM sub use case 1 and 2 are similar. And in most cases, the prediction performance of RRM sub use case 2 is slightly better than that of RRM sub use case 1, but the maximum difference is less than 0.5dB. In our understanding, this may be because: for RRM sub use case 1, the AI model output is intermediate result, instead of the final result. And the optimization procedure of AI model training for RRM sub use case 1 is to optimize RSRP difference between the all predicted BS beams and actual beams, while only top-3 beams are used for cell results derivation. But for RRM sub use case 2, the AI model output is layer 3 filtered cell results, which is more direct. And the optimization procedure is to optimize the RSRP difference between predicted cell results and actual cell results. Considering our metric is average L3 filtered results, the prediction accuracy of RRM sub use case 2 is slightly better.

Observation 8: The prediction accuracy of temporal domain measurement prediction Case A with RRM sub use case 1 and RRM sub use case 2 are similar, the maximum difference is less than 0.5dB.

Finally, we can find that the impact of UE speed, AI model and cell selection (best cell or sub-best cell) on the temporal domain measurement prediction case B are the same as temporal domain prediction case A. So, the observation 4~6 can also be applicable for temporal domain prediction case A.

Observation 9: The observation 4~6 for temporal domain measurement prediction case B are applicable for temporal domain prediction case A.

## 2.4 Simulation results for FR1 to FR1 inter-frequency (frequency domain)

Frequency domain measurement prediction is to predict the measurement results of one frequency based on the measurement results of the another frequency. Based on the agreements, our simulation focus on the scenario that the cell for measurement and the cell for prediction are co-located and in the same sector. And considering the adjacent frequency may suffer similar shadowing fading in the reality, a correlation model for shadowing fading is needed in the simulation. In our understanding, the correlation is affected by multiple factors, such as frequency difference between frequency for measurement and the frequency for prediction, correlation frequency domain length and soon. It is difficult to define a precise partial correlation inter-freq shadowing fading model. So in our simulation, we evaluate the frequency domain measurement prediction with full correlation and no correlation inter-freq shadowing fading model, which can be considered as a upper boundary and lower boundary of the prediction performance with partial correlation model.

The follows are the simulation results shown in form of simulation template table:

Table 2.4.1: Simulation template table for frequency domain measurement prediction

| Report parameters |  | ZTE |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 3 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | circle-bouncing model |
|  | UE speed (30,60,90,120 Km/h) |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) |  |
|  | Whether LOSsoft is modeled or not | not |
|  | spatial consistency option (A or B) | A |
|  | Number of TX beams | 4 |
|  | Number of RX beams | 1 |
|  | Measurement reduction rate(50%,…Note2) | \ |
|  | Observation window(Note3) | \ |
|  | Prediction window (Note3) | \ |
|  | Any other parameters (Note 4) | BS antenna configuration: -16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ UE antenna configuration:(1,1,2,1,1,1,1) Frequency Range: FR1@{4GHz,30KHz} FR1@{2GHz, 30KHz} |
| Data Size (Number of Samples) | Training/validity | (90000,5000) |
|  | Testing | 5000 |
| AI/ML model input/output | Model input (Note 5) | RRM sub use 2, layer 3 filtered cell result |
|  | Model output(Note 6) | RRM sub use 2, layer 3 filtered cell result |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | 0.486 K |
|  | Model complexity in model size (e.g. Mbyte) | 0.002 M |
|  | Computational complexity [FLOPs] | 4.797 K |
| Metrics | Average L3 cell level RSRP difference (dB) | See the table 2.4.3 |

Table 2.4.2: Simulation results for FR1 to FR1 frequency domain measurement prediction

| Inter-frequency shadow fading correlation | 30 km/h | 90 km/h |
| --- | --- | --- |
| full | 0.99 | 0.99 |
| no | 1.38 | 1.4 |

| - (a)CDF of RSRP difference in the case of 30km/h UE speed | (b)CDF of RSRP difference in the case of 90km/h UE speed |
| --- | --- |

Figure 2.4.1 CDF of RSRP difference for frequency domain measurement prediction

Comparing the simulation results in full correlation and no correlation inter-freq shadowing fading, we can find that no matter average RSRP difference value or RSRP difference CDF curve, the prediction accuracy of frequency domain prediction with full correlation inter-freq shadowing fading is better than that with no correlation model. It is easy to understand, the uncertainty of shadowing fading brings additional difficulty for prediction. But even with no correlation inter-freq shadowing model, the maximum average RSRP difference is only 1.4dB, which is acceptable.

Observation 10: The prediction accuracy of full correlation inter-freq shadowing fading model is better than that of no correlation model.

Observation 11: The maximum average RSRP difference of frequency domain prediction with no correlation inter-freq shadowing fading model is only 1.4dB, which is acceptable.

Besides, average L3 cell level RSRP difference for frequency domain measurement prediction with 30km/h UE speed and with 90km/h UE speed are similar, and their CDF curves are also close. In the case of full correlation inter-freq shadow fading model, the average RSRP difference with 30km/h UE speed is the same as that with 90km/h; in the case of no correlation inter-freq shadow fading model, the difference between the average RSRP difference with 30km/h UE speed and with 90km/h is only 0.02dB.

Observation 12: The prediction accuracy of frequency domain measurement prediction is not impacted by UE speed.

## 2.5 Simulation results for FR2 to FR2 intra-frequency spatial domain measurement prediction

Spatial domain prediction is to predict L3 cell level measurements by measuring sub set of configured SSB of the same cell. Only RRM sub case 1 and case 3 are considered. The follows are the simulation results shown in form of simulation template table:

Table 2.4.1: Spatial template table for frequency domain measurement prediction

| Report parameters |  | ZTE |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 3 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | circle-bouncing model |
|  | UE speed (30,60,90,120 Km/h) | 60km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | \ |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | \ |
|  | Whether LOSsoft is modeled or not | not |
|  | spatial consistency option (A or B) | A |
|  | Number of TX beams | 32 |
|  | Number of RX beams | 4 |
|  | Any other parameters (Note 4) | BS antenna configuration: Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Frequency Range: FR2 @ 30 GHz; SCS: 120 kHz System BW:80MHz |
| Data Size (Number of Samples) | Training/validity | (90000,5000) |
|  | Testing | 5000 |
| AI/ML model input/output | Model input (Note 5) | RRM sub use 1/3, layer 1 filtered beam result |
|  | Model output(Note 6) | RRM sub use 1/3, layer 1 filtered beam result |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | See the table 2.5.2 |
|  | Model complexity in model size (e.g. Mbyte) |  |
|  | Computational complexity [FLOPs] |  |
| Metrics | Average L3 cell level RSRP difference (dB) | See table 2.5.3 and table 2.5.4 |

Table 2.5.2: Model complexity for FR2 to FR2 intra-frequency spatial domain prediction

| RRM sub use case | MRRS | Model complexity in model size | Model complexity in a number of parameters | Computational complexity [FLOPs] |
| --- | --- | --- | --- | --- |
| RRM sub use case 1 | 75% (8-->32) | 0.004 M | 0.99 K | 4.064 K |
|  | 50% (16-->32) | 0.012 M | 3.222 K | 8.256 K |
| RRM sub use case 3 | 75% (8-->32) | 0.003 M | 0.771 K | 2.700 K |
|  | 50% (16-->32) | 0.010 M | 2.695 K | 6.644 K |

Table 2.5.3: Simulation results for intra-freq spatial domain on best cell

| RRM sub use case | MRRS | Average L3 cell level RSRP difference (dB) |
| --- | --- | --- |
| RRM sub use case 1 | 75% (8-->32) | 0.92 |
|  | 50% (16-->32) | 0.38 |
| RRM sub use case 3 | 75% (8-->32) | 1.29 |
|  | 50% (16-->32) | 0.89 |

| - CDF for RSRP difference in the case of 8-->32 beam pattern | - CDF for RSRP difference in the case of 8-->32 beam pattern |
| --- | --- |

Fig. 2.5.2 CDF of RSRP difference for spatial domain prediction on the best cell

Table 2.5.4: Simulation results for intra-freq spatial domain on sub-best cell

| RRM sub use case | MRRS | Average L3 cell level RSRP difference (dB) |
| --- | --- | --- |
| RRM sub use case 1 | 75% (8-->32) | 0.92 |
|  | 50% (16-->32) | 0.37 |
| RRM sub use case 3 | 75% (8-->32) | 1.27 |
|  | 50% (16-->32) | 0.89 |

| - CDF for RSRP difference in the case of 8-->32 beam pattern | - CDF for RSRP difference in the case of 8-->32 beam pattern |
| --- | --- |

Fig. 2.5.3 CDF of RSRP difference for spatial domain prediction on the sub-best cell

In the simulation, we study the impact of MRRS on the prediction accuracy. Although we have agreed to align the MRRS, the detailed value has not been determined yet. In our simulation, we consider 75% MRRS and 50% MRRS. According to the simulation results, we find that with the decrease of MRRS, the prediction accuracy can be improved largely. By increasing MRRS from 50% to 75%, the average RSRP difference increases at most 183%.

Observation 13: With the MRRS increase from 50% to 75%, the prediction accuracy of spatial domain prediction degrades largely. The average RSRP difference increases 183%.

Besides, we also compare the prediction accuracy of RRM sub use case 1 and RRM sub use case 3, and find that the RRM sub use case 1 can provide better prediction accuracy. This is because the AI model output of the RRM sub use case 3 is the L3 filtered cell results, which is impacted by both historical L3 results and current channel quality. However, the AI model input of the RRM sub use 3 is the L1 filtered beam results, which can only reflect the current channel quality. Besides, in the simulation for RRM sub use case 1, for simplicity, we only perform spatial domain prediction on the last sample of the UE trajectory. In other words, the old filtered result which is used to generate L3 result is from actual measurement.

Observation 14: The prediction accuracy of spatial domain measurement prediction with RRM sub use case 1 is better than that with RRM sub use case 3.

- Conclusion

Based on the above discussions, we have the following observations and proposals:

For FR1 to FR1 intra-frequency temporal domain case B:

Observation 1: With the increase of MRRT from 33.3% to 60%, the average RSRP difference increases at most 1.27dB, and the prediction accuracy for temporal domain measurement prediction case B decreases.

Observation 2: Regarding temporal domain measurement prediction B, even with the same MRRT, skipping pattern (e.g. [400ms,400ms] or [600ms,600ms]) can also impact the prediction accuracy, their average prediction error difference achieves up to 0.6dB.

Observation 3: Regarding temporal domain measurement prediction B, the prediction accuracy of temporal domain measurement prediction with spatial consistency option A is better than that with option B.

Observation 4: As UE speed decreases from 90km/h to 30km/h, the prediction accuracy of temporal domain measurement prediction Case B improves. The average and maximum RSRP difference decrease at most 32.2% and 33.3%, respectively.

Observation 5: For temporal domain measurement prediction case B, even under the same condition, different AI model can bring different prediction accuracy.

Observation 6: The prediction accuracy of temporal domain measurement prediction case B on the best cell is similar as that on the sub-best cell. And the difference between the average RSRP difference for the best cell and for the sub-best cell keeps within 0.14dB.

For FR2 to FR2 intra-frequency temporal domain case A:

Observation 7: With the same observation window length, the prediction accuracy is decreased with the increase of predicted time, and the average RSRP difference increases at most 127.1%.

Observation 8: The prediction accuracy of temporal domain measurement prediction Case A with RRM sub use case 1 and RRM sub use case 2 are similar, the maximum difference is less than 0.5dB.

Observation 9: The observation 4~6 for temporal domain measurement prediction case B are applicable for temporal domain prediction case A.

For FR1 to FR1 frequency domain prediction:

Observation 10: The prediction accuracy of full correlation inter-freq shadowing fading model is better than that of no correlation model.

Observation 11: The maximum average RSRP difference of frequency domain prediction with no correlation inter-freq shadowing fading model is only 1.4dB, which is acceptable.

Observation 12: The prediction accuracy of frequency domain measurement prediction is not impacted by UE speed.

For FR2 to FR2 intra-frequency spatial domain measurement prediction

Observation 13: With the MRRS increase from 50% to 75%, the prediction accuracy of spatial domain prediction degrades largely. The average RSRP difference increases 183%.

Observation 14: The prediction accuracy of spatial domain measurement prediction with RRM sub use case 1 is better than that with RRM sub use case 3.

- References

- Report of 3GPP TSG RAN WG2 meeting #125bis

- Report of 3GPP TSG RAN WG2 meeting #126