3GPP TSG-RAN2 Meeting #127	R2-2406423

Maastricht, Netherlands, Aug 19th-23rd, 2024

Source: 			ZTE Corporation

Title: 	Discussion on RRM measurement prediction

Agenda item:	8.3.2.2

Document for: 	Discussion and Decision

- Introduction

At the RAN 102 meeting, a new SID [1] on ‘Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR’ was approved. Some of the objects are shown below:

... Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - ...

It’s noted that both cell level measurement prediction and beam level measurement prediction are within the scope of RRM measurement prediction. In the past two meetings, we have studied the cell level measurement prediction. However, beam level measurement prediction has not been touched yet. Besides, during the post email discussion in the last meeting, we find that companies have different understandings on the L3 filtering period, which directly impacts the number of predicted/actual measured cell results within the prediction/ observation window. Therefore, in this contribution, we’d like to share our views on the beam level measurement prediction and L3 filtering period.

- Discussion

## 2.1 beam measurement prediction

In the current specification, when measurement report is triggered, the UE always needs to report the cell level measurement results, which can be used to trigger the handover procedure. And if configured, the UE also needs to report beam level measurement results to the network. The network selects the beam(s) (SSB or CSI-RS) with good quality based on the reported beam results, and allocates the CFRA resources to the UE in order to reduce data interruption.

For AI mobility, beam level measurement prediction is to predict beam level measurement results without actual measurement. Besides generating cell results, the beam level prediction can also be used to facilitate the target cell to allocate the RACH resource to the UE, as legacy.

Observation 1: Besides generating cell results, beam level measurement prediction can also be used to facilitate the target network to allocate RACH resource.

The existing measurement model defined in TS 38.300 is shown in Fig. 1. The part related to beam result derivation is indicated via red rectangle. As can be seen, the UE performs L1 filtering and L3 filtering on the raw beam results, then selects the Top X beam results to report to the network, where X is configured by the network.

Fig. 1 measurement model

Observation 2: In the existing measurement model, the raw beam results will experience L1 filtering and L3 filtering, then Top X L3 filtered results will be reported to the network.

Similar to L3 cell level measurement prediction, the possible ways to L3 beam level measurement prediction include:

Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results.

Case 2: To directly predict L3 filtered beam level results based on the L3 filtered beam level measurement results.

Case 3: To directly predict L3 filtered beam level results based on the L1 filtered beam level measurement results.

Case 1 is very similar to the RRM sub use case 1 of cell level measurement prediction. The only difference is how to process the predicted L1 filtered beam results later, while AI model is not impacted. Therefore, with case 1, the simulation platform for RRM sub use case 1 of cell results prediction can be reused directly, and our simulation work can be reduced largely. For case 2 and case 3, their AI model output is L3 filtered beam results directly, which does not require subsequent processing.

Proposal 1: To support L3 beam level measurement prediction, and consider the following cases:

Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results.

Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results.

Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results.

For case 1~3, the collected data are different. For example, regarding data collection for model training, the UE needs to report L1 filtered beam level measurement results for case 1, but needs to report L3 filtered beam level measurement results for case 2. Considering the training data are different, the trained AI model in case 1~3 are different. So, their prediction performance may be different. Companies are encouraged to provide the simulation results for comparing prediction performance of case 1~3.

Observation 3: The prediction accuracy of case 1~3 may be different, since the trained AI model are different.

Proposal 2: Companies are encouraged to provide simulation results for comparing the prediction accuracy of case 1~3.

Besides, since we already have lots of simulation work to do in the cell level prediction and the motivation of beam level prediction is to optimize handover procedure (e.g. to reduce handover delay). So, not all agreed simulation scenario in cell level prediction should be considered in beam level prediction, we can select some of them to study. Specifically, considering the number of Tx beams is limited in the FR1 scenario, the study of beam level prediction can be studied in FR2 scenario first. And taking the priority of evaluation scenario in cell level prediction in FR2 for reference, the evaluation of beam level prediction should focus on FR2-to-FR2 intra-frequency temporal domain prediction case A and FR2 to FR2 intra-frequency spatial domain prediction.

Proposal 3: Considering large simulation overhead, not all agreed simulation scenario in cell level prediction should be considered in beam level prediction, we can select some of them.

Proposal 4: The simulation for beam level prediction should focus on FR2-to-FR2 intra-frequency temporal domain prediction case A and FR2 to FR2 intra-frequency spatial domain prediction.

Regarding KPI, we have agreed to take average RSRP difference as prediction accuracy metric for RRM measurement prediction. So, one of the KPI for beam level measurement prediction shall be average L3 beam level RSRP difference. Besides, considering the motivation of beam level measurement prediction is to help the network to select the beam with good quality to allocate RACH resources, Top-K/1 beam prediction accuracy can also be considered as KPI, which is defined as the percentage of ‘the Top-1 genie-aided beam is one of the Top-K predicted beams’ as in AI-PHY study.

Proposal 5: For L3 beam level measurement prediction, consider average L3 beam level RSRP difference and Top-K/1 beam prediction accuracy as KPI.

## 2.2 L3 filtering period

During the post email discussion [POST126][031][AIMob], we find that companies have different understandings on L3 filtering period. With different understanding, the number of predicted/actual measured cell results within the prediction window/observation window are different, which impacts the prediction performance of AI model. In this sub-section, we’d like to discuss the understanding on the L3 filtering period.

Understanding 1 is that the L3 filtering is performed per measurement period

We take temporal domain prediction model in FR1 scenario for example, see the Fig.2:

Fig. 2: Understanding 1 in FR1 scenario

With understanding 1, the L3 filtered results are generated every measurement period (i.e. 200ms). And in the case that to predict the measurement results of future 200ms based on the measurement results of historical 400ms, there are 2 actual measured cell results within the observation window, and 1 predicted cell results within the prediction window.

Understanding 2 is that: the L3 filtering is performed per sampling period (or sampling period * UE Rx beam number for FR2 scenario)

Considering the impact of beam sweeping, we show the examples for understanding 2 in FR1 scenario and FR2 scenario, respectively.

Fig. 3: The understanding 2 in FR1 scenario

Fig. 4: The understanding 2 in FR2 scenario

With the understanding 2, the L3 filtered results are generated every 40ms in FR1 scenario and 80ms (20ms*4) in FR2 scenario. And in the same temporal domain prediction case, there are 10 actual measured cell results within the observation window and 5 predicted cell results within the prediction window.

Observation 4: There are two understanding of the L3 filtering period:

Understanding 1: The L3 filtering is performed per measurement period;

Understanding 2: The L3 filtering is performed per sampling period (or sampling period * UE Rx beam number in FR2).

And the understanding of L3 filtering period impacts the calculation method for L3 filtered cell results and the datasets for model training, which will further impact prediction accuracy of AI model. Therefore, in order to better compare the simulation results among companies, we suggest RAN2 to discuss and decide which understanding is correct in the AI mobility simulation.

Observation 5: The understanding of L3 filtering period will impact the prediction accuracy.

Proposal 6: RAN2 to discuss and decide which understanding for layer 3 filtering period is correct in the AI-mobility simulation.

- Conclusion and proposals

In this contribution, we have the following observations and proposals:

Observation 1: Besides generating cell results, beam level measurement prediction can also be used to facilitate the target network to allocate RACH resource.

Observation 2: In the existing measurement model, the raw beam results will experience L1 filtering and L3 filtering, then Top X L3 filtered results will be reported to the network.

Observation 3: The prediction accuracy of case 1~3 may be different, since the trained AI model are different.

Observation 4: There are two understanding of the L3 filtering period:

Understanding 1: The L3 filtering is performed per measurement period;

Understanding 2: The L3 filtering is performed per sampling period (or sampling period * UE Rx beam number in FR2).

Observation 5: The understanding of L3 filtering period will impact the prediction accuracy.

Proposal 1: To support L3 beam level measurement prediction, and consider the following cases:

Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results.

Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results.

Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results.

Proposal 2: Companies are encouraged to provide simulation results for comparing the prediction accuracy of case 1~3.

Proposal 3: Considering large simulation overhead, not all agreed simulation scenario in cell level prediction should be considered in beam level prediction, we can select some of them.

Proposal 4: The simulation for beam level prediction should focus on FR2-to-FR2 intra-frequency temporal domain prediction case A and FR2 to FR2 intra-frequency spatial domain prediction.

Proposal 5: For L3 beam level measurement prediction, consider average L3 beam level RSRP difference and Top-K/1 beam prediction accuracy as KPI.

Proposal 6: RAN2 to discuss and decide which understanding for layer 3 filtering period is correct in the AI-mobility simulation.

- Reference

- Report of 3GPP TSG RAN WG2 meeting #125bis