3GPP TSG-RAN2 Meeting #127	R2-2406424

Maastricht, Netherlands, Aug 19th-23rd, 2024

Source: 			ZTE Corporation

Title: 	Discussion on simulation assumption for RLF prediction

Agenda item:	8.3.4.1

Document for: 	Discussion and Decision

- Introduction

At the RAN 102 meeting, a new SID [1] on ‘Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR’ was approved. Some of the objects are as follows:

... Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] ...

In this contribution, we mainly discuss RLF specific methodology and simulation assumptions.

- Discussion

The radio link failure (RLF) results in the RRM performance deterioration. If the network can know whether the RLF will occur in the future, some measures can be taken in advance to avoid it. Thus, the RLF prediction is worthy for companies to study. In this section, we’d like to share our view on RLF prediction in the aspects of simulation assumption, RLF prediction model and metric.

## 2.1 Simulation assumption:

In order to better compare the simulation results between the companies, the calibration of simulation assumptions is necessary. For indirect RLF prediction, the aligned simulation parameters should be used for baseline case, model training, validation and inference. And for direct RLF prediction, the aligned simulation parameters should be used for at least baseline case for evaluating prediction accuracy.

Observation 1: For indirect RLF prediction, the aligned simulation parameters should be used for baseline case, model training, validation and inference.

Observation 2: For direct RLF prediction, the aligned simulation parameters should be used for at least baseline case to evaluate KPI.

Regarding the RLF simulation assumptions, we can reuse the parameters values provided in Table 5.2.1.3.1 of TR 36.839. And Qout and Qin represents the CQI threshold for reporting out-of-sync and in-sync indication, considering normally CQI is measured by SINR, we can consider Qout and Qin as SINR threshold in the simulation.

Table 5.2.1.3.1: The parameters for determine the RLFs and the PDCCH failures.

| Items | Description |
| --- | --- |
| Qout | -8 dB |
| Qin | -6 dB |
| T310 | 1s (the default value in 36.331) |
| N310 | 1 |
| T311 | Not used for calibration (since RLF recovery is not simulated in the calibration) |
| N311 | 1 |

Besides, according to the TR36.839:

#5.2.1.2 RLF modelling and definition of RLF states For the purpose of RLF monitoring, the basic L1 processing configurations in non-DRX mode should be: L1 sample rate is once every 10ms (i.e. radio frame), with the L1 samples filtered linearly over a sliding window of 200ms (i.e. 20 samples) for Qout and 100 ms (i.e. 10 samples) for Qin, respectively.

The simulation assumptions above can be reused, i.e. layer 1 sample rate is once every 10ms; evaluation period for Qout and Qin are 200ms and 100ms respectively.

These values also satisfy the RLM minimum requirements specified in TS 38.133.

Proposal 1: Reuse the simulation parameters in TR 36.839 for RLF prediction:

- SINR threshold: Qout = -8dB, Qin = -6dB;

- T310=1s, N310=1, N311=1;

- Layer 1 sample rate for RLM-RS is once every 10ms;

- Qout evaluation period: 200ms, Qin evaluation period: 100ms

In the last meeting, we have agreed that simulation assumption for RRM measurement prediction is the baseline for RLF prediction. However, in order to reduce simulation time and collect more data for model training/verification/inference, some adjustments on the simulation assumption are needed. The motivation of the adjustments is to make the channel condition more challenging, and collect more data of RLF. Besides, some simulation assumptions are not suitable for RLF prediction since SINR is considered in the RLF prediction. The following updates can be considered:

- Consider blockage in the channel model

- Although we have agreed to not consider blockage in the channel model for RRM measurement prediction, but for RLF prediction simulation, blockage is needed in order to introduce sudden radio link condition change. For the blockage model, companies can adopt one of the blockage model (blockage A or blockage B) specified in the clause 7.6.4 of TR 38.901, and report the adopted model along with the simulation results.

- Simulate traffic model

- Since we need to simulate the SINR value, it’s necessary to consider traffic model. Similar to Rel-18 AI-Phy study, the following traffic model can be considered in the simulation:

- Option 1: Full buffer

- Option 2: FTP model with detail assumptions (e.g., FTP model 1, FTP model 3)

Proposal 2: Regarding simulation assumptions for RLF prediction, consider the following updates:

- Consider blockage in the channel model: It’s up to companies to adopt blockage model A or B specified in clause 7.6.4 of TR 38.901;

- Simulate traffic model: The traffic model in AI-Phy study can be reused.

- Besides, in the procedure of generating datasets, we need to detect RLF in the simulation via actual measurement. The generated datasets can be used to train model, validate and infer for indirect RLF prediction, and can be used for at least baseline scenario to evaluate KPI for direct RLF prediction. The way to detect RLF in the simulation is: the UE physical layer measures the SINR value(s) of radio link monitoring reference signal (RLM RS) every indication period (e.g. 10ms). When the measured SINR value is less than Qout for all RLM RS, the physical layer indicates out-of-sync indication to RRC layer; when the measured SINR value(s) is larger than Qin for at least one RLM RS, the physical layer indicates in-sync indication to RRC layer.

In current specification, the RLM RS can be (1) radioLinkMonitoringRS provided by the network; (2) RS provided for the active TCI state for PDCCH reception. In both cases, when the UE moves, the network can modify the RLM RS based on the reported beam results. In the simulation, we can select the best beam of serving cell as RLM RS. When the UE detects the change of best beam, the RLM RS will also change. In the reality, when the UE detects the best beam is changed, the UE needs to report the beam results to the network, then the network will modify the RLM RS based on the reported beam results. There is a offset between the time that UE detects best beam change and the time of RLM RS change. In our understanding, we do not need to consider it in the simulation. Because the time offset is normally only a few milliseconds, but the length of monitoring window for Qout is 200ms, so it does not have obvious impact on the RLF derivation. In this case, the cause of RLF is that there are blockage or interference or coverage hole in the simulation environment, which cause the sudden radio link signal deterioration.

Observation 3: In the reality, there is a offset between the time that UE measures signals and the time of RLM RS change, which is not needed in the simulation.

Proposal 3: In the simulation, RLM RS is based on the best beam of the serving cell,

## 2.2 KPI

Indirect RLF prediction is to predict the SINR values of the RLM RS in the future based on historical SINR results first, then further determine whether the RLF will occur based on the predicted SINR value. We can consider precision and recall as KPI to evaluate RLF prediction accuracy. Precision is defined as the percentage of genie positive event in the all predicted positive event; and recall is defined as the percentage of predicted positive event in the all genie positive event. The detailed calculation method is as follow:

Precision = n3/(n1+n3)

Recall = n3/(n2+n3)

Where:

|  | Predicted negative | Predicted positive |
| --- | --- | --- |
| Actual negative | n0 | n1 |
| Actual positive | n2 | n3 |

Proposal 4: For indirect RLF prediction, precision and recall are considered as KPI to assess RLF prediction accuracy.

Besides, the prediction result of indirect RLF prediction is that whether the RLF will occur at one time instance. However,  it’s very difficult that the predicted RLF triggered time and the actual RLF triggered time are the same completely. And, even if they are not the same, when the predicted triggered time is close to actual triggered time, the prediction is also useful. So, a tolerate time can be considered, which is defined as the the maximum allowed time difference between predicted triggered time and actual triggered time. An example can be seen below:

As shown in the figure above, RLF is predicted to occur at the time instance t1, and the tolerate time is t0. If the actual RLF occurs within the time window [], the prediction is considered to be successful.

Proposal 5: For indirect RLF prediction, a tolerate time is considered when determining whether the predicted RLF is matched with actual RLF.

Regarding direct RLF prediction, we have agreed that RLF prediction result is the RLF probability. However, it’s difficult to evaluate the prediction accuracy, since the RLF prediction result is a probability value instead of a deterministic value. For example, if the RLF occurrence probability is 70%, how to compare with baseline scenario, and how to evaluate whether the probability value is accurate.

Proposal 6: For direct RLF prediction, RAN2 to discuss how to evaluate prediction accuracy.

- Conclusion and proposals

In this contribution, we have the following observations and proposals：

Observation 1: For indirect RLF prediction, the aligned simulation parameters should be used for baseline case, model training, validation and inference.

Observation 2: For direct RLF prediction, the aligned simulation parameters should be used for at least baseline case to evaluate KPI.

Observation 3: In the reality, there is a offset between the time that UE measures signals and the time of RLM RS change, which is not needed in the simulation.

Proposal 1: Reuse the simulation parameters in TR 36.839 for RLF prediction:

- SINR threshold: Qout = -8dB, Qin = -6dB;

- T310=1s, N310=1, N311=1;

- Layer 1 sample rate for RLM-RS is once every 10ms;

- Qout evaluation period: 200ms, Qin evaluation period: 100ms.

Proposal 2: Regarding simulation assumptions for RLF prediction, consider the following updates:

- Consider blockage in the channel model: It’s up to companies to adopt blockage model A or B specified in clause 7.6.4 of TR 38.901;

- Simulate traffic model: The traffic model in AI-Phy study can be reused.

Proposal 3: In the simulation, RLM RS is based on the best beam of the serving cell,

Proposal 4: For indirect RLF prediction, precision and recall are considered as KPI to assess RLF prediction accuracy.

Proposal 5: For indirect RLF prediction, a tolerate time is considered when determining whether the predicted RLF is matched with actual RLF.

Proposal 6: For direct RLF prediction, RAN2 to discuss how to evaluate prediction accuracy.

- Reference

- RP-234055:Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR FS_NR_AIML_Mob