3GPP TSG-RAN WG2 Meeting #127	R2-2406426

Maastricht, NL, Aug. 19th– Aug. 23th, 2024

Agenda Item:	8.2.3

Source: 	Xiaomi

Title:  	Discussion on A-IOT paging procedure

Document for:	Discussion and Decision

1	Introduction

In last meeting, RAN2 discussed the functionality and content aspects of A-IOT paging procedure, the following agreements were achieved.

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

In this contribution, we would like to discuss some other details related to A-IOT paging procedure and have corresponding observations and proposals.

2	Discussion

2.1 Allocation of D2R resources in A-IOT paging message

According to the above agreements, A-IOT paging message support different cases, e.g., single device, multiple devices and all devices. To reach a single device, which initiates a contention-free random access, an identifier e.g., permanent ID and dedicated resource/slot for the D2R response should be included. To reach multiple devices, which initiates a contention-based random access, a group identifier/no identifier and shared resource/slot e.g., Q like parameter should be included.

Since the transmission between device and BS reader is Uu resource, which is not visible to the CN/server, therefore it should be the BS reader to allocate the dedicated or shared resource/slot for the D2R response message from the triggered device and include the information of resource/slot in the A-IOT paging message from the BS reader to the device.

Meanwhile, RAN1 is still discussing the TDM/FDM mechanism to improve the access capacity, which may have impact on how to indicate the dedicated and/or shared resource for D2R response, therefore, the detailed design of the information signalling should be further studied and take RAN1 discussion into account.

Proposal 1: BS reader is responsible for the allocation of dedicated and/or shared resources in A-IOT paging message.

2.2 Temporary ID between BS reader and device

In RAN2#125bis meeting, RAN2 discussed the general procedures and achieved the following agreement

- Baseline procedure: Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS Step C: The device may perform the data communication with the reader as needed,: Details FFS

After reception of the A-IOT paging message, random access like procedure and/or data communication may follow. During these procedures, for each respond message from the device, an identifier should be included for validation, however always including a permanent ID e.g., EPC will increase power consumption of the device and also bring traffic burden on the interface. Therefore, it makes sense to apply a temporary ID for the message transmission between the device and the BS reader.

As shown in Figure 4, during the A-IOT paging for a single device, besides a permanent device ID and the resource/slot for reply, a temporary ID can be allocated to the device by the BS reader, the BS reader maintains a mapping between the permanent ID and the temporary ID for the device, which is used for the following message transmission between the device and the BS reader e.g., data communication. Regarding the D2R response from the triggered device to the BS reader, the temporary ID is included to identify the device and when forwarding the acknowledge message to the CN/server, the BS reader includes the permanent ID associated with the temporary ID based on the maintained mapping.

Figure 1: A-IOT paging procedure for single device with temporary ID

Proposal 2: RAN2 assumes a temporary ID is used for the message transmission between the device and the BS reader.

2.3 Visibility of A-IOT paging message

In RAN2#125bis meeting, RAN2 discussed the visibility of information required by the reader and achieved the following agreement.

- FFS about the level of visibility required by the reader and what information is necessary for AS layer operations.

As mentioned in the previous section, the A-IOT paging message from the BS reader to the device are different depending on whether the A-IOT paging procedure is initiated for a single device, a group of devices or all devices. To be more detailed, if the A-IOT paging procedure is for a single device, dedicated resource/slot should be indicated. While if the access procedure is for a group of devices or all devices, “Q” like parameter should be indicated for contention resolution. In addition, to assist the BS reader to set the value of “Q” properly, CN/server should also inform the BS reader of the number of the triggered devices if available, however this information is not mandatory, i.e., in some cases, even CN may not know the exact number of the triggered device, in this case the BS reader can set Q based on implementation and further adjust Q if needed. In addition, if the BS reader is responsible of allocating the temporary ID and maintains the mapping between the temporary ID and the permanent ID for the device, the permanent device identifier should be visible to the BS reader as well.

Proposal 3: During the A-IOT paging procedure, RAN2 assumes the following message is visible to BS reader.

- Whether the A-IOT paging message is for a single device or a group of devices/all devices

- The permanent device identifier, e.g., EPC

2.4 Paging type indication in A-IOT paging message

Actually, there were many proposals from last meeting to introduce a paging type to indicate the use case of the A-IOT paging message, e.g., inventory or command or inventory+command. However, in our view, no matter what use case the A-IoT paging message is to serve, the paging procedures share a common purpose to reach one or more target device for the upcoming A-IOT communication. From the reader’s perspective, there is no need to identify the use case triggered the paging procedure, the only useful information is to know whether the paging command is to trigger CFRA or CBRA to allocate the dedicated/shared resource for the follow-up D2R transmission. From the device’s perspective, it is also not necessary to be aware of the served use case of the paging procedure, the device only needs to know whether itself is paged or not based on the identifier included in the paging command and if it is paged, whether CFRA or CBRA is triggered, e.g., EPC like ID transmission for CFRA and RN16 like random ID transmission for CBRA.

Proposal 4: No need to introduce paging type indication to indicate the use case triggered the A-IOT paging procedure, e.g. inventory or command or inventory+command.

However as mentioned, above, it is necessary for both reader and device to identify whether the A-IOT paging message is for a single device or multiple devices, or whether the A-IOT paging message triggers CFRA or CBRA, to realize this intention, a paging type indication can be introduced.

Proposal 5: RAN2 to discuss to introduce paging type indication to indicate whether the A-IOT paging message triggers CFRA or CBRA.

2.5 Paging repetition

In last meeting, RAN2 agreed that the device can receive as long as there is enough energy and according to the SID, it takes the device up to several tens of seconds to charge. Since different A-IOT device types are supported, the charging time for the A IOT devices to start/resume A-IOT communication can vary. Then, it is quite possible some devices may miss the paging message due to large power consumption and/or long charging duration.

- The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study [RAN2, RAN1] - Duration of one device’s unavailability due to charging by energy harvesting can be assumed up to several tens of seconds - Note: this value can be revisited in future RAN plenary meetings, if necessary - TR 38.848 clause 5.6 statement on latency remains the case with respect to a single device, i.e.: “NOTE: The time for charging the Ambient IoT device storage (if present) is not included in the latency defined above. Time for energy harvesting, charging, etc. is regarded as an implementation issue only.”

In this case, to solve the device availability issue and paging reception failure issue, the reader implementation should be allowed to retransmit the paging message in case the number of received response from the device does not match the rough number of the triggered devices. However, whether/when to trigger paging repetition should be purely up to the reader implementation. For example, if the paging message received from CN triggers CFRA, the reader can resend the paging message if no response is received from the device after a certain duration. While if the paging message received from CN triggers CBRA, the reader can determine whether/when to re-trigger the paging procedure, based on the number of received response (e.g., RN16), the rough number of the triggered devices as indicated by CN and/or the value of Q like parameter etc.. In addition, it is also up to the reader to determine the paging retransmission times.

Proposal 6: RAN2 to discuss the support of re-trigger paging procedure. If re-trigger paging is supported, whether/when to send the paging repetition and how many times to resend the paging message is up to reader implementation.

If paging repetition is supported, another issue is for those devices that have already received the initial paging message and involved in the following CFRA/CBRA/command procedure, how to treat the unnecessary paging repetition. One possible solution is to introduce some indication e.g., similar as NDI in NR to indicate whether the paging message initiates a new inventory/command request or just a retransmission, where the indication should be determined and added by the reader. Another possible solution is from the device’s perspective, for example, if the device checks the identifier included in the new received paging message is as same as that if has already received, the device directly ignores the paging message. Or if the device is already within a paging procedure, e.g., during the random access or command procedure, the device ignores the paging command.

Proposal 7: RAN2 to discuss how devices that have already received a paging message to ignore the unnecessary paging repetition e.g., new indication in paging message or device to distinguish based on device identifier included in the paging message.

2.6 Periodical A-IOT paging procedure

In last meeting, RAN2 discussed the functionality of paging in A-IOT and agreed to not support legacy PO/PF and DRX cycle for the device.

Agreements

- Legacy paging message for device will not be supported.

- Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

However, in order to reduce the interaction between CN and BS reader and to support some periodic requirement, periodical inventory can be considered to be supported. A use case to support periodical inventory is to check the number/status of the devices periodically for frequent in-and-out-of-storage management. To be more detailed, CN configures the parameters related to periodical initial access/inventory e.g., operation period, identifier field, etc. BS reader triggers initial access/inventory and encapsulates the initial access command periodically according to the configuration from CN. Even the device does not support paging cycle and PO/PF, there is no problem to support the periodical initial access procedure, as long as the device is energy-harvest, the device is power-on and continues to monitor the initial access command.

Proposal 8: RAN2 to discuss and support the BS reader to trigger A-IOT paging procedure periodically according to the configuration from CN.

3	Conclusion

In this contribution, we discussed the procedures and signalling for the initial access in A-IOT and have corresponding proposals.

Proposal 1: BS reader is responsible for the allocation of dedicated and/or shared resources in A-IOT paging message.

Proposal 2: RAN2 assumes a temporary ID is used for the message transmission between the device and the BS reader.

Proposal 3: During the A-IOT paging procedure, RAN2 assumes the following message is visible to BS reader.

- Whether the A-IOT paging message is for a single device or a group of devices/all devices

- The permanent device identifier, e.g., EPC

Proposal 4: No need to introduce paging type indication to indicate the use case triggered the A-IOT paging procedure, e.g. inventory or command or inventory+command.

Proposal 5: RAN2 to discuss to introduce paging type indication to indicate whether the A-IOT paging message triggers CFRA or CBRA.

Proposal 6: RAN2 to discuss the support of re-trigger paging procedure. If re-trigger paging is supported, whether/when to send the paging repetition and how many times to resend the paging message is up to reader implementation.

Proposal 7: RAN2 to discuss how devices that have already received a paging message to ignore the unnecessary paging repetition e.g., new indication in paging message or device to distinguish based on device identifier included in the paging message.

Proposal 8: RAN2 to discuss and support the BS reader to trigger A-IOT paging procedure periodically according to the configuration from CN.

4	Reference

[1]	RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, 3GPP, March 2024.
[2]	RAN2#126 meeting minutes.

[3]	RAN2#125bis meeting minutes.