3GPP TSG-RAN WG2 Meeting #127		R2-2406427

Maastricht, Netherlands, 19th – 23rd Aug. 2024

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_IDLE and INACTIVE

Agenda Item:	8.4.2

Document for:	Discussion and Decision

- Introduction

In RAN2#125bis meeting, the following agreements for LP-WUS/WUR in RRC_IDLE/INACTIVE were made [1].

The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed. Working assumption: the LP-WUS configuration in SIB at least includes the following information: - LP-SS configuration - LP-WUS configuration - FFS on Entry/exit condition for LP-WUS monitoring The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

In RAN2#126 meeting, we further discussed the LP-WUS/WUR in RRC_IDLE/INACTIVE and reached the following agreements [2]:

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - LP-SS configuration - LP-WUS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it always configured Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. From RAN2 perspective, no new procedure is introduced for SI reception/updates.

In this contribution, we will discuss the issues for RRC_IDLE/RRC_INACTIVE procedures with LP-WUR, including LP-WUS configuration, the entry and exit condition, wake up procedure andsubgrouping.

- Discussion

  - LP-WUS configuration

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - LP-SS configuration - LP-WUS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

In RAN2#125bis and RAN2#126 meeting, it was agreed that the LP-WUS related configuration is provided via system information.

Regarding the specific system information for LP-WUS configuration, both the existing SIB(s) and new SIB are feasible. However, considering paging-related configuration and PEI information are included in SIB1, it is straightforward to follow the same design for LP-WUS configuration. Besides, introducing a new SIB will result in more specification work, e.g., the scheduling information of the new SIB and the on-demand request for the new SIB. Thus, SIB1 is preferred.

Proposal 1: LP-WUS configuration is provided in SIB1.

In RAN2#126 meeting, it was agreed that the entry/exit condition for LP-WUS monitoring is included in LP-WUS configuration. However, it is still FFS whether the entry/exit condition for LP-WUS monitoring is always configured in SIB. Besides, RAN1 also concluded that it is up to RAN2 discussion as below in the Working Assumption for the entry/exit condition for LP-WUS monitoring [3].

It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

In our view, the first issue that should be discussed is whether the entry/exit condition for LP-WUS monitoring is always configured, no matter whether it is in the SIB or RRC release message. Regarding this issue, some companies thought if the MR and LR have the same coverage, the entry/exit condition for LP-WUS monitoring seems not always need to be configured in RAN2#125bis meeting. According to the WID [4], the target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for Msg3, however, it depends on the real deployment on whether the coverage of LP-WUS and LP-SS is full coverage or partial coverage. For example, if the size of a cell is small, the coverage of LP-WUS and LP-SS can be full coverage.  On the contrary, if the size of a cell is large, the coverage of LP-WUS and LP-SS can be partial coverage.

However, in our understanding, the entry/exit condition for LP-WUS monitoring should be mandatorily configured if the network is intended to support the LP-WUS feature no matter what the coverage of LP-SS/LP-WUS is. For the partial coverage, the entry/exit condition for LP-WUS monitoring is anyway required to guarantee the paging reception when UE is out of coverage of LP-WUS signalling. For the full coverage, the entry/exit condition for LP-WUS monitoring is also required to guarantee the cell reselection, since LR can not perform neighbor cell measurement based on the WID [3], a mechanism is needed to confirm when the MR needs to wake up to perform the neighbor cell measurement.

Proposal 2: The entry/exit condition for LP-WUS monitoring is mandatorily configured if the network is intended to support the LP-WUS feature.

  - Entry/exit condition of using LP-WUS

Regarding the specific entry/exit condition for LP-WUS monitoring and UE behaviours, RAN1 has achieved the following Working Assumption [3].

Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. Note: This may be revisited based on the RAN2/RAN4 discussion.

And RAN2 has the following agreement in RAN2#126 meeting [2]:

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

Regarding the UE behaviours, it can be seen that even though the entry/exit condition is fulfilled, it is up to UE implementation whether to start/stop LP-WUS monitoring, i.e. UE “may” start LP-WUS monitoring, or “may” stop LP-WUS monitoring. For example, when the entry condition for LP-WUS monitoring is fulfilled, if there is expected UL data arrival later, MR could select not to go to ultra-deep sleep. Similarly, it is up to UE implementation to determine whether to stop the LP-WUS monitoring when the exit condition is fulfilled when the exit condition for LP-WUS monitoring is fulfilled, MR needs to perform legacy PO monitoring.

Observation 1:  It is up to UE implementation to determine whether to start the LP-WUS monitoring when the entry condition is fulfilled.

Observation 2:  It is up to UE implementation to determine whether to stop the LP-WUS monitoring when the exit condition is fulfilled.

According to the RAN2 agreement and RAN1 working assumption above, for the entry condition, whether it could be based on the measurement via LR is FFS, since the UE could switch on the LR to perform measurement before monitoring LP-WUS. In our view, it depends on the measurement accuracy via LR, measurement, and whether the measurement on MR and LR is comparable, which should be discussed/decided in RAN1/RAN4.

For the exit condition, whether it could be based on the measurement via MR could also be considered, since the entry and exit conditions based on measurement via different radio receivers may cause some ping-pong issues, e.g. the UE will exit the LP-WUS monitoring right after the entry condition is satisfied and UE enters the LP-WUS monitoring. Similarly, it also depends on the discussion on the measurement accuracy/metrics/whether MR and LR measurement is comparable in RAN1/RAN4.

Proposal 3: Regarding whether any measurement via LR is needed for entry condition and whether any measurement via MR is needed for exit condition, RAN2 to wait for RAN1/RAN4 progress.

Regarding the specific metric(s) for serving cell measurement via LR, RAN1 has achieved the following agreement [3].

Agreement From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS: - LP-RSRP - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols. - FFS: How to determine the received power of LP-SS in OOK ON symbols - LP-RSRQ - LP-RSRQ = LP-RSRP/LP-RSSI - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options: - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols. - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols. - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols. - FFS: LP-SINR Note: The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

Based on the RAN1 agreement, it can be seen that RSRP and RSRQ are supported for OOK-based LP-WUR. In legacy S-criteria/R-criteria for cell selection/reselection and RRM relaxation criteria, the measurement metric is based on RSRP and optionally on RSRQ. Besides, in some cases, RSRP is enough to reflect the cell quality, while it is up to network to configure whether RSRQ is needed. Thus, we think the same mechanism could be used to determine the measurement metrics for the entry/exit condition, i.e. the measurement metric on serving cell quality measured by MR and LR for entry/exit condition includes RSRP and optional RSRQ. Considering this issue is also related to RAN1/RAN4 discussion, it is better to send an LS to RAN1/RAN4 to confirm this.

Proposal 4: The metric for serving cell quality measured by MR for entry condition includes RSRP and optional RSRQ.

Proposal 5: RAN2 assumes the metric for serving cell quality measured by LR for exit condition includes RSRP and optional RSRQ. An LS is sent to RAN1/RAN4 for confirmation.

  - LP-WUS wake up procedure

The purpose of LP-WUS in RRC_IDLE/RRC_INACTIVE mode is to trigger the paging monitoring. Regarding the paging occasion associated with the paging triggered by LP-WUS, RAN1 achieved the following agreement in RAN1#116 meeting [5].

Agreement It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. - FFS: support of UE monitoring dynamic PO

The benefit for LP-WUS is to achieve significant UE power saving gain, while the paging latency is also an important metric for LP-WUS performance. As shown in Figure 1 below, if the dynamic PO is designed to be located in front of the legacy PO after UE receives the LP-WUS and wakes up from ultra-deep sleep state, the paging latency could be reduced.

Figure 1: Latency for LP-WUS based on dynamic PO

In the ideal situation, the dynamic PO is located just in/after the occasion that UE completes the ramp-up and synchronization procedure. Given the current typical DRX cycle is 1.28s, the dynamic PO solution could reduce paging latency by an average of around 600 ms. Although it's hard to get the perfect effect as the ideal case, we suppose with suitable dynamic PO designs, the reduced latency could be close to 600 ms when DRX cycle of 1.28s is applied. Thus, we think the dynamic PO is worth to be studied from the paging latency aspect.

Proposal 6: RAN2 to discuss whether dynamic PO is supported.

Besides, as legacy PEI design, the MR in ultra-deep sleep/monitoring LP-WUS should also wake up in case there is UL data arrival.

Proposal 7: MR should wake up for UE-initiated UL transmission, e.g. RACH/MO-SDT/RNA update/TA update, due to UL data and signaling arrival when using LP-WUS.

  - LP-WUS Subgrouping

In RAN2#125bis meeting, the subgrouping methods were agreed and the maximum number of subgroups is FFS[1]. And in RAN2#126 meeting, RAN2 determines the maximum number of subgroups for LP-WUS is no less than 8 [2]. In our view, the detail number would be further discussed and determined by RAN1 and there is no need to discuss further in RAN2.

RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

Regarding the subgrouping method for LP-WUS, RAN2 agreed that the PEI subgrouping method is taken as baseline, i.e. CN assigned and UE_ID based subgrouping. In the following part, we will analyse the specific details for CN assigned and UE_ID-based subgrouping.

For the CN assigned subgrouping, RAN2 has the following agreement in RAN2#126 meeting [2]:

RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

We think the similar mechanism and NAS signalling defined for PEI could be reused directly for LP-WUS subgrouping. Which may include the following characteristics:

- UE indicates its CN assigned subgrouping capability for LP-WUS subgrouping to CN during the registration procedure;

- CN assigns the subgrouping ID for LP-WUS to UE during the registration procedure and UE stores the subgrouping ID for LP-WUS in the UE context;

- UE determines whether the current serving cell supports the CN assigned subgrouping for LP-WUS based on the information in SI. If the serving cell supports the CN assigned subgrouping for LP-WUS, UE will monitor the LP-WUS with the CN assigned subgrouping ID.

Proposal 8: For CN assigned LP-WUS subgrouping, RAN2 assumes similar signalling defined for PEI should be reused for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion.

For the UE_ID based subgrouping, the similar formula as PEI which is as follows could be used as the baseline.

SubgroupID = (floor (UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID), where: - N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state as specified in clause 7.1 - Ns: number of paging occasions for a PF - UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192 - subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in system information

Proposal 9: For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

- UE_ID is related to 5G-S-TMSI,

- N is the number of total paging frames in DRX cycle,

- Ns is the number of the PO for a PF,

- Np is the number of subgroupNumForUEID for PEI, if configured; otherwise, Np is 1,

- subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Based on the RAN1 agreement above, it is possible that one UE monitors both LP-WUS and PEI.

Observation 3: A UE may monitor PEI after being woken up by LP-WUS.

In case LP-WUS and PEI are used together for one UE, it is expected that UE could achieve more power saving gain via two levels of subgrouping. According to the following Figure 2 and Figure 3, assuming there are 1024 UEs supporting both LP-WUS and PEI subgrouping sharing the same PO and the subgrouping number of LP-WUS and PEI are both 4, in case UE_ID#0 is paged:

- In Figure 1, LP-WUS subgrouping and PEI subgrouping are totally same, 256 UEs will be woken up by LP-WUS to monitor PEI, and all of them will be indicated by PEI to monitor the following PO.

- In Figure 2, LP-WUS subgrouping and PEI subgrouping are orthogonal, 256 UEs will be woken up by LP-WUS to monitor PEI, while only 64 UEs will be indicated by PEI to monitor the corresponding PO.

Figure 2: LP-WUS and PEI subgrouping are same

Figure 3: LP-WUS and PEI subgrouping are diverse/ orthogonal

It could be observed that if the subgroup ID of LP-WUS and PEI are the same, there is no benefit for PEI monitoring, instead it leads to more power consumption as UE has to monitor PEI after LP-WUS. However, if the subgroup ID of LP-WUS and PEI are independent, the false alarm rate could be further reduced with PEI.

Observation 4: False alarm rate could be further reduced if the subgrouping ID of LP-WUS and PEI are independent.

To achieve additional power saving gain as analyzed above and considering both LP-WUS and PEI support UE_ID based subgrouping and CN assigned subgrouping, the following options could be considered to achieve more power saving gain:

- For CN assigned subgrouping, it is expected that the CN could assign different subgrouping ID for LP-WUS and PEI for the same UE as much as possible. However, it is up to the SA2/CT1 design and up to network implementation to assign same or different subgrouping ID for the same UE.

- For UE_ID based subgrouping, different bits in 5G-S-TMSI could be used as UE_ID for LP-WUS and PEI subgrouping.

- For case CN assigned subgrouping is used for one feature (i.e. LP-WUS or PEI), while UE_ID based subgrouping is used for another feature, considering different subgrouping methods are used, it is expected that independent subgrouping ID is assigned/calculated for LP-WUS and PEI, naturally. It could be achieved by the restriction that the number of CN assigned subgrouping is same within the registration area, which is up to network implementation.

Proposal 10: In case LP-WUS is used together with PEI, it is expected independent subgroup ID is assigned for a UE in the LP-WUS subgrouping and PEI subgrouping.

Proposal 11: For UE_ID based subgrouping, different bits in 5G-S-TMSI are used as UE_ID for LP-WUS subgrouping and the PEI subgrouping.

- Conclusion

In this contribution, we discuss LP-WUS configuration, the entry and exit condition, wake up procedure, subgrouping and SI reception aspects. Based on the discussion, we have the following observations and proposals.

Observations：

Observation 1:  It is up to UE implementation to determine whether to start the LP-WUS monitoring when the entry condition is fulfilled.

Observation 2:  It is up to UE implementation to determine whether to stop the LP-WUS monitoring when the exit condition is fulfilled.

Observation 3: A UE may monitor PEI after being woken up by LP-WUS.

Observation 4: False alarm rate could be further reduced if the subgrouping ID of LP-WUS and PEI are independent.

Proposals：

LP-WUS configuration:

Proposal 1: LP-WUS configuration is provided in SIB1.

Proposal 2: The entry/exit condition for LP-WUS monitoring is mandatorily configured if the network is intended to support the LP-WUS feature.

Entry/Exit conditions for using LP-WUS:

Proposal 3: Regarding whether any measurement via LR is needed for entry condition and whether any measurement via MR is needed for exit condition, RAN2 to wait for RAN1/RAN4 progress.

Proposal 4: The metric for serving cell quality measured by MR for entry condition includes RSRP and optional RSRQ.

Proposal 5: RAN2 assumes the metric for serving cell quality measured by LR for exit condition includes RSRP and optional RSRQ. An LS is sent to RAN1/RAN4 for confirmation.

LP-WUS wake up procedure:

Proposal 6: RAN2 to discuss whether dynamic PO is supported.

Proposal 7: MR should wake up for UE-initiated UL transmission, e.g. RACH/MO-SDT/RNA update/TA update, due to UL data and signaling arrival when using LP-WUS.

LP-WUS Subgrouping:

Proposal 8: For CN assigned LP-WUS subgrouping, RAN2 assumes similar signalling defined for PEI should be reused for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion.

Proposal 9: For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

- UE_ID is related to 5G-S-TMSI,

- N is the number of total paging frames in DRX cycle,

- Ns is the number of the PO for a PF,

- Np is the number of subgroupNumForUEID for PEI, if configured; otherwise, Np is 1,

- subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Proposal 10: In case LP-WUS is used together with PEI, it is expected independent subgroup ID is assigned for a UE in the LP-WUS subgrouping and PEI subgrouping.

Proposal 11: For UE_ID based subgrouping, different bits in 5G-S-TMSI are used as UE_ID for LP-WUS subgrouping and the PEI subgrouping.

- References

- R2-2403732, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

- R2-2405702, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

- Draft_Minutes_report_RAN1#116b_v030

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo

- R1-2401937, Report of RAN1#116 meeting