3GPP TSG-RAN WG2 Meeting #127		R2- 2406428

Maastricht, Netherlands, 19th – 23rd Aug. 2024

Source:	vivo

Title:	Discussion on RRM measurement relaxation and offloading in RRC_IDLE and INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

- Introduction

In RAN#103 meeting, the revised WID for R18 low-power Wake-up Signal and Receiver for NR was agreed [1]. In the WID [1], the objective of the RRC_IDLE/INACTIVE modes is as follows:

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In RAN2#126 meeting, the following agreements for RRM measurement relaxation and offloading in RRC_IDLE and INACTIVE have been achieved [2].

Agreements on RRM measurement relaxation and offloading in RRC_IDLE and INACTIVE: - For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. - RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

In this contribution, we will further discuss the issues for MR serving/neighboring cell RRM measurements relaxation and serving cell RRM measurement offloading from MR to LP-WUR for RRC_IDLE/INACTIVE mode.

- Discussion

  - General

According to the WID [1], regarding the RRM measurement relaxation and offloading, we have the following objectives:

Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

It could be found that both RRM measurement relaxation and offloading are supported for the serving cell while only RRM measurement relaxation is supported for the neighboring cell. As for the specific definition of these terms, companies had different understandings for RRM measurement relaxation/offloading on serving cell in the previous meeting.

In our understanding, for RRM measurement relaxation/offloading on serving cell, there are two cases:

- Case 1: RRM measurement relaxation on serving cell: MR performs serving cell measurement relaxation + optional LR measurement;

In this case, MR performs relaxed serving cell measurement, while LR may or may not perform serving cell measurement. During the discussion, some companies thought this case could be called as “partial offload”. It is correct in case LR performs serving cell measurement. However, MR could also perform serving cell measurement relaxation without any LR measurement.

- Case 2: RRM measurement offloading on serving cell: No MR serving cell measurement + only LR performs measurement;

MR serving cell measurement can be stopped only if there is LR serving cell measurement.

For RRM measurement relaxation on neighboring cell, there is only one case:

- Case 3: RRM measurement relaxation on neighboring cell: only MR performs neighboring cell measurement further relaxation (including no MR neighboring cell measurement).

Whether the above cases exist or partially exist is being discussed in RAN4. We could wait for more progress from RAN4.

During Rel-18 SI phase, RAN2 reached the following conclusions regarding RRM measurement relaxation:

R2 assumes In ultra-deep-sleep, RRM measurement on serving cell via MR is relaxed (may include no measurement) if RRM measurement on LR is feasible/supported. FFS on the details, e.g. how to relax, in which condition. R2 assumes In ultra-deep-sleep, RRM measurement on neighboring cell via MR is relaxed (may include no measurement) if RRM measurement on LR is feasible/supported. FFS on the details, e.g. how to relax, in which condition. RAN2 has studied and concluded the feasibility for RRM measurement relaxation (including no measurement) for serving cell by MR and neighboring cell by MR at least if RRM measurement on LR for serving cell is feasible/supported. Details are to be decided in WI.

As usual, RAN2 should work on the relaxation criteria, while the detailed RRM relaxation/offloading mechanism, e.g. relaxation factor or relaxed requirement would be discussed and determined by RAN4. In RAN2#126 meeting, RAN2 agreed to specify the offloading criterion for serving cell in RAN2, and assume RAN4 will define the specific requirements for serving cell offloading. Similarly, for serving cell measurement relaxation, we think the serving cell MR measurement relaxation criteria should also be discussed in RAN2, and the specific requirements for serving cell measurement relaxation should  also be discussed by RAN4.

Proposal 1: For serving cell MR measurement relaxation (whether there is serving cell measurement via LR is FFS, and whether it is applicable for UE supporting LP-WUS is FFS), RAN2 should focus on specifying the relaxation criterion for serving cell.

Besides, in RAN2#126 meeting, RAN2 also agreed to further discuss the neighboring cell measurement relaxation criteria, we think the requirements for neighbour cell measurement relaxation should be determined by RAN4.

Proposal 2: RAN2 to wait for RAN4 to discuss/determine the detailed relaxation factors and requirements for serving/neighboring cell.

  - RRM relaxation/offloading criteria

Regarding the specific criteria for MR serving cell RRM measurement relaxation/offloading and MR neighboring cell measurement relaxation, it could simplily refer to the relaxation criteria defined for neighboring cell measurement decided in Rel-16/17, which include “serving cell quality” and “low mobility”.

Regarding “serving cell quality” criteria, considering RAN2 already made the following agreement [2] for the entry/exit conditions for LP-WUS monitoring as below, the “serving cell quality” criteria for RRM relaxation/offloading could be based on serving cell quality measurement via MR (FFS via LR).

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

Regarding “low mobility” criteria, it could be further discussed. For example, it is also related to whether the relaxation criteria for serving cell relaxation/offloading is the same as or separate from the entry condition for LP-WUS monitoring.

Proposal 3-1: The criteria for serving cell measurement relaxation/offloading include the serving cell quality, i.e. the serving cell quality via MR (FFS via LR) is better than a threshold. FFS whether other criteria, e.g. low mobility.

Regarding whether the relaxation criteria/threshold for serving cell relaxation/offloading is the same as or separate from the entry condition/threshold for LP-WUS monitoring, which is described as the following directions, RAN2 needs further discussion.

- Direction 1: the criteria/threshold for RRM relaxation/offloading are the same as the entry condition for LP-WUS monitoring.

- Direction 2: the criteria/ threshold for RRM relaxation/offloading are separate from the entry condition for LP-WUS monitoring.

After the study in Rel-18, the following conclusion was made in TR 38.869 [3]:

UE power saving gain cannot be observed if the existing Rel-18 MR RRM measurement periodicity for serving and neighbour cells are applied and UE MR enters in ultra-deep sleep during LP LP-WUS monitoring, therefore MR serving and neighbour cell measurement with further time domain relaxation than that is allowed in Rel-18 specification for IDLE/INACTIVE and/or at least serving cell RRM offloaded from MR to LR are beneficial.

It could be observed that there is no power saving gain if UE starts to monitor LP-WUS while the serving cell measurement relaxation/offloading or neighboring cell measurement relaxation has not been performed. In this way, if the criteria/threshold for RRM relaxation/offloading are separate from the entry condition for LP-WUS monitoring, i.e. Direction 2, there may be a risk that the entry condition for LP-WUS monitoring is fulfilled, but the RRM relaxation/offloading criteria have not been fulfilled, which is not expected.

Observation 1: There is no power saving gain that UE monitors LP-WUS without RRM measurement relaxation/offloading.

Thus, network configuration on the specific threshold of the RRM relaxation/offloading criteria and entry condition for LP-WUS monitoring should avoid such case without any power saving gain. In order to guarantee serving/neighboring cell measurement is relaxed when UE monitors LP-WUS, thresholds of the criteria for the serving/neighboring cell measurement relaxation should be lower than or equal to the threshold of the entry condition for LP-WUS monitoring. As for serving cell measurement offloading, considering the entry conditions of LP-WUS monitoring include at least serving cell quality via MR, serving cell measurement via MR can not be offloaded to LR before UE enters the LP-WUS monitoring state. Thus thresholds of the criteria for the serving cell measurement offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring.

Based on the analysis above, we provide two examples to illustrate the thresholds and UE behaviors for the serving cell measurement relaxation/offloading and neighboring cell measurement relaxation comparing to the threshold of entry condition on LP-WUS monitoring in the following figures.

Figure 1: Thresholds and UE behaviors for serving cell offloading/neighboring cell measurement relaxation and LP-WUS monitoring

Figure 2: Thresholds and UE behaviors for serving/neighboring cell measurement relaxation and LP-WUS monitoring

For simplicity, the serving cell measurement offloading and serving cell measurement relaxation are provided in Figure 1 and Figure 2, respectively. All the criteria for serving cell measurement relaxation/offloading and neighboring cell measurement relaxation and the entry condition for LP-WUS monitoring are based on the serving cell quality via MR.

For neighbor cell measurement relaxation, it refers to the neighboring cell measurement for intra-frequency/inter-frequency with equal or lower priority frequency. As for the inter-frequency with higher priority, it will be more complicated and can be further discussed later. When the serving cell quality based on the measurement via MR exceeds the legacy S-measure threshold, there is no neighboring cell measurement to be performed. Besides, it is plain and obvious that the threshold of the legacy S-measure threshold is higher than or equal to the threshold for the neighboring cell measurement relaxation (i.e. further relaxation is used to differentiate the RRM relaxation in Rel-16/17), since the neighboring cell measurement can be further relaxed only in case UE needs to perform neighboring cell measurement.

In Figure 1, there are two alternatives for the threshold of entry condition for LP-WUS monitoring and the threshold of serving cell offloading criteria, which could either be higher or lower than legacy S-measure threshold, i.e. Case 1 and Case 2. Besides, as the analysis above, the threshold of the criteria for the serving cell measurement offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring. For the green arrow, it represents that UE performs serving cell offloading if the serving cell offloading criteria are fulfilled.

For Case 1, i.e. the threshold of serving cell offloading criteria and the threshold of criteria for LP-WUS monitoring is higher than legacy S-measure threshold, when the entry condition for LP-WUS monitoring is fulfilled and the serving cell offloading criteria is fulfilled, UE will monitor LP-WUS and perform serving cell measurement offloading from MR to LR and UE won’t perform neighboring cell measurement. For Case 2, i.e. the threshold of serving cell offloading criteria and the threshold of criteria for LP-WUS monitoring is lower than legacy S-measure threshold, UE will monitor LP-WUS and perform serving cell measurement offloading from MR to LR, and UE performs relaxed neighboring cell measurement via MR.

In Figure 2, similar to Figure 1, there are two alternatives for the threshold of entry condition for LP-WUS monitoring and serving cell measurement relaxation criteria, i.e., Case 1 and Case 2. Besides, the threshold of entry condition for LP-WUS monitoring is higher than or equal to the threshold of the serving cell relaxation criteria. For Case 1, i.e. the threshold of serving cell relaxation criteria and the threshold of criteria for LP-WUS monitoring is higher than legacy S-measure threshold, when the entry condition for LP-WUS monitoring is fulfilled and the serving cell measurement relaxation is fulfilled, UE will monitor LP-WUS and perform serving cell measurement relaxation and no neighboring cell measurement. For Case 2, i.e. the threshold of serving cell relaxation criteria and the threshold of criteria for LP-WUS monitoring is lower than legacy S-measure threshold, UE will monitor LP-WUS and perform serving cell measurement relaxation, and MR neighboring cell measurement relaxation.

Proposal 3-2: Thresholds of the criteria for serving/neighboring cell measurement relaxation should be lower than or equal to the threshold of entry condition for LP-WUS monitoring.

Proposal 3-3: Thresholds of the criteria for serving measurement offloading should be higher than or equal to that of the entry condition for LP-WUS monitoring.

For further relaxation on neighboring cell measurement, according to the WID [1], the LR cannot perform neighboring cell measurement and only MR is concerned, which is the same as R16/17 neighboring cell measurement relaxation. Thus, the ‘not at cell edge’ and ‘low mobility’ criteria, similar as legacy, could be reused. In our view, the only difference is that the measurement relaxation for neighboring cell could be further relaxed than Rel-16/17, i.e. thresholds of R19 neighboring cell measurement relaxation should be stricter than that of R16/17 neighboring cell measurement relaxation, while the detailed relaxation factors and corresponding requirements would be discussed in RAN4.

Observation 2: LR doesn’t perform neighboring cell measurement, so the criteria for R19 neighboring cell measurement relaxation is similar to that of R16/17 neighboring cell measurement relaxation, i.e. ‘not at cell edge’ and ‘low mobility’.

Proposal 4: For R19 neighboring cell measurement relaxation, R16/17 neighboring cell measurement relaxation criteria (i.e. ‘not at cell edge’ and ‘low mobility’) can be reused as the baseline.

Proposal 5: Regarding the criteria ‘not at cell edge’ and ‘low mobility’, the corresponding thresholds of R19 neighboring cell measurement relaxation should be stricter than that of R16/17 neighboring cell measurement relaxation.

- Conclusion

In this contribution, we discuss the RRM relaxation of UE MR for both serving and neighboring cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR. Based on the discussion, we have the following observations and proposals.

General:

Proposal 1: For serving cell MR measurement relaxation (whether there is serving cell measurement via LR is FFS, and whether it is applicable for UE supporting LP-WUS is FFS), RAN2 should focus on specifying the relaxation criterion for serving cell.

Proposal 2: RAN2 to wait for RAN4 to discuss/determine the detailed relaxation factors and requirements for serving/neighboring cell.

RRM relaxation/offloading criteria:

Observation 1: There is no power saving gain that UE monitors LP-WUS without RRM measurement relaxation/offloading.

Observation 2: LR doesn’t perform neighboring cell measurement, so the criteria for R19 neighboring cell measurement relaxation is similar to that of R16/17 neighboring cell measurement relaxation, i.e. ‘not at cell edge’ and ‘low mobility’.

Proposal 3-1: The criteria for serving cell measurement relaxation/offloading include the serving cell quality, i.e. the serving cell quality via MR (FFS via LR) is better than a threshold. FFS whether other criteria, e.g. low mobility.

Proposal 3-2: Thresholds of the criteria for serving/neighboring cell measurement relaxation should be lower than or equal to the threshold of entry condition for LP-WUS monitoring.

Proposal 3-3: Thresholds of the criteria for serving measurement offloading should be higher than or equal to that of the entry condition for LP-WUS monitoring.

Proposal 4: For R19 neighboring cell measurement relaxation, R16/17 neighboring cell measurement relaxation criteria (i.e. ‘not at cell edge’ and ‘low mobility’) can be reused as the baseline.

Proposal 5: Regarding the criteria ‘not at cell edge’ and ‘low mobility’, the corresponding thresholds of R19 neighboring cell measurement relaxation should be stricter than that of R16/17 neighboring cell measurement relaxation.

- References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo

- R2-2405702, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

- 3GPP TR 38.869 v18.0.0