3GPP TSG-RAN WG2 Meeting #127		R2-2406429

Maastricht, Netherlands, 19th – 23rd Aug. 2024

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_CONNECTED

Agenda Item:	8.4.4

Document for:	Discussion and Decision

- Introduction

In RAN#103 meeting, the revised WID for R19 low-power Wake-up Signal and Receiver for NR was agreed [1]. In this WID, the objective of the RRC_CONNECTED mode is as follows:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

In RAN2#126 meeting, some progress has been made in RAN2 on the impacts on potion 1-1. In this contribution, we will further discuss the RAN2 impacts on RRC_CONNECTED procedures with LP-WUS/WUR based on RAN1 progress.

- Discussion

  - RAN2 impacts on different options

2.2.1 Background

In RAN1#116 and #116bis meetings, RAN1 has discussed the LP-WUS procedure to trigger PDCCH monitoring in RRC_CONNECTED and achieved some conclusions, which are summarized as follows [2]:

Agreement - Update the following agreement in RAN1#116 in red: Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 - Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

In RAN2#126 meeting, RAN2 discussed the LP-WUS/WUR in RRC_CONNECTED and agreed that RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1 as above [3].

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1.

Regarding the above three LP-WUS operation methods to trigger PDCCH monitoring, we have some progress on RAN2 impacts for Option 1-1 as below.

For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

However, for Option 1-2 and Option 1-3, the discussion was unfocused and the time is no enough in RAN2#126 meeting. Thus, we failed to reach agreements. In the following, we will continue to discuss RAN2 impacts on Option 1-2 and Option 1-3.

2.2.2 RAN2 impacts on Option 1-2

General

For Option 1-2: LP-WUS monitoring outside at least legacy C-DRX Active Time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. Regarding whether to monitor PDCCH in legacy C-DRX Active Time triggered by the start of drx-onDurationTimer for Option 1-2, RAN1 has agreed to further discuss the following two options:

- Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS;

- Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS.

Considering RAN1 agreed LP-WUS is monitored outside the active time and RAN1 didn’t discuss the C-DRX Active Time deeply, RAN2 should clarify the LP-WUS monitoring behaviour firstly for Option 1-2-1 and Option 1-2-2 in different duration. Correspondingly, how to monitor PDCCH triggered by LP-WUS and legacy C-DRX Active Time should also be discussed in RAN2.

Observation 1: The RAN2 impact on Option 1-2 is to clarify the LP-WUS monitoring, and the PDCCH monitoring behaviour triggered by LP-WUS and legacy C-DRX Active Time.

According to the MAC specification [4], the C-DRX Active Time includes not only the time duration when drx-onDurationTimer is running but also other cases, which are summarized below:

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or - drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or - ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or - there is an ongoing RACH-less LTM cell switch; or - there is an ongoing RACH-less handover in a terrestrial network.

In our understanding, the C-DRX Active Time other than the duration when drx-onDurationTimer is running is quite different from the C-DRX Active Time when drx-onDurationTimer is running with the following reasons.

The drx-onDurationTimer is configured to fit the traffic characteristic, e.g. arrival periodicity, which is not always expected there is PDCCH monitoring during drx-onDurationTimer. While the C-DRX Active Time other than the time duration when drx-onDurationTimer is running is designed for specific scheduling or necessary PDCCH monitoring, it is expected there is PDCCH monitoring during these C-DRX Active Time. For example, during drx-RetransmissionTimer, it is expected there is scheduling for retransmission. During ra-ContentionResolutionTimer or msgB-ResponseWindow, it is expected there is scheduling for Msg.4 or Msg.B. Other cases are similar.

Observation 2: Within C-DRX Active Time other than the duration when drx-onDurationTimer is running, it is expected there is always PDCCH scheduling; while within the C-DRX Active Time when drx-onDurationTimer is running, there may not be PDCCH scheduling.

Taking the current C-DRX pattern as the example, we will analyze the LP-WUS monitoring behaviour and PDCCH monitoring behaviour for both Option 1-2-1 and Option 1-2-2 in the duration outside the current C-DRX Active Time (e.g. the Part 1 in Fig.1 below) and C-DRX Active Time within the duration other than the time when drx-onDurationTimer is running (e.g. the Part 2 in Fig.1 below) and within the duration when drx-onDurationTimer is running (e.g. the Part 3 in Fig1 below) separately.

Figure 1: C-DRX time partitioning example

Regarding the duration outside the current C-DRX Active Time (e.g. the Part 1 in Fig.1), there is no difference between Option 1-2-1 and Option 1-2-2 according to RAN1 agreement, thus UE behaviours on LP-WUS monitoring should be for both Option 1-2-1 and Option 1-2-2 rightly. Since there is no PDCCH monitoring for a UE with current C-DRX pattern outside the legacy C-DRX Active Time, UE should monitor LP-WUS, i.e. during the Part I.

Regarding the legacy C-DRX Active Time when drx-onDurationTimer is running (e.g. the Part 3 in Fig.1), the PDCCH monitoring behaviours are different for Option 1-2-1 and Option 1-2-2 according to RAN1 agreement, corrspondingly, the LP-WUS monitoring behaviour for Option 1-2-1 and Option 1-2-2should also be different. In the following, we will discuss the detailed design.

Impacts on Option 1-2-1

For option 1-2-1, LP-WUS monitoring is performed only outside legacy C-DRX Active Time. After receiving LP-WUS, PDCCH monitoring is additionally triggered by LP-WUS. Regarding the C-DRX Active Time (including the Part 2 and Part 3 in Fig.1), legacy UE behaviour on PDCCH monitoring will not be impacted. The LP-WUS monitoring behaviour is shown as the Fig.2-1:

Figure 2-1: LP-WUS monitoring behaviour for Option 1-2-1

With the above, Option 1-2-1 on LP-WUS monitoring and PDCCH monitoring are proposed as follows:

Proposal 1: For Option 1-2-1, LP-WUS monitoring is performed outside legacy C-DRX Active Time according to the LP-WUS monitoring configuration.

Proposal 2: For option 1-2-1, PDCCH monitoring is additionally triggered by LP-WUS and triggered by legacy C-DRX Active Time (including drx-ondurationTimer is running and other legacy Active Time), i.e., PDCCH monitoring when drx-onDurationTimer is running is controlled by drx-ondurationTimer as legacy rather than LP-WUS.

Impacts on Option 1-2-2

For option 1-2-2, RAN1 has agreed that PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS. Considering the reason that legacy C-DRX Active Time includes drx-onDurationTimer is due to there is PDCCH monitoring in this duration, and RAN1 has agreed that there is no PDCCH monitoring in this duration, the duration for drx-onDurationTimer (e.g. Part 3-1 in Fig.2-2) should not be the “legacy C-DRX Active Time”.

Observation 3: For Option 1-2-2, drx-onDurationTimer should not be the “legacy C-DRX Active Time” as RAN1 agreed that there is no PDCCH monitoring in this duration.

Thus, for the duration of legacy drx-onDurationTimer (e.g. Part 3-1 in Fig.2-2), the PDCCH monitoring should be also controlled by LP-WUS, i.e. UE needs to monitor the LP-WUS and UE only monitors the PDCCH when the LP-WUS is received within drx-onDurationTimer duration.

Besides, similar as Option 1-2-1, RAN1 has agreed that UE should monitor LP-WUS outside the legacy C-DRX Active Time, e.g. during the Part 1 in Fig.2-2.

Regarding the C-DRX Active Time other than the time when drx-onDurationTimer is running (e.g. the Part 2 in Fig.2-2), since there will be PDCCH in most cases as explained above, to keep the simple design for LP-WUS in this release, it is assumed that the UE will keep monitoring PDCCH as legacy during the C-DRX Active Time other than the time when drx-onDurationTimer is running, i.e. PDCCH monitoring by the legacy timer, even there is no LP-WUS has been detected. Thus, within the C-DRX Active Time other than the time when drx-onDurationTimer is running (e.g. the Part 2 in Fig.2-2), there is no need to monitor LP-WUS.

Figure 2-2: LP-WUS monitoring behaviour for Option 1-2-2

With the above, Option 1-2-2 on LP-WUS monitoring and PDCCH monitoring are proposed as follows:

Proposal 3: For Option 1-2-2, LP-WUS monitoring is outside legacy C-DRX Active Time (e.g. Part 1 in Fig.2-2) and within the duration indicated by legacy drx-onDurationTimer (e.g. Part 3-1 in Fig.2-2), regardless whether the legacy drx-onDurationTimer is running or not.

Proposal 4: For option 1-2-2, PDCCH monitoring is triggered by LP-WUS and legacy C-DRX Active Time other than drx-ondurationTimer is running (e.g. Part 3-1 in Fig.2-2), i.e., PDCCH monitoring is not controlled by legacy drx-ondurationTimer.

For Option 1-2-2, whether legacy drx-onDurationTimer is running periodically or not needs to be further discussed. As mentioned above, considering there is no PDCCH monitoring in the duration of drx-onDurationTimer, there is no motivation to start the drx-onDurationTimer. But the corresponding behaviour except PDCCH monitoring could be further discussed if drx-onDurationTimer is not started. In Rel-16 DCP discussion, RAN2 has discussed the UE behaviour for the time which is current symbol n occurs within drx-onDurationTimer duration while associated drx-onDurationTimer is not started and no other reason, e.g. the grants/assignments to turn the time to be C-DRX Active Time, and RAN2 reached the following conclusions on UE behaviour:

- UE doesn’t transmit SRS;

- UE doesn’t report the semi-persistent CSI configured on PUSCH or semi-persistent CSI on PUCCH;

- Based on the network configuration, UE determines whether to report the periodic CSI. i.e UE determines whether to report the periodic CSI that is L1-RSRP on PUCCH based on the ps-TransmitPeriodicL1RSRP parameter to and UE determines whether to report the periodic CSI that is not L1-RSRP on PUCCH based on the ps-TransmitOtherPeriodicCSI parameter.

In our understanding, same issues for LP-WUS in Option 1-2-2 will occur if the drx-onDurationTimer won’t be started. And we think the same UE behaviour in DCP could be considered as the starting point for LP-WUS Option 1-2-2, while RAN2 could have further discussion on it if Option 1-2-2 is supported.

Proposal 5: For Option 1-2-2, the legacy drx-onDurationTimer won’t be running periodically. RAN2 will further discuss the UE behaviour within drx-onDurationTimer duration while associated drx-onDurationTimer is not started and the MAC entity is not in C-DRX Active Time.

Impacts on both Option 1-2-1 and Option 1-2-2

For Option 1-2-1 and Option 1-2-2, after UE detects the LP-WUS it will start the PDCCH monitoring. How to stop the PDCCH monitoring should be discussed by RAN2.

Observation 4: Another RAN2 impact on Option 1-2 is to determine how to stop the PDCCH monitoring triggered by LP-WUS.

Figure 3: PDCCH monitoring triggered by option 1-2 LP-WUS

RAN1 has the following agreement in RAN1#117 meeting [5], it could be observed that LP-WUS could work without existing features. In this case, the most straightforward method is to define a timer to control the stop of PDCCH monitoring triggered by LP-WUS, and the UE performs PDCCH monitoring until the timer is expired. Whether to introduce a new timer or reuse the legacy C-DRX timers, could be further discussed.

Agreement For RRC CONNECTED mode, LP-WUS can be configured without following existing features. - Rel-16 DCP - Rel-17 PDCCH skipping - Rel-17 SSSG switching - Rel-18 cell DTX Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX)

During the running of this timer triggered by LP-WUS, considering PDCCH monitoring should be performed. Whether the duration when this timer is running should be defined as Active Time could be further discussed as shown in Fig.3. Considering PDCCH monitoring in this duration, there is no need to monitor LP-WUS in this duration.

Proposal 6: For Option 1-2, introducing a timer to stop the PDCCH monitoring triggered by LP-WUS, and the UE performs PDCCH monitoring until the timer expires. FFS whether introducing a new timer or reusing legacy DRX timer.

Proposal 7: For Option 1-2, UE doesn’t monitor LP-WUS within the duration when UE monitors PDCCH triggered by LP-WUS. FFS whether the duration when PDCCH monitoring triggered by LP-WUS is defined as C-DRX Active Time.

Besides, we think LP-WUS could also work together with the existing feature, e.g. PDCCH skipping. In this case, PDCCH skipping indication could be also considered here to stop the PDCCH monitoring. More details could be further discussed based on RAN1 progress.

Proposal 8: For Option 1-2, when LP-WUS is used together with PDCCH skipping, the UE stops the PDCCH monitoring triggered by LP-WUS upon receiving the PDCCH skipping indication.

2.2.2 RAN2 impacts on Option 1-3

For Option 1-3: LP-WUS monitoring inside C-DRX Active Time according to the corresponding configuration to trigger PDCCH monitoring. For this option, RAN2 should further discuss what the C-DRX Active Time includes in this case for LP-WUS monitoring, and the UE behaviour on PDCCH monitoring triggered by LP-WUS.

Observation 5: RAN2 impact on Option 1-3 is to clarify where to monitor LP-WUS inside C-DRX Active Time and clarify the PDCCH monitoring behaviour after introducing the LP-WUS.

In our view, this solution is mainly used to resolve jitter issue in drx-onDurationTimer, which is useful for traffic with jitter, e.g. XR, to reduce the unnecessary PDCCH monitoring due to jitter while guaranteeing the service performance. In this case, the data packets may have a wide range of jitter and the network couldn’t predict when the data will arrive exactly, so the drx-onDurationTimer is configured with a long period to accommodate potential jitter, and the UE has to perform intensive PDCCH monitoring in the entire jitter range. The following figure shows the example of Option 1-3:

Figure 4: LP-WUS monitoring inside at least legacy C-DRX Active Time to trigger PDCCH monitoring

Observation 6: The LP-WUS procedures to trigger PDCCH monitoring Option 1-3 is useful for traffic with jitter, e.g. XR, as it can reduce unnecessary PDCCH monitoring due to jitter without loss of performance, e.g. latency.

Besides, according to the agreement in RAN1, it seems that for Option 1-3, UE should monitor LP-WUS in all C-DRX Active Time. However, in our view, there is no jitter issue in C-DRX Active Time other than the time when drx-onDurationTimer is running. Thus there is no need for UE to monitor LP-WUS and could monitor PDCCH directly inside the C-DRX Active Time other than the time when drx-onDurationTimer is running.

Proposal 9: For Option 1-3, UE only monitors the LP-WUS within the legacy drx-onDurationTimer duration.

Regarding the PDCCH monitoring period in this option, the UE starts to monitor LP-WUS instead of PDCCH from the beginning of drx-onDurationTimer, and the UE starts PDCCH monitoring via MR once LP-WUS is detected and indicates the PDCCH monitoring. With this, power saving gain without latency for data transmission could be obtained. The PDCCH monitoring within the other C-DRX Active Time won’t be impacted by LP-WUS Option 1-3.

Proposal 10: For Option 1-3, PDCCH monitoring within the drx-onDurationTimer duration is triggered by LP-WUS, and PDCCH monitoring in legacy Active Time other than the drx-onDurationTimer duration is as legacy.

  - Benefit analysis on Option 1-1, Option 1-2-1 and Option 1-2-2

2.3.1 Comparison between Option 1-2-1 and Option 1-2-2

The design target for LP-WUS in RRC_CONNECTED mode is to reduce unnecessary PDCCH monitoring, thus the most beneficial scenario for using LP-WUS is to control all uncertain PDCCH monitoring. As we have analyzed above, the legacy drx-onDurationTimer is configured by RRC and starts periodically, thus it may not perfectly match with the real arrival time of data burst, i.e. there may be no data transmission at the starting point of the duration or even during the whole period. LP-WUS could reduce the power consumption on unnecessary PDCCH monitoring within this duration.

However, Option 1-2-1 couldn’t bring power saving gain on this aspect considering PDCCH monitoring may be additionally triggered by LP-WUS outside the legacy C-DRX Active Time and it is only beneficial for latency performance. And it has to be used together with Option 1-1, which will bring extra complexity compared to Option 1-2-2. While Option 1-2-2 could work alone to obtain the power saving gain within this duration since PDCCH monitoring is only triggered by LP-WUS rather than legacy drx-onDurationTimer. Hence, Option 1-2-1 will bring extra complexity and power consumption compared to Option 1-2-2.

Observation 7: Option 1-2-1 has no power saving gain.

Observation 8: The restriction of Option 1-2-1 configured together with Option 1-1 will bring extra complexity compared to Option 1-2-2.

With the above, we think Option 1-2-1 could be excluded firstly.

Proposal 11: RAN2 assumes Option 1-2-1 is not supported for LP-WUS in RRC_CONNECTED mode.

2.3.2 Simulation results on Option 1-2-2

Since Option 1-1 and Option 1-2-2 have similar functions and application scenarios, we will compare the power saving gain (PSG) and user-perceived throughput (UPT) gain between Option 1-1 and Option 1-2-2 as well as the legacy power saving schemes introduced in legacy. The study output of simulation results on Option 1-1 and Option 1-2-2 in Rel-18 TR [6] is summarized in Annex.

To determine the procedures of LP-WUS triggering PDCCH monitoring, the options that can achieve better power saving and UPT performance should be prioritized. From Table 1 in Annex, Option 1-2-2 is the best option, which can achieve up to more than 60% power saving gain for deep sleep and up to more than 10% UPT gain for FTP traffic, up to more than 180% UPT gain for IM traffic according to the TR [6].

We further provided the performance comparison for the above options for LP-WUS triggering PDCCH monitoring in the following, as shown in Fig.5-1 and Fig.5-2. The detailed SLS assumptions and illustrations of LP-WUS replacement of DCP (i.e. option 1-1) and LP-WUS to trigger MR PDCCH monitoring (i.e. option 1-2-2) can be found in Annex.

In Fig.5-1, we provided evaluation results assuming DRX configuration of (DRX cycle-Inactivity_timer-Onduration_timer) = (80ms-8ms-8ms), and following cases were evaluated for FR1.

  - Always_On (i.e. No DRX)

  - Legacy DRX (best baseline by Rel-15)

  - Legacy DRX with DCP (best baseline by Rel-16)

  - Legacy DRX with DCP and Rel-17 PDCCH skipping (best baseline by Rel-17)

  - DRX with LP-WUS to replace DCP functionality and Rel-17 PDCCH skipping (LP-WUS option 1-1)

  - LP-WUS to trigger PDCCH monitoring from different MR sleep states (LP-WUS option 1-2-2)

Figure 5-1: Performance comparison for different options with 80ms DRX cycle for FR1

According to the results in Fig.5-1, The following observations can be obtained:

- “Legacy DRX with DCP and Rel-17 PDCCH skipping” (best baseline provided by Rel-17) can achieve 12.5% power saving gain with 35.8% UPT loss compared with Legacy DRX (best baseline by Rel-15)

- “DRX with LP-WUS to replace DCP functionality and Rel-17 PDCCH skipping (offset = 3/10 ms)” (LP-WUS option 1-1) can achieve additional 14.3% ~ 28.6% power saving gain with almost the same UPT, compared with “Legacy DRX with DCP and Rel-17 PDCCH skipping” (best baseline by Rel-17)

- “LP-WUS to trigger PDCCH monitoring from MR deep sleep” (LP-WUS option 1-2-2) can achieve additional 13.3% ~ 27.8% power saving gain with 101.7% ~ 125% UPT gain, compared with “DRX with LP-WUS to replace DCP functionality and Rel-17 PDCCH skipping (offset = 10 ~ 3ms for power saving gain and offset=3 ~10ms for UPT gain)” (LP-WUS option 1-1)

In Fig.5-2, we provided evaluation results assuming another DRX configuration of (DRX cycle-Inactivity_timer-Onduration_timer) = (160ms-100ms-8ms), and following cases were evaluated for FR1.

  - Always_On (i.e. No DRX)

  - Legacy DRX (best baseline by Rel-15)

  - Legacy DRX with DCP (best baseline by Rel-16)

  - Legacy DRX with DCP and Rel-17 PDCCH skipping (best baseline by Rel-17)

  - DRX with LP-WUS to replace DCP functionality and Rel-17 PDCCH skipping (LP-WUS option 1-1)

  - LP-WUS to trigger PDCCH monitoring from different MR sleep states (LP-WUS option 1-2-2)

Figure 5-2: Performance comparison for different options with 160ms DRX cycle for FR1

According to the results in Fig.5-2, the following observations can be obtained:

- “Legacy DRX with DCP and Rel-17 PDCCH skipping” (best baseline by Rel-17) can achieve 52.4% power saving gian with 71.4% UPT loss compared with Legacy DRX (best baseline by Rel-15)

- “DRX with LP-WUS to replace DCP functionality and Rel-17 PDCCH skipping (offset = 3/10 ms)” (LP-WUS option 1-1) can achieve additional 15% ~20% power saving gain with almost the same UPT, compared with “Legacy DRX with DCP and Rel-17 PDCCH skipping” (best baseline by Rel-17)

- “LP-WUS to trigger PDCCH monitoring from MR deep sleep” (LP-WUS option 1-2-2) can achieve additional 18.75% ~ 23.5% power saving gain with 203.9% ~ 220.5% UPT gain, compared with “DRX with LP-WUS to replace DCP functionality and Rel-17 PDCCH skipping (offset = 10 ~ 3ms for power saving gain and offset=3 ~10ms for UPT gain)” (LP-WUS option 1-1)

Based on the above observation, Option 1-2-2 could achieve more power saving gain than Option 1-1, and Option 1-2-2 has additional UPT gain than Option 1-1. Thus, Option 1-2-2 is the best option for LP-WUS in RRC_CONNECTED. Option 1-2-2 should be supported considering it has better performance than Option 1-1, and Option 1-1 could be excluded in this release since it has no additional gain compared to legacy DCP, while there is no need to introduce another similar function with no gain.

Proposal 12: RAN2 assumes Option 1-2-2 should be supported for LP-WUS in RRC_CONNECTED mode.

Proposal 13: RAN2 assumes Option 1-1 is not supported for LP-WUS in RRC_CONNECTED mode.

  - Enable and disable for LP-WUS monitoring

RAN1 has also agreed that gNB RRC signalling should be used to enable/disable the operation of PDCCH monitoring triggered by LP-WUS, but it is still FFS whether to support UE assistance for the enable/disable functionality.

For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance.

We think the enable/disable operation could rely on the configuration of LP-WUS as usual in RAN2. Regarding the additional UE assistance, according to the WID in [1], current RRM/RLM/BFD/CSI/BM measurement and corresponding report are performed by MR as legacy. In this way, network has enough information to enable/disable the LP-WUS functionality. Besides, the enable/disable operation is a long-term configuration. There is no need to dynamically change the configuration. Therefore, we think additional assistance information from UE to gNB for enable/disable operation is not necessary.

Proposal 14: In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

Regarding the RRC message to carry the LP-WUS configuration, RAN2 have agreed the dedicated RRC message is used.

The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In our understanding, it is straightforward to use the RRCResume and RRCReconfiguration messages as usual.

Proposal 15: LP-WUS configuration is configured in RRCResume/RRCReconfiguration message.

  - Activation and deactivation procedure for LP-WUS monitoring

Regarding the activation and deactivation for LP-WUS monitoring, RAN1 has the following options:

- In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

From RAN2 perspective, the meaning of enabling LP-WUS monitoring is to activate the LP-WUS functionality while UE couldn’t use LP-WUS if the LP-WUS functionality has some activation mechanisms. In this case, UE could use the LP-WUS functionality after receiving the LP-WUS activation signalling, and UE stops using the LP-WUS functionality upon receiving the LP-WUS deactivation signalling.

Considering the LP-WUS configuration overhead is comparable to other dedicated configurations, e.g. L3 measurement configuration, besides the LP-WUS monitoring is not an urgent behaviour, there is no motivation to introduce additional activation/deactivation procedure for LP-WUS monitoring. UE could use the LP-WUS when the LP-WUS monitoring is enabled, i.e., when UE receives the RRC dedicated signalling with LP-WUS configuration. And if the network doesn’t expect the UE to use the LP-WUS, it could just send the RRC signalling to disable the LP-WUS monitoring for the UE.

Proposal 16: RAN2 assumes UE could monitor LP-WUS upon LP-WUS is enabled, no additional indication/condition are needed for activation/deactivation of LP-WUS functionality. The final decision is up to RAN1.

- Conclusion

In this contribution, we discuss the LP-WUS triggers PDCCH monitoring in RRC_CONNECTED. Based on the discussion, we have the following observations and proposals.

Observations:

Observation 1: The RAN2 impact on Option 1-2 is to clarify the LP-WUS monitoring, and the PDCCH monitoring behaviour triggered by LP-WUS and legacy C-DRX Active Time.

Observation 2: Within C-DRX Active Time other than the duration when drx-onDurationTimer is running, it is expected there is always PDCCH scheduling; while within the C-DRX Active Time when drx-onDurationTimer is running, there may not be PDCCH scheduling.

Observation 3: For Option 1-2-2, drx-onDurationTimer should not be the “legacy C-DRX Active Time” as RAN1 agreed that there is no PDCCH monitoring in this duration.

Observation 4: Another RAN2 impact on Option 1-2 is to determine how to stop the PDCCH monitoring triggered by LP-WUS.

Observation 5: RAN2 impact on Option 1-3 is to clarify where to monitor LP-WUS inside C-DRX Active Time and clarify the PDCCH monitoring behaviour after introducing the LP-WUS.

Observation 6: The LP-WUS procedures to trigger PDCCH monitoring Option 1-3 is useful for traffic with jitter, e.g. XR, as it can reduce unnecessary PDCCH monitoring due to jitter without loss of performance, e.g. latency.

Observation 7: Option 1-2-1 has no power saving gain.

Observation 8: The restriction of Option 1-2-1 configured together with Option 1-1 will bring extra complexity compared to Option 1-2-2.

Proposals:

Impacts on Option 1-2-1

Proposal 1: For Option 1-2-1, LP-WUS monitoring is performed outside legacy C-DRX Active Time according to the LP-WUS monitoring configuration.

Proposal 2: For option 1-2-1, PDCCH monitoring is additionally triggered by LP-WUS and triggered by legacy C-DRX Active Time (including drx-ondurationTimer is running and other legacy Active Time), i.e., PDCCH monitoring when drx-onDurationTimer is running is controlled by drx-ondurationTimer as legacy rather than LP-WUS.

Impacts on Option 1-2-2

Proposal 3: For Option 1-2-2, LP-WUS monitoring is outside legacy C-DRX Active Time (e.g. Part 1 in Fig.2-2) and within the duration indicated by legacy drx-onDurationTimer (e.g. Part 3-1 in Fig.2-2), regardless whether the legacy drx-onDurationTimer is running or not.

Proposal 4: For option 1-2-2, PDCCH monitoring is triggered by LP-WUS and legacy C-DRX Active Time other than drx-ondurationTimer is running (e.g. Part 3-1 in Fig.2-2), i.e., PDCCH monitoring is not controlled by legacy drx-ondurationTimer.

Proposal 5: For Option 1-2-2, the legacy drx-onDurationTimer won’t be running periodically. RAN2 will further discuss the UE behaviour within drx-onDurationTimer duration while associated drx-onDurationTimer is not started and the MAC entity is not in C-DRX Active Time.

Impacts on both Option 1-2-1 and Option 1-2-2

Proposal 6: For Option 1-2, introducing a timer to stop the PDCCH monitoring triggered by LP-WUS, and the UE performs PDCCH monitoring until the timer expires. FFS whether introducing a new timer or reusing legacy DRX timer.

Proposal 7: For Option 1-2, UE doesn’t monitor LP-WUS within the duration when UE monitors PDCCH triggered by LP-WUS. FFS whether the duration when PDCCH monitoring triggered by LP-WUS is defined as C-DRX Active Time.

Proposal 8: For Option 1-2, when LP-WUS is used together with PDCCH skipping, the UE stops the PDCCH monitoring triggered by LP-WUS upon receiving the PDCCH skipping indication.

impacts on Option 1-3

Proposal 9: For Option 1-3, UE only monitors the LP-WUS within the legacy drx-onDurationTimer duration.

Proposal 10: For Option 1-3, PDCCH monitoring within the drx-onDurationTimer duration is triggered by LP-WUS, and PDCCH monitoring in legacy Active Time other than the drx-onDurationTimer duration is as legacy.

Benefit analysis on Option 1-1, Option 1-2-1 and Option 1-2-2

Proposal 11: RAN2 assumes Option 1-2-1 is not supported for LP-WUS in RRC_CONNECTED mode.

Proposal 12: RAN2 assumes Option 1-2-2 should be supported for LP-WUS in RRC_CONNECTED mode.

Proposal 13: RAN2 assumes Option 1-1 is not supported for LP-WUS in RRC_CONNECTED mode.

Enable and disable for LP-WUS monitoring

Proposal 14: In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

Proposal 15: LP-WUS configuration is configured in RRCResume/RRCReconfiguration message.

Activation and deactivation procedure for LP-WUS monitoring

Proposal 16: RAN2 assumes UE could monitor LP-WUS upon LP-WUS is enabled, no additional indication/condition are needed for activation/deactivation of LP-WUS functionality. The final decision is up to RAN1.

- References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo

- Chairman’s Notes of RAN1 #116bis.

- R2-2405702, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

- 3GPP TS 38.321: "NR Medium Access Control (MAC) protocol specification".

- Chairman’s Notes of RAN1 #117.

- TR 38.869 for LP-WUS WUR.

- Annex – Evaluation results

Table 1: Performance comparsion for different options for LP-WUS triggering PDCCH monitoring

| Option 1-1 | Option 1-2-2 | Option 1-3 |
| --- | --- | --- |
| For FTP and IM traffic and compared with existing power saving techniques: - PSG (power saving gain): Up to more than 30%; - UPT: less than 10% loss | For FTP and IM traffic and compared with existing power saving techniques: - When UE MR enters deep sleep during LP-WUS monitoring: - PSG: Up to more than 60%; - UPT: up to more than 10% gain for FTP traffic, up to more than 180% gain for IM traffic - When UE MR enters light sleep during LP-WUS monitoring: - PSG: lower or no UE power saving gain; - UPT gain: up to more than 50% gain For FTP traffic and compared with UE MR always-on PDCCH monitoring - PSG: Up to more than 40% - UPT: Similar UPT | For XR traffic and compared with existing power saving techniques: - PSG: up to more than 10% across different types of XR traffic and system load. - Capacity: The impact to the XR capacity is marginal when the UE MR enters the micro sleep state during LP-WUS monitoring. |
| Note1: the results refer to TR 38.869 conclusion part. Note 2: RAN1 has carried out system level evaluations for the UE power saving and UPT benefit of using LP-WUS/WUR for CONNETED mode UEs with different traffic types including XR, FTP and IM. Note 3: the above results of different LP-WUS monitoring schemes are provided by compared with the existing power saving techniques, including Rel-15 long/short C-DRX cycle, Rel-16 DCP, Rel-17 PDCCH skipping/SSSG switching and Rel-18 enhanced C-DRX for non-integer periodicities are used for in the evaluation comparison. |  |  |

Further details of simulations:

For option 1-1 (i.e. LP-WUS to replace DCP), the offset between LP-WUS monitoring occasion and the corresponding DRX onduration is 3ms and the window of LP-WUS monitoring is 1ms. Due to the short time offset, the MR has to ramp up from deep-sleep to light-sleep even before the LP-WUS monitoring in order to fulfill the 3ms wake-up latency if a positive LP-WUS is detected. However, if no positive LP-WUS is detected, MR can enter deep-sleep again but during this procedure, the MR sleep transition actually consumes additional power, as shown in the upper part of the figure below.

On the other hand, for LP-WUS to trigger MR PDCCH monitoring (i.e. option 1-2-2), MR does not have to have the above “sleep-state transition”, in other words, the MR is only wake-up from deep-sleep when triggered by positive LP-WUS detected, as shown in the lower part of the figure below.

SLS assumptions

| Parameter | Indoor Hotspot |
| --- | --- |
| Scenarios | Indoor Hotspot, 12 nodes in 50 m x 120 m |
| Channel model | InH |
| Carrier frequency | 4GHz |
| Bandwidth | 100 MHz, 1.72% Guard Band |
| Subcarrier spacing | 30 KHz |
| Frame structure | DDDSU (S: 10D:2G:2U) |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4), (dH,dV) = (0.5, 0.5)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,1/2,2,1,1;1,1/2), (dH, dV) = (0.5, N/A) λ |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| BS Power | 24 dBm per 20MHz |
| UE max Power | 23 dBm |
| UE Power | Max Tx power: 23 dBm, (P0 = -80, alpha = 0.8) |
| ISD | 20 m |
| BS height | 3 m |
| UE height | 1.5 m |
| Noise Figure | BS:5 dB, UE:9 dB |
| Max MCS | 256QAM |
| Device deployment | 100% indoor |
| Down-tilt | 90 degrees |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| Target BLER | 10% |
| UE speed | 3 km/h |

|  | FTP traffic |
| --- | --- |
| Model | FTP model 3 |
| Packet size | 0.5 Mbytes |
| Mean inter-arrival time | 200 ms |
| DRX setting | Period = 160 ms, Inactivity timer = 100 ms |