3GPP TSG-RAN WG2 Meeting #127		R2-2406430

Maastricht, Netherlands, 19th  – 23rd Aug. 2024

Source:	vivo

Title:	Discussion on inter-CU LTM

Agenda Item:	8.6.2

Document for:	Discussion and Decision

- Introduction

In the last meeting, the following agreements were made for inter-CU LTM:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

In this paper, we discuss the remaining open issues for inter-CU LTM during the entire LTM procedure. This includes the LTM preparation phase, the early sync phase, and the LTM execution phase. In the end, we had some high-level discussions on the scenarios of inter-CU LTM with DC.

- Discussion

  - LTM preparation phase

Reference configuration

In Rel-18, reference configuration is introduced to reduce the RRC signaling overhead for LTM and S-CPAC candidate cells configuration. For Rel-19 inter-CU LTM, we believe that the reference configuration mechanism can be reused. Since for S-CPAC, even if the candidate cells may belong to different SN, only one common reference configuration is used for all the candidate cells for simplicity. Besides, multiple reference configurations (e.g., per CU) may introduce additional signaling overhead for reference configuration which goes against the purpose of reducing signaling overhead. Therefore, we can still use one reference configuration for all the LTM candidate cells for inter-CU LTM.

Proposal 1: Reference configuration in Rel-18 is reused for inter-CU LTM, and only one reference configuration is provided for all the LTM inter-CU and intra-CU candidate cell configurations in one CG.

  - Early synchronization

UE-based TA measurement

In the last meeting, whether UE-based TA measurement can be supported for inter-CU LTM was discussed. Some companies believe that RAN4 has already defined the UE requirement for UE-based TA measurement and it applies to both intra-CU and inter-CU scenario. From our understanding, UE-based TA measurement is supported in synchronized cases in Rel-18 according to the reply LS by RAN4 [1]. Therefore, when the serving cell and candidate cells in Rel-19 from different CUs are synchronized,  UE-based TA measurement is supported obviously and the Rel-18 mechanism can be reused.

Observation 1: If the serving cell and candidate cells are from different CUs and are synchronized, UE-based TA measurement can be supported by reusing the Rel-18 mechanism.

Proposal 2: RAN2 confirms that UE-based TA measurement is supported in the synchronized inter-CU LTM scenario, and no specification impact is expected.

However, for Rel-19 inter-CU LTM, as the serving cell and the candidate cells may belong to different CUs and are more likely to be unsynchronized, it seems highly beneficial to support UE-based TA measurement for unsynchronized scenarios. The decision on whether UE-based TA measurement for unsynchronized cases can be supported and how to support it should be made by RAN1 and RAN4. Therefore, an LS should be sent to RAN1 and RAN4 to inquire about the feasibility and approach for supporting UE-based TA when the serving cell and the candidate cell are unsynchronized.

Proposal 3: RAN2 confirms that supporting UE-based TA measurement in the unsynchronized case is beneficial for inter-CU LTM from the RAN2 perspective.

Proposal 4: Send an LS to RAN1/4 to ask whether it is feasible to support UE-based TA measurement in unsynchronized cases.

  - LTM execution phase

L2 reset for mixture of intra-CU and inter-CU

In the last meeting, whether to introduce a new RRC configured Rel-19 ID to support the mixture of inter-CU and intra-CU LTM was discussed and postponed. In our opinion, a new set ID needs to be configured for each candidate cell and the serving cell to indicate whether UE should perform the inter-CU L2 reset operation, similar to the function of SecurityCellSetId for S-CPAC. Some companies propose waiting for the SA3 response for the security issue, as the UE may determine whether the current LTM is inter-CU or intra-CU based on whether the LTM Cell Switch Command MAC CE carriers the security key information (e.g., NCC) if the option 1 in [2] is agreed. However, there are cases for LTM without LTM cell switch command, including LTM fast recovery after RLF is declared or conditional LTM. Therefore, we still require an RRC preconfigured new set ID for the UE to differentiate between inter-CU and intra-CU LTM in these cases.

Proposal 5: Similar to S-CPAC, the network provides cell-specific set ID for each LTM candidate cell and the serving cell for UE to identify whether the LTM cell switch is intra-CU or inter-CU:

- If the new set IDs of the serving cell and the target cell have different values(i.e. inter-CU LTM), the UE performs MAC reset, RLC re-establishment, PDCP re-establishment, and security key update;

- Else(i.e. intra-CU LTM), the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

CFRA resource in LTM Cell Switch Command MAC CE

For RACH-based LTM, reserving CFRA resources for each UE will result in a huge waste of CFRA resources for the candidate cells. To resolve the issue in Rel-18, CFRA resources are supported to be indicated in the LTM cell switch command, which means the CFRA resource can be allocated when the S-DU determines to trigger LTM and prevent the waste of RACH resources.

To address the potential waste of CFRA resources caused by inter-CU LTM, we propose supporting the inclusion of CFRA resources in the LTM Cell Switch Command MAC CE for inter-CU LTM.

Proposal 6:  CFRA resources can be included in the LTM Cell Switch Command MAC CE for inter-CU LTM. Send an LS to inform RAN3.

  - Initial discussion on inter-CU LTM with DC

In the WID [3], the following DC scenarios are agreed to be supported as secondary priority:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3]

  - Prioritize the case when CU is acting as MN when DC is not configured

  - As secondary priority, support the case when NR-DC is configured and CU is acting as SN and MCG is unchanged

  - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released

Note: The case that LTM is configured in both MCG and SCG is excluded

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3]

  - Prioritize the case when CU is acting as MN when DC is not configured

  - As secondary priority, support the case when NR-DC is configured and CU is acting as SN and MCG is unchanged

  - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released

Note: The case that LTM is configured in both MCG and SCG is excluded

In the following, we have analyzed the specification impact of the two LTM DC scenarios mentioned in the WID, and we have also discussed the scenarios for the mixture of intra-CU LTM and inter-CU LTM in DC scenarios.

Scenario 1: CU is acting as SN and MCG is unchanged

From our perspective, almost all of the procedures for inter-CU LTM without DC can be reused. One crucial issue is how to support subsequent inter-SN LTM without MCG changed, which means the sk-Counter used for SN key updating should not be reconfigured after each time of inter-SN LTM. This issue is just the same with S-CPAC in Rel-19. Consequently, the solution implemented in S-CPAC for preconfiguring the sk-Counter list for each candidate SN can be leveraged for this purpose.

Proposal 7: For inter-CU LTM when CU is acting as SN and MCG is unchanged, the security update mechanism for SCPAC is reused, i.e. UE is preconfigured with one sk-Counter list for each candidate SN.

Scenario 2: CU is acting as MN and SCG is unchanged or SCG is released

- CU is acting as MN and SCG is released

For CU is acting as MN and SCG is released, there is no difference from inter-CU LTM without DC. The SCG configuration will be released after performing inter-CU MCG LTM, as per the SCG release indication in the MCG LTM candidate configuration.

- CU is acting as MN and SCG is unchanged

For CU is acting as MN and SCG is unchanged, as the SCG security key is derived from the MN security key, the SCG security key should be updated concurrently with the MN key update when the UE performs inter-CU MCG LTM.

Proposal 8: For inter-CU LTM when CU is acting as MN and SCG is unchanged, the SN key needs to be updated together with the MN key update.

Furthermore, we think RAN2 needs to clarify whether the SCG configuration is supported to be changed together with inter-MN LTM, such as modification of SCG radio bearers configuration. Our perspective is that the SCG configuration should not be supported to be changed together with inter-MN LTM. This is because it would require the MCG LTM reference configuration or the MCG LTM candidate configuration to carry the SCG configuration, which is very complex and does not align with the Rel-18 LTM RRC modelling.

Proposal 9: For inter-CU LTM when CU is acting as MN and SCG is unchanged, SCG configuration modification during the inter-MN LTM is not supported.

Scenario 3: Mixture of intra-CU and inter-CU LTM for MCG and SCG

In addition to the two DC scenarios mentioned in the WID, the following two scenarios for the mixture of intra-CU and inter-CU LTM can be supported naturally by configuring inter-CU and intra-CU LTM separately for the MCG and the SCG:

- Inter-CU MCG LTM and intra-CU SCG LTM configured together

- Intra-CU MCG LTM and inter-CU SCG LTM configured together

Given that intra-CU MCG LTM and intra-CU SCG LTM are supported in Rel-19 already, it will not introduce additional specification impact if the MCG is configured with inter-CU LTM and SCG is configured with intra-CU LTM, and vice versa.

Proposal 10: UE can be configured with mixture of intra-CU and inter-CU LTM for MCG and SCG. E.g., UE can be configured with intra-CU MCG LTM and inter-CU SCG LTM, or inter-CU MCG LTM and intra-CU SCG LTM together.

- Conclusion

In this paper, we discuss the inter-CU LTM remaining open issues and also the specification impact of inter-CU LTM in DC scenarios. We have the following observations and proposals:

LTM preparation phase

Proposal 1: Reference configuration in Rel-18 is reused for inter-CU LTM, and only one reference configuration is provided for all the LTM inter-CU and intra-CU candidate cell configurations in one CG.

Early synchronization

Observation 1: If the serving cell and candidate cells are from different CUs and are synchronized, UE-based TA measurement can be supported by reusing the Rel-18 mechanism.

Proposal 2: RAN2 confirms that UE-based TA measurement is supported in the synchronized inter-CU LTM scenario, and no specification impact is expected.

Proposal 3: RAN2 confirms that supporting UE-based TA measurement in the unsynchronized case is beneficial for inter-CU LTM from the RAN2 perspective.

Proposal 4: Send an LS to RAN1/4 to ask whether it is feasible to support UE-based TA measurement in unsynchronized cases.

LTM execution phase

Proposal 5: Similar to S-CPAC, the network provides cell-specific set ID for each LTM candidate cell and the serving cell for UE to identify whether the LTM cell switch is intra-CU or inter-CU:

- If the new set IDs of the serving cell and the target cell have different values(i.e. inter-CU LTM), the UE performs MAC reset, RLC re-establishment, PDCP re-establishment, and security key update;

- Else(i.e. intra-CU LTM), the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

Proposal 6:  CFRA resources can be included in the LTM Cell Switch Command MAC CE for inter-CU LTM. Send an LS to inform RAN3.

Initial discussion on inter-CU LTM with DC

Proposal 7: For inter-CU LTM when CU is acting as SN and MCG is unchanged, the security update mechanism for SCPAC is reused, i.e. UE is preconfigured with one sk-Counter list for each candidate SN.

Proposal 8: For inter-CU LTM when CU is acting as MN and SCG is unchanged, the SN key needs to be updated together with the MN key update.

Proposal 9: For inter-CU LTM when CU is acting as MN and SCG is unchanged, SCG configuration modification during the inter-MN LTM is not supported.

Proposal 10: UE can be configured with mixture of intra-CU and inter-CU LTM for MCG and SCG. E.g., UE can be configured with intra-CU MCG LTM and inter-CU SCG LTM, or inter-CU MCG LTM and intra-CU SCG LTM together.

- References

- R4-2314455, Reply LS beam application time and UE based TA measurement for LTM. Ericsson.

- R2-2403929, LS on security handling for inter-CU LTM in non-DC cases. RAN WG2.

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4. Apple Inc, China Telecom