3GPP TSG-RAN WG2 Meeting #127		R2-2406431

Maastricht, Netherlands, 19th – 23rd Aug. 2024

Source:	vivo

Title:		Discussion on LTM measurement event evaluation

Agenda Item:		8.6.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we will discuss the issues related to LTM measurement event evaluation, including:

- Whether to support event LTM1;

- Which Beam should be used for serving/candidate cell in event evaluation;

- Whether to support cell level measurement result;

- Event configuration signaling design.

- Discussion

  - Support of LTM1

In RAN2 #126 meeting, it was agreed for event LTM2/LTM3/LTM4/LTM5. For event LTM1 (Beam of serving cell becomes better than absolute threshold) it was FFS[1]:

- FFS on the need of Event LTM1.

We think LTM1, which is similar as LTM2 (only caring about beam of serving cell), is beneficial because it can give more information to NW so that the NW may adjust the beam-level measurement configuration (e.g. report configuration) based on that, e.g. NW can de-configure the measurement for candidate beams when the current beam is better than a threshold.

So, we think we can support LTM1 and anyway whether to configure LTM1 is based on NW implementation.

Observation 1: NW can have more information and may adjust the beam-level measurement configuration based on LTM1 and it is up to NW implementation whether to configure it or not.

Proposal 1: Event LTM1 (Beam of serving cell becomes better than absolute threshold) is supported for event triggered L1 measurement reporting.

  - Which Beam of serving/candidate cell for event evaluation

Another issue is the following FFS[1]:

- FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation.

For serving cell, the possible options are:

- Current beam (i.e. a beam corresponding to the indicated TCI state);

- Best beam.

RAN1 MIMO session discussed events related to the UE-initiated measurement reporting, and assumed current beam should be used for event evaluation. Meanwhile, the motivation to use ‘best beam’ of serving cell for event evaluation is also not clear as anyway the network can switch the UE to use the best beam as the current beam if the network wants to use the best beam for evaluation, thus using current beam is enough.

Observation 2: In RAN1 discussion about UE-initiated measurement reporting, current beam (i.e. a beam corresponding to the indicated TCI state) is used for serving cell beam quality evaluation.

Observation 3: It is up to network implementation to switch the UE to use the best beam as the current beam.

Proposal 2: Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

For neighboring cell, the possible options are:

- Best beam;

- Any beam.

The UE can be configured with multiple beams to be measured for one (or multiple, based on the signalling design) neighbouring cell. For example, if we configure Rel-19 beams which need to be measured based on the Rel-18 LTM-CSI-ResourceConfig as follows[2]:

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

It means e.g. the UE may be configured with:

- Beam 1, beam 2, beam 3, beam 4, beam 5…

- Candidate ID 1 for cell 1, candidate ID 1 for cell 1, candidate ID1 for cell 1, candidate ID 2 for cell 2, candidate ID 3 for cell 3…

So, it will result in the L1 measurement reporting latency if the UE waits until all beam 1/2/3 measurement is finished and uses the best beam among them for event evaluation regarding cell 1. And if it is allowed that multiple beams from multiple cells are associated to a same event, it is even more questionable why/how the UE uses the best beam to trigger the reporting for that event.

Observation 4: Use ‘best beam’ for event evaluation in L1 measurement reporting for neighbouring cell would cause delay for triggering because the UE needs to wait after finishing measurements for all beams of that cell to determine the best one.

On the other hand, using any beam of neighbouring cell which satisfies the condition of associated event for evaluation could ensure the L1 measurement to be timely reported. In the above example, it is totally fine that the UE triggers the L1 measurement reporting when beam 1 satisfies the condition and later to trigger the reporting for beam 3 as well if beam 3 also satisfies the condition. In this sense, we understand that ‘any beam’ is used for event evaluation in L1 measurement reporting for neighbouring cell.

Proposal 3: Any beam (i.e. any beam of neighbouring cell configured in L1 measurement resource configuration) is used for event evaluation in L1 measurement reporting for neighbouring cell.

  - Cell level measurement for event evaluation

First, we would like to clarify the concept of ‘cell level measurement’. In last meeting’s discussion, some companies mentioned that if we use ‘best beam’ of neighbouring cell to trigger the event evaluation, the UE somehow couldn’t consider it is a beam level evaluation because e.g. even best beam changes the UE may understand the event condition continuing to be met. However, we understand that when we discussed whether ‘cell level’ is supported, we were actually discussing to use measurement results of multiple beams to e.g. compare with some threshold in measurement event (like LTM4), i.e., the ‘cell level measurement’ should be interpreted as the consolidation of beam measurement results rather than the best beam of the cell.

Observation 5: The term ‘cell level measurement’ for L1 measurement reporting is equivalent to ‘consolidation of multiple beam measurement results’.

In our understanding, for L1 measurement reporting, beam level evaluation may be enough as anyway whether to trigger LTM cell switch is based on NW decision and the NW can have the whole picture for all beams. While the cell level measurement event may be needed only in some cases, e.g. in conditional LTM when the handover is triggered by UE itself and the defined event may need to consider measurement results for multiple beams to derive the cell quality.

Observation 6: Measurement event defined in beam level may be enough for L1 measurement reporting, but measurement event defined in cell level (i.e. to consider multiple beams) may also be needed in some cases e.g. conditional LTM.

Considering RAN WG work for conditional LTM will not start before RAN #105[3], we think for now we may just need to focus on the beam level measurement event.

Proposal 4: Postpone the discussion on cell level measurement for event evaluation to when discussion of conditional LTM starts.

  - Event configuration signaling design

In RAN2 #125bis meeting, it was agreed:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

To associate L1 measurement resource configuration with L1 LTM measurement event configuration, we understand the simplest and most logical way is to reuse the legacy Rel-18 method, i.e.

- Include the L1 LTM measurement event configuration in LTM-CSI-ReportConfig;

- Associate L1 measurement resource configuration with L1 LTM measurement event configuration by using the ‘ResourceConfigId’.

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

periodic-r18                                       SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

p0alpha                                            P0-PUSCH-AlphaSetId

},

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

},

...

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...

}

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-CSI-SSB-ResourceSet-r18         LTM-CSI-SSB-ResourceSet-r18,

...

}

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

By this way, the event-triggered measurement report configuration can be configured separately in a report configuration, which can be along with legacy periodic reporting by adding one new report type ‘event-triggered’.

Proposal 5: L1 LTM measurement event configuration is configured in LTM-CSI-ReportConfig, by adding one new report type ‘event-triggered’ with event parameters (e.g. TTT).

Moreover, as we agreed some beam level parameters (e.g. beam level offset) for event configuration, those parameters can be configured in L1 measurement resource configuration, e.g. LTM-CSI-SSB-ResourceSet, as the similar way in Rel-18.

Proposal 6: Beam level parameters (e.g. beam level offset) is configured in L1 measurement resource configuration.

With Proposal 5 and Proposal 6, the Rel-19 event configuration signalling (with reusing R18 framework) can be illustrated as follows:

- Conclusion

In this contribution, we discussed the issues related to LTM measurement event evaluation. We have the following Observations and Proposals:

Observation 1: NW can have more information and may adjust the beam-level measurement configuration based on LTM1 and it is up to NW implementation whether to configure it or not.

Observation 2: In RAN1 discussion about UE-initiated measurement reporting, current beam (i.e. a beam corresponding to the indicated TCI state) is used for serving cell beam quality evaluation.

Observation 3: It is up to network implementation to switch the UE to use the best beam as the current beam.

Observation 4: Use ‘best beam’ for event evaluation in L1 measurement reporting for neighbouring cell would cause delay for triggering because the UE needs to wait after finishing measurements for all beams of that cell to determine the best one.

Observation 5: The term ‘cell level measurement’ for L1 measurement reporting is equivalent to ‘consolidation of multiple beam measurement results’.

Observation 6: Measurement event defined in beam level may be enough for L1 measurement reporting, but measurement event defined in cell level (i.e. to consider multiple beams) may also be needed in some cases e.g. conditional LTM.

Proposal 1: Event LTM1 (Beam of serving cell becomes better than absolute threshold) is supported for event triggered L1 measurement reporting.

Proposal 2: Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

Proposal 3: Any beam (i.e. any beam of neighbouring cell configured in L1 measurement resource configuration) is used for event evaluation in L1 measurement reporting for neighbouring cell.

Proposal 4: Postpone the discussion on cell level measurement for event evaluation to when discussion of conditional LTM starts.

Proposal 5: L1 LTM measurement event configuration is configured in LTM-CSI-ReportConfig, by adding one new report type ‘event-triggered’ with event parameters (e.g. TTT).

Proposal 6: Beam level parameters (e.g. beam level offset) is configured in L1 measurement resource configuration.

- References

- RAN2 #126 chairman notes;

- 3GPP TS 38.331 V18.2.0 (2024-06);

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN meeting #103, Maastricht, Netherlands, March 18-21, 2024.