3GPP TSG-RAN WG2 Meeting #127		R2-2406433

Maastricht, Netherlands, 19th – 23rd Aug. 2024

Source:	vivo

Title:	Discussion on LS from SA2 on FS_XRM Ph2

Agenda Item:	8.7.1.1

Document for:	Discussion and Decision

- Introduction

In RAN2#127 meeting, an LS from SA2 is received with the below questions on XRM_Ph2 [1]:

SA2 has started to evaluate solutions for key issues for the study of XR and Media Services Ph2 (TR 23.700-70). SA2 thanks the LS replies for XRM Ph2 from RAN2, RAN3 and SA4 and SA2 would like to further coordinate on the following aspects: - Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension? - Question2 [for SA4 and RAN2]: There is some discussion about time to next burst. - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6? - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server? - Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

In this contribution, we will discuss the issues in the LS and provide the draft response from RAN2 point of view.

- Discussion

  - Indicating periodicity via in-band signaling

- Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

Periodicity defined in SA2 is evaluated by UPF based on the arrival time for each burst. Currently, it was indicated in CP from UPF through SMF to RAN node. In case the periodicity is changed dynamically, it could be also indicated in CP. Some companies may argue that the UP approach, i.e. from UPF to RAN, is faster than CP approach. But our understanding is: if the change frequency for periodicity is not so high, there is no difference between CP and UP approach; if the change frequency for periodicity is very high, it is actually not a periodic service, which is related to the question below. Thus, we think indicating periodicity via in-band signaling for dynamic changes of the periodicity is not needed from RAN2 point of view.

Proposal 1: RAN2 thinks indicating periodicity via in-band signaling for dynamic changes of the periodicity is not needed.

  - Time to next burst

- Question2 [for SA4 and RAN2]: There is some discussion about time to next burst. - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6? - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

According to the discussion in SA2, this question is for DL. In our understanding, DL is scheduled by DG, while PDCCH could be in the same slot of PDSCH. DL scheduling is always fast enough. We don’t see any use case of  the time to next burst for gNB scheduling. Besides, the possible jitter between the NG-RAN and the Application Server is not predictable, which means that the exact arrival time for the next burst at RAN node is not accuracy enough.

Actually, this issue was discussed in RAN2#122 meeting in Rel-18. The conclusion in RAN2 is: RAN can benefit from getting XR related information about End of Burst (1 bit). RAN2 have not identified any other dynamic information that is useful. It was sent to SA2 and SA4 in the LS [2].  So far, there is no new scenario that can be seen to change the above conclusion.

With this, we think the time to next burst is not useful for RAN resource scheduling.

Proposal 2: RAN2 thinks the time to next burst is not useful for RAN resource scheduling.

  - Reply LS to SA2

Based on the above discussion, a draft reply LS is provided in the Annex.

Proposal 3: RAN2 to discuss the draft reply LS in Annex.

- Conclusion

In this contribution, we discuss the issues in the LS from SA2 [1]. Based on the discussion, we have the following proposals:

Proposal 1: RAN2 thinks indicating periodicity via in-band signaling for dynamic changes of the periodicity is not needed.

Proposal 2: RAN2 thinks the time to next burst is not useful for RAN resource scheduling.

Proposal 3: RAN2 to discuss the draft reply LS in Annex.

- References

- R2-2406241/S2-2407351, LS on FS_XRM Ph2, SA2/vivo

- R2-2306572, LS response to N6 PDU Set Identification, RAN2/Intel

- Annex - draft reply LS

Title:	[Draft] reply LS on FS_XRM Ph2

Response to:	R2-2406241/S2-2407351

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	vivo [to be RAN2]

To:	SA2

Cc: 	RAN3, SA4

# Contact Person:

# Name:	              Chenli

E-mail Address:	 Chenli5g@vivo.com

# 1. Overall Description:

RAN2 would like to thank SA2 for the LS on FS_XRM Ph2 in [R2-2406241/S2-2407351].

RAN2 has discussed the questions in the LS, and concluded the following responses [To be updated based on the discussion conclusion]:

- Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

RAN2 thinks indicating periodicity via in-band signaling for dynamic changes of the periodicity is not needed.

- Question2 [for SA4 and RAN2]: There is some discussion about time to next burst. - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6? - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

RAN2 thinks the time to next burst is not useful for RAN resource scheduling.

# 2. Actions:

# To SA WG 2:

# RAN2 kindly request SA2 to take the above information into account during the future work, and provide feedback, if any.

# 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #127bis	 14th Oct. – 18th Oct. 2024		Hefei, China

TSG RAN WG2 Meeting #128		18th Nov. – 22nd Nov. 2024		Orlando, USA