3GPP TSG-RAN WG2 Meeting #127		R2-2406434

Maastricht, Netherlands, 19th – 23rd Aug. 2024

Source:	vivo

Title:	Discussion on LS from RAN3 on UL PSI based PDU discarding in NR-DC

Agenda Item:	8.7.1.1

Document for:	Discussion and Decision

- Introduction

In the LS from RAN3 to RAN2 [1], some questions regarding the UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE are raised by RAN3. In the following, our views are shared based on the related technical discussions and the answer to the question are provided in the draft reply LS from RAN2.

- Discussion

In the LS from RAN3, following statements and question were included in [1]:

RAN3 is currently discussing XR DC signaling enhancements for various bearer types. In case of NR-DC split bearer, MN DU and SN DU congestion is local to each node. Based on congestion situation in MN DU and/or SN DU, each of the nodes independently send PSI-Based SDU Discard Activation/Deactivation MAC CE to UE for activation/deactivation of UL PSI based PDU set discarding of the split bearer. when UE receives MAC CE from both nodes or from a single node, it is not clear how UE will activate/deactivate UL PSI based PDU set discarding when split bearer is configured. Thus, RAN3 kindly asks RAN2 to clarify the UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE.

According to discussion in RAN3#124 and RAN3#123bis, it was agreed that the PSI-Based SDU Discard Activation/Deactivation MAC CE would be sent independently without the co-ordination between SN and MN. As a result, it is inevitable that when UE receives a PSI-Based SDU Discard Activation MAC CE from one Node while there is no PSI-Based SDU Discard Activation MAC CE from the other Node or receive a PSI-Based SDU Discard Deactivation MAC CE from the other Node.

According to TS38.323 and TS38.331 as below, PSI-Based SDU discard is configured per-PDCP entity. The corresponding discard timer would be started if needed once the data received by PDCP from upper layer. This operation is done before the PDCP PDU construction and delivery to different nodes in case of split bearer configured. In other words, the PSI-Based SDU Discard operation can not be performed separately per MN/SN.

TS38.331 subclause 6.3.2 - PDCP-Config:

The value of this timer for a PDCP entity is always configured shorter than discardTimer, discardTimerExt or discardTimerExt2, whichever is used for the PDCP entity. The presence of this field indicates that PSI-based SDU discard is configured for a DRB.

TS38.323 5.2.1: At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall: - if discardTimerForLowImportance is configured and PSI based SDU discard is activated, and the PDCP SDU belongs to a low importance PDU Set: - start the discardTimerForLowImportance associated with this PDCP SDU; - else: - start the discardTimer associated with this PDCP SDU (if configured).

Observation: There is only one consistent PSI-Based discard operation for all received low importance data per PDCP entity in case of split bearer is configured.

Based on previous observation, there are three options on UE behavior for PSI-Based discard operation in case of split bearer.

Option 1: discardTimerForLowImportance would be started only when PSI-Based SDU Discard Activation MAC CE received from both MN and SN in case of split bearer.

For this option, it is assumed that if congestion happened in one node, it will automatically reduce the UL grant from this node, while UE UL data could also be scheduled normally via the other non-congested node. And the non-congested node can naturally help to alleviate the congestion impact to such UE UL data transmission. As a result, there is no need to active the PSI-Based SDU discard when one radio congested while the other is not. If the radios from both nodes of split bearer are congested, the PSI-Based SDU discard should be activated so that the low importance data will not further aggravate the radio congestion same as non-split bearer case.

The corresponding UE behavior could be as the following:

- If UE receives a PSI-Based SDU Discard Activation MAC CE from one node, it should wait the PSI-Based SDU Discard Activation MAC CE from the other node to start the timer discardTimerForLowImportance for low importance data from upper layer.

- If UE receives a PSI-Based SDU Discard Deactivation MAC CE from one node, it should not start any discardTimerForLowImportance no matter whether the PSI-Based SDU Discard from the other node is activated or deactivated.

Option 2: discardTimerForLowImportance would be started when PSI-Based SDU Discard Activation MAC CE received from either MN or SN in case of split bearer.

For option 2, once UE receive the PSI-Based SDU Discard Activation MAC CE from either MN or SN, it will follow the behavior as non-split bearer case. In such case, the congested radio could be alleviated to the most extent.

The corresponding UE behavior could be as the following:

- If UE receives a PSI-Based SDU Discard Activation MAC CE from one node (either MN or SN), it should start the timer discardTimerForLowImportance for low importance data from upper layer no matter whether the PSI-Based SDU Discard from the other node is activate or deactivate.

- If UE receives a PSI-Based SDU Discard Deactivation MAC CE from one node, it should check whether the PSI-Based SDU Discard from the other node is also deactivated. If so, not to start any discardTimerForLowImportance for low importance data.

Option 3: discardTimerForLowImportance would be started when PSI-Based SDU Discard Activation MAC CE received from the node configured by NW via RRC or MAC signaling (either MN or SN) in case of split bearer.

For this option, UE behavior could be the same as non-split bearer case which is simple for UE implementation. And network could configure an anchor node, e.g. either PDCP located node or the node with more radio resources estimated. This option may complex the network implementation and a new IE needs to be added for every time UE configured/reconfigured/SN change/MN change with PSI-Based SDU discard and split bearer configured simultaneously.

The corresponding UE behavior could be as the following:

- If UE receives a PSI-Based SDU Discard Activation MAC CE from the configured node, it should start the timer discardTimerForLowImportance for low importance data from upper layer

- If UE receives a PSI-Based SDU Discard Deactivation MAC CE from the configured node, it should not start any discardTimerForLowImportance for low importance data from upper layer

There is also a simplified option3 that UE always follow the PSI-Based SDU Discard Activation MAC CE from MN. In such option, when MN is congested and SN is not, UE would enable the PSI-Based SDU Discard in PDCP which has similar effect as option2. While when MN is not congested and SN is congested, UE would not enable the PSI-Based SDU Discard in PDCP which has similar effect as option1.

Option 3a: discardTimerForLowImportance would be started when PSI-Based SDU Discard Activation MAC CE received from the MN node in case of split bearer.

From the analysis above, also considering the motivation of introducing of PSI-Based SDU discard and split bearer, it is foreseen that in split bearer case, different congestion status in different node would not happen frequently. Thus, option 1, option 2 and option 3a could be considered, while since option 3 brings extra signaling and complexity, it could be de-prioritized.

Proposal 1: RAN2 to discuss the following options on UE behavior for PSI-Based SDU discard in case of split bearer and choose one of them.

- Option 1: discardTimerForLowImportance would be started only when PSI-Based SDU Discard Activation MAC CE received from both MN and SN in case of split bearer.

- Option 2: discardTimerForLowImportance would be started when PSI-Based SDU Discard Activation MAC CE received from any of MN or SN in case of split bearer.

- Option 3: discardTimerForLowImportance would be started when PSI-Based SDU Discard Activation MAC CE received from the node configured by NW via RRC or MAC signaling (either MN or SN) in case of split bearer.

- Option 3a: discardTimerForLowImportance would be started when PSI-Based SDU Discard Activation MAC CE received from the MN node in case of split bearer.

Based on the above discussion, a draft reply LS is provided in the Annex.

Proposal 2: RAN2 to discuss the draft reply LS in Annex.

- Conclusion

In this contribution, we discuss the issues in the LS from RAN3 [1]. Based on the discussion, we have the following observation and proposals:

Proposal 1: RAN2 to discuss the following options on UE behavior for PSI-Based SDU discard in case of split bearer and choose one of them.

- Option 1: discardTimerForLowImportance would be started only when PSI-Based SDU Discard Activation MAC CE received from both MN and SN in case of split bearer.

- Option 2: discardTimerForLowImportance would be started when PSI-Based SDU Discard Activation MAC CE received from any of MN or SN in case of split bearer.

- Option 3: discardTimerForLowImportance would be started when PSI-Based SDU Discard Activation MAC CE received from the node configured by NW via RRC or MAC signaling (either MN or SN) in case of split bearer.

- Option 3a: discardTimerForLowImportance would be started when PSI-Based SDU Discard Activation MAC CE received from the MN node in case of split bearer.

Proposal 2: RAN2 to discuss the draft reply LS in Annex.

- References

- R2-2406221/R3-243957, LS on UL PSI based PDU discarding in NR-DC, RAN3/Qualcomm

- Annex - draft reply LS

Title:	[Draft] reply LS on UL PSI based PDU discarding in NR-DC

Response to:	R2-2406221/R3-243957

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	vivo [to be RAN2]

To:	RAN3

Cc: 	-

# Contact Person:

# Name:	                Chenli

E-mail Address:	 Chenli5g@vivo.com

# 1. Overall Description:

RAN2 thanks RAN3 for the LS on UL PSI based PDU discarding in NR-DC in R2-2406221/R3-243957. RAN2 has discussed the questions in the LS, and concluded the following responses [To be updated based on the discussion conclusion]:

Questions for RAN2: RAN3 kindly asks RAN2 to clarify the UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE.

RAN2 response:

Considering if only one radio from the connected node is congested, UE could still be scheduled on the other non-congested radio for UL data transmission. Proper network implementation could grantee the data transmission, so it was agreed that UE will perform PSI-Based SDU discard operation only when the Activation MAC CE received from both MN and SN in case of split bearer configuration.

# 2. Actions:

# To RAN WG 3:

# RAN2 kindly request RAN3 to take the above information into account during the following work, and provide feedback, if any.

# 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #127bis	14th Oct. – 18th Oct. 2024		Hefei, China

TSG RAN WG2 Meeting #128		18th Nov. – 22nd Nov. 2024		Orlando, USA