3GPP TSG-RAN WG2 Meeting #127		R2-2406436

Maastricht, Netherlands, 19th – 23rd Aug. 2024

Source:	vivo

Title:	Discussion on scheduling enhancements for XR

Agenda Item:	8.7.4

Document for:	Discussion and Decision

- Introduction

In this contribution, the remaining issues regarding LCP enhancements and DSR enhancements are discussed based on the conclusions from previous meetings. In addition, some additional enhancements in relation to BSR/DSR and PDB/PSDB managements to improve the XR capacity and the XR user experience are also investigated.

- Discussion

In the following, the remaining issues regarding BSR/DSR and the additional issues are discussed respectively.

  - LCP enhancements

In RAN2#126[1], there were the following agreements regarding LCP enhancements:

Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. The solution should consider impact on UE complexity (as already indicated in SI objective description) For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH We try to avoid RAN1 impacts. RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial The solutions should not disallow non-delay critical data from using an UL grant. LCP prioritization within a logical channel will not be considered in RAN2 discussions FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

In the following, the open issues for LCP enhancements are discussed respectively for LCH priority enhancements, LCP restriction enhancements and resource allocation enhancements considering PDU set importance.

    - LCP priority enhancement

In RAN2#126 meeting, there is an FFS on whether the additional priority should be applied to only for the delay-critical data of the LCH or the whole LCH:

FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH

- If the additional priority is only applied to the delay-critical data, one LCH may have two applicable priorities during one MAC PDU construction. Upon one MAC PDU construction, i.e. the additional priority of an LCH is determined for the delay-critical data of this LCH and the delay-critical data of this LCH is included in the MAC PDU according to this additional priority. If the MAC PDU has capacity to accommodate more data after including all delay-critical data of the LCH into the MAC PDU, the original priority has to be applied for non-delay-critical data for the LCH. This means that the UE has to evaluate the prioritization between LCHs twice in correspondence to one MAC PDU construction, which causes considerable complexity and change at the UE side.

- If the additional priority is applied to the whole LCH, for an LCH configured with additional priority, whether additional priority or original priority should be applied needs to be determined per MAC PDU basis, i.e. only single priority (either original priority or the additional priority) can be applied for one LCH during one MAC PDU construction. Once the additional priority is determined for one LCH with delay-critical data upon one MAC PDU construction, the UE includes the delay-critical data and non-delay-critical data of this LCH into the MAC PDU according to the additional priority. In such way, the UE only evaluates the prioritization between LCHs once for one MAC PDU construction.

- Applying additional priority only for delay-critical data may result in the UE to evaluate the priority among LCHs twice for one MAC PDU construction.

With the above analysis, the priority should be applied to the whole LCH.

- Once the additional priority is determined for an LCH, the additional priority should be applied to all data of the LCH, including both delay critical and non-delay critical data.

- If the priority is determined for a LCH upon a MAC PDU construction, the resulted priority (either original priority or additional one) should be applied for the whole MAC PDU construction without change.

For a first LCH with delay-critical data, the gNB may issue UL grants to speed up the delay-critical data transmissions. After receiving the UL grants, the UE may prioritize the resource allocation for this LCH according to the additional priority of this LCH. However, when the UE has a large amount of delay-critical data for the first LCH, after the UE has allocated radio resource up to Bj for the first LCH, there are still remaining delay-critical data for the first LCH. According to existing multiplexing procedure, the UE will allocate the remaining resource for the second LCH of lower priority. After radio resource allocation for the second LCH, there may be no remaining capacity or the remaining capacity of the MAC PDU can be not enough to contain all the remaining delay-critical data of the first LCH.

In another instance, when there can be certain scheduling restriction, e.g. due to measurement gap (MG) overlapping with the PDB/PSDB window of a frame burst, the actual available time period(s) for transmission of the frame burst after DSR triggering could be further reduced due to the overlapping between the MG and the remaining time period. In this case, even when the UE get enough UL grants providing enough capacity for the delay-critical data of a LCH, the UE may not be able to include all delay-critical data of this LCH into the corresponding MAC PDUs due to the restriction of the Bj of this LCH and some capacity of the UL grants intending for the delay-critical data of this LCH are allocated to transmit the non-delay-critical data of other LCHs instead according to the existing multiplexing mechanism.

- For MAC PDU construction, the LCH with delay-critical data may not be allocated enough radio resource due to its buckets (Bj) exhausted.

To speed up the delay-critical data transmission, the UE should be allowed to allocate radio resources exceeding the rate limit (Bj limit).

- In addition to LCH priority adaptation, RAN2 should also consider the option to raise the rate limit (e.g. Bj) of the LCH with delay-critical data.

There is still one further open issue on whether to reuse the existing remaining time threshold or introduce a new threshold to determine the delay-critical data of an LCH. As far as our understanding, that the UE request UL grant via DSR is the last method to speed up the delay-critical data transmission. If the UE has a large amount of delay critical data, after DSR reception, the gNB may not be able to issue enough UL grants for the UE so that the UE can transmit all delay-critical data.

To reduce the risk that there is a large amount of delay-critical data (in accordance with the remaining time threshold of DSR) when DSR is triggered, a separate remaining time threshold should be introduced to determine the delay-critical data for LCP priority enhancements. With a separate remaining time threshold, the gNB the can configure a larger remaining time threshold for delay-critical data determination for LCH priority enhancement, which enable the UE to speed up the delay-critical data transmission earlier than DSR triggering.

- If a separate remaining time threshold for LCH priority adaptation larger than the existing remaining time threshold is configured, the risk of the occurrence of large amount of delay-critical data upon DSR triggering can be reduced.

- Introduce a separate remaining time threshold for delay critical data for LCP enhancement.

    - LCP restriction enhancement

In RAN2#126 meeting, LCP restriction enhancements were discussed and it was concluded that enhancement requiring RAN1 work should not be considered and the potential advantage for the LCP restriction enhancement should be justified. In the following, the potential benefits of LCP restriction relaxation are discussed with examples and the potential enhancements are provided.

For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial

In the current MAC specification [3], there are the following LCP restriction parameters which could restrict the LCH selection for an UL grant:

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel: - allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission; - maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission; - configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission; - allowedServingCells which sets the allowed cell(s) for transmission; - allowedCG-List which sets the allowed configured grant(s) for transmission; - allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission; - allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

Typically, the gNB configures these parameters to ensure that the data of an LCH can be mapped to proper UL grants which can ensure the QoS requirements of the LCH, including the data rate, the reliability (packet loss rate) and the latency bound.  In ideal case and with smart gNB implementation, the QoS requirements of an LCH can be fulfilled. However, in practice, one can not ensure that the preconfigured LCP restriction parameters are always optimal in all conditions considering the following factors:

- The radio conditions of a XR user can vary

- The available radio resource for XR users vary in relation to the system load variation

- The frame burst size may vary in a large range depending on the video compression level and different picture may have different compression level depending on the information included in the frames.

Considering these variations, even a smart gNB cannot guarantee that the preconfigured LCP restriction parameters are always optimal in case of the above variations. For instance, there could be a situation that the gNB cannot allocate enough optimal grants (i.e. the UL grants that fulfil all LCP restrictions for frame burst transmission, e.g. allowedSCS-List, maxPUSCH-Duration, allowedServingCells, etc.) for the delay-critical data or the remaining time for delay-critical data is not enough for the UE to request proper UL grants. In this case, if the UE is not allowed to use the available sub-optimal grants (i.e. the UL grants that does not fulfil some LCP restriction parameters for LCH of the frame burst), there is high risk of discarding delay-critical data due to remaining time exhausted.

In one more specific example, a XR UE has been configured with two carriers: Component Carrier 1 (CC 1) and CC 2. CC 1’s SCS is in the allowedSCS-List of LCH for XR frame burst transmission and the UL grants over CC1 are optimal grants for XR frame burst transmission, while CC 2’s SCS is not in the allowedSCS-List, i.e. the UL grants in CC1 is optimal UL grants for XR frame burst while the UL grants over CC2 is sub-optimal for XF frame burst. However, when there are multiple XR UEs having delay-critical data, and the gNB cannot provide enough UL grants over CC1 for all these XR UEs for delay-critical data transmission due to system congestion/overloading. In the meanwhile, there are still remaining resources in CC2. In such situation, if these XR UEs without enough optimal UL grants over CC1 are not allowed to use the sub-optimal UL grants over CC2 for delay-critical data transmission, the radio resources of CC2 would be wasted and at least some delay-critical data for these XR UEs would never get chance to be transmitted, i.e., have to be discarded. While if a XR UE is allowed to transmit its delay critical data using the sub-optimal UL grants, there is still considerable probability that these delay-critical data can be correctly received by the gNB in time, with which the user performance can obviously outperform the option to discard the delay-critical data without trying to use sub-optimal UL grants for delay-critical UL data transmission. Up to the implementation of gNB, some actions (e.g. transmit power boost, configure relatively lower MCS) could be taken to increase the probability of correct UL data transmission in one shot using sub-optimal grants.

- It can happen that there are no enough grants fulfilling all LCP restrictions (i.e. optimal grants) for the frame burst TX while there are still UL grants not fulfilling some LCP restrictions (i.e. sub-optimal grants) available.

- When there are no enough optimal grants while the UE is not allowed to use sub-optimal grants for XR frame burst TX, some delay-critical data of the frame burst has to be discarded and the remaining sub-optimal grants have to be wasted.

In order to improve both the system capacity and the XR user experience, LCP restriction relaxation should be supported to allow the UE to use sub-optimal UL grants according to preconfigurations:

- LCP restriction (e.g. allowedSCS-List, allowedServingCells, etc.) relaxation for an LCH according to the pre-configuration should be supported for delay-critical data transmission.

Similar to LCH priority enhancements, once LCP restriction is determined for one LCH, the determined LCP restriction relaxation can be applied for the whole LCH instead of the delay-critical data only, to avoid the complexity of using two sets of different LCP restriction parameters for an LCH in correspondence to one MAC PDU construction.

- Once LCP restriction relaxation is determined for an LCH, the resulted LCP restriction relaxation should be applied to the whole LCH, including both delay critical and non-delay critical data.

For the similar reason, the LCP restriction relaxation for one LCH can be determined per MAC PDU basis. When the UL grant is available, the UE determines whether LCP restriction relaxation is needed based on the pre-configurations. Once the LCP restriction parameters (relaxed ones or non-relaxed ones) are determined, these LCP restriction parameters should be used during the whole MAC PDU construction.

- If the LCP restriction is relaxed for an LCH upon a MAC PDU construction, the relaxed LCP restriction of this LCH should be applied for the whole MAC PDU construction without change.

    - Resource allocation considering PDU set Importance

According to the existing resource allocation procedure, the data that arrives the UE buffer earlier will be transmitted first when there is UL grant available, i.e. the PSI difference between different packets in one LCH queue is not considered in the resource allocation during the MAC PDU construction in the UE side. A UE may have both delay-critical data of PSI low importance and delay-critical data of PSI high importance. If the delay-critical data of PSI low importance arrives the UE buffer earlier than the delay-critical UL data, the delay-critical data of PSI low importance will be transmitted first. In such situation, if there are no enough UL grants to transmit all delay-critical data of the LCH, the discard risk of delay-critical data of high importance is higher than the delay-critical data of low importance. To relieve such situation, one can consider to allow the UE to prioritize the resource allocation for the delay-critical data of PSI high importance. If there is no enough radio resource for a frame burst at the end, the data of PSI with high importance can suffer lower packet loss risk than the data of PSI with low importance.

The similar solution can be also applied for non-delay-critical data with different PSI importance.

- RAN2 to study the resource allocation enhancements considering the PSI importance, for both delay-critical data and non-delay-critical data.

  - DSR enhancement

In RAN2#126, there were the following agreements regarding DSR enhancements:

Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. FFS whether DSR triggering is impacted FFS whether PDU set importance needs to be included

In RAN2-125bis[2], there are the following open issue which was not treated yet:

RAN2 will study enhancing existing DSR with additional information, e.g. multiple pairs of remaining time/buffer information, importance. FFS whether this only includes more information on delay-critical data or also information about non-delay critical data.

In the previous meetings, it was agreed that multiple pairs of remaining time/buffer information can be supported. However, there is still the open issue on whether the volume of non-delay-critical data should be also reported. As far as our understanding, reporting both non-critical data and critical-data can help the gNB to be aware of the total buffer status of a UE, which facilitate the gNB to consider the delay-critical data and non-delay-critical data jointly upon the resource allocation for this UE. For instance, if there is only a small amount of delay-critical data and non-delay-critical data, the gNB can schedule one or two grants to empty the UE buffer and allows the UE to go sleep before the next frame burst if necessary. In this case, it can reduce the iteration of UL grants and the corresponding HARQ feedbacks. If there is a large amount of delay-critical data and non-delay-critical data, the gNB can allocate sufficient UL grants for delay-critical data first to avoid packet discard and consider the balance between spectrum efficiency and the latency bound guarantee for non-delay-critical data. In this case, it can both the spectrum efficiency and the user experience.

Based on the above discussion, we think reporting non-critical data together with delay-critical data is meaningful for both user experience improvement and system capacity improvement. Considering the time limit of SI phase, we propose to keep this issue to WI phase. For example, to minimize the standardization effort, one unified DSR report can be considered to include both non-delay-critical data and delay-critical data. One possible approach is that the non-delay-critical data report can be considered as one part of the DSR with multiple pairs of remaining time/buffer information. In such way, only single new DSR format is needed.

- Whether/how to include non-delay-critical data information into DSR is left to discussion in WI phase.

For the option “multiple remaining time/buffer information”, we don’t see the need to introduce multiple delay thresholds for DSR triggering and reporting considering the complexity for both standardization and implementation. Furthermore, as a XR service has tight delay budget, it is less likely that the short remaining PDB/PSDB can allow multiple iterations of DSR reporting and delay-critical data scheduling in relation to one UL frame burst.

- The existing remaining time threshold is reused for triggering DSR with multiple pairs of remaining time/buffer information (i.e. no additional threshold for DSR triggering).

  - BSR enhancement

When the end packet of a frame burst is received by the UE from its application layer, there will be no further new data arrival for the frame burst. If a BSR in relation to the end packet arrival of this frame burst is reported to gNB, the gNB can know exactly how many resources are needed to empty the UE buffer in relation to the frame burst. This will facilitate the gNB to exactly allocate enough radio resources for the UE to finish the transmission of the frame burst without resource overshooting.

- RAN2 to consider new BSR triggering, e.g. upon the end packet arrival of a frame burst, as a candidate solution for scheduling enhancements.

  - PDB/PSDB adjustment

As discussed before, the overlapping between MG and the PDB/PSDB window of a frame burst could happen. Assuming that the MG duration is 6 ms, the PDB/PSDB is 10 ms, and the whole MG locates within the PDB/PSDB window, the actual available time within the PDB/PSDB window for DL/UL scheduling is 4 ms. It is great challenge for the gNB to schedule the DL/UL transmissions to successfully transmit the whole DL/UL frame burst, as 1) the gNB may not be able to allocate enough radio resource & 2) there is no enough time for the gNB to schedule the HARQ retransmissions if the initial HARQ transmissions fail.

- When the PDB/PDSB window of a frame burst is partly occupied by a measurement gap, the gNB may not be able to allocate enough radio resource for HARQ TX/ReTX within the PDB/PDSB window of the frame burst.

As discussed above, some XR traffic has very tight PDB/PSDB requirements, the gNB has to schedule the UL transmission to empty UE buffer in a very short period of PDB/PSDB. Due to the interference variation/ channel fading or overloading/congestion, some packet(s) of a frame burst may not be able to be successfully transmitted before the corresponding PDB/PSDB expiration.

- For XR traffic with tight PDB/PSDB requirements, some packet(s) of a frame burst may not be able to be transmitted before the corresponding PDB/PSDB expiration.

According to the current PDCP protocol, if some packet(s) of the frame burst cannot be transmitted upon the expiration of the discardTimer, these packet(s) will be discarded by the transmitter. However, for real-time XR services, usually GoP picture codec is applied and there is strong encoding/decoding dependency between near frames in order to achieve acceptable compression level of the frame bursts:

- P-frame burst is encoded in relation to the preceding I-frame burst and possibly some preceding P-frame bursts after the preceding I-frame.

- The B-frame burst in relation to neighboring P-frame bursts and the preceding I-frame burst.

Due to this inter-frame dependency, the discard of the outdated packets of a frame burst may have the following negative impacts:

- The packet discard of an I-frame burst degrades the video quality of all subsequent P/B frames until the next I-frame;

- The packet discard of a P-frame burst degrades the video quality of the neighboring B frames and the followed P/B-frames until the next I-frame.

During the discussion in Rel-18 SI, SA4 also confirms that the outdated packets may be still useful for application layer. In the above cases, if the outdated packets are not discarded but transmitted to the receiver, the receiver can still use the packets to help the decoding of the subsequent I/P/B frames, which avoid the video quality degradation propagation due to packet loss of a frame burst.

- If the outdated packets of a frame burst are not discarded but transmitted to the receiver, the receiver can recover the outdated frame for future frame burst decoding, which avoids the video quality degradation propagation until the next I-frame.

- RAN2 to study whether to transmit the outdated packet(s) to the receiver instead of simply discarding the outdated packet(s).

Alternatively, extending the PDB/PSDB period can be considered to avoid the unnecessary packet discard at transmitter, especially when the congestion has already been resolved or there is no congestion but the scheduling restriction, e.g. due to MG, occupies the transmission time.

- RAN2 to study the PDB/PSDB adjustment to avoid the discard of outdated packet(s), especially when there is no congestion in air interface.

- Conclusion

In this contribution, we have discussed the scheduling enhancements and potential solutions. Based on the discussions, there are the following observations:

Observation 1	Applying additional priority only for delay-critical data may result in the UE to evaluate the priority among LCHs twice for one MAC PDU construction.

Observation 2	For MAC PDU construction, the LCH with delay-critical data may not be allocated enough radio resource due to its buckets (Bj) exhausted.

Observation 3	If a separate remaining time threshold for LCH priority adaptation larger than the existing remaining time threshold is configured, the risk of the occurrence of large amount of delay-critical data upon DSR triggering can be reduced.

Observation 4	It can happen that there are no enough grants fulfilling all LCP restrictions (i.e. optimal grants) for the frame burst TX while there are still UL grants not fulfilling some LCP restrictions (i.e. sub-optimal grants) available.

Observation 5	When there are no enough optimal grants while the UE is not allowed to use sub-optimal grants for XR frame burst TX, some delay-critical data of the frame burst has to be discarded and the remaining sub-optimal grants have to be wasted.

In order to improve both the system capacity and the XR user experience, LCP restriction relaxation should be supported to allow the UE to use sub-optimal UL grants according to preconfigurations:

Observation 6	When the PDB/PDSB window of a frame burst is partly occupied by a measurement gap, the gNB may not be able to allocate enough radio resource for HARQ TX/ReTX within the PDB/PDSB window of the frame burst.

Observation 7	For XR traffic with tight PDB/PSDB requirements, some packet(s) of a frame burst may not be able to be transmitted before the corresponding PDB/PSDB expiration.

Observation 8	If the outdated packets of a frame burst are not discarded but transmitted to the receiver, the receiver can recover the outdated frame for future frame burst decoding, which avoids the video quality degradation propagation until the next I-frame.

Based on the above discussions and the observations, we have the following proposals:

Proposal 1	Once the additional priority is determined for an LCH, the additional priority should be applied to all data of the LCH, including both delay critical and non-delay critical data.

Proposal 2	If the priority is determined for a LCH upon a MAC PDU construction, the resulted priority (either original priority or additional one) should be applied for the whole MAC PDU construction without change.

Proposal 3	In addition to LCH priority adaptation, RAN2 should also consider the option to raise the rate limit (e.g. Bj) of the LCH with delay-critical data.

Proposal 4	Introduce a separate remaining time threshold for delay critical data for LCP enhancement.

Proposal 5	LCP restriction (e.g. allowedSCS-List, allowedServingCells, etc.) relaxation for an LCH according to the pre-configuration should be supported for delay-critical data transmission.

Similar to LCH priority enhancements, once LCP restriction is determined for one LCH, the determined LCP restriction relaxation can be applied for the whole LCH instead of the delay-critical data only, to avoid the complexity of using two sets of different LCP restriction parameters for an LCH in correspondence to one MAC PDU construction.

Proposal 6	Once LCP restriction relaxation is determined for an LCH, the resulted LCP restriction relaxation should be applied to the whole LCH, including both delay critical and non-delay critical data.

For the similar reason, the LCP restriction relaxation for one LCH can be determined per MAC PDU basis. When the UL grant is available, the UE determines whether LCP restriction relaxation is needed based on the pre-configurations. Once the LCP restriction parameters (relaxed ones or non-relaxed ones) are determined, these LCP restriction parameters should be used during the whole MAC PDU construction.

Proposal 7	If the LCP restriction is relaxed for an LCH upon a MAC PDU construction, the relaxed LCP restriction of this LCH should be applied for the whole MAC PDU construction without change.

Proposal 8	RAN2 to study the resource allocation enhancements considering the PSI importance, for both delay-critical data and non-delay-critical data.

Proposal 9	Whether/how to include non-delay-critical data information into DSR is left to discussion in WI phase.

Proposal 10	The existing remaining time threshold is reused for triggering DSR with multiple pairs of remaining time/buffer information (i.e. no additional threshold for DSR triggering).

Proposal 11	RAN2 to consider new BSR triggering, e.g. upon the end packet arrival of a frame burst, as a candidate solution for scheduling enhancements.

Proposal 12	RAN2 to study whether to transmit the outdated packet(s) to the receiver instead of simply discarding the outdated packet(s).

Proposal 13	RAN2 to study the PDB/PSDB adjustment to avoid the discard of outdated packet(s), especially when there is no congestion in air interface.

- References

- RAN2-126, Chair notes

- RAN2-125bis, Chair notes

- 3GPP TS 38.321 v18.1.0