3GPP TSG-RAN WG2 Meeting #127		R2-2406437

Maastricht, Netherlands, 19th – 23rd Aug. 2024

Source:	vivo

Title:	Discussion on RLC enhancement for XR

Agenda Item:	8.7.5

Document for:	Discussion and Decision

- Introduction

In RAN2#125bis, RAN2 agreed to discuss the following two directions for RLC AM enhancement for XR [1]:

- RAN2 will analyse solutions to ensure timely RLC retransmission(s) for XR - RAN2 will analyse how to avoid unnecessary retransmissions (e.g. to avoid reTx of out-dated packets)

And in RAN2#126, solutions for each direction are discussed and the following agreements were made [2]:

Avoiding unnecessary retransmissions - For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach - RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. - For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side - For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned Ensure timely retransmissions - To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism - Impact on capacity should be considered - RAN2 focuses on the enhancements for UL traffic

In this paper, we will discuss the two enhancement directions and also the solutions for each direction.

- Discussion

  - Enhancements to ensure timely RLC retransmission(s)

- Approach 1: Autonomous retransmission

Autonomous retransmission is a kind of proactive retransmission, allowing the transmitting AM RLC entity to autonomously perform RLC retransmissions without waiting for the RLC NACK feedback. In this approach, the transmitting AM RLC entity may decide to perform blind retransmissions based on traffic needs (e.g., the delay budget of the RLC SDU) or the HARQ feedback from the physical layer (e.g., the number of HARQ retransmission failures).

In XR, one of the common cases of late PDUs is congestion. Thus, the congestion scenario is a primary scenario that requires optimization. In a congested scenario, autonomous retransmission may have a negative impact on the system and worsen the congestion, as the transmitting RLC entity may perform unnecessary RLC retransmission, leading to the wastage of radio resources, even if the packet has already been successfully transmitted.

Observation 1: Autonomous retransmission may make the congestion worse.

For autonomous retransmission based on HARQ feedback, it is possible for the network to perform DL RLC autonomous retransmission based on the HARQ feedback. However, in the case of UL, there is no HARQ feedback and the UE is unable to determine whether the HARQ transmission is successful or the network just gives up the HARQ retransmission when the NDI is toggled. As a result, it is not feasible for the UE to perform UL autonomous RLC retransmissions based on the HARQ feedback.

Observation 2: Autonomous retransmission based on UL HARQ feedback is not feasible according to the existing HARQ mechanism.

Considering the negative effects of autonomous retransmission and the relatively large specification impact. We think there is no need to support the autonomous retransmission in Rel-19.

Proposal 1: Autonomous retransmission is not supported in Rel-19.

- Approach 2: Enhance the status report or polling mechanism to speed up the RLC retransmission

In this approach, several solutions have been proposed to reduce the time for STATUS reporting, so that the transmitting RLC entity can initiate retransmission for the NACK RLC SDU timely.

One solution is to introduce new STATUS reporting triggering mechanisms, such as introducing a new timer to decrease the time for the receiving RLC entity to initiate STATUS reporting [3] or making the transmitting RLC entity to initiate polling based on the delay budget of the RLC SDU [4].

Another solution is to reuse the existing mechanism for triggering STATUS reporting and configure smaller values for the for the existing timer (e.g., t-Reassembly, t-StatusProhibit, and t-PollRetransmit) and parameters (e.g, pollPDU and pollByte) to reduce the time for triggering STATUS reporting. In this solution, whether smaller values with finer granularity for the existing timer and parameters need to be introduced is FFS.

In our view, both solutions can effectively ensure timely retransmission. However, the second solution is preferred due to its simplicity. Given that only the header and ACK_SN are mandatory present in the STATUS PDU, more frequent STATUS reporting for the second solution is not expected to introduce too much radio resources wasting. Details on whether smaller values with finer granularity for the existing timer and parameters need to be introduced could be further discussed after this approach proceeds.

Proposal 2: Proper configuration and current mechanism for triggering status report or polling is enough for ensuring timely RLC retransmission. Whether smaller values with finer granularity for existing RLC STATUS reporting related timers (e.g., t-Reassembly, t-StatusProhibit, and t-PollRetransmit) or counters (e.g., pollPDU and pollByte) need to be introduced for faster STATUS report triggering is FFS.

  - Enhancements to avoid unnecessary retransmissions

To prevent the wastage of resources and the delay caused by unnecessary RLC retransmission, the RLC retransmission should be capable of being skipped or relaxed (e.g., reducing the retransmission number) for specific packets (e.g., packets that no longer meet the PDB/PSDB requirements or are about to exceed the PDB/PSDB requirements).

In RAN2#126 meeting, it was agreed two approaches for skipping/relaxing the RLC retransmissions.

- RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. - For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side - For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned

- Approach 1:Tx initiated approach

In this option, the UE determines whether RLC retransmission for the specific packets is unnecessary and should be skipped for UL traffic. The UE makes the decision maybe based on the delay information of each packet, such as the remaining time. Since the UL delay-related information is maintained at the UE side, it is natural and straightforward for the UE to make the decision.

Observation 3: Delay-related information for packets (e.g., remaining time) is maintained at the transmitter side, it is natural and straightforward for the transmitter side to decide whether to stop RLC retransmission for specific packets.

When the UE decides to stop RLC retransmission and discard the corresponding packets, the UE should inform the network of the obsolete RLC SDU(s), and the network RLC entity will update the state variables and advance the receiving window, just similar to the solution introduced for the PDCP SN gap report in Rel-18.

- Approach 2:Rx initiated approach

In this option, the receiver decides whether the packets in the transmitter should stop RLC retransmission and notify the transmitter. However, since the delay information of packets is maintained at the transmitter side and only the transmitter side knows whether the delay for a PDU/PDU set will meet the PDB/PSDB requirements or is about to exceed the PDB/PSDB requirements, it is difficult for the receiver to decide which packets should skip retransmission and when to skip.

In the previous meetings, some companies proposed introducing an additional timer per packet at the receiver and the receiver will notify the transmitter to skip RLC retransmission for the packet when the new timer expires. However, the solution is not feasible because the receiver does not know when the packet arrives at the transmitter buffer and it is hard to set the length of the timer. Besides, introducing a new timer at the receiver will bring a huge specification impact and is very complex.

Observation 4: Since the packet status is maintained at the transmitter, it is difficult for the receiver to determine whether an RLC SDU is obsolete or not.

With the above analysis for both Approach 1 (Tx initiated approach) and  Approach 2 (Rx initiated approach) above, we prefer to use the Approach 1 (Tx initiated approach) to avoid unnecessary RLC retransmissions.

Proposal 3: Approach 1 (Tx initiated approach) is supported for avoiding unnecessary RLC retransmissions.

- Conclusion

In this contribution, we discuss the two directions of RLC AM enhancements, and we have the following observations and proposals:

Enhancements to ensure timely RLC retransmission(s)

Observation 1: Autonomous retransmission may make the congestion worse.

Observation 2: Autonomous retransmission based on UL HARQ feedback is not feasible according to the existing HARQ mechanism.

Proposal 1: Autonomous retransmission is not supported in Rel-19.

Proposal 2: Proper configuration and current mechanism for triggering status report or polling is enough for ensuring timely RLC retransmission. Whether smaller values with finer granularity for existing RLC STATUS reporting related timers (e.g., t-Reassembly, t-StatusProhibit, and t-PollRetransmit) or counters (e.g., pollPDU and pollByte) need to be introduced for faster STATUS report triggering is FFS.

Enhancements to avoid unnecessary retransmissions

Observation 3: Delay-related information for packets (e.g., remaining time) is maintained at the transmitter side, it is natural and straightforward for the transmitter side to decide whether to stop RLC retransmission for specific packets.

Observation 4: Since the packet status is maintained at the transmitter, it is difficult for the receiver to determine whether an RLC SDU is obsolete or not.

Proposal 3: Approach 1 (Tx initiated approach) is supported for avoiding unnecessary RLC retransmissions.

- References

- R2_125bis_ChairNotes.

- R2_126_ChairNotes.

- R2-2402764, Considering the RLC enhancements for XR. ZTE Corporation, Sanechips.

- R2-2403090, RLC AM retransmission enhancement for XR. TCL