3GPP TSG-RAN WG2 Meeting #127		R2-2406438

Maastricht, Netherlands, 19th – 23rd Aug. 2024

Source:	vivo

Title:	Discussion on the impact of s-Measure on L1 measurement

Agenda Item:	7.4.2

Document for:	Discussion and Decision

- Introduction

In this paper, we discuss the impact of L3 measurement/s-MeasureConfig on the LTM L1 measurement and propose the corresponding solution.

- Discussion

  - LTM L1 measurement with L3 measurement

According to the LS from RAN4 [1], it has been agreed that UE is not required to perform L1 measurements on the neighbor cell for which UE has not performed L3 measurement:

- UE is not required to perform L1 measurements on unknown cell. Known cell definition: - In L1-RSRP measurement for neighbour cell, target cell is considered as known if the following conditions are met in this requirement: - The UE has performed L3 measurement on the target cell, and - FFS whether to add time constraint e.g., during the last [5] seconds - The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2 and 9.3. - Otherwise, it is unknown

And the definition of known cell is specified in RAN4 as following [2]:

The cell is considered as known if the following conditions are met in this requirement: - The UE has performed L3 measurement on the target cell during the last 5seconds, and - The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2. Otherwise, the cell is unknown.

Observation 1: According to RAN4 agreements/spec, L1 measurement requirements are only applied to the known candidate cells, for which UE has performed L3 measurement during the last 5 seconds.

And RAN2 has the following conclusion in RAN2#124 meeting:

- s-Measure does not apply to L1 LTM related measurements (in this release).

Besides, in last RAN2#126 meeting, RAN2 has agreed to capture the agreement in specification.

R2-2404437 Remaining issues on s-Measure and recovery after RLF due to RLC retransmission vivo discussion Rel-18 NR_Mob_enh2-Core DISCUSSION P1 - HW not current behaviour. LG agree with HW, already agreed that L1 meas doesn’t apply S-meas. Ericsson think this might not be captured in the TS. - Capture that s-Measure criterion is not applicable to L1 measurements, if not already clear (disc in CR update discussion)

Since the s-Measure criterion is not applicable to L1 measurements, we could assume that UE shall keep performing L1 LTM measurements on the LTM candidate cells once the LTM L1 measurement is configured and even if the s-MeasureConfig is fulfilled for L3 measurement.

Observation 2: UE shall perform the LTM L1 measurement once the LTM L1 measurement is configured and even if the s-Measure criterion for L3 measurement is fulfilled.

Take both the observation1 and 2 into account, the UE behavior for LTM L1 measurement is not clear when s-Measure criterion is fulfilled for L3 measurement.

When s-Measure criterion is fulfilled for L3 measurement, the UE will stop L3 measurements on neighboring cells (including LTM candidate cells). And according to Observation 1, the UE stops performing LTM L1 measurements when the neighboring LTM candidate cells become unknown (i.e. 5 seconds after UE stopping L3 measurements). However, according to the Observation 2, the UE still needs to continue the L1 measurement in this case.

Observation 3: UE measurement behaviour for L1 measurement is ambiguous in case s-Measure criterion is fulfilled for L3 measurement.

To avoid the impact of s-Measure on L1 LTM measurements, UE should always perform L3 measurements for LTM candidate cells to ensure the candidate cell is known even if s-Measure criterion is fulfilled. Hence, we propose:

Proposal 1: UE should always perform L3 measurement for LTM candidate cells even if s-Measure criterion is satisfied.

The corresponding TP is provided in Annex 1, we propose two options for this issue.

Proposal 2: If P1 is agreeable, RAN2 to adopt the TP option 1 or option 2 provided in Annex 1.

- Conclusion

In this contribution, we analyze the unclear UE behaviour for LTM L1 measurement due to s-measure for L1 measurement and L3 measurement is not inconsistent, the observations and proposal are as follows:

Observation 1: According to RAN4 agreements/spec, L1 measurement requirements are only applied to the known candidate cells, for which UE has performed L3 measurement during the last 5 seconds.

Observation 2: UE shall perform the LTM L1 measurement once the LTM L1 measurement is configured and even if the s-Measure criterion for L3 measurement is fulfilled.

Observation 3: UE measurement behaviour for L1 measurement is ambiguous in case s-Measure criterion is fulfilled for L3 measurement.

Proposal 1: UE should always perform L3 measurement for LTM candidate cells even if s-Measure criterion is satisfied.

Proposal 2: If P1 is agreeable, RAN2 to adopt the TP option 1 or option 2 provided in Annex 1.

- References

- R4-2321388, Reply LS on L1 measurements for LTM.

- 38133-i50_s9-9, "NR; Requirements for support of radio resource management".

- TS 38.331 v18.0.0, Radio Resource Control (RRC) protocol specification.

Annex:

### Option 1

TP for TS 38.331 on s-Measure on L1 measurement, two options are provided as following:

#### 5.5.3.1	General

----------------------skip unchanged part--------------------

NOTE 2:	In case the configurations for NR sidelink communication and CBR measurement are acquired via the E-UTRA, configurations for NR sidelink communication in SIB12, sl-ConfigDedicatedNR within RRCReconfiguration used in this clause are provided by the configurations in SystemInformationBlockType28, sl-ConfigDedicatedForNR within RRCConnectionReconfiguration as specified in TS 36.331[10], respectively.

NOTE 3:	If a UE that is configured by upper layers to transmit V2X sidelink communication is configured by NR with transmission resource pool(s) and the measurement objects concerning V2X sidelink communication (i.e. by sl-ConfigDedicatedEUTRA-Info), it shall perform CBR measurement as specified in clause 5.5.3 of TS 36.331 [10], based on the transmission resource pool(s) and the measurement object(s) concerning V2X sidelink communication configured by NR.

NOTE 4:	For V2X sidelink communication, each of the CBR measurement results is associated with a resource pool, as indicated by the poolReportId (see TS 36.331 [10]), that refers to a pool as included in sl-ConfigDedicatedEUTRA-Info or SIB13.

NOTE X: UE should always perform L3 measurement for LTM candidate cells to ensure the candidate cells are known cells (see TS 38.133 [14]), even if S-MeasureConfig is configured and satisfied.

### Option 2

#### 5.5.3.1	General

----------------------skip unchanged part--------------------

The UE shall:

----------------------skip unchanged part--------------------

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the reportType for the associated reportConfig is set to reportCGI and timer T321 is running:

3>	if useAutonomousGaps is configured for the associated reportConfig:

4>	perform the corresponding measurements on the frequency and RAT indicated in the associated measObject using autonomous gaps as necessary;

3>	else:

4>	perform the corresponding measurements on the frequency and RAT indicated in the associated measObject using available idle periods;

3>	if the cell indicated by reportCGI field for the associated measObject is an NR cell and that indicated cell is broadcasting SIB1 (see TS 38.213 [13], clause 13):

4>	try to acquire SIB1 in the concerned cell;

3>	if the cell indicated by reportCGI field is an E-UTRA cell:

4>	try to acquire SystemInformationBlockType1 in the concerned cell;

2>	if the ul-DelayValueConfig is configured for the associated reportConfig:

3>	ignore the measObject;

3>	for each of the configured DRBs, configure the PDCP layer to perform corresponding average UL PDCP packet delay measurement per DRB;

2>	if the ul-ExcessDelayConfig is configured for the associated reportConfig:

3>	ignore the measObject;

3>	for each of the configured DRBs, configure the PDCP layer to perform corresponding UL PDCP Excess Packet Delay delay measurement according to the configured threshold per DRB;

2>	if the reportType for the associated reportConfig is periodical, eventTriggered; or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the MCG VarMeasConfig and is indicated in the condExecutionCond or in the condExecutionCondPSCell associated to a condReconfigId in the MCG VarConditionalReconfig (for CHO, CPA, MN-initiated inter-SN CPC, or subsequent CPAC in NR-DC); or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCond associated to a condReconfigId in the SCG VarConditionalReconfig (for intra-SN CPC or subsequent CPAC); or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCondSCG associated to a condReconfigId in the MCG VarConditionalReconfig (for SN-initiated inter-SN CPC or subsequent CPAC in NR-DC); or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the triggerConditionSN associated to a condReconfigurationId in VarConditionalReconfiguration as specified in TS 36.331 [10] (for SN-initiated inter-SN CPC in EN-DC):

3>	if a measurement gap configuration is setup, or

3>	if the UE does not require measurement gaps to perform the concerned measurements:

4>	if s-MeasureConfig is not configured, or

4>	if s-MeasureConfig is set to ssb-RSRP and the NR SpCell RSRP based on SS/PBCH block, after layer 3 filtering, is lower than ssb-RSRP, or

4>	if s-MeasureConfig is set to csi-RSRP and the NR SpCell RSRP based on CSI-RS, after layer 3 filtering, is lower than csi-RSRP:

5>	if the measObject is associated to NR and the rsType is set to csi-rs:

6>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured:

7>	derive layer 3 filtered beam measurements only based on CSI-RS for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on CSI-RS for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to NR and the rsType is set to ssb:

6>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured:

7>	derive layer 3 beam measurements only based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on SS/PBCH block for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from thassociated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to E-UTRA:

6>	perform the corresponding measurements associated to neighbouring cells on the frequencies indicated in the concerned measObject, as described in 5.5.3.2;

5>	if the measObject is associated to UTRA-FDD:

6>	perform the corresponding measurements associated to neighbouring cells on the frequencies indicated in the concerned measObject, as described in 5.5.3.2;

5>	if the measObject is associated to L2 U2N Relay UE:

6>	perform the corresponding measurements associated to candidate Relay UEs on the frequencies indicated in the concerned measObject, as described in 5.5.3.4;

4>	if s-MeasureConfig is set to ssb-RSRP and the NR SpCell RSRP based on SS/PBCH block, after layer 3 filtering, is not lower than ssb-RSRP, or

4>	if s-MeasureConfig is set to csi-RSRP and the NR SpCell RSRP based on CSI-RS, after layer 3 filtering, is not lower than csi-RSRP:

5>	if the measObject is associated to NR and the LTM candidate configurations is configured for the UE :

6> for the listed cell within the measObject which is the LTM Candidate cell whose LTM-CandidateId included in the LTM-CSI-ResourceConfig associated with the LTM-CSI-ReportConfig configuration of the serving cell:

7>	if the rsType is set to csi-rs:

8>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured:

9>	derive layer 3 filtered beam measurements only based on CSI-RS for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a;

8>	derive cell measurement results based on CSI-RS for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

7>	if the rsType is set to ssb:

8>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured:

9>	derive layer 3 beam measurements only based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a;

8>	derive cell measurement results based on SS/PBCH block for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

4>	if the measRSSI-ReportConfig is configured in the associated reportConfig:

5>	perform the RSSI and channel occupancy measurements on the frequency configured by rmtc-Frequency in the associated measObject;

NOTE 0:	The network avoids configuring UEs supporting only CHO and/or Rel-16 CPC with measurements not referred to by any execution condition.

----------------------skip unchanged part--------------------