3GPP TSG-RAN WG2 Meeting #127		R2-2406441

Maastricht, Netherlands, 19th – 23rd Aug. 2024

Source:	vivo, China Telecom, China Unicom, Guangdong Genius

Title:	Discussion on issues for emergency call with barringExemptEmergencyCall

Agenda Item:	7.24.2.2

Document for:	Discussion and Decision

- Introduction

To allow Redcap/eRedCap/2Rx XR UE to initiate an emergency call in a cell that is barred via cellBarredRedCap1Rx/ cellBarredRedCap2Rx/cellBarred2RxXR in SIB1, a barring exemption indication was introduced in RAN2#126 meeting [1]. However, there are still some remaining issues with the current mechanism. In the contribution, we will discuss the issues in detail.

For simplicity, in the following sections, we take RedCap UE as an example in the discussion, while the observations and proposals are also applicable to eRedCap UE and 2Rx XR UE.

- Discussion

  - Issues

Issue 1: Unreasonable UE behaviors for RedCap and non-RedCap UE

In RAN sharing scenario, a cell may broadcast more than one plmn-IdentityLists, some with trackingAreaCode, others without trackingAreaCode.

TS38.331 [2] trackingAreaCode Indicates Tracking Area Code to which the cell indicated by cellIdentity field belongs. The absence of the field indicates that the cell only supports PSCell/SCell functionality (per PLMN) or is an NTN cell..

If no trackingAreaCode is provided for the UE’s PLMN (i.e. selected PLMN or registered PLMN or PLMN of the equivalent PLMN list), a normal (i.e. non-RedCap) UE shall consider the cell as barred according to the current specification as following:

TS38.331 [2] ##### 5.2.2.4.2 Actions upon reception of the SIB1 Upon receiving the SIB1 the UE shall: … 2> if the UE is neither a RedCap nor an eRedCap UE, or for TDD if the UE is an (e)RedCap UE, or for FDD if the UE is an (e)RedCap UE and halfDuplexRedCapAllowed is present, or if the UE is an (e)RedCap UE and the (e)RedCap UE supports full-duplex FDD operation on this band: 3> if neither trackingAreaCode nor trackingAreaList is provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list: 4> consider the cell as barred in accordance with TS 38.304 [20]; 4> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

As a result, the UE is not allowed to initiate emergency call in the cell. To initiate an emergency call, the UE may select a different PLMN with providing trackingAreaCode in the RAN sharing cell and re-evaluate the barring condition again according to TS38.304 as below.

TS38.304 [3] ### 5.3.1 Cell status and cell reservations … NOTE 2: If barring of a cell is triggered by the condition of trackingAreaCode and trackingAreaList not being provided, as specified in TS 38.331 [3], the barring only applies to this PLMN and the UE can re-evaluate the barring condition again due to selection of another PLMN.

In Rel-18, with the introduction of barringExemptEmergencyCall [1], a RedCap UE considers a cell as acceptable cell when the following conditions are fulfilled:

TS38.304 [2]: ### 5.3.1 Cell status and cell reservations … When cell status "barred" is indicated or to be treated as if the cell status is "barred", - The UE is not permitted to select/reselect this cell, not even for emergency calls except for the below cases: - When cellBarredRedCap1Rx is set to "barred" in SIB1, a RedCap UE that supports only 1Rx branch can consider the cell as acceptable cell, only if cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to "barred" and in SIB1, barringExemptEmergencyCall is present and, if the RedCap UE supports only half duplex FDD operation, halfDuplexRedCapAllowed is set to "true"; or

However, since the barringExemptEmergencyCall is common for all the PLMNs broadcasted in one RAN sharing cell, a RedCap UE may think it is permitted to regard the cell as an acceptable cell even if the UE has selected a PLMN without providing trackingAreaCode. Consequently, the RedCap UE may try to initiate an emergency call through the PLMN that has not provided a trackingAreaCode. While the barringExemptEmergencyCall is not applied to non-RedCap UE, so the non-RedCap UE will treat such cell as barred and is not allowed to initiate emergency call in the cell as mentioned above. This is unreasonable, as RedCap UE seems more advanced than normal UE (i.e. non-RedCap UE).

We have the following observation:

Observation 1: The presence of barringExemptEmergencyCall leads to unreasonable different behaviors for RedCap UE and non-RedCap UE on initiating an emergency call in a RAN sharing cell:

- non-RedCap UE is not allowed to initiate an emergency call through a PLMN without providing trackingAreaCode.

- RedCap UE is allowed to initiate an emergency call through a PLMN without providing trackingAreaCode.

Issue 2: RRC Setup failure through PLMN without trackingAreaCode

In the RRC Setup procedure in TS38.331 [2], the PLMN chosen by the UE should be included in the RRCSetupComplete message as below. The PLMN information is used by gNB to route the dedicatedNAS-Message included in the RRCSetupComplete message to the correct AMF. The absence of trackingAreaCode for a PLMN may due to the gNB has no connection (or be unable to forward the dedicatedNAS-Message) to the AMF serving the selected PLMN.

TS38.331 [2]

Hence, if a RedCap UE selects an PLMN without providing trackingAreaCode to initiate an emergency call, the setup of the emergency call would fail.

Observation 2: If a RedCap UE selects an PLMN without providing trackingAreaCode to initiate an emergency call, the setup of the emergency call would fail.

  - Solution

Take the above into account, we think the case without providing barringExemptEmergencyCall for UE’s PLMN should be excluded for (e)RedCap/2Rx XR UE to initiate an emergency call in a cell with barringExemptEmergencyCall indication.

Proposal 1: A cell is considered as barred cell by a UE if no trackingAreaCode is provided for any of the PLMNs associated with the UE (i.e. the selected PLMN or the registered PLMN or PLMN of the equivalent PLMN list) in SIB1, irrespective of whether the barringExemptEmergencyCall is present or not in SIB1.

A corresponding TP on TS 38.304 is provided in Annex.

Proposal 2: If Proposal 1 is agreeable, RAN2 to adopt the corresponding TP provided in Annex.

- Conclusion

In this contribution, we discuss the issue that a RedCap/eRedCap/2Rx XR UE may errorly initiate emergency call according to the barring exemption indication in a cell associated with PLMNs without providing trackingAreaCode. Based on the discussion, we have the following observations and proposal:

Observation 1: The presence of barringExemptEmergencyCall leads to unreasonable different behaviors for RedCap UE and non-RedCap UE on initiating an emergency call in a RAN sharing cell:

- non-RedCap UE is not allowed to initiate an emergency call through a PLMN without providing trackingAreaCode.

- RedCap UE is allowed to initiate an emergency call through a PLMN without providing trackingAreaCode.

Observation 2: If a RedCap UE selects an PLMN without providing trackingAreaCode to initiate an emergency call, the setup of the emergency call would fail.

Proposal 1: A cell is considered as barred cell by a UE if no trackingAreaCode is provided for any of the PLMNs associated with the UE (i.e. the selected PLMN or the registered PLMN or PLMN of the equivalent PLMN list) in SIB1, irrespective of whether the barringExemptEmergencyCall is present or not in SIB1.

Proposal 2: If Proposal 1 is agreeable, RAN2 to adopt the corresponding TP provided in Annex.

- References

- R2-2405956/R2-2405957/R2-2405958, Introduction of barring exemption for RedCap UEs for emergency calls, Apple.

- TS 38.331 v18.1.0, Radio Resource Control (RRC) protocol specification.

- TS 38.304 v18.2.0, User Equipment (UE) procedures in Idle mode and RRC.

Annex: TP for TS 38.304 v18.2.0

5.3.1	Cell status and cell reservations

Omit unrelated part.

-------------------------------------------------------start of change------------------------------------------------------------

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls except for the below cases:

-	When cellBarredRedCap1Rx is set to "barred" in SIB1, a RedCap UE that supports only 1Rx branch can consider the cell as acceptable cell, only if cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to "barred" and in SIB1, barringExemptEmergencyCall is present, if the RedCap UE supports only half duplex FDD operation, halfDuplexRedCapAllowed is set to "true" and, trackingAreaCode is provided for the selected PLMN or the registered PLMN or PLMN of the equivalent PLMN list in SIB1; or

-	When cellBarredRedCap2Rx is set to "barred" in SIB1, a RedCap UE that supports 2Rx branches can consider the cell as acceptable cell, only if cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to "barred" and in SIB1, barringExemptEmergencyCall is present, if the RedCap UE supports only half duplex FDD operation, halfDuplexRedCapAllowed is set to "true" and, trackingAreaCode is provided for the selected PLMN or the registered PLMN or PLMN of the equivalent PLMN list in SIB1; or

-	When cellBarred-eRedCap1Rx is set to "barred" in SIB1, an eRedCap UE that supports only 1Rx branch can consider the cell as acceptable cell, only if cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to "barred" and in SIB1, barringExemptEmergencyCall is present, if the eRedCap UE supports only half duplex FDD operation, halfDuplexRedCapAllowed is set to "true" and, trackingAreaCode is provided for the selected PLMN or the registered PLMN or PLMN of the equivalent PLMN list in SIB1; or

-	When cellBarred-eRedCap2Rx is set to "barred" in SIB1, an eRedCap UE that supports 2Rx branches can consider the cell as acceptable cell, only if cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to "barred" and in SIB1, barringExemptEmergencyCall is present, if the eRedCap UE supports only half duplex FDD operation, halfDuplexRedCapAllowed is set to "true" and, trackingAreaCode is provided for the selected PLMN or the registered PLMN or PLMN of the equivalent PLMN list in SIB1; or

-	When cellBarred2RxXR is present in SIB1, a 2Rx XR UE can consider the cell as acceptable cell, only if the cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to "barred" and in SIB1, barringExemptEmergencyCall is present and, trackingAreaCode is provided for the selected PLMN or the registered PLMN or PLMN of the equivalent PLMN list in SIB1;

-------------------------------------------------------end of change------------------------------------------------------------

Omit unrelated part.