3GPP TSG-RAN WG2 Meeting #127	R2-2406443

Maastricht, Netherlands, Aug 18th – 23rd, 2024

Agenda item:		8.7.5

Source:		Samsung

Title:			RLC Enhancements for XR

Document for:	Discussion & Decision

# 1 Introduction

In this contribution, we discuss RLC AM enhancements for XR, based on the agreements made in the last meeting.

- RAN2#126 Agreements Timely RLC AM Retransmissions: To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic Unnecessary RLC AM Retransmissions Avoidance: For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy FFS how Rx side determines that an SDU should be abandoned

# 2 Formulating the problem

At RLC AM transmitting side, an RLC SDU can be in three states:

- State 1: waiting for initial transmission.

- State 2: waiting for Ack after being (re)transmitted.

- State 3: waiting for retransmission (after being Nacked).

Fig. 1. Three states an RLC SDU can be in.

From RLC point of view, the latency pertaining to the retransmission (when needed) mainly consists of

- waiting time in State 2: until receiving STATUS report covering its status from peer RLC entity, and

- waiting time in State 3: until being submitted to lower layer (MAC).

One operation that lengthens the waiting time in State 2 could be RLC polling latency. In legacy RLC AM, polling can be delayed until RLC SDUs or bytes thereof delivered to lower layer are accumulated up to a certain threshold configured by NW. Such mechanism provides NW with a way to manipulate how often the polling and the STATUS report that follows should be involved, but at the same time, entails an extra polling latency.

Observation 1: A delay-critical RLC SDU can suffer polling latency before requesting STATUS report from peer RLC entity, which delays its retransmission when needed.

For state 3, a Nacked RLC SDU in retransmission buffer may suffer queing delay until being submitted to lower layer, caused by the preemption from other Nacked RLC SDUs previously stored in the retransmission buffer. Note that legacy RLC AM does not discard RLC SDUs that has been submitted to lower layer as shown in Excerpt 1, to avoid the occurence of RLC SN gap. It means that the retransmission of a delay-critical RLC SDU can be preempted by that of other RLC SDU(s) previously stored in retransmission buffer but with expired discardTimer.

Excerpt 1:

38.322 ## 5.4 SDU discard procedures When indicated from upper layer (i.e. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

Observation 2: The retransmission of a delay-critical RLC SDU can be unnecessarily delayed by that of other RLC SDU(s) with expired discardTimer.

- Exploring the solution space

## Timely RLC AM Retransmissions

Table I presents a comparison of different approaches to enable timely RLC AM retransmissions, as were considered in RAN2#126.

TABLE I

| Approach | Attribute / Condition | Pros | Cons |
| --- | --- | --- | --- |
| Autonomous Retransmissions | Blind retransmission | - Quick retransmission | - In-efficient and huge impact on capacity i.e. resource wastage |
|  | Upon failure of a number of HARQ transmissions | - Early determination & Local NACK | - Not straight-forward to map a RLC PDU and HARQ TB - Involves complicated and cumber-some cross-layer interactions - Large specification impact |
|  | Upon remaining time of the PDU becomes lesser than a threshold | - Pre-emptive retransmission | - Unnecessary - same can be achieved with a suitable configuration of polling timers |
|  | Upon availability of spare transmisison resources | - Opportunistic approach | - Not full-proof and deterministic |
| Enhanced Status Report | Stopping StatusProhibit timer for small PDB | - Early status reporting for small PDB | - Unnecessary - same can be achieved with suitable status reporting configurations by NW |
| Enhanced polling | Dedicated polling thresholds and associated counters for delay critical RLC SDUs | - Polling enhancements for delay-critical SDUs - Differentiated treatment | - New configurations needed for delay critical but manageable |

Autonomous retransmission implies a transmitter can speed up retransmission without having to wait for receiving a status report from the receiver. Though the proposal looks attractive; however, there can be drastic impacts on the capacity as a large number of unnecessary and unregulated retransmissions could take place. As also described in Table I, it is not easy to regulate such autonomous transmissions based on specific conditions (e.g. HARQ transmisions, timers or spare transmission resources).

Observation 3: With autonomous retransmission, there can be drastic impacts on the capacity as a large number of unnecessary and unregulated retransmissions could take place. It is not easy to regulate such autonomous transmissions based on specific conditions (e.g. HARQ transmisions, timers or spare transmission resources).

Proposal 1: No enhancements are pursued for autonomous RLC AM retransmissions.

Enhanced status report requires the receiver should be able to provide a status report quickly to the transmitter. More particularly, this is to support stopping of t-StatusProhibit timer so as to be able to report status when small PDB is involved. However, such provisions can be easily achieved by the NW with suitable configurations of the status reporting e.g. t-StatusProhibit timer. Moreover, receiver can only determine about the need for timely retransmission from the perceived SN gap in the receiving window i.e. the receiver is not well aware about the retransmission requirements as compared to the transmitter.

Observation 4: Enhancements for status reporting for small PDB (e.g. stopping of t-StatusProhibit) are not needed as network configuration can take care with suitable configurations. Receiver is not well aware about the need for timely retransmissions as compared to the transmitter.

Proposal 2: No enhancements are pursued for status reporting in order to aid timely RLC AM retransmissions.

Further, we discuss the enhanced polling approach to facilitate timely RLC AM retransmissions.

As shown in Excerpt 2, there are three cases in legacy RLC AM to trigger a polling at the transmitting side:

- Case 1: the number of initially transmitted RLC SDUs or the bytes thereof, have been accumulated up to a certain thresholds (pollPDU or pollByte), respectively, whichever comes first

- Case 2: there is no data to (re)transmit

- Case 3: transmitting window stalling happens

Excerpt 2:

38.322 5.3.3 Polling ... Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte: - include a poll in the AMD PDU as described below. Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); - include a poll in the AMD PDU as described below. ...

Among the three cases, a special care can be taken for Case 1, in reducing the polling latency, especially, when there are delay-critical RLC SDUs involved. E.g., dedicated thresholds (i.e., pollPDU and pollByte) for delay-critical RLC SDUs and the associated counters can be introduced, in order to reduce the polling latency for delay-critical RLC SDUs.

Proposal 3: Dedicated polling thresholds and the associated counters are introduced for delay-critical RLC SDUs.

## Unnecessary RLC AM Retransmission Avoidance

Table II presents a comparison of different approaches to avoid unnecessary RLC retransmissions, as considered in RAN2#126.

TABLE II

| Approach | Attribute / Condition | Pros | Cons |
| --- | --- | --- | --- |
| Transmitter initiated | RLC SN gap reporting mechanism | - Tx-initiated approach can be effectively designed based on actual delay incurred by SDU - Rx operation is easy with just state variables update | - New signalling for RLC SN gap reporting (though it is quite aligned to existing PDCP SN gap reporting) |
| Receiver initiated | Indicate abandoned SDUs in status report upon t-Reassembly expiry | - No impact on transmitter operation | - Rx operation becomes complex to determine and address abandoned SDU(s) - Does not allow for determining and allowing needed retransmissions - Receiver is not precisely aware about actual delay incurred by SDU - Status reporting is restricted by t-StatusProhibit |

Receiver initiated approach implies the receiver can inform the transmitter in the status report about the abandoned SDUs so that transmitter can update its transmit state variables (i.e. advance the transmission window) and avoid retransmission of the obsolete SDUs. More particularly, the abandoned SDUs are to be informed as ACK when expiry of t-Reassembly occurs. There are certain problems with receiver initiated approach that makes the receiver operation complex to determine and address abandon SDUs:

- Approach does not allow for determinign and allowing needed retransmissions and just abandons the SDUs outrightly upon t-Reassembly expiry when reception failure for an AMD PDU is detected.

- Receiver is not precisely aware about the actual delay incurred by the SDU at the transmitter. Due to scheduling reasons, the actual delay incurred by the SDU is only be best known to the transmitter.

- Status reporting is also restricted by t-StatusProhibit timer to avoid excessive signalling. That is, a delay may be involved in reporting the status.

Observation 5: Receiver initiated approach does not allow for determining and allowing needed retransmissions and abandons the SDUs outrightly upon t-Reassembly expiry when reception failure for an AMD PDU is detected. Moreover, receiver is not precisely aware about the actual delay incurred by the SDU at the transmitter. Status reporting is also restricted by t-StatusProhibit timer.

Proposal 4: Receiver initiated approach is not considered for unnecessary RLC retransmissions avoidance.

Next we discuss Transmitter initiated approach. Significantly, transmitter initiated approach can be effectively designed based on the actual delay incurred by SDU, as contrary to receiver initiated approach.

Note that in Release 18 XR, RAN2 has introduced PDCP SN gap reporting to reduce the reordering latency caused by already discarded PDCP SDUs, encountered at receiving PDCP entity.

In legacy RLC, an RLC SDU that has been discarded at PDCP, if it has been submitted to lower layer, will not be discarded at RLC. It seems unnecessary to retransmit such RLC SDUs, since there is no more value to deliver it. Such unnecessary RLC retransmission will delay the other RLC SDUs’ (re)transmissions, especially, for delay-critical RLC SDUs. Also, such operation will decrease the overall radio resource utilization from application point of view.

To avoid such unnecessary retransmissions, RAN2 should consider discarding RLC SDUs that have already been submitted to lower layer.

Proposal 5: Upon PDCP discard indication, RLC AM can discard RLC SDUs already submitted to lower layer, even if RLC SN gap can occur.

Since RLC AM is designed for loss-less delivery, legacy RLC AM will not work if discarding an RLC SDU already submitted to lower layer, which causes RLC SN gap, i.e., window stalling will occur at both transmitting and receiving sides of RLC AM.

To overcome window-stalling issue caused by RLC SN gap, RAN2 can consider introducing an RLC SN gap reporting mechanism to inform receiving side of RLC AM of the RLC SN gap. The approach can be quite aligned with PDCP SN gap reporting and can be greatly benefited w.r.t. the specification effort and detailing perspective.

Proposal 6: RLC SN gap reporting mechanism can be considered for RLC AM to handle RLC SN gap.

# 4 Conclusion

Based on the discussion above, we have the following observations and proposals:

Observation 1: A delay-critical RLC SDU can suffer polling latency before requesting STATUS report from peer RLC entity, which delays its retransmission when needed.

Observation 2: The retransmission of a delay-critical RLC SDU can be unnecessarily delayed by that of other RLC SDU(s) with expired discardTimer.

Timely RLC AM Retransmission

Observation 3: With autonomous retransmission, there can be drastic impacts on the capacity as a large number of unnecessary and unregulated retransmissions could take place. It is not easy to regulate such autonomous transmissions based on specific conditions (e.g. HARQ transmisions, timers or spare transmission resources).

Proposal 1: No enhancements are pursued for autonomous RLC AM retransmissions.

Observation 4: Enhancements for status reporting for small PDB (e.g. stopping of t-StatusProhibit) are not needed as network configuration can take care with suitable configurations. Receiver is not well aware about the need for timely retransmissions as compared to the transmitter.

Proposal 2: No enhancements are pursued for status reporting in order to aid timely RLC AM retransmissions.

Proposal 3: Dedicated polling thresholds and the associated counters are introduced for delay-critical RLC SDUs.

Unnecessary RLC AM Retransmission Avoidance

Observation 5: Receiver initiated approach does not allow for determining and allowing needed retransmissions and abandons the SDUs outrightly upon t-Reassembly expiry when reception failure for an AMD PDU is detected. Moreover, receiver is not precisely aware about the actual delay incurred by the SDU at the transmitter. Status reporting is also restricted by t-StatusProhibit timer.

Proposal 4: Receiver initiated approach is not considered for unnecessary RLC retransmissions avoidance.

Proposal 5: Upon PDCP discard indication, RLC AM can discard RLC SDUs already submitted to lower layer, even if RLC SN gap can occur.

Proposal 6: RLC SN gap reporting mechanism can be considered for RLC AM to handle RLC SN gap.