3GPP TSG-RAN WG2 Meeting #127										R2-2406444

Maastricht, Netherlands, Aug 19th–23rd, 2024

Agenda item:		8.5.2

Title: 	On-demand SSB SCell operation in connected mode

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

The following objective has been included in NES enhancement in R19 [1]:

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this paper, we share some understanding on the general solution from RAN2 perspective.

- Discussion

  - Understanding on the scenarios

There has been discussion in RAN1 [2] on the scenarios with the following agreements made:

------------------------------------------------------RAN1#116bis Agreements------------------------------------------------------

Agreement

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

- For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner.

  - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered.

- Notes:

  - Scenario #2A refers to

    - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

  - Scenario #3A refers to

    - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

  - Scenario #3B refers to

    - “When SCell activation is completed and SCell is activated” or

    - “After SCell activation is completed and SCell is activated”

  - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications.

  - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

------------------------------------------------------RAN1#116bis Agreements------------------------------------------------------

And the understanding on the first 4 scenarios with no FFS are as follows:

Figure 1. Case#1 and #2 for Scenario#2

- Scenario #2 and Case #1

There is no always-on SSB on the cell.

On-demand SSB is triggered when SCell is configured to a UE but before the UE receives SCell activation command.

- Scenario #2 and Case #2

Always-on SSB is periodically transmitted on the cell

On-demand SSB is triggered when SCell is configured to a UE but before the UE receives SCell activation command.

Figure 2. Case#1 and #2 for Scenario#2a

- Scenario #2A and Case #1

There is no always-on SSB on the cell.

On-demand SSB is triggered when UE receives SCell activation command.

- Scenario #2A and Case #2

Always-on SSB is periodically transmitted on the cell.

On-demand SSB is triggered when UE receives SCell activation command.

In RAN2, we can start with the scenarios identified by RAN1 with no FFS marked. And we will wait for RAN1 progress on other scenarios.

Proposal 1: RAN2 start with the following scenarios and cases for on demand SSB SCell operation with down-selection in future discussion:

- Scenario #2 and Case #1:

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

Case #1: No always-on SSB on the cell

Case #2: Always-on SSB is periodically transmitted on the cell

Scenario #2: On-demand SSB is triggered when SCell is configured to a UE but before the UE receives SCell activation command.

Scenario #2A: On-demand SSB is triggered when UE receives SCell activation command.

Proposal 2: RAN2 will wait for RAN1’s progress on other scenarios.

  - On demand SSB procedure

The on demand SSB procedure can be triggered when UL or DL traffic arrives and more activate SCells are needed.

Proposal 3: The on demand SSB procedure can be triggered by PCell when DL traffic arrives or by UE when UL traffic arrives.

    - Case #1: There is no always-on SSB on the cell

For both Scenario #2 and #2A, we understand the SCell(s) supporting on demand SSB can be added but deactivated at first.

In the existing SCell addition and activation mechanism, when a SCell is configured but deactivated, the UE does not need to receive the corresponding PDCCH or PDSCH, but would still monitor SSB.

While for on demand SSB procedure, we expect UE not to monitor SSB from these cells as there is no always-on SSB on the cell, which can be achieved by indicating the on demand SSB SCells during SCell addition and modification.

For case#1 when there is no always on SSB, UE needs to be aware of the situation and then not monitor SSB.

Proposal 4: For case#1, the SCell(s) supporting on demand SSB would be added but deactivated at first.

Proposal 5: For case#1, UE needs to be aware of for which SCell(s) the on demand SSB is supported and would not monitor SSB from these cells. NW can indicate the on demand SSB SCells during SCell addition and modification.

Figure 3: On demand SSB procedure for case#1

For Scenario#2, when DL traffic arrives, the on demand SSB procedure can be triggered by network, for example, the PCell wake up SCells and indicate to UE that the corresponding SCell would start to transmit SSB (for case#1). UE would then measure these SCells and network can decide to activate one or more of them for UE. When UL traffic arrives, UE can report to the PCell that more SCells are needed and would like to trigger on demand SSB procedure to measure more SCells to be activated later. PCell would then wake up SCells and indicate to UE that the corresponding SCell would start to transmit SSB (for case#1).

In this case, a separate signaling is need to indicate to UE that the corresponding SCell would start to transmit SSB (for case#1).

For Scenario#2A, when DL traffic arrives, the on demand SSB procedure can be triggered by network, for example, the PCell wake up SCell(s) and activate for UE. When UL traffic arrives, UE can report to the PCell that more SCells are needed and would like to trigger on demand SSB procedure to measure more SCells to be activated later. PCell would then wake up SCells and activate for UE.

In this case, the SCell activation MAC CE can be reused to trigger on demand SSB and activate SCells at the same time.

Proposal 6: For case#1, NW should indicate to UE which SCell(s) would start to transmit SSB. Introduce new signaling or reuse SCell activation/deactivation MAC CE for this purpose.

    - Case #2: Always-on SSB is periodically transmitted on the cell

Figure 4: On demand SSB procedure for case#2

For case#2, the situation is slightly different as there would be always-on SSB periodically transmitted on the cell.

There is an issue: Whether UE would monitor the SSB from on demand SSB SCells, we need to consider.

If no, then the similar procedure as proposed above for case#1 applies:

Figure 5. On demand SSB procedure for case#2 without SSB monitoring at the beginning

--The SCell(s) supporting on demand SSB would be added but deactivated at first. UE needs to be aware of for which SCell(s) the on demand SSB is supported and would not monitor SSB on such cells. NW can indicate the on demand SSB SCells during SCell addition and modification.

--The on demand SSB procedure can be triggered by PCell when DL traffic arrives or by UE when UL traffic arrives.

--NW should indicate the SSB transmission update to UE and UE would start to monitor SSB of these cells upon reception of the indication.

Proposal 7: For case#2, if UE is not required to monitor SSB from on demand SSB SCells before triggered:

--The SCell(s) supporting on demand SSB would be added but deactivated at first.

--UE needs to be aware of for which SCell(s) the on demand SSB is supported and would not monitor SSB from these cells. NW can indicate the on demand SSB SCells during SCell addition and modification.

--NW should indicate the SSB transmission update to UE and UE would start to monitor SSB of these cells upon reception of the indication.

If Yes, then we need to consider the following options:

- Option 1: The on demand SSB procedure is triggered by NW, e.g. PCell.

In this option, the on demand SSB SCells can be added and deactivated as the existing procedure and UE would then not need to receive the corresponding PDCCH or PDSCH, but would still monitor SSB. UE does not need to be aware of these SCells as no specific handling is required.

Upon triggering of the on demand SSB procedure, NW should indicate to UE the change of the SSB transmission, e.g. with more frequency SSB transmission, and provide the corresponding SSB configuration to UE.

Figure 6. NW triggered on demand SSB for case#2 with SSB monitoring at the beginning

- Option 2: The on demand SSB procedure is triggered by UE.

In this option, the on demand SSB SCells can be added and deactivated as the existing procedure but still need to be aware of the on demand SCells if we would like UE to trigger the on demand SSB procedure.

Figure 7.UE triggered on demand SSB for case#2 with SSB monitoring at the beginning

Proposal 8: For case#2, if UE monitor SSB from on demand SSB SCells before triggered:

--The SCell(s) supporting on demand SSB would be added but deactivated at first.

--To support UE triggered procedure, UE needs to be aware of for which SCell(s) the on demand SSB is supported. NW can indicate the on demand SSB SCells during SCell addition and modification. If UE triggered procedure is not supported, such indication is not needed.

--NW should indicate the SSB transmission update to UE and UE would monitor SSB of these cells with updated SSB transmission.

- Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

Scenarios:

Proposal 1: RAN2 start with the following scenarios and cases for on demand SSB SCell operation with down-selection in future discussion:

- Scenario #2 and Case #1:

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

Case #1: No always-on SSB on the cell

Case #2: Always-on SSB is periodically transmitted on the cell

Scenario #2: On-demand SSB is triggered when SCell is configured to a UE but before the UE receives SCell activation command.

Scenario #2A: On-demand SSB is triggered when UE receives SCell activation command.

Proposal 2: RAN2 will wait for RAN1’s progress on other scenarios.

On demand SSB procedure:

Proposal 3: The on demand SSB procedure can be triggered by PCell when DL traffic arrives or by UE when UL traffic arrives.

Proposal 4: For case#1, the SCell(s) supporting on demand SSB would be added but deactivated at first.

Proposal 5: For case#1, UE needs to be aware of for which SCell(s) the on demand SSB is supported and would not monitor SSB from these cells. NW can indicate the on demand SSB SCells during SCell addition and modification.

Proposal 6: For case#1, NW should indicate to UE which SCell(s) would start to transmit SSB. Introduce new signaling or reuse SCell activation/deactivation MAC CE for this purpose.

Proposal 7: For case#2, if UE is not required to monitor SSB from on demand SSB SCells before triggered:

--The SCell(s) supporting on demand SSB would be added but deactivated at first.

--UE needs to be aware of for which SCell(s) the on demand SSB is supported and would not monitor SSB from these cells. NW can indicate the on demand SSB SCells during SCell addition and modification.

--NW should indicate the SSB transmission update to UE and UE would start to monitor SSB of these cells upon reception of the indication.

Proposal 8: For case#2, if UE monitor SSB from on demand SSB SCells before triggered:

--The SCell(s) supporting on demand SSB would be added but deactivated at first.

--To support UE triggered procedure, UE needs to be aware of for which SCell(s) the on demand SSB is supported. NW can indicate the on demand SSB SCells during SCell addition and modification. If UE triggered procedure is not supported, such indication is not needed.

--NW should indicate the SSB transmission update to UE and UE would monitor SSB of these cells with updated SSB transmission.

- Reference

- RP-240170 Revised WID for Enhancements of network energy savings for NR

- Draft Report of 3GPP TSG RAN WG1 #116b v0.3.0

- Annex - RAN1 Agreements on on-demand SSB

RAN1#116

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.

Agreement

- For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

  - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

  - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

    - This does not preclude SCell for which activation is completed

    - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement

- For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB.

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options.

  - FFS: How to define time instance A/B and the value of N per option.

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement).

RAN1#116bis

Agreement

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

- For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner.

  - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered.

- Notes:

  - Scenario #2A refers to

    - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

  - Scenario #3A refers to

    - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

  - Scenario #3B refers to

    - “When SCell activation is completed and SCell is activated” or

    - “After SCell activation is completed and SCell is activated”

  - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications.

  - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Agreement

For a cell supporting on-demand SSB SCell operation,

- Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not.

- For on-demand SSB on the cell, downselect between the following alternatives

  - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not.

  - Alt-2: On-demand SSB is limited to non-cell-defining SSB.

    - FFS: Further limitations to on-demand SSB

Agreement

For a cell supporting on-demand SSB SCell operation,

- L1 and/or L3 measurement based on on-demand SSB is supported for the cell.

  - FFS further details on L1 and/or L3 measurement

Agreement

The following agreement from RAN1#116 is modified (in red)

- For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

Agreement

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

RAN1#117

Agreement (Signalling)

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell.

- Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

- FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell.

  - This DCI signaling does not provide SCell activation/deactivation.

  - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc.

- FFS: Scenarios where the above signalings are applicable

Agreement (Contents) (amended as shown in red in Friday session)

For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer signaling is supported.

- Frequency of the on-demand SSB

- SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

- Periodicity of the on-demand SSB

- FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE

- FFS: Whether the RRC is newly introduced or existing RRC is reused

Agreement

- At least support L1 measurement based on on-demand SSB

  - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

    - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB.

Agreement

- For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

  - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - FFS: Whether the value of T is predefined or indicated/configured by gNB

  - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE.

- Sub-carrier spacing of the on-demand SSB

- Physical Cell ID of the on-demand SSB

- Location of on-demand SSB burst

- Downlink transmit power of on-demand SSB

- FFS: Other parameters

- FFS: Whether each of above parameters is configured/indicated explicitly or not