3GPP TSG-RAN WG2 Meeting #127										R2-2406447

Maastricht, Netherlands, Aug 19th – 23rd, 2024                       Revision of R2-2404860

Agenda item:		8.4.2

Title: 	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In RAN#102 meeting, a new WID on Low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved. In RAN#103 meeting, the WID was further refined in [1] with the following RAN2 related objectives:

The objectives of the work item are the following: - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

In RAN2#125bis meeting, the following agreements have been made for procedure and configuration of LP-WUS for IDLE and INACTIVE mode [2]:

- The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed. - Working assumption: the LP-WUS configuration in SIB at least includes the following information: - LP-SS configuration - LP-WUS configuration - FFS on Entry/exit condition for LP-WUS monitoring - The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

In RAN2#126 meeting, the following agreements have been made for procedure and configuration of LP-WUS for IDLE and INACTIVE mode [3]:

- RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. - The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - LP-SS configuration - LP-WUS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it is always configured) - Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. - Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. - RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. - RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. - From RAN2 perspective, no new procedure is introduced for SI reception/updates.

In this contribution, we will further discuss the RAN2 related issues on procedure and configuration of LP-WUS for IDLE/INACTIVE modes and give our proposals.

- Discussion

2.1 Provision of LP-WUS related configuration

During the SI stage, it has been discussed that the utilization of LP-WUS may be subject to the coverage performance of LP-WUS. But, in the description for WID objectives, it has been captured that “the target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3”. Based on the RAN1 agreement in RAN1#116bis, the LP-SS design and LP-WUS design have been discussed, but has not concluded yet. Considering that LP-SS configuration and LP-WUS configuration highly depend on RAN1 agreement for LP-SS and LP-WUS design, RAN2 can postpone the discussion for LP-WUS and LP-SS related configuration until there is agreement for the LP-SS and LP-WUS design in RAN1.

Proposal 1: RAN2 can postpone the discussion on LP-WUS and LP-SS related configuration until there is agreement for the LP-SS and LP-WUS design in RAN1.

2.2 Activation/deactivation of LP-WUS

Since it has been agreed in RAN2#125bis that LP-WUS related configuration for IDLE/INACTIVE state is provided via system information, the provision of LP-WUS configuration via SIB can be seen as an implicit activation to apply LP-WUS configuration for all the UEs supporting this function. Meanwhile, if the LP-WUS configuration is no longer broadcast via SIB, it can be assumed that application of LP-WUS configuration (and also the LP-WUS monitoring) is deactivated.

However, it’s also possible that the UE would not apply LP-WUS configuration immediately on reception of LP-WUS configuration and the application of LP-WUS configuration will be activated later, e.g., per-UE via a dedicated signaling, e.g., RRC connection release message. During SI stage, such process has been discussed but no decision has been made.

The main motivation to activate/deactivate LP-WUS for a specific UE or part of UEs via dedicated signaling is that it may be not suitable to always activate the LP-WUS monitoring for all the UEs as soon as the LP-WUS configuration is broadcast. For example, LP-WUS can be applied to diverse UE types, e.g., IIoT UE, (e)Redcap UE, NR UE with XR traffic and so on, the device type or traffic type of UE may be another consideration for activating or deactivating LP-WUS for a UE. For example, the UEs with delay-sensitive services may not be suitable to monitor LP-WUS considering the possible large wake up latency caused by LP-WUS, which may have unexpected impacts on the delay-sensitive service. The exact latency that may be increased by LP-WUS is still under evaluation in RAN1, e.g., to take into account the LP-WUS processing time, MR transition time for ramp up, Time/frequency synchronization of MR etc. Per our understanding, if the additional latency caused by LP-WUS is far larger than the duration that random access procedure may use, e.g. if the increased latency is several seconds or more, the LP-WUS in IDLE/INACTIVE modes should be activated/deactivated by UE dedicated signaling, e.g., to activate LP-WUS only for the UE without delay-sensitive services or to deactivate LP-WUS when the UE starts to perform delay-sensitive services.

In a summary, from RAN2 perspective, we can see the need to activate/deactivate the application of LP-WUS configuration via dedicated signaling. But this can be later decided after the RAN1 has clear conclusion on the potential latency caused by LP-WUS monitoring.

Observation 1: LP-WUS monitoring may result in additional latency compared to legacy paging procedure and hence, it is useful to support per UE activation of LP-WUS monitoring in addition to the per cell configuration.

Proposal 2: In addition to the SIB signaling for LP-WUS configuration, dedicated activation/deactivation of LP-WUS per UE should also be supported.

2.3 LP-WUS monitoring procedure and LP-WUS subgrouping

In RAN1#116 meeting [5], the following agreements have been achieved on the wake up procedure:

Agreement For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not. Agreement It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. - FFS: support of UE monitoring dynamic PO Conclusion For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

According to the RAN1 agreement, both “LP-WUS+ Paging” and “LP-WUS+PEI+paging” alternatives can be supported. Moreover, it can be seen that even though the PEI is configured, the UE may or may not monitor PEI. The result is that gNB cannot know whether a UE monitors PEI or not when the the condition for UE to monitor PEI is met. Therefore, the gNB should send LP-WUS, PEI and paging when both the condition for UE to monitor LP-WUS and the condition for UE to monitor PEI are met.

Observation 2: Per the RAN1 agreements, gNB should send LP-WUS, PEI and paging when when both the condition for UE to monitor LP-WUS and the condition for UE to monitor PEI are met.

When LP-WUS+PEI+Paging is used, the mapping between LP-WUS subgroup and PEI subgroup has the following possibilities.

- When one LP-WUS subgroup is mapped to one PEI subgroup (as shown in figure 1a), the PEI monitoring cannot reduce the false PO monitoring compared to the case without PEI monitoring (as shown in figure 1b) but the PEI monitoring will increase the UE power consumption. Therefore, the PEI monitoring should be avoided when one LP-WUS subgroup is mapped to one PEI subgroup.

Figure 1a. 1:1 with PEI monitoring                          Figure 1b. 1:1 without PEI monitoring

- When more than one LP-WUS subgroups are mapped to one PEI subgroup (as shown in figure 2a), the PEI monitoring cannot reduce the false PO monitoring compared to the case without PEI monitoring (as shown in figure 2b) eiter and the PEI monitoring will increase the UE power consumption.

Figure 2a. N:1 with PEI monitoring                          Figure 2b. N:1 without PEI monitoring

- When one LP-WUS subgroup is mapped to more than one PEI subgroups (as shown in figure 3a), the PEI monitoring can reduce the false PO monitoring compared to the case without PEI monitoring (as shown in figure 3b) and can reduce the UE power consumption.

Figure 3a. 1:N with PEI monitoring                          Figure 3b. 1:N without PEI monitoring

Observation 3: If the LP-WUS is activated, PEI monitoring is beneficial for decreasing false wakeup only when one LP-WUS subgroup is mapped to more than one PEI subgroups.

Thus, when the condition for UE to monitor PEI is met, the LP-WUS subgrouping should consider the PEI subgrouping. E.g. for UE ID based LP-WUS subgrouping, the PEI subgrouping should be considered, the following formula can be used to determine the LP-WUS subgroup in this case:

LP-WUS SubgroupID = floor (UE_ID/(N*Ns*subgroupsNumofPEI)) mod subgroupsNumForUEIDofLPWUS) + subgroupsNumofLPWUS - subgroupsNumForUEIDofLPWUS

Where:

subgroupsNumofPEI: total number of subgroups for both CN assigned PEI subgrouping (if any) and UE_ID based PEI subgrouping (if any) in a PO, which is broadcasted in system information.

subgroupsNumofLPWUS: total number of subgroups for both CN assigned LP-WUS subgrouping (if any) and UE_ID based LP-WUS subgrouping (if any) in a PO, which is broadcasted in system information

subgroupsNumForUEIDofLPWUS: number of subgroups for UE_ID based LP-WUS subgrouping in a PO, which is broadcasted in system information

Considering that in some cases, the condition for UE to monitor LP-WUS is met but the condition for UE to monitor PEI is not met, e.g. some UEs may only support LP-WUS and does not support PEI, in which case it is not necessary to care about the PEI subgroups when determining the LP-WUS subgroup, the following formula can be used to determine the LP-WUS subgroup in this case:

LP-WUS SubgroupID = floor (UE_ID/(N*Ns)) mod subgroupsNumForUEIDofLPWUS) + subgroupsNumofLPWUS - subgroupsNumForUEIDofLPWUS

Where:

subgroupsNumofLPWUS: total number of subgroups for both CN assigned LP-WUS subgrouping (if any) and UE_ID based LP-WUS subgrouping (if any) in a PO, which is broadcasted in system information

subgroupsNumForUEIDofLPWUS: number of subgroups for UE_ID based LP-WUS subgrouping in a PO, which is broadcasted in system information

Proposal 3:  Whether the condition for UE to monitor PEI is met should be considered to determine the LP-WUS subgroup.

Proposal 3a: When the condition for UE to monitor PEI is met, the UE determines the LP-WUS subgroup ID by the formula:

LP-WUS SubgroupID = floor (UE_ID/(N*Ns*subgroupsNumofPEI)) mod subgroupsNumForUEIDofLPWUS) + subgroupsNumofLPWUS - subgroupsNumForUEIDofLPWUS

Proposal 3b: When the condition for UE to monitor PEI is not met, the UE determines the LP-WUS subgroup ID by the formula:

LP-WUS SubgroupID = floor (UE_ID/(N*Ns)) mod subgroupsNumForUEIDofLPWUS) + subgroupsNumofLPWUS - subgroupsNumForUEIDofLPWUS

2.4 Time offset between LP-WUS and PO

In the RAN1#116 meeting [5], the following has been agreed regarding the time gap between LP-WUS reception and MR to start PDCCH monitoring for RRC CONNECTED mode:

Agreement For RRC CONNECTED mode, minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following - LP-WUS processing time - MR transition time for ramp up - Time/frequency synchronization of MR - FFS whether UE can report supported minimum time gap from candidate values FFS: Whether the minimum time gap values can be more than one.

Although there is no similar agreement for LP-WUS in IDLE/INACTIVE modes, the similar agreement is necessary to define the time gap between LP-WUS reception and MR to start PDCCH monitoring in IDLE/INACTIVE modes.

If LP-WUS+PEI+Paging is used, UE should monitor LP-WUS first, and if LP-WUS is detected, it then monitors PEI, the time gap between end of LP-WUS monitoring and MR monitoring should be relative to the PDCCH for first PEI. If UE skips the PEI monitor by its implementation, UE needs to take into account the above time gap when it determines the PO for monitoring paging.

If LP-WUS+Paging is used, UE should monitor LP-WUS first and if LP-WUS is detected, it then monitors paging or DCI, the LP-WUS timing offset should be relative to the PDCCH for first paging or DCI.

Proposal 4a: For LP-WUS monitoring in IDLE/INACTIVE modes, it is necessary to define the time gap between LP-WUS reception and the start for MR to monitor PDCCH. The following options can be discussed:

- When UE supports PEI and PEI is activated, LP-WUS timing offset is relative to the PDCCH for PEI;

- If UE does not support PEI or PEI is not activated, LP-WUS timing offset is relative to the PDCCH for PO.

Proposal 4b: The value of minimum time gap between LP-WUS reception and the start for MR to monitor PDCCH is up to RAN1 to discuss and decide.

2.5 UE capability

With reference to the discussion in previous sections, if no activation/deactivation indication is supported in dedicated signaling, as soon as that the LP-WUS configuration is broadcast via SIB, UEs that support LP-WUS in IDLE/INACTIVE modes would immediately apply the LP-WUS configuration and start to monitor the LP-WUS signal.

For this case, the paging gNB needs to be aware of whether a UE supports LP-WUS and then can know whether LP-WUS needs to be sent to the UE when CN has paging for UE. In other word, only when the LP-WUS configuration has been broadcast in the paging gNB and only for the UEs supporting LP-WUS monitor, the gNB needs to send LP-WUS to the relevant UEs. Therefore, the UE capability on supporting LP-WUS in IDLE/INACTIVE modes should be reported to the NW, e.g., within UE radio capability and further transmitted to the paging gNB via Paging message or RAN Paging.

Proposal 5: UE capability on supporting LP-WUS in IDLE/INACTIVE modes needs to be reported to serving gNB, e.g., via UECapabilityInformation message.

If activation/deactivation of LP-WUS in IDLE/INACTIVE modes via dedicated signaling is not supported, the paging gNB needs to be aware of UE’s capability of LP-WUS in IDLE/INACTIVE mode and decide whether to send LP-WUS to UE.

Proposal 6a: If activation/deactivation of LP-WUS in IDLE/INACTIVE modes via dedicated signaling is not supported, UE capability on supporting LP-WUS should be provided to the paging gNB e.g., via NGAP PAGING and RAN PAGING message.

If activation/deactivation of LP-WUS in IDLE/INACTIVE modes by dedicated signaling is supported, the gNB firstly needs to be aware of the UE capability of supporting LP-WUS in IDLE/INACTIVE modes and then can be able to activate/deactivate the LP-WUS for the UE by dedicated signaling.

Meanwhile, for the paging gNB, to only know the UE capability would be not sufficient as UE may not have been activated the use of LP-WUS, it also needs to know whether the LP-WUS is activated for this UE. Therefore, whether the LP-WUS monitoring has been activated for this UE (which can only be performed on UE supporting LP-WUS) needs to be transmitted to the paging gNB by Paging or RAN Paging.

Proposal 6a: If activation/deactivation of LP-WUS in IDLE/INACTIVE modes by UE dedicated signaling is supported, LP-WUS activation/deactivation indication should be provided to the paging gNB, e.g., via NGAP PAGING and RAN PAGING message.

- Conclusion

Provision of LP-WUS related configuration

Proposal 1: RAN2 can postpone the discussion on LP-WUS and LP-SS related configuration until there is agreement for the LP-SS and LP-WUS design in RAN1.

Activation/deactivation of LP-WUS

Observation 1: LP-WUS monitoring may result in additional latency compared to legacy paging procedure and hence, it is useful to support per UE activation of LP-WUS monitoring in addition to the per cell configuration.

Proposal 2: In addition to the SIB signaling for LP-WUS configuration, dedicated activation/deactivation of LP-WUS per UE should also be supported.

LP-WUS monitoring procedure and LP-WUS subgrouping

Observation 2: Per the RAN1 agreements, gNB should send LP-WUS, PEI and paging when when both the condition for UE to monitor LP-WUS and the condition for UE to monitor PEI are met.

Observation 3: If the LP-WUS is activated, PEI monitoring is beneficial for decreasing false wakeup only when one LP-WUS subgroup is mapped to more than one PEI subgroups.

Proposal 3:  Whether the condition for UE to monitor PEI is met should be considered to determine the LP-WUS subgroup.

Proposal 3a: When the condition for UE to monitor PEI is met, the UE determines the LP-WUS subgroup ID by the formula:

LP-WUS SubgroupID = floor (UE_ID/(N*Ns*subgroupsNumofPEI)) mod subgroupsNumForUEIDofLPWUS) + subgroupsNumofLPWUS - subgroupsNumForUEIDofLPWUS

Proposal 3b: When the condition for UE to monitor PEI is not met, the UE determines the LP-WUS subgroup ID by the formula:

LP-WUS SubgroupID = floor (UE_ID/(N*Ns)) mod subgroupsNumForUEIDofLPWUS) + subgroupsNumofLPWUS - subgroupsNumForUEIDofLPWUS

Time offset between LP-WUS and PO

Proposal 4a: For LP-WUS monitoring in IDLE/INACTIVE modes, it is necessary to define the time gap between LP-WUS reception and the start for MR to monitor PDCCH. The following options can be discussed:

- When UE supports PEI and PEI is activated, LP-WUS timing offset is relative to the PDCCH for PEI;

- If UE does not support PEI or PEI is not activated, LP-WUS timing offset is relative to the PDCCH for PO.

Proposal 4b: The value of minimum time gap between LP-WUS reception and the start for MR to monitor PDCCH is up to RAN1 to discuss and decide.

UE capability

Proposal 5: UE capability on supporting LP-WUS in IDLE/INACTIVE modes needs to be reported to serving gNB, e.g., via UECapabilityInformation message.

Proposal 6a: If activation/deactivation of LP-WUS in IDLE/INACTIVE modes via dedicated signaling is not supported, UE capability on supporting LP-WUS should be provided to the paging gNB e.g., via NGAP PAGING and RAN PAGING message.

Proposal 6a: If activation/deactivation of LP-WUS in IDLE/INACTIVE modes by UE dedicated signaling is supported, LP-WUS activation/deactivation indication should be provided to the paging gNB, e.g., via NGAP PAGING and RAN PAGING message.

- References

- RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, RAN#103, March, 2024

- Draft_R2_125bis_Meeting_Report_v1, RAN2#125bis, April, 2024

- Draft_R2_126_Meeting_Report_v1, RAN2#126, May, 2024

- Draft_Minutes_report_RAN1#116b_v030, RAN1#116bis, April, 2024

- Draft_Minutes_report_RAN1#116_v020-202402, RAN1#116, February, 2024

- Draft_Minutes_report_RAN1#117_v010, RAN1#117, February, 2024

- Annex

RAN1#116bis meeting agreements [4]

Agreement Regarding the LP-WUS information for idle/inactive UEs, at least consider the following: - Option 1: A bitmap with each bit corresponding to [one or more] subgroups - Option 2: A codepoint value corresponding to one or more subgroup(s) - Option 3: Multiple codepoint values with each corresponding to one or more subgroup(s) - Combination of above options are not precluded - FFS how to carry LP-WUS information, e.g., by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS. - FFS how to carry LP-WUS information by overlaid OFDM sequences. - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used - Other options are not precluded Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, at least consider the following： - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 2: A codepoint value corresponding to one or part of UE identity, e.g., C-RNTI - Option 3: A codepoint value corresponding to [one or more] UEs - Option 4: Multiple codepoint values with each corresponding to [one or more] UE(s) - Option 5: Multiple bit blocks with each corresponding to [one or more] UE(s) - Combination of above options are not precluded. - FFS how to carry LP-WUS information, e.g, by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS. - FFS how to carry LP-WUS information by overlaid OFDM sequences. - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used - FFS details of LP-WUS information to trigger PDCCH monitoring (e.g. whether above is applicable to one or more serving cells) Agreement Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam. - Option 1: K = 1 - Option 2: K can be larger than or equal to 1 - FFS if more than 1 LP-WUS is transmitted from the same beam, whether the information in these multiple LP-WUS is always the same or can be different Agreement From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS: - LP-RSRP - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols. - FFS: How to determine the received power of LP-SS in OOK ON symbols - LP-RSRQ - LP-RSRQ = LP-RSRP/LP-RSSI - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options: - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols. - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols. - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols. Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion.

RAN1#117 meeting agreements [6]

Agreement It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options: - Option 1: UEs monitoring the same PO monitor the same LO - Option 2: UEs corresponding to different POs monitor the same LO - Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. - Combinations of the above options can be considered. From Thursday session Agreement For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth. Agreement For LP-RSSI definition for LP-RSRQ, down-select between the following two options: - Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols. Note: Above does not constrain LP-SS sequence design for OOK. Agreement For idle/inactive mode, the maximum number of information bits (excluding CRC) in a LP-WUS is Z, where Z <= [8 or 16]. - FFS the exact value of Z Agreement For idle/inactive mode, the maximum number of subgroups per PO is X, where 8 <= X <= 256. - FFS the exact value of X.