3GPP TSG-RAN WG2 Meeting #126											R2-2406448

Maastricht, Netherlands, Aug 19th – 23rd, 2024                            Revision of R2-2404861

Agenda item:		8.4.3

Title: 	RRM measurement relaxation and offloading in RRC_IDLE and RRC_INACTIVE mode

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In RAN #102 meeting, a new WID on Low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved in [1].

In RAN2#126 meeting, the following agreements have been made for RRM measurement relaxation and offloading in RRC_IDLE and RRC_INACTIVE mode [3]:

- For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. - RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

In this contribution, we will discuss the RAN2 related issue on RRM measurement relaxation and offloading for IDLE/INACTIVE modes and give our proposals.

- Discussion

  - Serving cell measurement relaxation

Based on the current agreement in RAN2 that the RRM measurement of the neighbour cell can only be performed by MR, it can be assumed that RRM measurement relaxation is to relax the serving cell measurement when the LP-SS based measurement is performed. In NB-IoTand eMTC, serving cell measurement relaxation is supported, e.g. NW can configure numDRX-CyclesRelaxed in SIB, and the UE supporting WUS shall relax serving cell measurement requirement defined with the DRX cycle from the DRX cycle length of DRX_cycle to the DRX cycle length of min (DRX_cycle * numDRX-CyclesRelaxed, 10.24s), the similar mechanism can be used for NR serving cell measurement relaxation.

Proposal 1: The NB-IoT serving cell measurement relaxation mechanism is used as the baseline for NR serving cell measurement relaxation, e.g. NW can configure numDRX-CyclesRelaxed in SIB, and the NR UE supporting LP-SS based measurement may relax serving cell measurement requirement from M1*N1*DRX_cycle to min( M1*N1*DRX_cycle * numDRX-CyclesRelaxed, 10.24s).

In NB-IoTand eMTC, the coverage of WUS is the same as the coverage of cell. But in the NR, the coverage of LP-WUS may be less than or equal to the cell coverage, and the NR serving cell measurement relaxation can only be performed only in the coverage of the LP-WUS. Thus, cell edge condition(e.g. whether in the coverage of LP-WUS) should be considered for the NR serving cell measurement relaxation, e.g. only the RSRP of the serving cell is larger than an RSRP threshold and the RSRQ of the serving cell is larger than an RSRQ threshold(ifconfigured), the NR serving cell measurement relaxation can be performed.

Proposal 2: Cell edge condition (e.g. whether the UE is in the coverage of LP-WUS) should be considered for the NR serving cell measurement relaxation, e.g. only the RSRP of the serving cell is larger than an RSRP threshold and the RSRQ of the serving cell is larger than an RSRQ threshold (if configured), the NR serving cell measurement relaxation can be performed.

The serving cell measurement relaxation means that UE will relax the serving cell measurement based on SSB, but the UE should perform serving cell measurement based on LP-SS, whether the same measurement accuracy can be achieved for the serving cell in the serving cell measurement relaxation state and serving cell measurement non-relaxation state depends on the RAN4 evaluation. If the same measurement accuracy can be achieved, then the serving cell measurement result in the serving cell measurement relaxation state can also be used for evaluating the cell reselection condition, and “low mobility” condition need not to be considered for the NR serving cell measurement relaxation.

Proposal 3: Wait for RAN4 input on whether other conditions are required for serving cell measurement relaxation.

  - Neighbor cell measurement relaxation

Since RAN2 understands that the RRM measurement of the neighboring cell can only be performed by MR, the neighboring cell measurement accuracy has not changed when introducing LP-WUS, thus the neighbor cell measurement relaxation should not be changed, e.g. the legacy Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’ can reused for  neighbour cell measurement relaxation.

Proposal 4: The legacy Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’ are reused for neighbour cell measurement relaxation.

  - Serving cell measurement offloading

In order to achieve the UE power saving gain by LP-WUS/WUR, the RRM measurement on serving cell and neighbouring cell via MR may be skipped when UE is using LP-WUS or MR is in ultra-deep sleep. However, UE still needs to ensure that IDLE/INACTIVE mode measurements are possible during this state. In order to avoid or minimize the performance degradation of mobility handling in RRC_IDLE or RRC_INACTIVE when MR is stopped, RAN2 has already agreed in SI stage [2] that: ultra deep sleep state could be entered when the serving cell quality of both LP-SS and MR SSB is larger than a threshold, e.g. UE is not at cell edge. As legacy, the relaxed measurement criterion for UE not at cell edge [3] can be used to decide whether the UE is at cell edge. Whether a threshold is separately defined for LP-SS and MR SS can be further considered, e.g. depending on the measurement performance of LP-WUS defined by RAN4.

-------------------------------Relaxed measurement criterion for UE not at cell edge in 38.304------------------------------

The relaxed measurement criterion for UE not at cell edge is fulfilled when:

-	Srxlev > SSearchThresholdP, and,

-	Squal > SSearchThresholdQ, if SSearchThresholdQ is configured,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	Squal = current Squal value of the serving cell (dB).

-------------------------------Relaxed measurement criterion for UE not at cell edge in 38.304------------------------------

Proposal 5: RAN2 confirms that serving cell quality threshold based on LP-SS and MR SSB is used for UE to decide whether to enter MR serving cell measurement offloading for UE in IDLE/INACTIVE modes, e.g. to decide whether the UE is at cell edge. And whether the threshold should be separately defined for LP-SS and MR SSB or not needs RAN4 input.

Furthermore, some UEs may move quickly, and the resumption of Main Radio may require longer duration (e.g. several seconds), the serving cell measurement relaxation of UE MR based on LP-WUS should consider UE mobility, e.g. only the UEs with low mobility can apply the serving cell measurement relaxation of UE MR. As legacy, the Relaxed measurement criterion for UE with low mobility [3] can be used to decide whether the UE is in low mobility.

------------------------------Relaxed measurement criterion for UE with low mobility in 38.304------------------------------

The relaxed measurement criterion for UE with low mobility is fulfilled when:

-	(SrxlevRef – Srxlev) < SSearchDeltaP,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows:

-	After selecting or reselecting a new cell, or

-	If (Srxlev - SrxlevRef) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP:

-	The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell.

------------------------------Relaxed measurement criterion for UE with low mobility in 38.304------------------------------

Proposal 6a: The relaxed measurement criterion for UE with low mobility can be reused for UE to decide whether to enter MR serving cell measurement offloading for UE in IDLE/INACTIVE mode.

- Case 1: UE has not perform MR serving cell measurement offloading and may perform MR serving cell measurement on MR SSB and LP-SS

When the UE is not performing MR serving cell measurement offloading based on LP-SS, the UE may perform RRM measurements based on MR SSB and LP-SS, the UE can decide whether it is in low mobility state based on MR SSB and/or LP-SS change.

Proposal 6b: When the UE is not performing MR serving cell measurement offloading, serving cell quality change based on MR SSB and LP-SS measurement can be used to determine whether the UE is in low mobility state.

- Case 2: UE has performed MR serving cell measurement offloading and perform MR serving cell measurement only on LP-SS

While the UE performing MR serving cell measurement offloading, UE perform serving cell measurement based on LP-SS only. If the UE determines that it is neither in stationary state nor is at the cell edge based on the LP-SS measurement, the UE starts to perform serving cell measurements based on main radio (e.g. MR SSB).

Proposal 6a: The UE performing MR RRM measurement offloading should resume measurements based on MR for serving cell measurements when the UE is neither in low mobility state nor at the cell edge.

As legacy, the Relaxed measurement criterion for UE with low mobility (e.g. the LP-SS RSRP change value is less than a threshold) can be used to decide whether the UE is in low mobility, and relaxed measurement criterion for UE not at cell edge (e.g. the LP-SS RSRP value is lower than an RSRP threshold and the LP-SS RSRQ value is lower than an RSRQ threshold, if configured) can be used to decide whether the UE is at the cell edge.

Proposal 6b: When the UE is performing serving cell measurement based on LP-SS only, whether it is in low mobility state is determined based on whether LP-SS RSRP change is less than a threshold, and whether it is at the cell edge is determined based on whether the LP-SS RSRP value is lower than a threshold and the LP-SS RSRQ value is lower than an RSRQ threshold, if configured.

- Conclusion

Serving cell measurement relaxation

Proposal 1: The NB-IoT serving cell measurement relaxation mechanism is used as the baseline for NR serving cell measurement relaxation, e.g. NW can configure numDRX-CyclesRelaxed in SIB, and the NR UE supporting LP-SS based measurement may relax serving cell measurement requirement from M1*N1*DRX_cycle to min( M1*N1*DRX_cycle * numDRX-CyclesRelaxed, 10.24s).

Proposal 2: Cell edge condition (e.g. whether the UE is in the coverage of LP-WUS) should be considered for the NR serving cell measurement relaxation, e.g. only the RSRP of the serving cell is larger than an RSRP threshold and the RSRQ of the serving cell is larger than an RSRQ threshold (if configured), the NR serving cell measurement relaxation can be performed.

Proposal 3: Wait for RAN4 input on whether other conditions are required for serving cell measurement relaxation.

Neighbor cell measurement relaxation

Proposal 4: The legacy Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’ are reused for neighbour cell measurement relaxation.

Serving cell measurement offloading

Proposal 5: RAN2 confirms that serving cell quality threshold based on LP-SS and MR SSB is used for UE to decide whether to enter MR serving cell measurement offloading for UE in IDLE/INACTIVE modes, e.g. to decide whether the UE is at cell edge. And whether the threshold should be separately defined for LP-SS and MR SSB or not needs RAN4 input.

Proposal 6a: The UE performing MR RRM measurement offloading should resume measurements based on MR for serving cell measurements when the UE is neither in low mobility state nor at the cell edge.

Proposal 6b: When the UE is performing serving cell measurement based on LP-SS only, whether it is in low mobility state is determined based on whether LP-SS RSRP change is less than a threshold, and whether it is at the cell edge is determined based on whether the LP-SS RSRP value is lower than a threshold and the LP-SS RSRQ value is lower than an RSRQ threshold, if configured.

- References

- RP-240801 Revised WID Low-power wake-up signal and receiver for NR (LP-WUS_WUR) , RAN#103

- Draft_R2_126_Meeting_Report_v1, RAN2#126, May, 2024

- 3GPP TR 38.869 V18.0.0 (2023-12), Study on low-power wake-up signal and receiver for NR

- 3GPP TR 38.304 V18.1.0 (2024-03), NR;User Equipment (UE) procedures in Idle mode and RRC Inactive state