3GPP TSG-RAN WG2 Meeting #126										R2-2406449

Maastricht, Netherlands, Aug 19th – 23rd, 2024                      Revision of R2-2404862

Agenda item:		8.4.4

Title: 	Procedures for LP-WUS in RRC_CONNECTED

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In RAN#102 meeting, a new WID on Low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved. In RAN#103 meeting, the WID was further refined in [1] with the following objective for LP-WUS in connected:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

In RAN2#126 meeting, the following agreements have been made for RRM measurement relaxation and offloading in RRC_IDLE and RRC_INACTIVE mode [3]:

- In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. - For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. - RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. - The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In this contribution, we will further discuss the RAN2 related issues on procedure of LP-WUS for RRC_CONNECTED state and give our proposals.

- Discussion

2.1 Activation/deactivation for LP-WUS monitoring in RRC_CONNECTED state

It has been agreed that the LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message. Considering that the coverage of LP-WUS may be less than or equal to the cell coverage, even the LP-WUS related configuration is provided, the LP-WUS may not be monitored, e.g. when the UE is at the cell edge. The LP-WUS monitoring activation/deactivation should be considered.

Usually, there are two solutions for LP-WUS monitoring activation/deactivation:

- Solution 1: similar as RRC_IDLE/RRC_INACTIVE state, a cell edge condition (e.g. RSRP threshold) is configured to UE, and UE monitors LP-WUS when it is not at the cell edge. For this solution, Network cannot know whether the UE is monitoring the LP-WUS and should always send LP-WUS, whichever the UE monitors it or not.

- Solution 2: Network activate or deactivate the LP-WUS explicitly, e.g. with RRC message, DCI or MAC CE. For this solution, UE need monitor LP-WUS only when it is activated by network, and network can activate or deactivate the LP-WUS based on the CQI info or UE assistance information reported by UE.

To keep alignment between UE LP_WUS monitoring and gNB LP_WUS transmission, solution 2 is preferred.

Proposal 1:LP_WUS for RRC_CONNECTED is activated or deactivated by MAC CE.

## 2.2 LP-WUS monitoring schemes with C-DRX configured

RAN1 has identified the following 3 options for the LP-WUS monitoring with C-DRX configured

About Option 1-1 in RAN1 agreement for LP-WUS monitoring for RRC_CONNECTED state.

- Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

  - This option may replace DCP functionality

In RAN2 #126 meeting, the following agreements have been made for Option 1-1:

- For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. - RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. - The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

Option 1-1 can be taken as the baseline for LP-WUS monitoring with C-DRX configured, and RAN2 can study the LP-WUS monitoring with C-DRX configured on top of the Option-1-1.

About Option 1-2 in RAN1 agreement for LP-WUS monitoring for RRC_CONNECTED state

- Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

  - PDCCH monitoring possibly irrespective of drx-onDurationTimer

    - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

      - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX

    - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

This option can be used to decrease the service delay, e.g. when DL data burst arrives outside the C-DRX active time, LP-WUS can trigger the UE to enter the C-DRX active time and monitor PDCCH for DL scheduling.

Since RAN1 has decided not to support LP-WUS without C-DRX configured, but legacy C-DRX on-duration timer can be started only in some fixed occasion, which may lead large delay for non-periodic traffic if large C-DRX cycle, drx-onDurationTimer and drx-InactivityTimer are configured. Considering that not all the traffic are semi-static, e.g. the bursts do not always arrive periodically at a certain duration, it is difficut to configure a suitable C-DRX cycle, drx-onDurationTimer and drx-InactivityTimer to cover all the possible burst arrive occasion, e.g. if the value drx-onDurationTimer and drx-InactivityTimer are set too long, the UE power will be cost too much; if the value drx-onDurationTimer and drx-InactivityTimer are set too short, the burst amy arrive aoutside the C-DRX active time, and the data transmission delay will be too much. With the LP-WUS monitoring outside the legacy C-DRX active time, small values of drx-onDurationTimer and drx-InactivityTimer can be set for the traffic with non-periodic traffic so that UE only monitor LP-WUS when there is no burst arriving, which can save UE power and the delay for non-periodic traffic can be decreased much. Considering that LP-WUS monitoring will cost little UE power, both the latency reduction and UE power saving gain can be achieved.

Observation 1: Using Option 1-2, both the latency reduction and UE power saving gain can be achieved.

With Option 1-2, the LP-WUS monitoring occasion should be defined. Similar as the C-DRX start occasion, a LP_WUS cycle and LP-WUS start offset can be defined. And for compatible with option1-1, the LP_WUS cycle should be a factor of C-DRX cycle, and the time offset in option 1-1 is used, e.g. if LP-WUS is configured and activated, the UE monitors the LP-WUS at the SFN and subframe determined by:

[ (SFN × 10) + subframe number -( drx-StartOffset - timeOffset)] modulo (LP_WUS-cycle)) =0:

Where LP_WUS-cycle and timeOffset are configured by network, LP_WUS-cycle is an integer factor of C-DRX-cycle and timeOffset is the time GAP between LP-WUS occasion and the start of drx-onDurationTimer.

Proposal 2: Option 1-2 is supported, e.g. UE can monitor LP-WUS with a LP-WUS cycle during the non-active time of C-DRX cycle if LP-WUS for RRC_CONNECTED state is configured and activated.

Proposal 2a: UE starts drx-OnDurationTimer after a timeOffset when it detects LP-WUS.

Proposal 2b: UE determines the LP-WUS monitoring occasion with the similar formula as that to determine the start of drx-onDurationTimer, e.g. the UE monitors LP-WUS during the non-active time of C-DRX cycle if:

[(SFN × 10) + subframe number-(drx-StartOffset - timeOffset)] modulo (LP_WUS-cycle)) = 0

About Option 1-3 in RAN1 agreement for LP-WUS monitoring for RRC_CONNECTED state

    - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

This option can be used to decrease the UE power when intermittent DL data burst arrives at gNB, e.g. in the C-DRX active time, only if LP-WUS is detected, UE monitors PDCCH. Since LP-WUS consumes little UE power, a small C-DRX cycle, large drx-OndurationTimer and/or large drx-inactiveTimer can be configured to keep the UE always in the DRX active time. This solution can also reduce the UE power consumption, but the following issue should be considered: Whether the LP-WUS will be monitored when the HARQ timer is running. If LP-WUS will be monitored when the HARQ timer is running, the UE performance should be evaluated, e.g. whether UE can monitor LP-WUS and perform main radio Tx/Rx simultaneously; if LP-WUS will be monitored only after the HARQ timer expires, the LP-WUS monitoring may lead PDCCH monitoring delay, and further lead traffic transmission delay, which depends on the RAN1 design for LP-WUS, e.g. the time GAP between LP-WUS monitoring and PDCCH monitoring.

Observation 2: Option 1-3 can also reduce UE power consumption, meanwhile keep small data transmission latency, but the UE performance and the PDCCH monitoring delay caused for LP-WUS monitoring should be evaluated by RAN1.

Proposal 3: Option 1-3 is not supported, e.g. UE does not monitor LP-WUS during the active time of C-DRX cycle.

- Conclusion

Proposal 1:LP_WUS for RRC_CONNECTED is activated or deactivated by MAC CE.

Observation 1: Using Option 1-2, both the latency reduction and UE power saving gain can be achieved.

Proposal 2: Option 1-2 is supported, e.g. UE can monitor LP-WUS with a LP-WUS cycle during the non-active time of C-DRX cycle if LP-WUS for RRC_CONNECTED state is configured and activated.

Proposal 2a: UE starts drx-OnDurationTimer after a timeOffset when it detects LP-WUS.

Proposal 2b: UE determines the LP-WUS monitoring occasion with the similar formula as that to determine the start of drx-onDurationTimer, e.g. the UE monitors LP-WUS during the non-active time of C-DRX cycle if:

[(SFN × 10) + subframe number-(drx-StartOffset - timeOffset)] modulo (LP_WUS-cycle)) = 0

Observation 2: Option 1-3 can also reduce UE power consumption, meanwhile keep small data transmission latency, but the UE performance and the PDCCH monitoring delay caused for LP-WUS monitoring should be evaluated by RAN1.

Proposal 3: Option 1-3 is not supported, e.g. UE does not monitor LP-WUS during the active time of C-DRX cycle.

- References

- RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, RAN#103, March, 2024

- Draft_Minutes_report_RAN1#116b_v030, RAN1#116bis, April, 2024