3GPP TSG-RAN WG2 Meeting #127	R2-2406451

Maastricht, Netherlands, August 19th– 23rd, 2024

Agenda Item:	7.7.2

Source: 	vivo

Title:  	Remaining Issues on FR2-NTN Support

Document for:	Discussion and Decision

- Introduction

In RAN2#125bis and RAN2#126, the issue on NTN specific capability appliable to FR2-NTN was discussed, and finally agreed to add clarification on the applicability to the FR2-NTN band in addition to the FR1 NTN band.

In the contribution, we will further discuss the remaining issues on FR2-NTN.

- Discussion

  - Enhanced RRM requirements for measurements in IDLE/INACTIVE for FR2-NTN band

The following capability was introduced in R17 to indicate whether UE supports the enhanced RRM requirements for measurement for NTN bands [1]. But since in R17, no FR2-NTN band is supported, so it was restricted to the FR1 FDD NTN bands only.

Enhanced RRM requirements for measurements in IDLE and INACTIVE modes It is optional for UE to support enhanced RRM requirements for measurements for NTN bands (FR1 only and FDD only) in RRC_IDLE/RRC_INACTIVE as specified in TS 38.133 [5]. If UE does not support this feature, other NTN measurement requirements (as specified in TS 38.133 [5], clause 4.2C.2 for RRC_IDLE and clause 5.1C.2 for RRC_INACTIVE) are applied.

Observation 1: The R17 UE capability to indicate whether UE supports the enhanced RRM requirements is restricted to FR1 NTN bands since the FR2-NTN bands are not supported in R17.

Besides, in R18, RAN4 introduce the RRM requirements for FR2-NTN, e.g., by reusing the FR1 requirements including both the enhanced requirements and other NTN requirements [2][3]. However, the above UE capability defined by RAN2 in R17 cannot be reused since it is limited to FR1 bands. To address the issue, it is proposed to introduce another new UE capability to indicate whether the UE support the enhanced RRM requirements for FR2-NTN bands, and it is only appliable to R18 UEs. The corresponding draft TP can be found in the annex 1.

The requirements in clause 4.2C shall apply for the quasi-earth fixed cell and the earth moving cell. The requirements in clause 4.2C apply to FR1-NTN and FR2-NTN as defined in TS 38.101-5. The requirements in clause 4.2C apply to FR2-NTN with the following assumption: - no inter-satellite measurement is configured; - single SAN Tx beam per radio cell in DL; - same UE Rx beam is used for both serving and neighboring cells which belong to the same satellite.

Proposal 1: Introduce a new R18 UE capability without signalling for enhanced FR2-NTN bands RRM requirements.

Proposal 2: RAN2 sends an LS to check with RAN4 on the introduction of the new R18 UE capability.

  - Clarification on the term FR2-1 (band) and FR2-NTN (band)

According to TS38.101-2 [4], in TN network, the FR2 band can be further divided into FR2-1 and FR2-2 band.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specification are identified as described in Table 5.1-1. Whenever the FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be considered, unless otherwise stated. Table 5.1-1: Definition of frequency ranges Frequency range designation; Corresponding frequency range. FR1; 410 MHz – 7125 MHz. FR2; FR2-1; 24250 MHz – 52600 MHz. FR2-2; 52600 MHz – 71000 MHz.

Further in R18, the satellite operation bands for FR2 were introduced, which is referred as to the FR2-NTN band [5]. And according to the NOTE 2, in case the term FR2 band is used, the FR2-NTN band can also be considered to be included. If the FR2-1 is used, it cannot indicate that the FR2-NTN is also included.

Table 5.1-1: Definition of NTN frequency ranges Frequency range designation; Corresponding frequency range. FR1-NTN1; 410 MHz – 7125 MHz. FR2-NTN2; 17300 MHz – 30000 MHz. NOTE 1: [NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.] NOTE 2: [NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.].

Observation 2: According to the current RAN4 spec, the term “FR2” also indicates the FR2-NTN, while the term “FR2-1” cannot indicate the FR2-NTN.

When we discussed the NTN capability, some of the TN capability definitions are reused. However, they only refer to the term FR2-1 (bands) when describing which band the capability is applicable. The following are some examples extracted from the latest TS38.306 i20 [1].

The UE may support different functionalities between FDD and TDD, and/or between FR1 and FR2. The UE shall indicate the UE capabilities as follows. In the table of UE capability parameter in subsequent clauses, "Yes" in the column by "FDD-TDD DIFF" and "FR1-FR2 DIFF" indicates the UE capability field can have a different value for between FDD and TDD or between FR1 and FR2 and "No" indicates if it cannot. "(Incl FR2-2 DIFF)" in the column by "FR1-FR2 DIFF" indicates the UE capability field can have a different value for between FR2-1 and FR2-2.

channelBWs-DL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-DL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For IAB-MT, to determine whether the IAB-MT supports a channel bandwidth of 100 MHz, the network checks channelBW-DL-IAB-r16. For FR1, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For IAB-MT the third / rightmost bit (for 200MHz) is ignored. To determine whether the IAB-MT supports a channel bandwidth of 200 MHz, the network checks channelBW-DL-IAB-r16. For FR1, the leading/leftmost bit in channelBWs-DL-v1590 indicates 70MHz, the second leftmost bit indicates 45MHz, the third leftmost bit indicates 35MHz, the fourth leftmost bit indicates 100MHz and all the remaining bits in channelBWs-DL-v1590 shall be set to 0. The fourth leftmost bit (for 100MHz) is not applicable for bands n41, n48, n77, n78, n79 and n90 as defined in TS 38.101-1 [2]. For each band, (e)RedCap UEs shall indicate supporting the maximum of those channel bandwidths that are less than or equal to 20 MHz for FR1 and less than or equal to 100 Mhz for FR2, taking restrictions in TS 38.101-1 [2] and TS 38.101-2 [3] into consideration. For each band, NTN capable UEs shall indicate the supported channel bandwidths for FR1, taking restrictions in TS 38.101-5 [34] into consideration. This feature is applicable only for FR1 and FR2-1 band, otherwise it is absent.

multiPUSCH-SingleDCI-FR2-1-SCS-120kHz-r17 Indicates whether the UE supports multi-PUSCH scheduling by single DCI for the operation with 120kHz SCS in FR2-1 with non-contiguous allocation.

multiPDSCH-SingleDCI-FR2-1-SCS-120kHz-r17 Indicates whether the UE supports multi-PDSCH scheduling by single DCI for the operation with 120kHz SCS in FR2-1 and HARQ enhancements for both type 1 and type 2 HARQ codebook.

NOTE 1: For a UE capability which cannot be differentiated between FR2-1 and FR2-2, 'FR2 TDD' in Table B-1 includes both 'FR2-1 TDD' and 'FR2-2 TDD'. NOTE 2: For a UE capability which can be differentiated between FR2-1 and FR2-2, 'FR2 TDD' in Table B-1 only means 'FR2-1 TDD'.

Moreover, there are also some RRC parameters that refer only to the FR2-1 bands (not capturing the FR2-NTN bands), and hence the corresponding interpretation/definitions for FR2 NTN is unclear. The following is an example extracted from the TS 38.331.

TS 38.331: BWP ::= SEQUENCE { locationAndBandwidth INTEGER (0..37949), subcarrierSpacing SubcarrierSpacing, cyclicPrefix ENUMERATED { extended } OPTIONAL -- Need R } subcarrierSpacing Subcarrier spacing to be used in this BWP for all channels and reference signals unless explicitly configured elsewhere. Corresponds to subcarrier spacing according to TS 38.211 [16], table 4.2-1. The value kHz15 corresponds to µ=0, value kHz30 corresponds to µ=1, and so on. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

Observation 3: According to the TS 38.306 and TS 38.331, only the term FR2-1(bands) is referred even if the UE capability also applies to the FR2-NTN (bands).

To address the issue, two options can be considered.

Option 1: Check the TS38.306 and TS 38.331 specs case by case for each capability, and add the missing FR2-NTN if the capability or the description can also apply for NTN FR2- NTN but it only refers to the FR2-1;

Option 2: Add a clarification that “NTN bands within this frequency range of FR2-NTN are regarded as a FR2-1 band when it is referenced”, so that when we refer FR2-1 (bands), the FR2-NTN (bands) is also considered to be referred.

If we go with option 1, careful review of the spec is needed so as to find all places where such an issue exits, and this may be not easy given there are so many UE capabilities there. And further if in a later phase, a new frequency band is added, maybe similar work may also need to be repeated. Further from RAN2 perspective, option 2 is feasible without any technical issue. The only issue is this is not aligned with the RAN4 spec (where they do not think the FR2-1 can also represent the FR2-NTN), there may be risks. Considering the spec impacts and the further spec maintenance workload, it is preferred to select option 2, and send LS to RAN4 to ask them to check whether the understanding is correct.

Proposal 3: For 38.306 and 38.331, it is RAN2's understanding that NTN bands within FR2-NTN can be regarded as a FR2-1 band.

Proposal 4: If proposal 2 is agreed, RAN2 sends LS to check with RAN4 whether the understanding is correct.

Regarding proposals 3 and 4, a draft LS is attached in the annex 2.

  - Clarification on NTN related UE capabilities and parameters for FDD-FR2 NTN bands

In RAN2#125bis and RAN2#126, based on the contribution [6][7], some NTN related UE capabilities for NTN FDD-FR2 were discussed and some clarification is added in the field description to indicate the support of the NTN FDD-FR2, the main reason to do such clarification is FR2-NTN is introduced in R19 and such FR2-NTN bands are in FDD mode, while in legacy the FR2 bands are in TDD mode only.

However, we reviewing the latest TS38.306 i20 [1], we observed that there are also some UE capabilities also appliable to NTN, but it only refers the term TDD-FR2-1 bands, e.g., configuredUL-GrantType1-v1650, configuredUL-GrantType2-v1650, enhancedSkipUplinkTxConfigured-v1660, enhancedSkipUplinkTxDynamic-v1660, pusch-RepetitionMultiSlots-v1650, pusch-RepetitionTypeA-v16c0, rlm-Relaxation-r17 and bfd-Relaxation-r17.

The following is one example extracted from the TS38.306 [1].

configuredUL-GrantType1-v1650 Indicates whether the UE supports Type 1 PUSCH transmissions with configured grant as specified in TS 38.214 [12] with UL-TWG-repK value of one. This applies only to non-shared spectrum channel access. For shared spectrum channel access, configuredUL-GrantType1-r16 applies. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. The UE only includes configuredUL-GrantType1-v1650 if configuredUL-GrantType1 is absent.

Proposal 5: Updating the following description of the NTN related UE capabilities to indicate the support of NTN FDD-FR2 bands:

- configuredUL-GrantType1-v1650;

- configuredUL-GrantType2-v1650;

- enhancedSkipUplinkTxConfigured-v1660;

- enhancedSkipUplinkTxDynamic-v1660;

- pusch-RepetitionMultiSlots-v1650;

- pusch-RepetitionTypeA-v16c0;

- rlm-Relaxation-r17;

- bfd-Relaxation-r17.

- Conclusions

This contribution discussed the remaining issues on FR2-NTN in Rel-18 specs. Observations and Proposals are summarized as follows,

Observation 1: The R17 UE capability to indicate whether UE supports the enhanced RRM requirements is restricted to FR1 NTN bands since the FR2-NTN bands are not supported in R17.

Proposal 1: Introduce a new R18 UE capability without signalling for enhanced FR2-NTN bands RRM requirements.

Proposal 2: RAN2 sends an LS to check with RAN4 on the introduction of the new R18 UE capability.

Observation 2: According to the current RAN4 spec, the term “FR2” also indicates the FR2-NTN, while the term “FR2-1” cannot indicate the FR2-NTN.

Observation 3: According to the TS 38.306 and TS 38.331, only the term FR2-1(bands) is referred even if the UE capability also applies to the FR2-NTN (bands).

Observation 3: According to the TS38.306, only the term FR2-1(bands) is referred even the UE capability also applies to the FR2-NTN (bands).

Proposal 3: For 38.306 and 38.331, it is RAN2's understanding that NTN bands within FR2-NTN can be regarded as a FR2-1 band.

Proposal 4: If proposal 2 is agreed, RAN2 sends LS to check with RAN4 whether the understanding is correct.

Proposal 5: Updating the following description of the NTN related UE capabilities to indicate the support of NTN FDD-FR2 bands:

- configuredUL-GrantType1-v1650;

- configuredUL-GrantType2-v1650;

- enhancedSkipUplinkTxConfigured-v1660;

- enhancedSkipUplinkTxDynamic-v1660;

- pusch-RepetitionMultiSlots-v1650;

- pusch-RepetitionTypeA-v16c0;

- rlm-Relaxation-r17;

- bfd-Relaxation-r17.

- Reference

- TS38.306 i20, TS38.821, Solutions for NR to support Non-Terrestrial Networks (NTN).

- TS38.133 i60, TS38.821, Solutions for NR to support Non-Terrestrial Networks (NTN).

- R4-2317374, RRM WF and CR work split for NR_NTN_enh.

- TS38.101-2, NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone.

- TS38.101-5, NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements.

- R2-2403069, Discussion on NTN FR2 UE capability, ZTE Corporation, Sanechips

- R2-2405080, Consideration on remaining FR2 UE capability issues	ZTE, Corporation, Sanechips, Eutelsat Group.

- Annex 1 TP on TS38.306(Proposal 1)

5.6	RRM measurement features

Definitions for feature

Enhanced inter-frequency IDLE/INACTIVE measurements for HST FR2 It is optional for UE to support RRM requirement for inter-frequency measurements in idle and inactive mode to support FR2 high speed up to 350 km/h, as specified in TS 38.133 [5]. A UE supporting this feature shall also indicate support of PC6 in ue-PowerClass-v1700.

Enhanced RRM requirements for measurements in IDLE and INACTIVE modes It is optional for UE to support enhanced RRM requirements for measurements for NTN bands (FR1 only and FDD only) in RRC_IDLE/RRC_INACTIVE as specified in TS 38.133 [5]. If UE does not support this feature, other NTN measurement requirements (as specified in TS 38.133 [5], clause 4.2C.2 for RRC_IDLE and clause 5.1C.2 for RRC_INACTIVE) are applied.

Enhanced RRM requirements for measurements in IDLE and INACTIVE modes for FR2 It is optional for UE to support enhanced RRM requirements for measurements for FDD FR2-NTN bands in RRC_IDLE/RRC_INACTIVE as specified in TS 38.133 [5]. If UE does not support this feature, other NTN measurement requirements (as specified in TS 38.133 [5], clause 4.2C.2 for RRC_IDLE and clause 5.1C.2 for RRC_INACTIVE) are applied.

Enhanced RRM requirements for measurements in IDLE and INACTIVE modes for ATG It is optional for the UE in RRC_IDLE/RRC_INACTIVE to support the enhanced inter-frequency cell re-selection requirements for ATG (as specified in TS 38.133 [5], Table 4.2D.2.4-2). If UE does not support this feature, other measurement requirements as specified in TS 38.133 [5], Table 4.2D.2.4-1 are applied.

High speed inter-frequency IDLE/INACTIVE measurements It is optional for UE to support high speed inter-frequency measurements in RRC_IDLE/RRC_INACTIVE as specified in TS 38.133 [5].

Location-based measurement initiation It is optional for the UE in RRC_IDLE/RRC_INACTIVE to support location based RRM measurements of neighbour cells in NTN quasi-Earth fixed cell as specified in TS 38.304 [21].

Location-based measurement initiation for NTN Earth-moving cell It is optional for the UE in RRC_IDLE/RRC_INACTIVE to support location based RRM measurements of neighbour cells in NTN Earth-moving cell as specified in TS 38.304 [21].

Relaxed measurement It is optional for UE to support relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21].

Rel-17 relaxed measurement for RRC_IDLE/RRC_INACTIVE It is optional for (e)RedCap UE to support Rel-17 relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21].

Skipping TN measurements It is optional for the UE in RRC_IDLE/RRC_INACTIVE to support skipping the neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage as specified in TS 38.304 [21].

Time-based measurement initiation It is optional for the UE in RRC_IDLE/RRC_INACTIVE to support time based RRM measurements of neighbour cells in NTN quasi-Earth fixed cell as specified in TS 38.304 [21].

Time-based measurement initiation for NTN Earth-moving cell It is optional for the UE in RRC_IDLE/RRC_INACTIVE to support time based RRM measurements of neighbour cells in NTN Earth-moving cell as specified in TS 38.304 [21].

- Annex 2 Draft LS to RAN4(Proposal 3)

Title:	[Draft] LS on clarification on the term FR2-1 (band) and FR2-NTN (band)

Response to:	-

Release:	Release 18

Work Item:	NR_NTN_enh-Core

Source:	vivo (to be RAN2)

To:	RAN4

Cc:              RAN1

Contact Person:

Name:	Yitao Mo

E-mail Address:	yitao.mo@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 discussed and found in the current RAN2 spec TS 38.306 and TS 38.331, only the term FR2-1 or the FR2-1 bands is used within some NTN related UE capabilities description, it lacks the reference to FR2-NTN. And it is RAN2’s understanding that NTN bands within this frequency range of FR2-NTN are regarded as a FR2-1 band when referenced in TS 38.306 and TS 38.331. RAN2 would like to check with RAN4 whether this understanding is correct or feasible.

2. Actions:

To RAN4

ACTION:	 RAN2 respectfully asks RAN4 to provide feedback on the above questions.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2#127bis	2024-10-14 to 2024-10-18		Hefei, CN

TSG-RAN WG2#128	2024-11-18 to 2024-11-22 	Orlando , US