3GPP TSG-RAN WG2 Meeting #127	R2-2406452

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:	8.11.2

Source:	Huawei, HiSilicon

Title:	Impacts on the random access by the evolution of duplex operation

Document for: Decision

# 1	Introduction

Based on the revised WID RP-241614[1], the enhancements for random access operation in SBFD symbols are to be considered as following:

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

In this contribution, we will analyse the RAN2 impacts to support random access in SBFD symbols by UEs in RRC CONNECTED mode and RRC_IDLE/INACTIVE mode.

# 2	Discussion

## 2.1 RACH events

Currently, the TS 38.300 specifies the events to trigger the RACH procedure as following:

The random access procedure is triggered by a number of events: - Initial access from RRC_IDLE; - RRC Connection Re-establishment procedure; - DL or UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure (see clause 18.0) is ongoing, when UL synchronisation status is "non-synchronised"; - UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available; - Handover, except for when RACH-less HO is configured; - SR failure; - Explicit request by RRC upon synchronous reconfiguration; - RRC Connection Resume procedure from RRC_INACTIVE; - To establish time alignment for a primary or a secondary TAG; - Request for Other SI (see clause 7.3); - Beam failure recovery; - Consistent UL LBT failure on SpCell; - SDT in RRC_INACTIVE (see clause 18); - Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning; - Early UL synchronization with an LTM candidate cell; - RACH-based LTM cell switch.

RAN2 needs to discuss whether RACH procedure for all the above cases can be supported in SBFD symbols, considering that random access in SBFD symbols is aiming to reduce the random access latency, to reduce the PRACH collision probability and to improve the coverage of PRACH and Msg3. On the other hand, using SBFD symbols for RACH may cause extra interference to other UEs in the same area.

Proposal 1: RAN2 to discuss for which triggering events, random access procedure in SBFD symbols is supported.

## 2.2 RACH Configuration

In the previous RAN1 meetings, there were some agreements and working assumption achieved about random access operation in SBFD symbols:

Working assumption: For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols. Agreement Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

Based on the RAN1 conclusions, random access in SBFD symbols is supported for UEs in both RRC CONNECTED mode and RRC_IDLE/INACTIVE mode.

From RAN2 point of view, how to provide the RACH configuration for SBFD resource to UE in RRC CONNECTED mode and RRC_IDLE/INACTIVE mode should be discussed. As a baseline, for RRC_IDLE/INACTIVE mode, providing the RACH configuration for SBFD resource via SIB should be supported as the cell common configuration. On top of that, if RAN1 makes further conclusion that it is beneficial to also have UE specific configuration, the RACH configuration for SBFD can be provided via dedicated RRC signaling for RRC_CONNECTED UEs or RRC_INACTIVE UEs.

Proposal 2: RAN2 to confirm that the network provides the RACH configuration for SBFD via SIB. Whether to provide the RACH configuration for SBFD via dedicated RRC signalling is pending on RAN1’s progress.

On RACH configuration approaches, RAN1 has achieved the following agreement:

Agreement For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

To support random access in SBFD symbols for SBFD-aware UEs, gNB should configure PRACH resources in SBFD symbols. Based on the above agreement, there are two options:

Option 1 with Alt 1-1: Use one single RACH configuration, and only based on the existing parameters of the single RACH configuration.

Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration.

Option 1: One single RACH configuration       Option 2: Introduce a separate additional RACH configuration

Figure 1. Signalling design for the new PRACH resource configuration

As illustrated in Figure 1, Option 1 is to reuse the same configuration of PRACH for non SBFD-aware UEs; and for Option 2, a separate RACH configuration with a full set of parameters (preamble format, periodicity, time-frequency location, number of FDMed ROs, etc) is used for PRACH resource in SBFD symbols.

While Option 2 has full flexibility to configure the PRACH resources, the Option 1 has a smaller signalling overhead. For Option 1, the frequency-time location configuration of PRACH for (legacy) non SBFD-aware UEs can be reused to indicate the frequency-time location of new PRACH resources, i.e., using the legacy prach-ConfigurationIndex and msg1-FDM to indicate the time pattern of new PRACH resource and the number of FDMed ROs for SBFD-aware UEs. With this option, some new rules may be needed to ensure that the frequency-time location of PRACH for SBFD-aware UEs are confined within the UL subband in SBFD symbols. While RAN1 is discussing such rules, RAN2 can confirm the above RAN1 agreement on RACH configuration and start to discuss the related signalling design.

Proposal 3: RAN2 to confirm that RACH configuration for SBFD can be either based on the legacy RACH parameters or a new set of RACH parameters, and to discuss the signalling design for both options.

When the new PRACH resource configuration for SBFD-aware UEs (new PRACH) and the PRACH resource configuration for non SBFD-aware UEs (legacy PRACH) are both provided, the behavior of SBFD-aware UEs on PRACH resource selection should be clarified. To be specific, whether an SBFD-aware UE is allowed to use the PRACH resources for non SBFD-aware UEs. If allowed, under which conditions/when shall the SBFD-aware UE select the new PRACH resources and when to select the PRACH resources for non SBFD-aware UEs. It is understood that the RSRP threshold of measured SSB or the interference level can be considered.

Proposal 4: RAN2 to consider the criterion for the SBFD-aware UE to select between the PRACH resource in SBFD symbols and the PRACH resource in non-SBFD symbols.

Another aspect to discuss is the UE fall-back behaviour with regard to PRACH resource selection. After multiple times of RACH failure with resource in SBFD symbols, to avoid severe UE-to-UE CLI, adjacent-cell interference and resource wasting, UE shall be allowed to switch from the PRACH resources in SBFD symbols to the PRACH resources in non-SBFD symbols.

Proposal 5: RAN2 to discuss new fall-back behaviour if UE fails on the PRACH resource in SBFD symbols.

# 3	Conclusion

Based on the above discussion we have the following proposals:

Proposal 1: RAN2 to discuss for which triggering events, random access procedure in SBFD symbols is supported.

Proposal 2: RAN2 to confirm that the network provides the RACH configuration for SBFD via SIB. Whether to provide the RACH configuration for SBFD via dedicated RRC signalling is pending on RAN1’s progress.

Proposal 3: RAN2 to confirm that RACH configuration for SBFD can be either based on the legacy RACH parameters or a new set of RACH parameters, and to discuss the signalling design for both options.

Proposal 4: RAN2 to consider the criterion for the SBFD-aware UE to select between the PRACH resource in SBFD symbols and the PRACH resource in non-SBFD symbols.

Proposal 5: RAN2 to discuss new fall-back behaviour if UE fails on the PRACH resource in SBFD symbols.

# 4	Reference

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)