3GPP TSG-RAN WG2 Meeting #127	R2-2406453

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	8.2.2 Functionality aspects

Source: 	Semtech

Title: 	Considerations for functionality aspects

Document for:	Discussion/Decision

# Introduction

In RAN#102, the A-IoT Rel-19 SID [2] was approved based on the Ambient IoT RAN led study technical report [1]. The SID was updated in RAN #103 [3]. The motivation of Ambient IoT is to support devices that are ultra-low cost and ultra-low power. This tdoc discusses the considerations for functionality aspects.

# Charging Impacts

Both of the Ambient IoT devices defined in the overall objectives of the SID [2,3] include energy storage.

- The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices:

- ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm,DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

While energy harvesting methods are not part of the SID objectives, the impact of energy harvesting on device availability has been endorsed as part of the study [4,5].

RP-240854	Moderator's summary on R19 Ambient IoT	Huawei

Replaces

RP-240827

conclusion: proposal 3v2 is endorsed, proposal 2 is endorsed, proposal v5v2 is endorsed

Proposal 2 (endorsed above)

- Confirm that study of design of energy harvesting signal/waveform is out of SI scope in Rel-19

- The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study

  - One device’s charging by energy harvesting can be assumed up to several tens of seconds

    - Note: this value can be revisited in future RAN plenary meetings, if necessary

  - TR 38.848 clause 5.6 statement on latency remains the case with respect to a single device, i.e.: “NOTE: The time for charging the Ambient IoT device storage (if present) is not included in the latency defined above. Time for energy harvesting, charging, etc. is regarded as an implementation issue only.”

- No SID revision is necessary

From RAN2 #126 [6]:

Agreements

1	As baseline, the “inventory only” case is supported by the procedure:

-	Step A: A-IoT paging;

-	Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS

2	As baseline, the “inventory and command” case is supported by the procedure:

-	Step A: A-IoT paging;

-	Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS

-	Step C: reader to device data transmission (e.g. the R2D command), and

-	Step D: corresponding device to reader data transmission (e.g. the feedback).  FFS whether this is optional, pending other WG discussions.

Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message.  This doesn’t mean that inventory and command are received by the reader at the same time from upper layer.

3	From RAN2 point of view we will study “Command only” use case.

FFS the options on how to support it :

Initial trigger message from the reader contains the command.  Final feasibility depends on SA2 and SA3 work/conclusions.

Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

4	The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

5	“Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

1	A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.   “Msg2” usage/presence can be further discussed

Agreement

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

One of the impacts of the A-IoT devices relying on the energy harvesting for device operation is the reliability of the available energy for operations. The device energy storage can reduce the impact, but energy storage is expensive so A-IoT devices would have limited storage. To limit the energy storage requirements on the devices, the reader should have procedures to deal with devices with unreliable energy capability.

For example, the “inventory and command” use case (above) has multiple communication transaction steps and includes the processing for the “command” requested by the reader. It is possible that when the reader starts a procedure with the A-IoT device via a page, the device may have insufficient energy to complete the entire procedure. If the transaction fails part way, the reader will need to redo the transaction, assuming that the failure has been detected. Alternatively, the device could ignore the reader’s page (after assessing the energy storage) but then the reader would need to re-try and possibly increase the area of paging, etc. It could also lead to the reader to declare that the device is unreachable.

For the “inventory and command” use case, there is a need for mitigation procedures for the situation that an A-IoT device has insufficient energy storage to complete the procedure. However, in the “inventory only” case, the transaction is limited hence may need further study on what can be done.

RAN2 to study mitigation procedures of when an A-IoT device has insufficient energy storage to complete the transaction (for “inventory and command” and perhaps “inventory only”).

# Segmentation and Reassembly

From RAN2 #125bis [7]:

Agreements for user plane

1	SDAP is not supported for UP protocol stack.

2	PDCP layer is not needed.  FFS how to handle AS security (if needed pending SA3 dicsussion) and any other really needed functionalities.

3	RLC layer is not needed.   FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size).  RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed.

4	No HARQ and RLC AM

5	FFS about the level of visibility required by the reader and what information is necessary for AS layer operations.

6	RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL).  FFS how to handle the general QoS requirements from SA2

As RAN2 discusses the need for segmentation and reassembly in the AS layer, the device complexity aspects and the application requirements need to be considered. Supporting AS layer segmentation and reassembly would add complexity to the A-IoT device, both in increased memory and processing. From an implementation perspective, it might even result in the same data residing both in the application part of the A-IoT device (i.e., the source of the data) and the AS layer to buffer for transmission.

There are A-IoT applications that will require larger user data transfers (e.g., “inventory and command” use cases). This could perhaps be solved by allowing the application layer to perform the segmentation (i.e., it would send a few commands in a row). However, the AS layer would need to allow for efficient transmissions of back-to-back data, without re-paging for each transaction.

RAN2 to consider not supporting segmentation and reassembly in the AS layer.

# Summary

- RAN2 to study mitigation procedures of when an A-IoT device has insufficient energy storage to complete the transaction (for “inventory and command” and perhaps “inventory only”).

- RAN2 to consider not supporting segmentation and reassembly in the AS layer.

# References

TR 38.848 Study on Ambient IoT (Internet of Things) in RAN, 2023

RP-234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”

RP-240854 “Moderator's summary on R19 Ambient IoT”, RAN #103

RAN #103 Meeting Report, draft_MeetingReport_RAN_103_240321_eom.rtf

RAN2 #126, RAN2 Chair notes, May 2024.

RAN2 #125bis, RAN2 Chair notes, April 2024.