3GPP TSG-RAN WG2 Meeting #127	R2-2406454

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	8.2.4 A-IoT Random Access

Source: 	Semtech

Title: 	Considerations for Random Access

Document for:	Discussion/Decision

# Introduction

In RAN#102, the A-IoT Rel-19 SID [2] was approved based on the Ambient IoT RAN led study technical report [1]. The SID was updated in RAN #103 [3]. The motivation of Ambient IoT is to support devices that are ultra-low cost and ultra-low power. This tdoc discusses the considerations for random access.

# Contention resolution and access failure

In the RAN2 #125bis [4] it was agreed that the random-access procedure is triggered by the reader:

Agreement

- RAN2 confirms slotted-ALOHA is the baseline for Ambient IoT random access

- We will study the support for access triggering for a single device, group of devices, or all devices.    RAN2 to discuss the contention-based and contention-free access procedures and detailed solutions.

- Random Access is triggered by the reader

- Reader provides the information that the device needs to respond to the random access trigger.  FFS what those parameters are

- Study the solution and benefits of both 2-step like random access procedure and 4-step like random access procedure.  FFS the details on each procedure and how we call it.

- Handling of contention resolution failure and access failure at the device will be studied in RAN2, including failure detection and re-access.  FFS details

- For the very first access message from the device to reader in random access an ID is included.  RAN2 to discuss whether a temporary identifier is included, or the permanent device ID is included (considering other WGs input as well).

From RAN2 #126 [5]:

Agreements

1	RAN2 will study the following cases for AIoT paging message:

- a message containing an ID of a single A-IoT device.

- a message containing a group ID that maps to multiple A-IoT devices.

- a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message.

- a message containing multiple IDs of A-IoT devices.  Need to confirm the need for this use case based on SA2 discussion.

What device ID and group ID and scenarios is depending on SA2 discussion.

2	AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message).  FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

3	From RAN2 perspective, we assume the device can receive as long as there is enough energy.  We will wait for RAN1 further progress on device monitoring details.

In the last RAN2 meeting, it was agreed that RAN2 would study A-IoT paging messages addressed to multiple devices. Once situation where this would be used is in inventory control (e.g., warehouse, etc.) where the reader would page groups of devices. However, with multiple devices triggering random access procedure after the page, the A-IoT system will need a clear procedure to minimize or avoid collisions. One solution maybe for the A-IoT device to select a random delay after the trigger before starting the random-access transmission. The delay value range could be provided to the A-IoT device from the reader.

Study how ambient IoT devices can reduce collisions after a random-access trigger from the reader.

# Summary

- Study how ambient IoT devices can reduce collisions after a random-access trigger from the reader.

# References

TR 38.848 Study on Ambient IoT (Internet of Things) in RAN, 2023

RP-234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”

RAN2 #125bis, RAN2 Chair notes, April 2024.

RAN2 #126, RAN2 Chair notes, May 2024.