3GPP TSG-RAN2#127							             R2-2406457

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on LSs for XR

Agenda item:	8.7.1.1

Document for: Discussion and Decision

# Introduction

There are two XR related LSs received, e.g.  SA2 LS on FS_XRM Ph2[1] and RAN3 LS on UL PSI based PDU discarding in NR-DC [2]

In this contribution, we will further discuss the related issue and give our proposals.

# Discussion

## 2.1 About LS for XRM ph2[1]

On Question 1: SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

In the current specification, the periodicity is provided per QoS flow, e.g. in TSCAI, and the UL periodicity can also be provided by UAI, which are semi-static, and mainly used for semi-static resource scheduling and/or configuration, e.g. for SPS configuration, CG configuration and/or C-DRX configuration. If the periodicity of the XR traffic changes rather dynamically and needs to be included in GTP-U header, then configuration/reconfiguration for SPS is not practical and the gNB has to rely on dynamic scheduling instead. Thus , there is no use case for dynamic periodicity seen, we think the periodicity provided by TSCAI and UL periodicity provided by UAI are enough, it is not necessary to provide periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity.

Observation 1: From RAN2 perspective, the periodicity provided in TSCAI and UL periodicity provided by UAI are enough, it is not necessary to provide periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity.

On Question 2: Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

If the exact BAT of the next burst can be provided before the burst arriving, it is beneficial for gNB to schedule the radio resource, especially for dynamic scheduling case.

Observation 2: From RAN2 perspective, the exact BAT of the next burst is beneficial for gNB to schedule the radio resource.

Proposal 1: send reply LS for the SA2 LS[1] to indicate:

- For Q1: From RAN2 perspective, it is not necessary to provide periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity.

- For Q2: RAN2 think the exact BAT of the next burst is beneficial for gNB to schedule the radio resource.

## 2.2 About LS on UL PSI based PDU discarding in NR-DC[2]

RAN3 has asked RAN2 to clarify the UE behaviour for handling uplink PSI based PDU discarding when MN and/or SN sends the PSI-based SDU discard Activation/Deactivation MAC CE.

According to the current RAN2 specs, the MAC entity doesn’t distinguish whether the MAC CE for PSI-Based SDU discard activation/deactivation MAC CE is received from the MN or SN. Thus, the MAC entity will simply forward the MAC CE received (either from MN or SN) to the corresponding PDCP. If both MN and SN send the MAC CE, then the state of the discard behaviour for the given PDCP entity will be according to the latest MAC CE received from either the MN or SN.

Observation 3: From RAN2 perspective, the MAC entity doesn’t distinguish whether the PSI-Based SDU Discard Activation/Deactivation MAC CE was received from MN or SN.

Observation 4: In case of split bearer, if both MN and SN send the PSI-Based SDU Discard Activation/Deactivation MAC CE, then the UE will act on both and the final state of the PSI-Based SDU discard for the given DRB will be according to the latest MAC CE received at the UE side.

Based on the above, RAN2 should reply to RAN3 as follows:

Proposal 2: Reply to RAN3 that:

- The MAC entity doesn’t distinguish whether the PSI-Based SDU Discard Activation/Deactivation MAC CE was received from MN or SN

- In case of split bearer, if both MN and SN send the PSI-Based SDU Discard Activation/Deactivation MAC CE, then the UE will act on both and the final state of the PSI-Based SDU discard for the given DRB will be according to the latest MAC CE received at the UE side.

# Conclusion

The following observations/proposals are made:

Observation 1: From RAN2 perspective, the periodicity provided in TSCAI and UL periodicity provided by UAI are enough, it is not necessary to provide periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity.

Observation 2: From RAN2 perspective, the exact BAT of the next burst is beneficial for gNB to schedule the radio resource.

Proposal 1: send reply LS for the SA2 LS[1] to indicate:

- For Q1: From RAN2 perspective, it is not necessary to provide periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity.

- For Q2: RAN2 think the exact BAT of the next burst is beneficial for gNB to schedule the radio resource.

Observation 3: The MAC entity doesn’t distinguish whether the PSI-Based SDU Discard Activation/Deactivation MAC CE was received from MN or SN.

Observation 4: In case of split bearer, if both MN and SN send the PSI-Based SDU Discard Activation/Deactivation MAC CE, then the UE will act on both and the final state of the PSI-Based SDU discard for the given DRB will be according to the latest MAC CE received at the UE side.

Proposal 2: Reply to RAN3 that:

- The MAC entity doesn’t distinguish whether the PSI-Based SDU Discard Activation/Deactivation MAC CE was received from MN or SN

- In case of split bearer, if both MN and SN send the PSI-Based SDU Discard Activation/Deactivation MAC CE, then the UE will act on both and the final state of the PSI-Based SDU discard for the given DRB will be according to the latest MAC CE received at the UE side.

# References

- S2-2407351_LS for XRM ph2, SA2#163, May 2024

- R3-243957_ LS on UL PSI based PDU discarding in NR-DC, RAN3#124, May 2024