3GPP TSG-RAN WG2 Meeting #127	R2-2406458

Maastricht, Netherlands 19 – 23 August, 2024

Source: 			ZTE Corporation, Sanechips

Title: 	SDT signalling optimization for partial context transfer

Agenda item:		7.24.1

Document for: 	Discussion and Decision

- Introduction

RAN3 sent an LS to RAN2 (R3-242198) informing us about a possible solution for signalling optimization for partial context transfer and asked RAN2 to inform about any potential RAN2 impact or issue with the solution discussed in RAN3. Further to this, at RAN2#126, RAN2 discussed the implications of sending RRCSetup message during SDT and agreed the following:

=>	Reply to LS:

-	No specification impact (for Rel-17 and 18)

-	However, there are RAN2 issues, like data loss (explain the issue)  and Uu signaling overhead due to security being cleared (explain that as from Rel-15 …)

Subsequently, during the post meeting email discussion, it was debated whether a MAC reset is needed if RRCSetup is received during SDT. The LS reply was eventually postponed as we couldn’t conclude on whether or not a MAC reset is needed. In this contribution, we discuss the open aspects of this further and propose a response to RAN3.

- Discussion

When the UE receives RRCRelease message, MAC is reset and if the UE subsequently resumes the connection, then the gNB and the UE MAC state (for NDI for each HARQ process) is aligned. However, if SDT is initiated, then the UE will use some HARQ processes for data transfer and if subsequently RRCSetup is received in the middle of the SDT session, then the MAC state for each of the HARQ process needs to be aligned between the gNB and the UE. In the LS from RAN3, it was mentioned that only Xn signalling will be impacted with the so-called network based solution, where RRCSetup message is sent during SDT. However, this means that the new gNB has to identify the existing UE context in the DU for the ongoing SDT session and use the RRCSetup procedure in combination with the UE context modification procedure at the DU instead of using UE context setup procedure which is typically used for RRCSetup. This results in a new procedure at the network side.

Observation 1: For RRCSetup, the network uses UE context setup procedure at the DU currently and if the DU context needs to be kept (i.e. avoiding MAC reset) then a new procedure to keep UE context at the DU will be needed and RAN3 should investigate whether this needs a new F1 signalling procedure

On the other hand, if RRCSetup is sent after some data has been exchanged between the network and the UE (especially DL data) – e.g. as in MBS case, then MAC is reset and hence a new DU context is created for the UE on the DU by the network.

Observation 2: If any data is exchanged between the network and the UE during INACTIVE state (e.g. MBS) then MAC reset is performed upon reception of RRCSetup and network creates a new DU context for the UE (which is the general procedure for any RRCSetup today)

Given the above if RRCSetup is to be sent during SDT session, then it would be good to perform MAC Reset on the UE side and to create a new context for the UE on the network side as is currently the case (and aligned with the MBS).

However, if we go this way, then this is not compatible with legacy SDT operation. So, if MAC reset is to be performed, then the UE procedure is different to Rel-17 behaviour and hence, it is not possible for the network to send RRCSetup to the UE during SDT after exchange of DL data. Thus, either a new capability is needed for the UE if we enable this feature or we should clarify that the network is not allowed to send RRCSetup after sending any DL data.

Observation 3: If MAC reset is needed for the case of the RRCSetup during SDT, then a new UE capability is needed since this is not compatible anymore with Rel-17 SDT operation

Based on the above, we propose the following:

Proposal 1: Capture in RAN2 specs that the network should not send RRCSetup if DL data is sent to the UE

Proposal 2: Reply to RAN3 as follows:

- A MAC Reset is needed if RRCSetup procedure is initiated in the middle of the SDT session and this needs new UE capability and is hence not compatible with Rel-17 operation.

- Network based solution would result in data loss since UE will discard any pending UL data and setup a new RRC connection.

- Network based solution would result in additional signalling to reestablish AS context and AS security context both of which are released upon receiving RRCSetup

- Hence RAN2 agreed that the network should not send RRCSetup message after sending any DL data during SDT

- Conclusion

Observation 1: For RRCSetup, the network uses UE context setup procedure at the DU currently and if the DU context needs to be kept (i.e. avoiding MAC reset) then a new procedure to keep UE context at the DU will be needed and RAN3 should investigate whether this needs a new F1 signalling procedure

Observation 2: If any data is exchanged between the network and the UE during INACTIVE state (e.g. MBS) then MAC reset is performed upon reception of RRCSetup and network creates a new DU context for the UE (which is the general procedure for RRCSetup today)

Observation 3: If MAC reset is needed for the case of the RRCSetup during SDT, then a new UE capability is needed since this is not compatible anymore with Rel-17 SDT operation

Proposal 1: Capture in RAN2 specs that the network should not send RRCSetup if DL data is sent to the UE

Proposal 2: Reply to RAN3 as follows:

- A MAC Reset is needed if RRCSetup procedure is initiated in the middle of the SDT session and this needs new UE capability and is hence not compatible with Rel-17 operation.

- Network based solution would result in data loss since UE will discard any pending UL data and setup a new RRC connection.

- Network based solution would result in additional signalling to reestablish AS context and AS security context both of which are released upon receiving RRCSetup

- Hence RAN2 agreed that the network should not send RRCSetup message after sending any DL data during SDT