3GPP TSG RAN WG2 Meeting #127                                                           R2-2406470

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.5.3

Source:	Intel Corporation

Title:	Remaining details to enable on-demand SIB1

Document for:	Discussion and decision

# Introduction

This document discusses the remaining open details required to enable the support of on-demand SIB1 (OD-SIB1) considering RAN2 agreements.

# Discussion

## Idle mode operation

RAN2 discussed UE operation when acquiring OD-SIB1 and concluded the following:

RAN2#126

- 6.	If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- 7.	After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

- 9.	NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- 10.	How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

- 12.	After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

Even though legacy operation is baseline, we understand current scenario under discussion potentially introduces a large time between when UE camps in a Cell A and UE switches to camp in a neighbouring NES Cell and still needs to request/acquire OD-SIB1 in NES Cell (as shown on Figure 1).

Figure 1. Scenario 1a (or case (2) from RAN1)

Moreover, RAN1 also concluded that it is up to RAN2 whether any operations may need to be enabled or disabled for OD-SIB1 as shown in below conclusion captured in RAN1#117.

- Conclusion:  For further study of on-demand SIB1 in idle/inactive mode, enabling or disabling specific operations (e.g. paging, RACH receiving, OSI request …) of the NES cell with on-demand SIB1 is up to RAN2.

### UE reachability

When UE waits to camp in NES Cell in the scenario just explained in Figure 1, UE would not be able to acquire paging or critical SI (e.g., ETWS notifications) in NES Cell until OD-SIB1 is acquired. This time might be different than in legacy operation as UE needs to request OD-SIB1 and wait for its transmission before being able to complete its cell reselection into NES Cell and acquire notification of paging or critical SI. In RAN2#126, RAN2 agreed that legacy behaviour is agreed which would mean to confirm that this behaviour is acceptable. However, this level of details was not actually discussed last meeting. Therefore, we suggest to at least discuss it once including whether any enhancement or requirement should be defined to allow UE to still be reachable during these in-between state (i.e. when UE is attempting to move to NES Cell, but it is still trying to acquire OD-SIB1).

- When UE in RRC_IDLE/INACTIVE moves from a Cell A to a NES Cell and still needs to request/acquire OD-SIB1 in NES Cell, to confirm that it is acceptable for this UE not to monitor/receive paging or critical SI (e.g., ETWS notification) in this NES Cell until OD-SBI1 is received. NOTE: UE’s unreachability most likely will be longer than in legacy cell reselection scenarios.

  - If Proposal 1 is not agreeable, to discuss whether a new behaviour is required for Rel-19 NES UEs to guarantee its reachability (i.e., monitoring/reception of paging or critical SI (e.g. ETWS)) when UE in RRC_IDLE/INACTIVE moves from a Cell A to a NES Cell and still needs to (or is) request/acquire OD-SIB1 in NES Cell

### Prioritization associated to NES Cell (for NES UEs)

It is possible to exclude cells from reselection using intra/interFreqExcludedCellList.  While this doesn’t bar the cells, it can be used to prevent a legacy UE in neighbouring cells from reselecting an NES cell.  As barring is already provided by other SIB fields, intra/interFreqExcludedCellList legacy mechanism is sufficient to achieve the objective of a legacy UE to down-prioritize or exclude an NES Cell. On summary, a new signaling is necessary to be able to exclude cells but specifically for NES UEs.

- To define the required signaling for the network to indicate to Rel-19 NES UEs when to exclude specific cells from its neighbouring cell list (including both intra-/inter- frequencies) which may be different than the one broadcasted for NES UEs. I.e., To define an intra/interFreqExcludedCellList specific for Rel-19 NES UEs which would be used by these UEs instead than legacy one.

### Cell Reselection

RAN2 assumes legacy cell (re)selection operation applicable for OD-SIB1 operation. From RAN2 specification point of view, the term of “camping” on a cell is used after successful (re)selecting and acquisition of minimum SI as shown in below references.

TS 38.300 captures the following:

For a UE to be allowed to camp on a cell it must have acquired the contents of the Minimum SI from that cell. There may be cells in the system that do not broadcast the Minimum SI and where the UE therefore cannot camp.

…

Minimum SI comprises basic information required for initial access and information for acquiring any other SI. Minimum SI consists of:

  - MIB contains cell barred status information and essential physical layer information of the cell required to receive further system information, e.g. CORESET#0 configuration. MIB is periodically broadcast on BCH.

  - SIB1 defines the scheduling of other system information blocks and contains information required for initial access. SIB1 is also referred to as Remaining Minimum SI (RMSI) and is periodically broadcast on DL-SCH or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED.

TS 38.304 captures the following:

Camped on a cell: UE has completed the cell selection/reselection process and has chosen a cell. The UE monitors system information and (in most cases) paging information.

In addition, the term “considered for camping” is used only in stage-2 to indicate when UE is not required to acquire minimum SI from another cell/frequency layer.

TS 38.300 captures the following:

For a cell/frequency that is considered for camping by the UE, the UE is not required to acquire the contents of the minimum SI of that cell/frequency from another cell/frequency layer. This does not preclude the case that the UE applies stored SI from previously visited cell(s).

For NES operation, when UE chooses a NES cell with OD-SIB1 (which would require that cell (re)selection has successfully completed) but UE is yet to acquire its OD-SIB1. Our suggestion is to use an specific term (e.g., “attempting to camp”) to avoid confusions while this does not mean that a new state/transition is defined. This “attempting to camp" term would knowledge the in-between/longer transition in which UE requests and waits for the OD-SIB1. After it is clarified how UE behaviour during this “attempting to camp” period, RAN2 could revisit whether to reuse the same wording as in TS 38.300 “considered for camping”. Summarizing our scenario over time:

- While RRC_IDLE/INACITVE UE is performing cell (re)selection and chooses a NES cell.

- UE sends UL request for the transmission of OD-SIB1 (i.e., UE sends UL WUS to request OD-SIB1).

- UE waits and receives OD-SIB1. After receiving OD-SIB1, UE can camp in that NES Cell (if not barred).

While UE is “attempting to camp” in an NES Cell (i.e., UE does not have minimum SI), it would not be able to monitor paging or critical SI in NES Cell until the corresponding OD-SIB1 is acquired as it was previously explained.

Legacy cell (re)selection operation is reused for NES Cells with OD-SIB1 (i.e., no new states or states transitions are defined for UEs in RRC_IDLE/INACTIVE) as agreed in RAN2#126.

- The term “attempting to camp” is defined as follows: UE is “attempting to camp” on a NES Cell when UE choses that NES cell using legacy cell (re)selection procedure, but UE is still requesting/waiting to acquire OD-SIB1.

  - After UE successfully receives OD-SIB1 for that NES Cell and if is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell (i.e., acquisition of OD-SIB1 is successful even though it is not broadcasted periodically).

  - FFS whether the term “considered for camping” (which is currently used in TS 38.300) could be re-used instead of the “attempting to camp” currently considered for OD-SIB1 operation. This FFS could be discussed after/if the OD-SIB1 features moves to WI phase.

### Barring associated to NES Cell (for NES UEs)

RAN2 agreed that existing mechanism are sufficient to bar legacy UEs to access NES Cells as shown in above agreements. This would mean that a different barring bit should be used if/when NES UEs are not to be barred (i.e., to allow the NES UEs). This could be similar to Rel-18 NES UEs as shown in below reference.

cellBarredNES-r18                ENUMERATED {notBarred}         OPTIONAL,  -- Need R

cellBarredNES This field indicates the cell barring status for UEs supporting nes-CellDTX-DRX as described in 5.2.2.4.2.

1>	if the UE supports nes-CellDTX-DRX and it is in RRC_IDLE or in RRC_INACTIVE, or if the UE supporting nes-CellDTX-DRX is in RRC_CONNECTED while T311 is running:

2>	if cellBarred in the acquired MIB is set to barred:

3>	if cellBarredNES is absent in the acquired SIB1:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

In addition, the following two open question/topics would at least need discussion in RAN2:

- Topic 1) Which kind of information the non-barring indication applies to?

The following options could be considered:

- Option (1.a) Single (i.e., common) non-barring indication applies to all Rel-19 NES features (which would include OD-SIB1, OD-SSB, and adaptation on time for common channels/signaling).

- Option (1.b) Multiple (i.e., different) non-barring indications are defined for specific features or group of features supported by or specific to Rel-19 NES UEs.

Similar to Rel-18 NES, option (1.a) seems sufficient unless there is a specific requirement that requires providing different access handling, which at this point is not identified.

- Topic 2) Which non-barring indication is used?

The following options could be considered:

- Option (2.a) Reusing Rel-18 NES non-barring indication for Rel-19 NES UEs/features.

- Option (2.b) Defining a new non-barring indication for Rel-19 NES UEs/features.

Both options are feasible but depends on which network/deployments are been targeted. Therefore, we suggest to discussing both options with the understanding that option (2.b) might be the one that would provide the maximum flexibility if network, UEs and operators may want to treat differently the access of Rel-18 and Rel-19 NES UEs.

- Rel-19 NES Cell needs to be able to indicate when a Rel-19 NES Cell UEs are notBarred (i.e., allowed) while legacy UEs are barred (i.e., when setting cellBarred to barred).

  - If Proposal 4 is agreed, to discuss which kind of information the non-barring indication applies to. For this, a single (i.e., common) non-barring indication applies to all Rel-19 NES features (which would include OD-SIB1, OD-SSB, and adaptation on time for common channels/signaling) seems sufficient (option 1.a).

  - If Proposal 4.1 is agreed, to discuss which non-barring bit is used. For this, we suggest discussing option (2.a) reusing cellBarredNES-r18 indication (i.e., the scope of cellBarredNES-r18 is extended to also cover/address Rel-19 NES UEs) or option (2.b) defining a new bit to indicate when Rel-19 NES UEs are not barred.

## Validity of OD-SIB1

RAN2 agreed that upon being notified of a SI change in a NES Cell (e.g., via paging), UE is expected to always be able to receive OD-SIB1 (without having to perform an explicit request).

- 15.	Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

During RAN2#126 meeting, it was also discussed how UE would know when OD-SIB1 becomes invalid or obsolete after being acquired. The concept of “valid SIB1” would be new one as legacy SIB1 is always and periodically broadcasted for a given cell. On other hand, other SIBs have the concept of “valid stored version”.

TS 38.331 captures the following:

When the UE acquires a MIB or a SIB1 or an SI message in a serving cell as described in clause 5.2.2.3, and if the UE stores the acquired SIB, then the UE shall store the associated areaScope, if present, the first PLMN-Identity in the PLMN-IdentityInfoList for non-NPN-only cells or the first NPN identity (SNPN identity in case of SNPN, or PNI-NPN identity in case of PNI-NPN) in the NPN-IdentityInfoList for NPN-only cells, the cellIdentity, the systemInformationAreaID, if present, and the valueTag, if present, as indicated in the si-SchedulingInfo for the SIB. If the UE stores the acquired posSIB, then the UE shall store the associated areaScope, if present, the cellIdentity, the systemInformationAreaID, if present, the valueTag, if provided in assistanceDataSIB-Element, and the expirationTime if provided in assistanceDataSIB-Element. The UE may use a valid stored version of the SI except MIB, SIB1, SIB6,

SIB7 or SIB8 e.g. after cell re-selection, upon return from out of coverage or after the reception of SI change indication. The valueTag and expirationTime for posSIB is optionally provided in assistanceDataSIB-Element, as specified in TS 37.355 [49].

In summary, RAN2 should discuss whether a validity time is used for OD-SIB1 and if so, the simplest and sufficient approach would be to reuse legacy concept as a baseline. The only exception should be H-SFN which should not trigger a SI notification change of SIB1.

- To re-use validity time concept (which is currently defined for other SIBs) for OD-SIB1. I.e., when Rel-19 NES UE acquires OD-SIB1, this UE uses it as a valid stored version.

- Legacy notification of SI change can be reused for OD-SIB1. NOTE: network is excluded to trigger a notification of SI change due to changes of the H-SFN value.

## UL WUS configuration (i.e., to request OD-SIB1 in NES Cell)

RAN2 already agreed that cell A broadcasts to UE a UL WUS configuration to allow the UE requesting OD-SIB1 in the NES Cell.

RAN2#125bis

- UL WUS configuration includes at least below information:

  - RACH configuration

- A UE needs to know a UL WUS configuration to request SIB1 of which cell.

RAN2#126

- 5.	Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB

For the details on the required UL WUS configuration, our suggestion is to wait for RAN1 progress as they are currently discussing them.

RAN1 is already discussing details of the UL WUS configuration required for the UE to be able to request and acquire OD-SIB1 in a NES Cell.

- Wait for RAN1 conclusion the details of the UL WUS configuration.

On other hand, RAN2 should discuss the expected network behavior if there is a change of the UL WUS configuration considering the following:

- UL WUS configuration associated to an NES Cell could be conveyed via one or more neighboring cells (i.e., Cell A).

It should be up to the NES Cell (i.e., the network/operators) to coordinate/update the UL WUS configuration of all the applicable neighboring cells.

- The new/updated UL WUS configuration associated to one NES Cell needs to also be provided to Rel-19 NES UE already camping on that NES Cell.

NES Cell should also be able to convey its own UL WUS configuration, when required. When so, this would target Rel-19 NES UEs already camping on that NES Cell (i.e., which had already acquired successfully OD-SIB1).

- How to inform Rel-19 NES UEs about the new/updated UL WUS configuration associated to one NES Cell. This mechanism might be similar as the one used to inform Rel-19 NES UEs about updates on OS-SIB1 which is discussed in previous section.

Coordinate when to start using the new/updated UL WUS configuration associated to one NES Cell which could be provided via multiple cells (including itself) would be too complex. Instead, it seems simpler and sufficient to rely on network implementation. I.e., during the transition period it is up to NES Cell needs to handle UEs using the old and new UL WUS configuration.

Summarizing, the handling of above scenario is up to network implementation although NES Cell should be able to broadcast new/updated UL WUS configuration for UEs already camping on this NES Cell. In addition, it might be good to inform RAN3 about this agreement if/when the work moves to WI phase.

- NES Cell should be able to provide UL WUS configuration to Rel-19 NES UEs which are already camping on it, e.g. for purpose of provided updates that maybe done to that UL WUS configuration.

- When changes/updates of the UL WUS configuration are to be done, any cell that broadcasts this configuration shall update this using existing SIB update mechanisms and no new rules need to be defined on when UE should start using this new configuration. It is  out of RAN2 scope (i.e., details are up to RAN3 and/or network implementation) how neighbour cells are informed of changes of the UL WUS configuration associated to a NES Cell.

## Msg.3 request of OD-SIB1

RAN2 agreed that Msg1 based on-demand kind of RACH procedure is at least reused as shown below:

RAN2#126:

- 4.	For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

- 8.	RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

RAN2#125bis:

- RAN2 assume that RACH procedure is reused for UE to request on-demand SIB1.

- UL WUS configuration includes at least below information:

  - RACH configuration

- A UE needs to know a UL WUS configuration to request SIB1 of which cell.

- Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

For on-demand SI, Msg 3 based procedure was defined as UE could request the transmission of one or multiple SIBs. For OD-SIB1, RAN1 agreed the assumption for further study that at least dedicated RACH resources are assumed, as shown below:

- For UL WUS design for SIB1 request, at least dedicated PRACH resource is the assumption for further study in RAN1

  - FFS: Details on time, frequency, and/or PRACH preamble resources for UL WUS

  - FFS: whether RACH resource for SIB1 request could be used for an initial access procedure and/or an on-demand SI procedure

Therefore whether Msg.3 based procedure is or not required depends on how the usage of dedicated PRACH resources for Rel-19 NES UEs is defined. I.e., whether they are defined as a mandatory feature to support by UE and to be used by the network and operators.

Some companies propose to use Msg.3 request to combine UE’s request for transmission of OD-SIB1 and other SIBs. This seems an optimization which might not be essential considering that UE needs to first acquire OD-SIB1 to know the information corresponding to other on-demand SIBs.

- Whether to define/allow OD-SIB1 via Msg.3 based towards NES Cell depends on how RAN1 defines the usage/support of dedicated PRACH resources for Rel-19 NES UEs. I.e., if the usage of dedicated PRACH resources for Rel-19 NES UEs is defined as optional from network point of view, OD-SIB1 via Msg.3 based would need to also be allowed.

## Acquisition of OD-SIB1

RAN1 has already agreed that at least (as a starting point) Rel-19 NES UE monitors for OD-SIB1 transmission during certain time window after requesting its transmission to the NES Cell, as shown in below RAN1#117 agreements:

- At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, after UE transmits the UL WUS in idle/inactive mode, RAN1 assumes following as a starting point:

- Option 1: One or more type 0 PDCCH monitoring occasions for on demand SIB1 within a time window

    - FFS: How the search space zero configuration is provided (e.g. from  searchSpaceZero  in  MIB  or  from a new search space that is indicated by UL-WUS configuration)

    - FFS: Details of the time window, including at least the starting time and duration

    - FFS: Whether/how to support transmission of on-demand SIB1 with the association with SSB(s) based on a received UL-WUS

More concrete details will need to be defined but we suggest waiting for RAN1 further progress or even the WI phase (assuming RAN agrees to include it as part of WI phase).

RAN1 has already agreed that UE monitors for OD-SIB1 transmission within a time window after requesting the NES Cell for its transmission.

- Wait for RAN1 conclusion on how the time window is defined to monitor for the OD-SIB1 after requesting it (as they are already discussing it).

## Case 3 scenario (i.e., Cell A provides OD-SIB1 for NES Cell)

RAN2 agreed to wait for RAN1 progress on whether to support case 3 scenario. During RAN2#126 meeting discussion, some companies raised concerns on how UE knows if OD-SIB1 is or not valid and how to inform all UEs about a new/updated OD-SIB1 if this is provided via Cell A. In our understanding, the content of SIB1 does not need to change frequently and UE could assume it valid unless network indicates otherwise, or its validity time expires since it was previously acquired/stored. In addition, when NES Cell changes the information of the SIB1, NES Cell needs to also be able to inform any Rel-19 NES UEs already camping on that NES Cell about the new/updated SIB1 e.g., during the next BCCH modification period.

- If Proposal 5 is agreed, case 3 scenario (i.e., Cell A provides OD-SIB1 for NES Cell) seems simple for RAN2 to support it with the understanding that UEs already camping on an NES Cell should also be updated when SIB1 changes (by using SIB update Paging and broadcasting OD-SIB1 when required).

# Conclusion

The observations captured are the following:

Observation 1.	Legacy cell (re)selection operation is reused for NES Cells with OD-SIB1 (i.e., no new states or states transitions are defined for UEs in RRC_IDLE/INACTIVE) as agreed in RAN2#126.

Observation 2.	RAN1 is already discussing details of the UL WUS configuration required for the UE to be able to request and acquire OD-SIB1 in a NES Cell.

Observation 3.	RAN1 has already agreed that UE monitors for OD-SIB1 transmission within a time window after requesting the NES Cell for its transmission.

The proposals captured are the following:

UE reachability

Proposal 1.	When UE in RRC_IDLE/INACTIVE moves from a Cell A to a NES Cell and still needs to request/acquire OD-SIB1 in NES Cell, to confirm that it is acceptable for this UE not to monitor/receive paging or critical SI (e.g., ETWS notification) in this NES Cell until OD-SBI1 is received. NOTE: UE’s unreachability most likely will be longer than in legacy cell reselection scenarios.

Proposal 1.1.	If Proposal 1 is not agreeable, to discuss whether a new behaviour is required for Rel-19 NES UEs to guarantee its reachability (i.e., monitoring/reception of paging or critical SI (e.g. ETWS)) when UE in RRC_IDLE/INACTIVE moves from a Cell A to a NES Cell and still needs to (or is) request/acquire OD-SIB1 in NES Cell

Prioritization associated to NES Cell (for NES UEs)

Proposal 2.	To define the required signaling for the network to indicate to Rel-19 NES UEs when to exclude specific cells from its neighbouring cell list (including both intra-/inter- frequencies) which may be different than the one broadcasted for NES UEs. I.e., To define an intra/interFreqExcludedCellList specific for Rel-19 NES UEs which would be used by these UEs instead than legacy one.

Cell Reselection

Proposal 3.	The term “attempting to camp” is defined as follows: UE is “attempting to camp” on a NES Cell when UE choses that NES cell using legacy cell (re)selection procedure, but UE is still requesting/waiting to acquire OD-SIB1.

Proposal 3.1.	After UE successfully receives OD-SIB1 for that NES Cell and if is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell (i.e., acquisition of OD-SIB1 is successful even though it is not broadcasted periodically).

Proposal 3.2.	FFS whether the term “considered for camping” (which is currently used in TS 38.300) could be re-used instead of the “attempting to camp” currently considered for OD-SIB1 operation. This FFS could be discussed after/if the OD-SIB1 features moves to WI phase.

Barring associated to NES Cell (for NES UEs)

Proposal 4.	Rel-19 NES Cell needs to be able to indicate when a Rel-19 NES Cell UEs are notBarred (i.e., allowed) while legacy UEs are barred (i.e., when setting cellBarred to barred).

Proposal 4.1.	If Proposal 4 is agreed, to discuss which kind of information the non-barring indication applies to. For this, a single (i.e., common) non-barring indication applies to all Rel-19 NES features (which would include OD-SIB1, OD-SSB, and adaptation on time for common channels/signaling) seems sufficient (option 1.a).

Proposal 4.2.	If Proposal 4.1 is agreed, to discuss which non-barring bit is used. For this, we suggest discussing option (2.a) reusing cellBarredNES-r18 indication (i.e., the scope of cellBarredNES-r18 is extended to also cover/address Rel-19 NES UEs) or option (2.b) defining a new bit to indicate when Rel-19 NES UEs are not barred.

Validity of OD-SIB1

Proposal 5.	To re-use validity time concept (which is currently defined for other SIBs) for OD-SIB1. I.e., when Rel-19 NES UE acquires OD-SIB1, this UE uses it as a valid stored version.

Proposal 6.	Legacy notification of SI change can be reused for OD-SIB1. NOTE: network is excluded to trigger a notification of SI change due to changes of the H-SFN value.

UL WUS configuration (i.e., to request OD-SIB1 in NES Cell)

Proposal 7.	Wait for RAN1 conclusion the details of the UL WUS configuration.

Proposal 8.	NES Cell should be able to provide UL WUS configuration to Rel-19 NES UEs which are already camping on it, e.g. for purpose of provided updates that maybe done to that UL WUS configuration.

Msg.3 request of OD-SIB1

Proposal 9.	When changes/updates of the UL WUS configuration are to be done, any cell that broadcasts this configuration shall update this using existing SIB update mechanisms and no new rules need to be defined on when UE should start using this new configuration. It is  out of RAN2 scope (i.e., details are up to RAN3 and/or network implementation) how neighbour cells are informed of changes of the UL WUS configuration associated to a NES Cell.

Proposal 10.	Whether to define/allow OD-SIB1 via Msg.3 based towards NES Cell depends on how RAN1 defines the usage/support of dedicated PRACH resources for Rel-19 NES UEs. I.e., if the usage of dedicated PRACH resources for Rel-19 NES UEs is defined as optional from network point of view, OD-SIB1 via Msg.3 based would need to also be allowed.

Acquisition of OD-SIB1

Proposal 11.	Wait for RAN1 conclusion on how the time window is defined to monitor for the OD-SIB1 after requesting it (as they are already discussing it).

Case 3 scenario (i.e., Cell A provides OD-SIB1 for NES Cell)

Proposal 12.	If Proposal 5 is agreed, case 3 scenario (i.e., Cell A provides OD-SIB1 for NES Cell) seems simple for RAN2 to support it with the understanding that UEs already camping on an NES Cell should also be updated when SIB1 changes (by using SIB update Paging and broadcasting OD-SIB1 when required).