3GPP TSG RAN WG2 Meeting #127                                                           R2-2406472

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.7.1.1

Source:	Intel Corporation

Title:	RAN2 views and responses to LSs from SA2, RAN3 and SA4

Document for:	Discussion and decision

# Introduction

This document provides our company’s views (including, when applicable, suggested response as well as foreseen RAN2 impacts) for the following incoming LSs from SA2 [1], RAN3 [2] and SA4 [3].

# Discussion

## Views on SA2 incoming LS S2-2407351 [1]

### [Q1] Periodicity via in-band signaling

SA2 provides the following regarding whether to share periodicity via in-band signaling:

Question1 [for SA4, RAN2 and RAN3]:

- SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

- To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

This topic is similar to previous SA2 question on whether to support dynamic redundancy ratios which RAN2 responded in LS [4] as follows:

Provide feedback on the impact on NG-RAN to support dynamic redundancy ratios, i.e., a different ratio of PDUs that need to be successfully transferred to the UE for different PDU Sets within the same QoS flow?

- Answer: So far RAN2 has not yet had sufficient time to analyse the proposal in detail. Based on initial analysis, RAN2 thinks that it may have negative impact on RAN performance due to intentional dropping of packets. Further analysis is required to fully assess those impacts. In addition, RAN2 thinks that it may introduce additional complexity for RAN on both UL and DL. For DL, while no impact on RAN2 specifications is foreseen, it may have impact on network implementation complexity. For UL, there might be potential impact on RAN2 specifications.

This dynamic change of the periodicity that provides the new value over UP would add complexity and can be challenging for UE/RAN to use it in real time. If change of periodicity information were shared via a semi-static behaviour (over control plane kind of information), this would be helpful assuming that the changes are provided earlier, and the new periodicity is used for certain time afterwards.

- Suggested RAN2 response to [Q1 in SA2 LS S2-2407351]: On whether to share periodicity via in-band signaling (i.e., GTP-U), RAN2 understands that this may introduce additional complexity for RAN on DL (as well as UL if it were enabled). For DL, while no impact on RAN2 specifications is foreseen, it can impact network implementation complexity due to the potential frequent changes which might be difficult to accommodate for. For UL, there might be some impact on RAN2 specifications (e.g. to convey this information over the air). If any, inputs on the changes of periodicity provided less frequently e.g., in a semi-static way (i.e., via CP) might be less complex to use.

### [Q2] time to next burst

SA2 provides the following regarding whether to share information about the next burst

Question2 [for SA4 and RAN2]: There is some discussion about time to next burst.

- To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6?

- Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

This topic is like previous SA2 question on whether the size of incoming burst is useful for RAN resource scheduling which RAN2 responded in LS [5] as follows:

Regarding Question 4 on whether the size of incoming burst is useful for RAN resource scheduling, RAN2 understands this question is for DL, so only discussed the DL. RAN2 understands the size of incoming burst is useful for gNB resource scheduling if it can be provided early enough, e.g. in the first packet of the burst

Therefore, we suggest replying similarly to SA2 explaining that the question/response focuses in DL traffic and indeed knowing the time of next burst can indeed be helpful if it is known early enough, e.g. as part of previous burst.

- Suggested RAN2 response to [Q2 in SA2 LS S2-2407351]: On whether time to next burst is useful for RAN resource scheduling, RAN2 understands this question is for DL and the time to next burst is useful for gNB resource scheduling if it can be provided early enough, e.g. as part of previous burst.

## Views on RAN3 incoming LS R3-243957 [2]

RAN3 provides the following regarding XR DC signaling enhancements for various bearer types:

In case of NR-DC split bearer, MN DU and SN DU congestion is local to each node. Based on congestion situation in MN DU and/or SN DU, each of the nodes independently send PSI-Based SDU Discard Activation/Deactivation MAC CE to UE for activation/deactivation of UL PSI based PDU set discarding of the split bearer. when UE receives MAC CE from both nodes or from a single node, it is not clear how UE will activate/deactivate UL PSI based PDU set discarding when split bearer is configured.

Thus, RAN3 kindly asks RAN2 to clarify the UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE.

During Rel-18 XR, split bearer and PSI discard was discussed in RAN2#125 and RAN2#125bis and the following agreements were taken:

RAN2#125

- For PSI-Based SDU Discard Activation/Deactivation MAC CE, Di field indicates the activation/deactivation status of the PSI-based SDU discard of DRB i, where i is the ascending order of the DRB ID among the DRBs configured with PSI-based SDU discard and with RLC entity(ies) associated with this MAC entity.

RAN2#125bis

- No further optimizations will be supported in Rel-18 for UL split-bearer

On this regard, the following was also captured accordingly in TS 38.321:

3	Activation/deactivation of PSI-based SDU discard

The network activates and deactivates PSI-based SDU discard by sending the PSI-Based SDU Discard Activation/Deactivation MAC CE described in clause 6.1.3.73. The PSI-based SDU discard is initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync.

The MAC entity shall for each DRB configured with discardTimerForLowImportance:

1>	if a PSI-Based SDU Discard Activation/Deactivation MAC CE is received activating the PSI-based SDU discard for the DRB:

2>	indicate the activation of the PSI-based SDU discard for the DRB to upper-layers.

1>	if a PSI-Based SDU Discard Activation/Deactivation MAC CE is received deactivating the PSI-based SDU discard for the DRB:

2>	indicate the deactivation of the PSI-based SDU discard for the DRB to upper-layers.

….

6.1.3.73		PSI-Based SDU Discard Activation/Deactivation MAC CE

The PSI-Based SDU Discard Activation/Deactivation MAC CE is identified by MAC subheader with an one-octet eLCID as specified in Table 6.2.1-1b.

It has a fixed size and consists of one octet defined as follows (Figure 6.1.3.73-1):

-	Di: This field indicates the activation/deactivation status of the PSI-based SDU discard of DRB i, where i is the ascending order of the DRB ID among the DRBs configured with discardTimerForLowImportance and with RLC entity(ies) associated with this MAC entity. The Di field set to 1 indicates that the PSI-based SDU discard shall be activated for DRB i. The Di field set to 0 indicates that the PSI-based SDU discard shall be deactivated for DRB i.

Figure 6.1.3.73-1: PSI-Based SDU Discard Activation/Deactivation MAC CE

When PSI based discard is activated, upper layers would trigger the start of a specific timer when other conditions are met as shown below on the reference from 38.323:

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

-	if discardTimerForLowImportance is configured and PSI based SDU discard is activated, and the PDCP SDU belongs to a low importance PDU Set:

-	start the discardTimerForLowImportance associated with this PDCP SDU;

-	else:

-	start the discardTimer associated with this PDCP SDU (if configured).

NOTE 0:	Identification of PSI of a PDU Set and determination of low importance PDU Set are left up to UE implementation.

While for split bearers (i.e., same DRB ID is used), it was preferable that both (i.e., the MN and SN) can provide the PSI-Based SDU Discard Activation/Deactivation MAC CE to the UE independently. In this regard, the UE would act on the corresponding MAC CE received (from MCG and/or SCG). From RAN2 point of view, when one of the paths has PSI based discard activated and other conditions are met, the UE will perform discard of low importance PDU Sets upon expiry of the corresponding timer. On summary, in Rel-18, RAN2 did not optimize the UE discard/handle when there is a conflict between the commands received from both nodes (i.e., MN and SN).

On the other hand, RAN3 has asked this open question as part of Rel-19 XR understanding that RAN2 decision was also driven due to the lack of time and agreement during Rel-18 XR. Therefore, we suggest for RAN2 to consider optimizing the corresponding UE operation on this e.g., as it was suggested in [6] which proposed the following:

- A Rel-19 UE with UL split-bearer only activates PSI-based SDU discard when it has received MAC CEs activating the PSI-based SDU discard from both MN and SN.

- A Rel-19 UE with UL split-bearer deactivates PSI-based SDU discard when it receives the MAC CE deactivating the PSI-based SDU discard from either MN or SN.

On summary, considering RAN3 question we suggest for RAN2 considering to also enhance this operation in Rel-19.

Rel-18 XR allows either or both MN or SN to provide PSI-Based SDU Discard Activation/Deactivation MAC CE when required (i.e., independently). UE behaviour treats the DRB as congested when MN and/or SN provides the PSI discard activation command (i.e., even if the activation comment comes from one of the nodes only).  RAN2 discussed and agreed not to enhance this.

- To agree on enhancing UE operation in Rel-19 XR with UL split-bearer when using PSI-based SDU discard as follows:

  - A Rel-19 UE with UL split-bearer only activates PSI-based SDU discard when it has received MAC CEs activating the PSI-based SDU discard from both MN and SN.

  - A Rel-19 UE with UL split-bearer deactivates PSI-based SDU discard when it receives the MAC CE deactivating the PSI-based SDU discard from either MN or SN.

- Suggested RAN2 response to [Q1 in RAN3 LS R3-243957]:

  - Rel-18 TS 38.321 in §6.1.3.73 explains how PSI-Based SDU Discard Activation/Deactivation MAC CE can be sent by either MN or SN independently to provide the maximum network flexibility/need. When either MN or SN provide a PSI-Based SDU Discard Activation, UE enables the mechanism that may trigger the discard of low importance PDU Sets as it is specified in TS 38.321 and 38.323.

  - If Proposal 3 is agreed (including Proposal 3.1 and Proposal 3.2), to also inform RAN3 on those corresponding new RAN2 agreements to be implemented as part of Rel-19 XR WI.

## Views on SA4 incoming LS S4-241370 [3]

It is important to review in a little detail SA4 latest inputs even though RAN2 is on CC in the SA4 LS [3] as this might impact some of the discussions ongoing for Rel-19 XR. More specifically, SA4 response on the PDU Set Importance (PSI) and the dependencies among PDU Set:

Question1 [for SA4, RAN2 and RAN3]: PDU Set correlation information (Sol#23) provides the dependency relationship among PDU Sets. Does SA4, RAN2 and RAN3 see any improvement with adding inter-PDU set correlation information to assist RAN making PDU set discarding decision as comparing to the existing (R18) PDU Set information that is already provided by the AS?

- Reply 1: Release 18 TS 26.522 clause 4.2.6.2 contains guidelines that assign PDU Set Importance implicitly taking the dependencies between PDU Sets into account. There is no consensus to add another solution besides the PDU Set importance in SA4. However, SA4 may further study this.

RAN2 is only CC in SA4 LS S4-241370 but it is important to discuss RAN2 potential impact from SA4 specification on TS 26.522 in §4.2.6.2.5 which contains guidelines on how the PDU Set Importance (PSI) is assigned as well as implicitly taking the dependencies between PDU Sets into account.

The following points could be highlighted from §4.2.6.2 in TS 26.522:

- SA4 TS 26.522 describes how PSI can be allocated and interpretated by RAN when having to perform discard (based on the value) in §4.2.6.2.1. Moreover, the video frames seem to be covered also here when using the language of reference and non-reference frames. In addition, SA4 seems to still be discussing how immerse audio is categorized.

4.2.6.2.1	General

In general, whenever the RAN needs to discard packets (e.g., under congestion situations), it is better to discard packets of lower importance rather than discarding packets randomly. If a discarded packet is critical for the media stream, the QoE may be severely degraded. For this reason, the PDU Set Importance (PSI) field can be used to mark PDU Sets with their importance level. The PSI field can then be used by the RAN to discard PDU sets. In case of congestion, PDU Sets with higher PSI values are more likely to be discarded.

PDU Sets that contain audio data should be assigned a lower PSI value (i.e., higher importance) compared with PDU Sets that contain other media types.

NOTE 1:  PDU Sets that carry immersive audio data are not necessarily assigned a lower PSI value compared with the other media PDU Sets. The PSI value of immersive audio PDU Sets is FFS.

PDU Sets that contains the reference frames present in the video bitstream should be assigned a lower PSI value compared with PDU Sets that contain non-reference frames.

NOTE 2:  It is assumed that the video bitstream uses referencing structures that have no coding delay caused by out-of-order output, as typically done for low-delay applications.

- Two sub-clauses provide suggested guidance for common video codecs: §4.2.6.2.2 for H.264/AVC Codec and §4.2.6.2.3 for H.265/HEVC Codec.

- Another sub-clause explains how the value of PSI is interpretated for discard based on affected PDU Sets in §4.2.6.2.4. In addition to the implicit information (via the value assigned to the PSI), there can be an explicit transfer of information about the attached frames (although it is unclear what this may refer to e.g. is this related to the size range or size of propagation error that they explain later on).

4.2.6.2.4	PSI based on affected PDU Sets

When the transport layer is forced to perform immediate dropping/discarding of a PDU Set but has a freedom of selection among the PDU Sets, the PDU Set with smaller degrees of artifact would be the better choice in most cases. Dropping of a PDU Set may corrupt the decoded output of itself and the other PDU Sets though they may already be transmitted perfectly to the receiving end or yet in a queue waiting to be transmitted. The degrees of artifact can be explicitly transferred as the number of affected frames which precedes/follows the PDU Set, or can be implicitly transferred as the importance value where the lower value means the higher PDU Sets are affected while higher values proportionally mean less number of PDU Sets are affected, for example. By considering such a quantization of various affected PDU Sets can be translated into importance field, using 4 bits to represent 16 possible size ranges is recommended.

The information on the size of propagation error which caused by the dropping of each PDU Set may be provided by the application layer. The information may present the size of error propagation implicitly with a proportional mapping of error propagation size to an index such as the importance of the PDU Set in the media stream.

The importance value of a PDU Set in PDU Set Information (PSI) RTP HE is set as follows:

-    The error propagation size is mapped to importance field value. The higher the error propagation size of a PDU Set, that PDU Set is more important, and it shall be assigned the lower PSI value. PDU Sets with low error propagation are of less importance and the PSI value for such PDU Sets shall be higher compared to PDU sets with higher error propagation size.

- Another sub-clause in §4.2.6.2.5 explains the inter-dependencies in for generic XR application.

4.2.6.2.5	PSI mapping based on PDU Set dependencies

RTP senders should consider that multiplexed RTP streams are treated as a single Multimedia Session and set the PSI field accordingly, i.e., the PSI field for one bitstream that depends on other RTP stream(s) in the same Multimedia Session may need to be set taking the PSI field for PDU Sets in other multiplexed RTP streams into account. In some cases, dependencies can exist across bitstreams even when they are not multiplexed, particularly for XR services.

In case of such dependencies, it may not be sufficient to set the PSI values based on codecs and media types alone. PSI values shall be set in this case based on the following, which are listed in an increasing order of importance, i.e., decreasing order of PSI values.

-    The PDU Set is considered not necessary for the processing of any other PDU Set. Such PDU Sets should be assigned the highest PSI values 14-15. When multiplexing, if a PDU Set is assigned PSI value of 15, similar PDU Sets of all streams should be assigned the PSI value 15 to prevent unfair treatment. If interdependency is known, e.g., in stereo streams (left eye is more important than right eye), then the more important stream can be assigned the PSI value 14.

-     In H.264/AVC, these include the PDU Sets with an NRI value equal to b00 in the NAL unit header.

-     In H.265/HEVC, the NAL unit header does not contain a field like NRI that indicates the relative transport priority. Hence, it is up to the application to identify such PDU Sets.

-    The PDU Set is necessary for the processing of some PDU Sets of the stream to which it belongs. Such PDU Sets should be assigned a PSI value in the range 9-13 (inclusive). The lower end of the range should be used for IDR/IRAP pictures since they are more important for decoding of the bitstream.

-     In H.264/AVC, these include:

-     IDR pictures with nal_unit_type equal to 5

-     Non-IDR pictures with nal_unit_type in the range 1 to 4 (inclusive)

-     In H.265/HEVC, these include:

-     IRAP pictures with nal_unit_type field assigned a value in the range 16 to 23 (inclusive)

-     RADL or RASL pictures with nal_unit_type in the range 6 to 9 (inclusive)

-    The PDU Set is necessary for the processing of all the other PDU Sets of the stream to which it belongs. Such PDU Sets should be assigned a PSI value in the range 6-8 (inclusive).

-     In H.264/AVC, these include:

-     SPS, PPS, i.e., NAL units with the nal_unit_type field equal to 7, 8, 13 or 15

-     In H.265/HEVC, these include:

-     SPS, PPS, VPS, i.e., NAL units with the nal_unit_type field in the range 32 to 34 (inclusive)

-    The PDU Set is necessary for the processing of some PDU Sets of the stream to which it belongs and also necessary for the processing of some PDU Sets of some other streams to which it does not belong. Such PDU Sets should be assigned a PSI value in the range 4-5 (inclusive).

NOTE 1:  Values in this and lower range shall be used for assigning PSI values to PDU Sets in multiplexed streams or if dependencies exist across non-multiplexed bitstreams. Use cases for those cases are FFS. In case only a single RTP stream is present, the ranges provided by the previous bullet points shall be used.

NOTE 2:  Considerations for multiplexed audio streams are FFS.

-    The PDU Set is necessary for the processing of all PDU Sets of the stream to which it belongs and also of some other streams to which it does not belong. Such PDU Sets should be assigned a PSI value in the range 2-3 (inclusive).

-    The PDU Set is necessary for the processing of all PDU Sets of all streams. Such PDU Sets should be assigned the lowest PSI value 1.

During Rel-18 XR, RAN2 has only differentiated between PDU Sets that are considered as low and high importance but it has not considered the 16 possible values that PSI may be allocated. Moreover RAN2 has also not considered the dependency between PDU Sets within same or different XR traffic stream(s).

SA4 Rel-18 TS 26.522 in §4.2.6.2 explains how the 16 different levels of PSI may be assigned depending on the kind of PDU Set, i.e., its importance (which includes the kind of traffic/application as well as its dependencies) within its XR traffic/stream and across all XR traffic/streams.

- Suggested RAN2 “impact” from [SA4 LS S4-241370] RAN2 scope for Rel-19 XR enhancements should take into consideration SA4 guidance defined in TS 26.522 §4.2.6.2 on how PSI is assigned depending on the kind of PDU Set and its dependency within its XR traffic/stream and across all XR traffic/streams.

  - If Proposal 5 is agreed, to send an LS to SA4/SA2 informing on the agreement and to also ask whether explicit indication could be defined to help UE/RAN with the identification of the dependent PDU Sets within its XR traffic/stream and across all XR traffic/streams belonging to the same Multimedia Session.

# Conclusion

The observations captured are the following:

Observation 1.	Rel-18 XR allows either or both MN or SN to provide PSI-Based SDU Discard Activation/Deactivation MAC CE when required (i.e., independently). UE behaviour treats the DRB as congested when MN and/or SN provides the PSI discard activation command (i.e., even if the activation comment comes from one of the nodes only).  RAN2 discussed and agreed not to enhance this.

Observation 2.	RAN2 is only CC in SA4 LS S4-241370 but it is important to discuss RAN2 potential impact from SA4 specification on TS 26.522 in §4.2.6.2.5 which contains guidelines on how the PDU Set Importance (PSI) is assigned as well as implicitly taking the dependencies between PDU Sets into account.

Observation 3.	SA4 Rel-18 TS 26.522 in §4.2.6.2 explains how the 16 different levels of PSI may be assigned depending on the kind of PDU Set, i.e., its importance (which includes the kind of traffic/application as well as its dependencies) within its XR traffic/stream and across all XR traffic/streams.

The proposals captured are the following:

Proposal 1.	Suggested RAN2 response to [Q1 in SA2 LS S2-2407351]: On whether to share periodicity via in-band signaling (i.e., GTP-U), RAN2 understands that this may introduce additional complexity for RAN on DL (as well as UL if it were enabled). For DL, while no impact on RAN2 specifications is foreseen, it can impact network implementation complexity due to the potential frequent changes which might be difficult to accommodate for. For UL, there might be some impact on RAN2 specifications (e.g. to convey this information over the air). If any, inputs on the changes of periodicity provided less frequently e.g., in a semi-static way (i.e., via CP) might be less complex to use.

Proposal 2.	Suggested RAN2 response to [Q2 in SA2 LS S2-2407351]: On whether time to next burst is useful for RAN resource scheduling, RAN2 understands this question is for DL and the time to next burst is useful for gNB resource scheduling if it can be provided early enough, e.g. as part of previous burst.

Proposal 3.	To agree on enhancing UE operation in Rel-19 XR with UL split-bearer when using PSI-based SDU discard as follows:

Proposal 3.1.	A Rel-19 UE with UL split-bearer only activates PSI-based SDU discard when it has received MAC CEs activating the PSI-based SDU discard from both MN and SN.

Proposal 3.2.	A Rel-19 UE with UL split-bearer deactivates PSI-based SDU discard when it receives the MAC CE deactivating the PSI-based SDU discard from either MN or SN.

Proposal 4.	Suggested RAN2 response to [Q1 in RAN3 LS R3-243957]:

Proposal 4.1.	Rel-18 TS 38.321 in §6.1.3.73 explains how PSI-Based SDU Discard Activation/Deactivation MAC CE can be sent by either MN or SN independently to provide the maximum network flexibility/need. When either MN or SN provide a PSI-Based SDU Discard Activation, UE enables the mechanism that may trigger the discard of low importance PDU Sets as it is specified in TS 38.321 and 38.323.

Proposal 4.2.	If Proposal 3 is agreed (including Proposal 3.1 and Proposal 3.2), to also inform RAN3 on those corresponding new RAN2 agreements to be implemented as part of Rel-19 XR WI.

Proposal 5.	Suggested RAN2 “impact” from [SA4 LS S4-241370] RAN2 scope for Rel-19 XR enhancements should take into consideration SA4 guidance defined in TS 26.522 §4.2.6.2 on how PSI is assigned depending on the kind of PDU Set and its dependency within its XR traffic/stream and across all XR traffic/streams.

Proposal 5.1.	If Proposal 5 is agreed, to send an LS to SA4/SA2 informing on the agreement and to also ask whether explicit indication could be defined to help UE/RAN with the identification of the dependent PDU Sets within its XR traffic/stream and across all XR traffic/streams belonging to the same Multimedia Session.

# Reference

- S2-2407351, LS on FS_XRM Ph2; From SA2; To: RAN2, SA4, RAN3; May 2024.

- R3-243957, LS on UL PSI based PDU discarding in NR-DC; From R3; To: R2; May 2024.

- S4-241370, LS Reply on FS_XRM Ph2, From: SA4, To: SA2, Cc: RAN2, RAN3, May 2024.

- R2-2405781, Reply LS on Application-Layer FEC Awareness at RAN, From: RAN2, To: SA2, Cc: RAN3, SA4, May 2024.

- R2-2405779, Reply LS to SA2 on FS_XRM PH2, From: RAN2, To: SA2, Cc: RAN3, SA4, May 2024.

- R2-2402387, Issues related to PSI-based discard, Qualcomm Incorporated, April 2024.