3GPP TSG RAN WG2 Meeting #127                                                           R2-2406473

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.7.2

Source:	Intel Corporation

Title:	UE/RAN enhancements considering multi-modal awareness

Document for:	Discussion and decision

# Introduction

This document discusses possible UE and RAN enhancements to enable Multi-modal awareness as it was agreed in RAN2#126 to “support Multi-Modality awareness in RAN in Rel-19 for UL and DL”. RAN2 conclusion will also require SA2 input on whether/what multi-modal information can be provided as requested in LS [1].

# Discussion

## Multi-modal kind of information

Multi-modal kind of information could be available in UE AS layer and RAN and considering RAN2#126 discussion (which were also included as possible examples in the LS [1] sent to SA2), this information could be used as follows:

- Identification of the different QoS flows that belong to the same multi-modal service, such as multi-modal identifier already defined by SA2 (i.e., MMSID).

- Inter-related synchronization between different QoS flows that belong to the same multi-modal service defined for example, as a maximum tolerable/synchronization delay/threshold.

SA2 input is required on which multi-modal information could be available in UE and/or RAN.

Assuming that some multi-modal kind of information is available, this could improve RAN2 related operations, e.g.:

- Reduce unnecessary traffic over the air by skipping (re)transmission of packets when “inter-related packets” are unnecessary. This “inter-related packets” refer to packets from different QoS flows belonging to the same multi-modal service that are associated/related considering the synchronization between those different XR flows. Unnecessary packets are when their related ones fail or are discarded, such, when not meeting their associated delay budget or expires their transmission for one of the related XR traffic flows.

- UL scheduling enhancements to prioritize or coordinate (re)transmissions of inter-related packets of different XR traffic flows. The description of “inter-related packets” of previous point (a) also applies here with the difference that UE and RAN use this information to prioritize or coordinate data transmission of the those related XR traffic flows of the specific multi-modal service.

- UE power saving by better aligning the C-DRX with the different/simultaneous XR traffic flows. Each XR traffic flow belonging to a multi-modal service can have its own and different QoS requirements and traffic characteristics. This may be different periodicities or active traffic duration among others.

- Awareness of multi-modal information in UE AS layer and RAN. If UE AS layer and RAN had awareness of multi-modal information between the different XR traffic flows (DRBs), further enhancements could also be defined to improve the selection of the optimum configuration(s), scheduling, prioritization, etc.

Multi-modal information could be used by RAN2 to (a) reduce traffic over the air (e.g., by skipping (re)transmission of packets when “inter-related packets” are unnecessary), (b) enhance UL scheduling (e.g., to prioritize or coordinate (re)transmissions of inter-related packets), (c) improve UE’s power consumption (e.g., by better aligning the C-DRX is with the different/simultaneous XR traffic flows) and (c) awareness of multi-modal service information in UE AS layer and RAN (e.g., to improve operations such as selection of configurations).

To be able to continue RAN2 discussion, we assume that some multi-modal information will be known/available in UE and RAN (i.e., equality for UL and DL traffic). Moreover, when discussing any AS/RAN enhancements, we suggest assuming the following:

- Each XR traffic flow with different QoS requirements that belongs to the same multi-modal service can be mapped to its own QoS flow and correspondingly to its own DRB (i.e., alternative 111 is allowed when mapping multiple/different XR traffic flows of one multi-modal services to QoS flows and DRBs).

- RAN and UE AS layer can have visibility on the association/relation between different XR traffic flows belonging to a Multi-modal Service, including when different QoS flows belong to one common multi-modal service and potentially, some synchronization information

- Baseline assumptions to discuss possible RAN2 enhancements considering multi-modal kind of information (even though SA2 input is finally needed):

  - Each XR traffic flow with different QoS requirements that belongs to the same multi-modal service can be mapped to its own QoS flow and correspondingly to its own DRB (i.e., alternative 111 is allowed when mapping multiple/different XR traffic flows of one multi-modal services to QoS flows and DRBs).

  - RAN and UE AS layer can have visibility on the association/relation between different XR traffic flows belonging to a Multi-modal Service, including when different QoS flows belong to one common multi-modal service and potentially, some synchronization information.

## Enhancements for inter-related XR traffic by reducing unnecessary traffic over the air and enhancing UL scheduling

Different XR traffic (e.g., audio, visual, tactile, haptic) is assumed periodic with some potential associated jitter (except for pose/control in UL which does not have jitter). Moreover, when those different XR traffic belong to one multi-modal service for one UE, synchronization information may be available between those inter-related XR traffic, such as maximum/tolerable delays between them. If so, this synchronization information of a multi-modal service could be used in a coordinated manner by UE/RAN (i.e., it should be prioritized or coordinate the delivery successful or not of the inter-related XR traffic) or this multi-modal associations could be considered when performing discard e.g., under congestion.  Figure 1 provides an example on how the inter-delay related info. may be applied between packets from three different XR traffic flows (A, B, C) that belongs to the same multi-modal service.

Figure 1. Example of inter-related maximum delays at the RX side for traffic (B) and (C) with (A) (where traffic flows (A), (B) and (C) belongs to a given multi-modal service).

- To agree on enhancing the UE AS layer and RAN operation, for at least the discard and UL scheduling, in relation to inter-related packets of different XR traffic flows belonging to the same multi-modal service.

## UE’s power consumption improvement

From power consumption point of view, it seems helpful to also enhance UE’s C-DRX operation considering that majority of XR traffic flows can be assumed periodic with certain jitter as explained above. We suggest for RAN2 to define new mechanisms that allow a UE to use multiple C-DRX configurations (or part of the configuration) in order to better fit the current/ongoing XR data traffic, as shown in Figure 2.

Figure 2. Combined C-DRX active time for multiple C-DRX configurations in used

Another aspect to discuss is whether those multiple DRX parameters (or configurations) can be (a) applied/used at the same time (i.e, in parallel) or (b) only pre-configured and UE switches between them (based on UE or network trigger). To get the maximum benefit when using multiple CDRX parameters/configurations, it seems preferable if UE uses them (1) in parallel or (2) switches without network indication but based on some pre-defined/configured rules/events or (3) with minimal signaling (e.g., L1 or maybe MAC CE) to trigger the change. If approach (3) has support, RAN1 input would be required to understand whether L1 signaling could be easily enhanced if multiple CDRX parameters/configurations are applied/used at the same time (i.e., in parallel) depending on the XR traffic flow.

- To defines a mechanism which allows the usage of multiple C-DRX configurations (or parameters) within a DRX group of a MAC entity. FFS whether they are used in parallel or whether certain trigger is used to switch between them.

# Conclusion

The observations captured are the following:

Observation 1.	SA2 input is required on which multi-modal information could be available in UE and/or RAN.

Observation 2.	Multi-modal information could be used by RAN2 to (a) reduce traffic over the air (e.g., by skipping (re)transmission of packets when “inter-related packets” are unnecessary), (b) enhance UL scheduling (e.g., to prioritize or coordinate (re)transmissions of inter-related packets), (c) improve UE’s power consumption (e.g., by better aligning the C-DRX is with the different/simultaneous XR traffic flows) and (c) awareness of multi-modal service information in UE AS layer and RAN (e.g., to improve operations such as selection of configurations).

The proposals captured are the following:

Multi-modal kind of information

Proposal 1.	Baseline assumptions to discuss possible RAN2 enhancements considering multi-modal kind of information (even though SA2 input is finally needed):

Proposal 1.1.	Each XR traffic flow with different QoS requirements that belongs to the same multi-modal service can be mapped to its own QoS flow and correspondingly to its own DRB (i.e., alternative 111 is allowed when mapping multiple/different XR traffic flows of one multi-modal services to QoS flows and DRBs).

Proposal 1.2.	RAN and UE AS layer can have visibility on the association/relation between different XR traffic flows belonging to a Multi-modal Service, including when different QoS flows belong to one common multi-modal service and potentially, some synchronization information.

Enhancements for inter-related XR traffic by reducing unnecessary traffic over the air and enhancing UL scheduling

Proposal 2.	To agree on enhancing the UE AS layer and RAN operation, for at least the discard and UL scheduling, in relation to inter-related packets of different XR traffic flows belonging to the same multi-modal service.

UE’s power consumption improvement

Proposal 3.	To defines a mechanism which allows the usage of multiple C-DRX configurations (or parameters) within a DRX group of a MAC entity. FFS whether they are used in parallel or whether certain trigger is used to switch between them.

# Reference

- R2-2405782, LS on multi-modality awareness at RAN, To: SA2; CC R3, SA4; May 2024.