3GPP TSG RAN WG2 Meeting #127                                                           R2-2406474

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.7.4

Source:	Intel Corporation

Title:	Scheduling enhancements using delay related information

Document for:	Discussion and decision

# Introduction

This document continues discussion on how to enhance UL scheduling using delay/deadline information and DSR.

# LCH enhancements (including prioritization, selection)

The following are RAN2 agreements associated to LCH enhancements including prioritization, restriction or selection.

RAN2#125bis agreements:

- RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied:

  - Alternative 1: Enhance LCP restrictions/LCH selection.

  - Alternative 2: Enhance LCH prioritization.

- RAN2 should consider potential impact on traffic from SRBs.

RAN2#126 agreements:

- LCP enhancements – LCH prioritization

  - Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR.

  - The solution should consider impact on UE complexity (as already indicated in SI objective description)

  - For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline:

    - Use additional priority configured to LCHs in case of these LCHs with delay-critical data

    - FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH

- LCP enhancements – granularity

  - LCP prioritization within a logical channel will not be considered in RAN2 discussions

  - FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

- LCP enhancements – LCH restrictions

  - We try to avoid RAN1 impacts.

  - RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way.

  - For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial

  - The solutions should not disallow non-delay critical data from using an UL grant.

LCP mechanism does not change the priorities after data is assigned to one channel. However, it might be desirable to “change” (or adapt) the priority/handling associated to the packets after being in UE’s buffers under specific situations e.g., when the packet delay budget (PDB) requirement is tight. Figure 1 provides an example on how logical channel prioritization of buffered data may work based on legacy behaviour (in the left) or with a new prioritization on top (in the right) where data in LCH2 with priority p2 gets prioritized for transmission over other in LCH1 with priority p1. This example shows how prioritization within a logical channel is not used (i.e., both green and pink labeled data available in LCH2 is prioritized even though the pink one is the one that triggered the event).

Figure 1. LCH prioritization of buffered data as per legacy (left) or with a new prioritization on top (right)

On summary, when logical prioritization mechanism allows to change or adapt the prioritization associated to data available in UE’s buffers after it is assigned to one specific logical channel, it is important to deliver this data in order, while maintaining the benefits of the current LCH prioritization mechanism such as to prevent starvation among logical channels, to maintain a clean separation between logical channels and not to add too much complexity on the LCH prioritization mechanism.

Resource allocation can be changed/adapted for specific packets that require a higher priority of transmission such that these packets can be transmitted before data of other logical channels but without treating differently packets stored within a specific logical channel.

## Triggering events to change priority

Different trigger events could be considered to change the priority of transmission for a logical channel considering the buffered data:

- Trigger (1) Maximum/tolerable delay related information considering PDB, PSDB or remaining time delay. This remaining delay helps to understand the urgency with which the corresponding data needs to be sent before it becomes invalid in Tx side.

- Trigger (2) PDUs belonging to the same PDU Set. When prioritizing transmission of data, it could be preferable to send all packets belonging to the same PDU Set at the same time instead of only parts of them.

- Trigger (3) Importance of the data. When prioritizing transmission of data, it may be preferable to do so for data identified as critical data or high importance of PDU Sets. During Rel-18 XR, “delay critical PDCP SDU” (and correspondingly “delay critical RLC SDU”) and “low importance PDU Set” were defined. This kind of information could be potentially used for this trigger (3).

- Trigger (4) Multi-modal related information. When prioritizing transmission of data, it could be preferable to send related packets of different XR traffic flows belonging to the same multi-modal service. However, this topic is dependent on SA2 input and RAN2 conclusion of the study on whether it is helpful for RAN and UE AS layer to enhance its operation considering multi-modal kind of information.

Until RAN2 further progress on the details related to the multi-modality awareness in RAN, we suggest not to consider the trigger event (4). At the minimum, the trigger event (1) shall always be used (when configured) for UE to determine when to change the priority associated to a logical channel and potentially could be left up to network decision whether trigger events (2) and (3) should also be met on top of that. The network decision on whether to also configured trigger events (2) and (3) would depend on the characteristics/requirements of the different XR application.

- The following trigger events can change the prioritization of transmission associated to a logical channel based on its buffered data: (1) maximum tolerable delay (i.e., measured as remaining time is below certain threshold), (2) PDUs belonging to the same PDU Set (i.e., to prioritize sending all packets belonging to the same PDU Set at the same time), and (3) importance of data (i.e., data categorized as delay critical or belonging to high importance PDU Set).

  - The trigger event used to change the priority of transmission associated to a logical channel is configurable per UE and per LCH.

    - When network configures a different/new priority of transmission associated to a logical channel, trigger event (1) is always be used. The trigger event (2) and (3) can be optionally configured by network on top, when applicable.

## Operation to change the priority

The following explain two options how prioritization of transmissions could be changed for specific buffered data (without discussing the possible trigger events just explained):

- Solution (1) Reflective QoS mechanism (network-controlled mechanism). Legacy reflective QoS mechanism could be re-used on top the new trigger events just explained to change its priority associated to the LCH. This solution would not require much specification work as it would rely on network action to change temporarily the priority of the DRB by sending a packet in that QoS flow via a different DRB in DL and that makes the UE to switch the traffic in UL to the same new DRB as the DL packet. The network decision could be based on information provided by the BSR, delay information in the DSR or other information internal to the network.

- Solution (2) UE autonomous temporary change of the logical channel priority (UE-controlled mechanism). Solutions could be discussed that allow a temporary change the priority associated to specific packet and potentially all other packets on the same logical channel that were earlier than this one on the queue, as shown in T1 and T2 behaviour of below Figure 2. For this, the aim should be to enable a mechanism that minimizes impacts to legacy LCH prioritization that still prevent starvation among the logical channels and provide a clean separation of data from the logical channels.

Figure 2. Example of UE’s autonomous and temporary change of LCH prioritization for transmission

- To change the priority for earlier transmission of specific packet associated logical channel (after a specific packet has already been associated with a specific logical channel), to consider/discuss: solution (1) to reuse reflective QoS mechanism or solution (2) to define a simple mechanism that allows a temporary change of its current priority. NOTE: solution (2) only changes the priority associated to a logical channel until transmitting the affected packets (i.e., those packets which met the trigger event(s) explained in Proposal 1).

# DSR enhancements

The following are RAN2 agreements associated to LCH enhancements including prioritization, restriction or selection.

RAN2#125bis agreements:

- RAN2 will study enhancing existing DSR with additional information, e.g. multiple pairs of remaining time/buffer information, importance. FFS whether this only includes more information on delay-critical data or also information about non-delay critical data.

RAN2#126 agreements:

- DSR enhancements – multiple thresholds and grouping of data

  - Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG.

  - FFS whether DSR triggering is impacted

  - FFS whether PDU set importance needs to be included

- DSR enhancements – whether to include non-delay critical data

## Background on Rel-18 DSR

Firstly, it is useful to highlight few points from Rel-18 DSR operation before discussing how to enhance it.

- Rel-18 DSR MAC CE includes the delay information (i.e. the Remaining Time and Buffer Size fields) of all LCGs which have pending DSRs when the MAC PDU containing this DSR MAC CE is to be built. Few additional points to highlight:

- The Remaining Time indicates the shortest remaining value of running PDCP discardTimer among all PDCP SDUs that are buffered for an LCG but have not been transmitted in any MAC PDU, at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE.

- There is a flag (i.e., referred as BT) per LCG to indicate (i.e., when set to 1) that the corresponding LCG is configured with additionalBS-TableAllowed and the buffer size indicated by the corresponding Buffer Size field is not zero.

- The Buffer Size indicates the total amount of delay-critical UL data for an LCG according to the data volume calculation procedure specified in TS 38.322 and TS 38.323 for the associated RLC and PDCP entities, respectively, after the MAC PDU has been built.

- DSR is triggered when a remaining time of the running PDCP discardTimer is below certain threshold.

If an LCG is configured for delay status reporting, the MAC entity shall for each LCH within the LCG:

1>	if the smallest remaining value of the running PDCP discardTimer among all the PDCP SDUs buffered for the LCH that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and

1>	if there is no DSR pending for the LCH:

2>	trigger a DSR for the LCH.

- PDCP discardTimer applies for case (b.1) any PDU (which can belong to any PSI level if it belongs to a PDU Set) when PSI-Discard is not configured/activated, or case (b.2) only to PDUs belonging to high importance PDU Sets when PSI-Discard is configured/activated.

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

if discardTimerForLowImportance is configured and PSI based SDU discard is activated, and the PDCP SDU belongs to a low importance PDU Set:

start the discardTimerForLowImportance associated with this PDCP SDU;

else:

start the discardTimer associated with this PDCP SDU (if configured).

Therefore, when PDUs belonging to low importance PDU Sets and PSI-Discard is configured/activated case (b.3), those SDUs trigger the start of a different PDCP discard timer, i.e. discardTimerForLowImportance. This would never trigger DSR reporting. It may be unclear whether it is or not critical to report this information understanding that PSI based discard is only used by network when suffering congestion in which case low priority PDU Set are not considered as critical.

Summarizing, the following observations highlights key aspects to considering when discussing desirable/further Rel-19 enhancements to the discard mechanism.

Rel-18 DSR MAC CE only includes the delay information (i.e. the Remaining Time and Buffer Size fields) per applicable LCG and this indicates the shortest remaining value of running PDCP discardTimer associated to a LCG with total amount of delay-critical UL data

PDCP discardTimer is used to trigger DSR. I.e., DSR is not triggered by SDUs that use PDCP discardTimerForLowImportance instead than discardTimer.

The following summarizes how discard enhancements were defined in Rel-18 XR:

  - [case (b.1)] When PSI based SDU discard is not activated:

    - PDCP discardTimer is used for any SDUs (i.e. there is no different behaviour based on PSI level).

  - [case (b.2)] When discardTimerForLowImportance is configured and PSI based SDU discard is activated:

    - PDCP discardTimer can only be used for SDUs belonging to a high importance PDU Set.

  - [case (b.3)] When discardTimerForLowImportance is configured, PSI based SDU discard is activated and SDU belongs to a low importance PDU Set,

    - PDCP discardTimer is not used for SDUs belongs to a low importance PDU Set.

## Importance of a PDU Set in the enhanced DSR

Considering Rel-18 specification, when PSI based SDU discard is not activated [case (b.1) just explained], both low and high importance SDUs will trigger the DSR and hence network would not know if what triggered the DSR belong to high or low importance PDU Sets. This kind of information could be helpful for the network to better prioritize the scheduling of the UE’s buffered data that is close to expiry. For example, if the DSR was triggered by an SDU of low importance, network may not want to prioritise this data.

- DSR is enhanced to also include importance level of the PDU Set. This importance level of the PDU Set (i.e., high or low) is associated with the SDU with the smallest of the PDCP discardTimer that triggered DSR.

## Low importance PDU Set in the enhanced DSR

When network is under congestion scenarios and network configures and actives both PSI based SDU discard and discardTimerForLowImportance [case (b.2)], it does not seem critical for UE to report information of the remaining time of SDU belongs to a low importance PDU Set (understanding that their transmission may not be as critical/important in comparison to other important data).

- DSR is not enhanced in Rel-19 to include remaining time information of SDU belongs to a low importance PDU Set when discardTimerForLowImportance is configured and PSI based SDU discard is activated (which should only be configured under congestion scenarios).

## Triggering of the enhanced DSR

RAN2 agreed that the enhance DSR can report multiple pairs of remaining time and buffer size for one LCG instead of only including the smallest of the remaining times, as defined in Rel-18 (and shown in above reference). For case (b.1), it is important to clarify whether any of the remaining times are to be reported or only those which are below the threshold and those that belongs to a high importance PDU Set.

- When reporting multiple remaining time (with their corresponding buffer size) in the enhanced DSR, UE only reports information associated to an SDU belonging to a high importance PDU Set / delay-critical UL data which has its PDCP discardTimer below the configured remainingTimeThreshold.

In addition, RAN2 could discuss whether a buffer size threshold may want to be defined as an additional trigger of these enhanced DSR and if so, whether it could be used by itself of in addition to the triggering condition associated to the remainingTimeThreshold

- To discuss whether a new buffer size threshold is also used to trigger the enhanced DSR reporting.

# Conclusion

The observations captured are the following:

LCH enhancements

Observation 1.	Resource allocation can be changed/adapted for specific packets that require a higher priority of transmission such that these packets can be transmitted before data of other logical channels but without treating differently packets stored within a specific logical channel.

Background on Rel-18 DSR

Observation 2.	Rel-18 DSR MAC CE only includes the delay information (i.e. the Remaining Time and Buffer Size fields) per applicable LCG and this indicates the shortest remaining value of running PDCP discardTimer associated to a LCG with total amount of delay-critical UL data

Observation 3.	PDCP discardTimer is used to trigger DSR. I.e., DSR is not triggered by SDUs that use PDCP discardTimerForLowImportance instead than discardTimer.

Observation 4.	The following summarizes how discard enhancements were defined in Rel-18 XR:

o	[case (b.1)] When PSI based SDU discard is not activated:

	PDCP discardTimer is used for any SDUs (i.e. there is no different behaviour based on PSI level).

o	[case (b.2)] When discardTimerForLowImportance is configured and PSI based SDU discard is activated:

	PDCP discardTimer can only be used for SDUs belonging to a high importance PDU Set.

o	[case (b.3)] When discardTimerForLowImportance is configured, PSI based SDU discard is activated and SDU belongs to a low importance PDU Set,

	PDCP discardTimer is not used for SDUs belongs to a low importance PDU Set.

The proposals captured are the following:

** LCH enhancements (including prioritization, selection) **

Triggering events to change priority

Proposal 1.	The following trigger events can change the prioritization of transmission associated to a logical channel based on its buffered data: (1) maximum tolerable delay (i.e., measured as remaining time is below certain threshold), (2) PDUs belonging to the same PDU Set (i.e., to prioritize sending all packets belonging to the same PDU Set at the same time), and (3) importance of data (i.e., data categorized as delay critical or belonging to high importance PDU Set).

Proposal 1.1.	The trigger event used to change the priority of transmission associated to a logical channel is configurable per UE and per LCH.

Proposal 1.1.1.	When network configures a different/new priority of transmission associated to a logical channel, trigger event (1) is always be used. The trigger event (2) and (3) can be optionally configured by network on top, when applicable.

Operation to change the priority

Proposal 2.	To change the priority for earlier transmission of specific packet associated logical channel (after a specific packet has already been associated with a specific logical channel), to consider/discuss: solution (1) to reuse reflective QoS mechanism or solution (2) to define a simple mechanism that allows a temporary change of its current priority. NOTE: solution (2) only changes the priority associated to a logical channel until transmitting the affected packets (i.e., those packets which met the trigger event(s) explained in Proposal 1).

** DSR enhancements **

Importance of a PDU Set in the enhanced DSR

Proposal 3.	DSR is enhanced to also include importance level of the PDU Set. This importance level of the PDU Set (i.e., high or low) is associated with the SDU with the smallest of the PDCP discardTimer that triggered DSR.

Low importance PDU Set in the enhanced DSR

Proposal 4.	DSR is not enhanced in Rel-19 to include remaining time information of SDU belongs to a low importance PDU Set when discardTimerForLowImportance is configured and PSI based SDU discard is activated (which should only be configured under congestion scenarios).

Triggering of the enhanced DSR

Proposal 5.	When reporting multiple remaining time (with their corresponding buffer size) in the enhanced DSR, UE only reports information associated to an SDU belonging to a high importance PDU Set / delay-critical UL data which has its PDCP discardTimer below the configured remainingTimeThreshold.

Proposal 6.	To discuss whether a new buffer size threshold is also used to trigger the enhanced DSR reporting.