3GPP TSG RAN WG2 Meeting #127                                                           R2-2406475

# Maastricht, The Netherlands, 19 – 23 Aug, 2024

Agenda item:	8.7.5

Source:	Intel Corporation

Title:	RLC AM enhancements for XR traffic

Document for:	Discussion and decision

# Introduction

RAN2 had extended discussion on RLC retransmission enhancements for XR and made the following agreements [1]:

Avoiding unnecessary retransmissions For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned Autonomous retransmissions To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

In this contribution, we continue the discussion on the possible solutions considered to provide timely RLC re-transmission, as well as, on the TX and RX initiated approaches for avoiding unnecessary retransmissions presenting our company views.

# Discussion

In the last meeting, there was extended discussion on the need for enhancing RLC operation by considering both autonomous retransmission (in a proactive fashion), as well as how to avoid triggering unnecessary retransmissions. In both cases, we think it would be good to first clarify what criteria shall be used by the RLC layer to determine whether the retransmission should be triggered autonomously or not triggered at all. Note that currently, RLC sublayer does not take into consideration the remaining Packet Delay Budget (PDB) associated to a packet. For example, current RLC (re)transmission procedure does not consider the packet delay budget associated with its corresponding packet and whether the (re)transmissions may be useful or not at the receiver. Figure 1 below shows how PDU 2 is retransmitted by the TX entity even though the remaining PDB is low (which may end up in a discard at the RX even if it is received successfully). In summary, current RLC operation may lead to inefficiencies in retransmission decisions and may also result in a wastage of network resources, especially for time-sensitive traffic such as XR applications.

Figure 1. Issue with RLC (re)transmissions without awareness of the associated PDB

RLC AM mechanism does not take into account the remaining packet delay budget of the data traffic when performing (re)transmissions. If this new information were considered, RLC operation could be enhanced to ensure a timely RLC (re-)transmissions and avoid unnecessary (re-)transmissions.

- RLC AM (re)transmission mechanism is enhanced based on the remaining delay budget associated with each packet.

## Timely RLC re-transmission(s)

One of the scenarios where enhancements are needed relate to autonomous RLC retransmissions. Three different options were captured for discussion/consideration in the last meeting:

- Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered)

- Retransmission based on enhanced status report

- Retransmission based on enhanced polling

In the following discussion, we focus on the first option, which does not require any explicit triggering received in Tx side from Rx (i.e., without status report). In this case, RLC could enhance its operation when using the remaining delay budget of a packet (e.g. when the remaining PDB exceeds certain threshold by proactively performing re-transmission(s). i.e., if the UE has not yet received a NACK for a given unacknowledged packet (to trigger retransmission), the UE can autonomously decide to perform a (proactive) retransmission. Figure 2 depicts an example where one PDU is proactively retransmitted when its remaining delay is below a threshold (here referred as threhlow).

Figure 2. Example scenario of proactive RLC re-transmission

This operation could help avoiding or alleviating the occurrence of the scenario explained in section 2.1 (i.e., when the status report and the triggered retransmission is too late). It is also possible that the retransmission is not actually needed but even if it is the case, this should not be a major issue as the receiving entity can already detect if a received packet is a duplicate or not; i.e., there is no special handling required at the receiver (i.e., the RX entity can simply discard the duplicated PDU). Also, depending on the configuration of the threshold for remaining PDB, the network can control the signaling overhead associated with such autonomous retransmissions in the UL. Therefore, we think the impact to overall capacity should be small and easily manageable based on proper configuration.

Proactive RLC AM re-transmission would still be under network control (e.g. configuring the threshold for the remaining time that would trigger this retransmission) which could limit the impact to overall capacity and limit the signaling overhead required.

To enable this proactive RLC re-tx operation, Tx side would need to keep track of the remaining delay budget associated to each packet at the RLC. PDCP layers currently performs a similar operation via the PDCP discardTimer or discardTimerForLowImportance.

- discardTimer. This timer is configured only for DRBs. The duration of the timer is configured by upper layers TS 38.331. In the transmitter, a new timer is started upon reception of an SDU from upper layer.

- discardTimerForLowImportance. This timer is configured only for DRBs. The duration of the timer is configured by upper layers TS 38.331. In the transmitter, a new timer is started upon reception of an SDU belonging to a low importance PDU Set from upper layer.

One option is to reuse these PDCP discard timers to inform RLC when the remaining delay budget associated to each packet is below a configurable threshold. Another option is to create a new timer at the RLC Tx side specific for the handling of RLC (re)transmission (or in the case of next enhancement skipping). This new RLC based timer has the advantage that it is specific for each RLC SDU/PDU and might be easier to specify the relation between an RLC-specific timer and the handling of the corresponding RLC SDU/PDUs. On the other hand, RAN2 usually avoid defining similar mechanisms across different layers unless it is strictly necessary if a technical concern or large complexity is found. Therefore, we suggest considering as baseline (initial assumption) to reuse legacy PDCP discard based timer(s) first.

- Proactive RLC AM re-transmission is performed when the remaining delay budget associated to each packet is below a configurable threshold (referred for the discussion as T1).

  - If Proposal 2 is agreed (i.e., to trigger proactive RLC AM re-tx), to re-use PDCP discard timers (i.e., discardTimer and discardTimerForLowImportance) to inform RLC when PDCP discard timer is below a configured threshold.

## Avoiding unnecesary RLC re-transmission(s)

In the last meeting, two different approaches were discussed when it comes to avoiding unnecessary re-transmissions.

For the TX initiated approach, the TX RLC AM entity needs to know whether to skip retransmission for a SDU which has not yet been acknowledged. To do so, the TX entity actually determines that the reception of this SDU(s) is not useful anymore and it then notifies the RX entity (e.g., via SN gap reporting) that the retransmission of these SDU(s) will be skipped. This then allows the RX entity to stop triggering NACK for those SDUs and updated the receiver window accordingly to avoid stalling.

Figure 3 depicts an example where selective skipping of RLC retransmission is performed following a TX initiated approach, i.e., when the remaining delay budget associated to each packet (referred as t-pdb) is below a configurable threshold (referred as threshhigh).

Figure 3. Example scenario of selective skipping of RLC (re)transmission via TX initiated approach

In contrast, with the RX initiated approach, it is the RX RLC AM entity which is responsible for determining whether a certain SDU or SDUs shall not be retransmitted, presumably because they are out of date and no longer needed at the receiver. This can be realized by the RX entity not transmitting a NACK for those SDU(s) to the TX entity, thereby informing the TX entity that the SDU(s) was successfully received, and the TX window can keep updating.

In both the schemes, it is imperative to discuss how the TX/RX entity is able to determine whether a given SDU is “out of date” and does not need to be retransmitted. For the case of the TX initiated approach, as we have discussed above, there is a clear relationship with the remaining PDB, i.e. how much more delay the packet can tolerate before it becomes obsolete. For the case of RX initiated case, it is harder to determine since the RX entity is not aware of the remaining PDB of a packet which it is yet to receive. Some companies have mentioned that a counter or timer can be defined at the RX entity for this purpose, but this still leaves the question of how this timer is set/configured. For instance, will a similar discard timer be defined in the RX that needs to be kept track per packet level or will an additional/shorter time window be defined that will be kept per packet level. Also, how would RX side keep track of this understanding that packets can be transmitted out of order in some scenarios.

| Scheme | Pros | Cons |
| --- | --- | --- |
| TX initiated approach | TX entity is easily able to determine whether a packet is “out-of-date” based on remaining PDB (as it is a similar modeling to legacy discard timer operation) [Limited complexity] | The SN gap at the RX side needs to be addressed i.e. via SN gap reporting. FFS if Rel-18 SN gap reporting is sufficient [Limited complexity] |
| RX initiated approach | No additional SN gap reporting needed (other than RLC status report) [No additional complexity] | Not easy/clear how to determine the need for triggering retransmission of a given PDU at the RX side [Larger complexity] |

Table 1 Analysis of TX and RX initiated approaches for skipping RLC retransmissions

From the table above, it can be seen that both schemes have their pros and cons but could be made to work. In our view, we prefer the TX initiated approach, since the fundamental issue of how to determine whether a given SDU retransmission is useful or not can be better addressed using this scheme and is more aligned to legacy RLC operation at the packet level. I.e., Tx side can keep track of the remaining delay budget associated to each packet at the RLC, which could be done, e.g. by reusing PDCP discard timers (as discussed in section 2.1). Selective skipping of RLC (re)transmission considering the remaining delay time seems helpful and feasible to avoid unnecessary (re)transmission of packets over the air (which are discarded in the Tx side). For this, RAN2 could discuss whether Rel-18 PDCP SN gap reporting is sufficient, or additional enhancements are needed.

When performing selective skipping of RLC AM (re)transmission, RLC SN gaps may appear which may be solved reusing or, if required, enhancing further Rel-18 PDCP SN gap reporting functionality.

Both TX initiated and RX initiated approach are feasible modelling to enable the skipping enhancement for RLC AM (re)transmission. However RX initiated approach seems to introduce a larger/functional complexity when RX side needs to determine whether a given SDU retransmission is useful or not, i.e. to keep track of the remaining delay budget associated to each packet at the RLC layer.

- The TX initiated approach is used to enable the skipping enhancement for RLC AM (re)transmission.

- Selective skipping of RLC AM (re)transmission is performed when the remaining delay budget associated to each packet is below a configurable threshold (referred for the discussion as T2).

- If Proposal 3 is agreed (i.e., TX initiated approach is used) and Proposal 4 is agreed (i.e. to trigger skipping of RLC AM re-(tx) using remaining delay budget of a packet), the PDCP discard timer (i.e., discardTimer and discardTimerForLowImportance) is reused to inform RLC when PDCP discard timer is below T2 for specific SDU.

## Joint enhancement efforts at RLC

Either of the enhancements just discussed could be used individually or together as Figure 4 depicts. This Figure 4 explains how over time different regimes/operations for RLC (re)transmission may be more suitable based on the remaining delay/PDB of the associated packet

Figure 4.  Usage of different RLC AM (re)transmission regimes based on remaining delay budget of a packet

Summarizing, we suggest considering mechanism possible enhancements to RLC AM (re)transmission as complementary targeting different goals.

- Network can configure different threshold when enabling the usage of autonomous RLC retransmission (when remaining delay time associated with a packet is below T1 but above T2) and the usage of selective skipping of RLC (re)transmission (when remaining delay time associated with a packet is below T2).

  - FFS whether T2 is or not the same as legacy value of the PDCP discard timer (including both discardTimer and discardTimerForLowImportance).

# Conclusion

This contribution continues the discussion on need for RLC retransmissions and makes the following observations and proposals:

- RLC AM mechanism does not take into account the remaining packet delay budget of the data traffic when performing (re)transmissions. If this new information were considered, RLC operation could be enhanced to ensure a timely RLC (re-)transmissions and avoid unnecessary (re-)transmissions.

Proactive RLC AM re-transmission would still be under network control (e.g. configuring the threshold for the remaining time that would trigger this retransmission) which could limit the impact to overall capacity and limit the signaling overhead required.

When performing selective skipping of RLC AM (re)transmission, RLC SN gaps may appear which may be solved reusing or, if required, enhancing further Rel-18 PDCP SN gap reporting functionality.

Both TX initiated and RX initiated approach are feasible modelling to enable the skipping enhancement for RLC AM (re)transmission. However RX initiated approach seems to introduce a larger/functional complexity when RX side needs to determine whether a given SDU retransmission is useful or not, i.e. to keep track of the remaining delay budget associated to each packet at the RLC layer.

- RLC AM (re)transmission mechanism is enhanced based on the remaining delay budget associated with each packet.

- Proactive RLC AM re-transmission is performed when the remaining delay budget associated to each packet is below a configurable threshold (referred for the discussion as T1).

  - If Proposal 2 is agreed (i.e., to trigger proactive RLC AM re-tx), to re-use PDCP discard timers (i.e., discardTimer and discardTimerForLowImportance) to inform RLC when PDCP discard timer is below a configured threshold.

- The TX initiated approach is used to enable the skipping enhancement for RLC AM (re)transmission.

- Selective skipping of RLC AM (re)transmission is performed when the remaining delay budget associated to each packet is below a configurable threshold (referred for the discussion as T2).

- If Proposal 3 is agreed (i.e., TX initiated approach is used) and Proposal 4 is agreed (i.e. to trigger skipping of RLC AM re-(tx) using remaining delay budget of a packet), the PDCP discard timer (i.e., discardTimer and discardTimerForLowImportance) is reused to inform RLC when PDCP discard timer is below T2 for specific SDU.

- Network can configure different threshold when enabling the usage of autonomous RLC retransmission (when remaining delay time associated with a packet is below T1 but above T2) and the usage of selective skipping of RLC (re)transmission (when remaining delay time associated with a packet is below T2).

  - FFS whether T2 is or not the same as legacy value of the PDCP discard timer (including both discardTimer and discardTimerForLowImportance).

# References

| [1] | RAN2#126, Chairman Notes. |
| --- | --- |