3GPP TSG-RAN WG2 Meeting #127												R2-2406481

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:	8.7.5

Source: 	Sharp

Title:  	Analysis of RLC AM Enhancements

Document for: 	Discussion

# Introduction

In RAN2#126 meeting, RAN2 made the following agreements:

Avoiding unnecessary retransmissions

For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives:

- Rx initiated approach

- Tx initiated approach

RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer.

For Tx initiated approach:

    - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs

    - Tx side stops retransmit obsolete SDUs

    - Rx side updates state variables according to the information from Tx side

For Rx initiated approach:

    - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter.

    - Tx side just processes the status report as in legacy

    - FFS how Rx side determines that an SDU should be abandoned

Autonomous retransmissions

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options:

    - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered)

    - Retransmission based on enhanced status report

    - Retransmission based on enhanced polling

- FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism

Impact on capacity should be considered

RAN2 focuses on the enhancements for UL traffic

This document discusses detailed mechanisms for those candidate solutions.

# Discussion:

## Avoiding Unnecessary Retransmissions of Outdated Packets

There are two directions of potential solutions: TX-initiated mechanism and RX-initiated mechanism. In this section, we analyse the potential spec impact and limitations for both mechanisms.

< Triggering Condition of Indication >

In TX-initiated mechanism, the transmitter decides whether the packet is outdated. The decision is basically based on PDCP discard timer which starts at the arrival in AS layer. Usually, the expiry of the discard timer is considered as the definition of outdated packet. Thus, it will be a reuse of the existing timer operation without change. One may argue that a shorter timer than PDCP discard timer is needed. However, if the PDCP discard timer is running, we can say that the packet is usually not outdated. So, no further complexity is necessary.

In RX-initiated mechanism, the receiver decides whether the packet is outdated. Since the receiver does not know the exact arrival time precisely, it should only rely on out-of-order reception. The existing t-reassembly timer cannot be reused, as the length of the timer is set to sufficient HARQ retransmissions whose duration is much smaller than required delay of the outdated packet. Thus, a new triggering condition with longer timer than t-reassembly should be captured in RLC specification. It may be either a timer or counter.

RX-initiated mechanism requires a new triggering mechanism whereas TX-initiated mechanism can reuse the existing PDCP discard timer.

Since the RX-initiated mechanism should rely on out-of-order reception, it inherits some delay until at least one out-of-order delivery is detected. Figures 1 and 2 compares detection of TX-based mechanism and RX-based mechanism. Figure 1 assumes that RX-based mechanism knows the outdated packet after the expiry of t-assembly, although it is the theoretical (not so practical) minimum time to declare the outdated packet. The figures show that RX-initiated mechanism has unnecessary delay until the variable update. Also, the buffered delay at the TX side cannot be taken into account. It is reasonable to set multiple expiries of t-reassembly timer should be used for the detection of outdated packet. In this case, the first expiry may not trigger to set the reception status as ACKed, then the delay may increase or unnecessary retransmission may not be avoided.

RX-initiated mechanism has inaccuracy of the detection of outdated packet, e.g., delayed detection.

Figure 1. Delayed detection of outdated packet of RX-initiated mechanism

Figure 2. Immediate detection of outdated packet of TX-initiated mechanism

< Indication to Stop Retransmissions >

The major spec impact of TX-initiated mechanism is an indication that the transmitter (UE in uplink) does not perform retransmissions for outdated packets and the receiver (gNB in uplink) updates the state variables. In other words, a new control PDU format may be needed. In our view, probably RLC Status Report format can be reused, so additional efforts will not be small.

In contrast, RX-initiated mechanism does not need a new indication. Instead, the receiver set the reception status of the outdated packet as ACKed to prohibit the unnecessary retransmission.

Both mechanisms require some change to move the RLC reception window at the detection of outdated packet.

TX-initiated mechanism requires a new RLC control PDU format to indicate outdated packets, like RLC Status Report.

< Obsolete Packet Discard in PDCP Duplication >

There is another case of obsolete packets, i.e., PDCP duplication. PDCP duplication can be configured for AM DRB. In this case, successful delivery (i.e. RLC ACK reception) of one leg triggers discard of the same packet of the other leg. However, RLC cannot discard the packet in case that the initial transmission was already started. TX-initiated mechanism based on PDCP’s discard indication makes it possible, regardless of initial transmission. However, RX-initiated mechanism cannot detect the successful delivery of the other leg.

In PDCP duplication, RX-initiated mechanism cannot avoid the unnecessary retransmission when the same PDCP packet is successfully transmitted on the other leg.

|  | TX-initiated | RX-initiated |
| --- | --- | --- |
| Triggering condition | PDCP discard timer (reuse) | New RX timers or counter based on out-of-order delivery |
| Accuracy of detection | Accurate | Additional delay or inaccuracy |
| Indication to avoid retransmissions | New control PDU | RLC status report (reuse) |
| Applicability of duplication | Applicable | Can’t detect |

Table 1. Summary of TX-initiated vs RX-initiated

Table 1 summarizes the comparison analysis throughout this section. The analysis concludes that TX-initiated mechanism is more accurate and effectively avoids the unnecessary retransmission. On the other hand, RX-initiated mechanism does not need to introduce a new control PDU format, whereas a new triggering condition is necessary. In our view, the complexity of TX-initiated mechanism is not big, as we can define status report-like control PDU. It would be better to have an accurate mechanism.

RAN2 to adopt TX-initiated mechanism to avoid unnecessary retransmission.

Discard Indication from PDCP triggers the indication, when the PDCP SDU is already transmitted in the lower layer (i.e., MAC).

## Timely Transmission

According to the discussion in RAN2#126, there are three solutions on the table:

- Autonomous retransmission without status report

- Retransmission based on enhanced status report

- Retransmission based on enhanced polling

All candidate solutions aim to accelerate retransmissions to guarantee the timely transmission. More specifically, all the solutions are targeted to enable early retransmissions under certain conditions such the remaining time until the required delivery time is less than a threshold value. Also, RAN2 agreed to focus on the enhancements for UL traffic.

Enhanced status report would mean that when the remaining time until the required delivery time is short (e.g., delay-critical data), the receiver triggers a status report early and asks the UE to perform retransmission early. In this solution, the gNB decides when to send the status report. In uplink, it is fully up to gNB implementations which mean that it is possible currently. Thus, no further spec impact is expected.

In uplink, retransmission based on enhanced status report is already possible and no further spec impact is expected.

Enhanced polling would mean that when the remaining time until the required delivery time is short (e.g., delay-critical data), the transmitter triggers a polling early to trigger the status report early and the UE performs retransmission early. This mechanism may need to modify only polling procedure. However, actual early retransmission will have delay from setting a poll until status report reception at the UE. It is not appropriate for XR traffic which has delay constraint. Moreover, if the status report experiences congestion in downlink, the early retransmission might not be possible. Also, the polled PDU may experience congestion or retransmissions. Thus, it is not considered as a good solution.

Retransmission based on enhanced polling has delay until the reception of the status report and may experience congestion.

Autonomous retransmission without status report is a more direct way than enhanced polling. Also, autonomous retransmission without status report is useful when either uplink or downlink is experiencing congestion. The only expected spec impact would be when the autonomous retransmission is triggered in RLC.

One may argue that the autonomous retransmission wastes the uplink resource. However, the autonomous retransmission is triggered only for urgent data to reduce performance degradation. Excessive resource waste does not occur for most of the time, and RLC ACK usually prohibits the unnecessary retransmission.

Autonomous retransmission without status report is useful for congestion scenario and potential resource waste can be under network control.

Based on the observations above, we see that RAN2 should introduce an RLC autonomous retransmission mechanism without status report.

RAN2 to introduce an RLC autonomous retransmission mechanism without status report.

Retransmission based enhanced status report and retransmission based on enhanced polling are excluded.

# Conclusions

Based on the discussion given above, we have the following proposals:

Avoiding unnecessary retransmissions

- RAN2 to adopt TX-initiated mechanism to avoid unnecessary retransmission.

Discard Indication from PDCP triggers the indication, when the PDCP SDU is already transmitted in the lower layer (i.e., MAC).

- Autonomous retransmissions

RAN2 to introduce an RLC autonomous retransmission mechanism without status report.

Retransmission based enhanced status report and retransmission based on enhanced polling are excluded.