3GPP TSG-RAN WG2 Meeting #127                                                                   R2-2406482

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	CATT

Title:	Discussion on the functionality aspects for Ambient IoT

Agenda Item:	8.2.2

Document for:	Discussion

1. Introduction

This contribution further discusses the functionalities required for A-IoT devices, including indication of device message size/status, segmentation/reassembly/higher layer repetition, and security related aspects.

2. Discussion

2.1 Indication of device message size/status

- Whether to need indication of message size/status

On the functionality discussion in last meeting, we had agreed to not support legacy NR BSR/SR for A-IoT. Whether to have further indication for message size/status by the device to reader, it still remains the following controversy,

legacy NR BSR/SR is not needed for A-IoT communication. FFS whether further indication of device message size/status is needed

Note that whether to need the message size/status depends on the potential use cases for A-IoT. According to 38.848,

The design target of maximum message size is approximately 1000 bits to be received by the Ambient IoT device, and approximately 1000 bits to be transmitted from the Ambient IoT device, based on the maximum application layer packet size.

Therefore, the UL data transmitted to reader is more or like small data. It seems workable to relay on implementation to ensure enough resource within one UL scheduling, so the device is not expected to have remaining data after performing one UL transmission for the response of the inventory or command service. But the drawback of this way is the reader always needs to provide large enough resource regardless of the data volume buffered by the device, which is not resource-efficient.

Observation 1: If the device does not send any message size/status indication to reader, the reader needs to always provide enough large resource regardless of the data volume in the device, which is not resource-efficient.

Therefore, it has benefits that the reader is aware of the data volume that needs to send by the device. The candidate options of the indication to reader are message size or message status.

| Indication Type | Indication content | Reader scheduling |
| --- | --- | --- |
| Option 1: Message size | The data volume in the device | Based on reader loading, whether to provide large enough resource for the data volume |
| Option 2: Message status | Whether to have remaining data in the device | Provide blind resource scheduling |

As summarized above, the indication of message size helps the reader to perform efficient scheduling based on reader loading, while the indication of message status will result in blind scheduling by the reader, i.e., the reader still does not know whether the current resource is enough for the next transmission.

Proposal 1: The device can send the message size indication to the reader for facilitating the resource scheduling by the reader.

- Reader control on message size indication

Upon reception of inventory or command service by the device, the device needs to response the UL data to the reader. The data volume differs with different SA1 use cases. In some use cases, the data volume may be small and the reader can ensure large enough resource so that the data can be sent to the reader through one UL transmission. For some potential use cases, e.g., the reader collects the sensor’s data for a period of time, the reader may not provide large enough resource for UL transmission due to loading situation, and thus may require the device to send the message size to facilitate the subsequent scheduling. Therefore, it can be up to the reader to indicate the device whether to send the message size to the reader.

Proposal 2: It’s up to reader control whether the device to report the message size to the reader.

2.2 Segmentation/reassembly/higher layer repetition

RAN2 discussed the segmentation and reached the agreements as below:

Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective.   FFS how we handle segmentation case (if needed)

So we will further discuss how to handle the segmentation case.

- The need of segmentation

As shown in the Table 1, the message size requirements for SA1 use cases of indoor inventory and indoor command are from 96 bits to 100 bytes.

Table 1: Message size and potential contents for SA1 use cases of indoor inventory and indoor command

| RAN representative Use Cases (rUCs) | SA1 use cases / traffic scenarios | Message size | Message contents |
| --- | --- | --- | --- |
| rUC1: indoor inventory | 5.16 Automated supply chain distribution | <100 bytes | Inventory information (such as: product serial number. product types, product location, etc). |
|  | 6.1 Traffic scenario on flower auction | 96 bits | Only an identifier for the tag is sent (Electronic Product Code (EPC)). |
|  | 6.3 Electronic shelf label | 100 bytes | The message payload size is calculated based on the capacity of 50 Unicode characters for item description and pricing on the electronic shelf label. |
| rUC4: indoor command | 6.3 Electronic shelf label | 100 bytes | The message payload size is calculated based on the capacity of 50 Unicode characters for item description and pricing on the electronic shelf label. |

The candidate transport block sizes such as {128, 256, 512, 1024} bits for PDRCH are still under RAN1 discussion.

Although the reader may indicate the message size to the reader suggested by our Proposal 1, it may occur that the channel quality is not good enough, which cannot meet the expectation for the message size. For example, when the size of the UL message (e.g. 800bits) exceeds the transport block size of the PDRCH (e.g. 512bits), the UL segmentation is required.

Observation 2: Segmentation is required when the size of information is larger than the transport block size.

However, buffer is not mandatory in this case, because the response of inventory or command comes from the stored information in the device rather than from the buffer information. The device can opt to initially transfer only the first half or a portion of the message that fits the transport block size (e.g. 512bits in this case), along with an indication to the reader that there is a remainder of the message to be transmitted in a subsequent transmission. This indication is crucial for the reader to be aware of the pending transmission.

Upon receiving a subsequent command from the reader, the device then proceeds to transfer the remaining information. This approach ensures that large UL messages are handled in manageable segments when the channel condition is not good enough.

Note that Buffer-free segmentation by device avoids the increasing memory cost that typically accompanies legacy segmentation performed by the UE.

Observation 3: Buffer-free UL segmentation by device, avoids the increasing memory cost that typically accompanies legacy segmentation performed by the UE.

Proposal 3: Buffer-free UL segmentation by device is feasible for device without increasing cost. Support the functionality of segmentation for UL transmission by device.

- How to handle segmentation from device to reader

When segmentation from device to reader is supported, reassembly in the reader is required as well. When the reader fails to decode a certain segmentation, the reassembly fails accordingly. But from our view, the retransmission of the segmentation from device to reader is not mandatory. Decoding failure caused by segmentation can be considered to be same as message reception failure, i.e., the message was not successfully received by the reader. Therefore, retransmission is not requested to resolve message failures caused by segmentation.

Proposal 4: Retransmission is not supported for segmentation. Decoding failure caused by segmentation can be considered as message reception failure.

2.3 Security related aspects

SA3 has discussed key issues: Protection for disabling device in 3GPP TR 33.713 V0.2.0 (2024-05) [1].

Based on operator policy, there are two categories of disabling device operations, i.e. permanent disabling of the capability and temporary disabling of the capability. For example, if the ambient IoT device follows the spoofed permanent/temporary disable device operation from an attacker, the ambient IoT devices will not respond to the network either permanently, or for a period of time, leading to the Denial of Service (DOS). The means to securely disable the ambient IoT device(s)’s capability to transmit RF signals shall be supported which is concluded in SA3.

5.1.3 Potential security requirements The means to securely disable the Ambient IoT device(s)’s capability to transmit RF signals shall be supported. Editor’s Note: Whether the solutions for this key issue are the same or different from those for communication protection issue is FFS. Editor’s Note: Security solutions for this Key Issue should be aligned with the conclusion of Ambient IoT system architecture in SA2.

The availability of ambient IoT devices will be impacted if the disabling device operation is not securely performed.

Observation 4: The operation on disabling device and the related information between reader and device should also be protected by higher layer.

Besides, the issue on privacy by protecting A-IoT device identifiers is studied by SA3 which is captured in 3GPP TR 33.713 V0.2.0 (2024-05) [1].

In A-IoT services, the identifiers of A-IoT device are used to identify the device. If the identifiers associated with a device are not privacy protected (e.g., exposed over the air), an attacker (e.g., an over-the-air attacker) can identify and track an A-IoT device based on the identifiers associated with the A-IoT device. Thus, this key issue is to investigate potential mechanisms to privacy protect the A-IoT device identifiers.

5.3.2 Security Threats An attacker can identify, monitor and track an AIoT device based on the identifiers associated with the AIoT device if the identifiers are not privacy protected. Editor’s Note: It is FFS how the above threat affects various use cases. Editor’s Note: Security threat and requirement for potential exposure of quantity of devices after adversary broadcasts an inventory message is FFS. 5.3.3 Potential security requirements Mechanisms for mitigating privacy threats (described above) by identifying, linking, and tracking the identifiers of AIoT Device(s) shall be supported. Editor’s Note: AIoT use cases that do not need the above privacy protection mechanisms are FFS.

So we can conclude based on the progress in SA3:

- The privacy by protecting A-IoT device identifiers is required.

- The A-IoT use cases that do not need the above privacy protection mechanisms are FFS.

Observation 5: The A-IoT device identifiers should be protected by higher layer and should not be exposed over the air. FFS the use case of being exposed over the air.

According to the Observation 4 and Observation 5 on the progress of security in SA3, we proposed that:

Proposal 5-1: The A-IoT device identifier (e.g., group ID, device ID) cannot be exposed over the air, i.e., conveyed in NAS layer over the air. FFS the use case of being exposed over the air, depending on SA3.

Proposal 5-2: The command of disabling device cannot be visible to reader, i.e., conveyed in NAS layer over the air. FFS whether other commands visible to reader or not, depending on SA3.

Proposal 5-3: Message/service type (e.g., inventory or command) and number of devices from CN can be visible to reader.

This issue on authorization for 5G ambient IoT services is studied by SA3, which is captured in 3GPP TR 33.713 V0.2.0 (2024-05) [1].

In the Topology 2 as defined in TR 38.769 [2], the UE acting as the intermediate node is responsible for transferring the information between AIoT device and 5GS. If the authorization of intermediate node is not supported, the attacker can play the role of intermediate node and arbitrarily deny 5G AIoT service. Therefore, it is necessary to study how to authorize the UE for acting as the intermediate node.

Therefore, it is necessary to study how to support authorization of the UE for acting as the intermediate node from RAN2 perspective.

Observation 6: if the UE acting as the intermediate node is authorized via NAS, there is no impact on the interface between gNB and CN.

Proposal 6: RAN2 to study how to support the authorization of UE for the intermediate node.

3. Conclusion

Based on the analysis in section 2, the observations and proposals are summarized as following:

Observation 1: If the device does not send any message size/status indication to reader, the reader needs to always provide enough large resource regardless of the data volume in the device, which is not resource-efficient.

Observation 2: Segmentation is required when the size of information is larger than the transport block size.

Observation 3: Buffer-free UL segmentation by device, avoids the increasing memory cost that typically accompanies legacy segmentation performed by the UE.

Observation 4: The operation on disabling device and the related information between reader and device should also be protected by higher layer.

Observation 5: The A-IoT device identifiers should be protected by higher layer and should not be exposed over the air. FFS the use case of being exposed over the air.

Observation 6: if the UE acting as the intermediate node is authorized via NAS, there is no impact on the interface between gNB and CN.

Indication of device message size/status

Proposal 1: The device can send the message size indication to the reader for facilitating the resource scheduling by the reader.

Proposal 2: It’s up to reader control whether the device to report the message size to the reader.

Segmentation/reassembly/higher layer repetition

Proposal 3: Buffer-free UL segmentation by device is feasible for device without increasing cost. Support the functionality of segmentation for UL transmission by device.

Proposal 4: Retransmission is not supported for segmentation. Decoding failure caused by segmentation can be considered as message reception failure.

Security related aspects

Proposal 5-1: The A-IoT device identifier (e.g., group ID, device ID) cannot be exposed over the air, i.e., conveyed in NAS layer over the air. FFS the use case of being exposed over the air, depending on SA3.

Proposal 5-2: The command of disabling device cannot be visible to reader, i.e., conveyed in NAS layer over the air. FFS whether other commands visible to reader or not, depending on SA3.

Proposal 5-3: Message/service type (e.g., inventory or command) and number of devices from CN can be visible to reader.

Proposal 6: RAN2 to study how to support the authorization of UE for the intermediate node.

4. References

- 3GPP TR 33.713 Study on Security Aspect of Ambient IoT Services in 5G (Release 19) V0.2.0 (2024-05)