3GPP TSG-RAN WG2 Meeting #127	R2-2406483

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.2.3

Source: 	CATT

Title: 	Discussion on Paging for Ambient IoT

Document for: 	Discussion and Decision

1	Introduction

In this contribution, the A-IoT paging aspects and content required for A-IoT for the different identified procedures will be further discussed.

2	Content for the A-IoT paging message

2.1	 Analyzation on content for inventory procedure

Identity of target A-IoT device(s)

In RAN2#126 [1], it has been agreed that RAN2 will study the cases that device ID or group ID contained in A-IoT paging message to reach A-IoT device(s).

1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

For the cases of paging single A-IoT device and a group of A-IoT devices, if the device ID/ group ID is contained in the A-IoT paging message in format of AS IE, it can be read by anyone, which will bring security concerns. One way to solve this issue is to contain the device ID/ group ID in format of upper layer packet, e.g., NAS data. The upper layer packet is generated by CN and sent to reader when an inventory procedure is triggered. Then, the reader will generate an A-IoT paging message in radio interface, including the received upper layer packet, and transparent to A-IoT devices. After receiving the A-IoT paging message, the device will determine whether the A-IoT paging message is for itself by decoding the upper layer packet.

Observation 1: For the inventory procedure, if the device ID/ group ID contained in A-IoT paging message is implemented as AS layer IE, there may be security concerns.

Proposal 1: For the inventory procedure, the device ID/ group ID in A-IoT paging message is contained in an upper layer packet, e.g. NAS container. The definition of upper layer ultimately depends on SA2.

Configuration used for determination of access occasion

In RAN2#126 [1], the following agreement was reached.

2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

In “A-IoT Random Access” part, CBRA and CFRA mechanisms are still under discussing. From our perspective, it is complex and has no obvious benefit to configure dedicate resource for multiple devices in the A-IoT paging message, that CFRA should be only used for single device case and CBRA should be used for multiple devices case, the detailed analyzation refers to section 2.2 of our contribution [2]. In the following, the discussion is expanded on cases of CFRA for single device and CBRA for multiple devices.

- In case of CBRA for multiple devices: Considering different inventory services may have different number of target devices, the reader can assign appropriate access resource for devices to avoid serious collisions or waste of resource. Hence, the resource is better to be explicit configured instead of preconfigured statically. The resource is shared among the target A-IoT devices.

- In case of CFRA for single device: Both implicit configuration and explicit configuration can work. If the implicit configuration is implemented, the target device can use the first access occasion directly. If the explicit configuration is implemented, the reader can configure only one access occasion for this device. For the latter option, the formats of the A-IoT paging message for different cases, i.e., CBRA and CFRA, are aligned. We have a little preference on the latter option.

Observation 2: It is complex and has no obvious benefit to configure dedicate resource for multiple devices in the A-IoT paging message, that CFRA should be only used for single device case and CBRA should be used for multiple devices case.

Proposal 2: For the inventory procedure, in case of CBRA for multiple devices and CFRA for single device, the configuration used to determine access resource is explicitly configured via the A-IoT paging message. And the configured resource is shared among all target device(s).

In RAN1#116-bis, RAN1 has agreed to study time-domain multiple access, frequency-domain multiple access and code-domain multiple access. However, there is no further progress in RAN1.

Agreement Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS. Agreement Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS. Agreement Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices

At present, RAN2 only considers time-domain multiple access, i.e., device perform access within different time slot/frame. To achieve that, reference to RFID mechanism, configuration of Q, N may need to be indicated to devices, which can be used by device to select an occasion to perform access. The detailed mechanism depends on the discussion in “A-IoT Random Access” part of RAN2

For the detailed configuration used for random access to be contained in the A-IoT paging message, RAN2 could postpone the discussion until there is progress in RAN1 and the “A-IoT Random Access” part of RAN2.

Proposal 3: For the inventory procedure, postpone the discussion on detailed configuration used by devices for random access until there is progress in RAN1 and the “A-IoT Random Access” part in RAN2.

2.2	 Analyzation on content for inventory + command procedure

Reference to [3], the possible overall procedures of inventory + command are shown. According to these procedures, it can be seen that, the reader will perform inventory at first, at this step, there is no difference comparing with the inventory procedure. Hence, the discussion and conclusions in section 2.1 are also applicable for the inventory + command procedure.

Observation 3: For the inventory + command procedure, the reader will firstly perform inventory by sending an A-IoT paging message. This step is exactly the same as the inventory procedure.

Proposal 4: Proposals 1 through 3 for the inventory procedure are applicable to the inventory + command procedure.

2.3	 Analyzation on content for command only procedure

For the command only procedure, the identity of target A-IoT device(s) and configuration used by A-IoT devices for determination of access occasion are also needed. Hence, the discussion and conclusions in section 2.1 are also applicable to the command only procedure.

Observation 4: For the command only procedure, the identity of target A-IoT device(s) and configuration used by A-IoT devices for determination of access occasion are also required within the A-IoT paging message.

Proposal 5: Proposals 1 through 3 for the inventory procedure are applicable to the command only procedure.

A special issue for command only procedure is whether the command can be contained in the A-IoT paging message. In RAN2#126, the following agreement was achieved.

3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

From our perspective, if the command raised by CN/AF can be contained in A-IoT paging message, it should be contained in an upper layer packet with security protect. We could wait for SA2 and SA3 progress on whether the command can be contained in A-IoT paging message.

Proposal 6: If the command can be contained in A-IoT paging message, it should be contained in an upper layer packet. Wait for SA2 and SA3 progress on whether the command can be contained in A-IoT paging message.

2.4	 Signalling structure of the A-IoT paging message

Based on the discussion in section 2.1~2.3, it can be seen that a unified A-IoT paging message can applicable to these three procedures. The message contains an upper layer packet and configuration used to determine access resource. The upper layer packet includes the target device ID/group ID and may also include command for command only procedure.

Proposal 7: A unified A-IoT paging message is applicable to the identified procedures, including inventory, inventory + command and command only. The A-IoT paging message comprises the following elements:

- An upper layer packet: includes the target device ID/group ID, FFS command.

- Configuration parameters used by device to determine access resource.

Apart from the above mentioned elements, there is also discussion on including other information in A-IoT paging message, e.g., indication of service type and reader ID. At present, these discussions seem not critical for the basis function of A-IoT paging message. Hence, we propose to discuss these issues in WI phase if needed.

Proposal 8: Whether additional information is contained in A-IoT paging message can be discussed in WI phase.

3	A-IoT paging procedure

Monitoring of the A-IoT paging message

In RAN2#126, it has been agreed that the device can receive as long as it has enough energy.

3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

Then, based on this agreement, the issue of how to guarantee the target device could receive the A-IoT paging message is equivalent to how to guarantee the target device has energy. This issue is depends on RAN1. For example, carrier wave can be used to provide energy for Type-1 and Type-2 devices with energy storage. In this situation, the A-IoT device can collect energy during the switching time before backscattering the response signal. We could wait for RAN1’s progress and come back to check whether any enhancement is needed by RAN2.

Proposal 9: Ensuring that the device does not miss the A-IoT paging message is primarily depends on RAN1. RAN2 can check whether any enhancement is needed when there is progress in RAN1.

Device behaviour upon detecting the A-IoT paging message

After detecting the A-IoT paging message, the device will determine whether the A-IoT paging message is for itself. If yes, the device should respond to the reader in the following random access procedure for the subsequent signalling/data transmission. Otherwise, the device will do nothing. The issue is how device determines whether to respond to the A-IoT paging message. Based on the following agreement achieved in RAN2#126 on the identity used for paging device(s), this issue is analyzed for different cases.

1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

- For the first bullet, if the device ID is equal to the single device ID in the A-IoT paging message, the device should respond to the A-IoT paging message.

- For the second bullet, if the device belongs to the target group ID, the device should respond to the A-IoT paging message. For example, if the group ID is a mask of the device ID, then the device compares the mask with specific bits of the device ID. However, the format of group ID is still under discussion of SA2, RAN2 needs wait for progress of SA2.

- For the third bullet, if the A-IoT paging message does not contain ID, the device should respond to the A-IoT paging message.

- For the forth bullet, this use case can be discussed if it is confirmed by SA2.

Proposal 10: Upon receiving the A-IoT paging message, the device should response to the reader in the following random access procedure, if the A-IoT paging message is for itself. In detail:

- If the device ID is equal to the single device ID in the A-IoT paging message, the device should respond to the A-IoT paging message.

- If the A-IoT paging message does not contain ID, the device should respond to the A-IoT paging message.

- For the case of group ID and multiple IDs, waits for progress of SA2.

4	Conclusion

In this contribution, paging functionality for A-IoT is discussed with giving corresponding observations and proposals:

Content for the A-IoT paging message

Observation 1: For the inventory procedure, if the device ID/ group ID contained in A-IoT paging message is implemented as AS layer IE, there may be security concerns.

Proposal 1: For the inventory procedure, the device ID/ group ID in A-IoT paging message is contained in an upper layer packet, e.g. NAS container. The definition of upper layer ultimately depends on SA2.

Observation 2: It is complex and has no obvious benefit to configure dedicate resource for multiple devices in the A-IoT paging message, that CFRA should be only used for single device case and CBRA should be used for multiple devices case.

Proposal 2: For the inventory procedure, in case of CBRA for multiple devices and CFRA for single device, the configuration used to determine access resource is explicitly configured via the A-IoT paging message. And the configured resource is shared among all target device(s).

Proposal 3: For the inventory procedure, postpone the discussion on detailed configuration used by devices for random access until there is progress in RAN1 and the “A-IoT Random Access” part in RAN2.

Observation 3: For the inventory + command procedure, the reader will firstly perform inventory by sending an A-IoT paging message. This step is exactly the same as the inventory procedure.

Proposal 4: Proposals 1 through 3 for the inventory procedure are applicable to the inventory + command procedure.

Observation 4: For the command only procedure, the identity of target A-IoT device(s) and configuration used by A-IoT devices for determination of access occasion are also required within the A-IoT paging message.

Proposal 5: Proposals 1 through 3 for the inventory procedure are applicable to the command only procedure.

Proposal 6: If the command can be contained in A-IoT paging message, it should be contained in an upper layer packet. Wait for SA2 and SA3 progress on whether the command can be contained in A-IoT paging message.

Proposal 7: A unified A-IoT paging message is applicable to the identified procedures, including inventory, inventory + command and command only. The A-IoT paging message comprises the following elements:

- An upper layer packet: includes the target device ID/group ID, FFS command.

- Configuration parameters used by device to determine access resource.

Proposal 8: Whether additional information is contained in A-IoT paging message can be discussed in WI phase.

A-IoT paging procedure

Proposal 9: Ensuring that the device does not miss the A-IoT paging message is primarily depends on RAN1. RAN2 can check whether any enhancement is needed when there is progress in RAN1.

Proposal 10: Upon receiving the A-IoT paging message, the device should response to the reader in the following random access procedure, if the A-IoT paging message is for itself. In detail:

- If the device ID is equal to the single device ID in the A-IoT paging message, the device should respond to the A-IoT paging message.

- If the A-IoT paging message does not contain ID, the device should respond to the A-IoT paging message.

- For the case of group ID and multiple IDs, waits for progress of SA2.

5	Reference

- Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan.

- R2-2406484 Discussion on the Random Access for Ambient IoT, CATT.

- R2-2404231 Discussion on stage 2 overall procedures for Ambient IoT, CATT.