3GPP TSG-RAN WG2 Meeting #127                                                        R2-2406486

Maastricht, NL, 19th – 23rd Aug, 2024

Source:              CATT

Title:	             Random Access in SBFD symbols

Agenda Item:	8.11.2

Document for:	Discussion and Decision

Introduction

According to [1], one of the objectives of SBFD is as below:

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

In this contribution, we will discuss how to support random access in SBFD symbols based on RAN1 agreements.

Discussion

RAN1 background

In RAN1#116 and RAN1#116bis meeting, regarding to the RACH configuration, the following options were discussed:

- Option 1: Use one single RACH configuration with possible enhancement.

  - Alt 1-1: only based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon).

  - Alt 1-2: based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon) and newly introduced parameter(s).

- Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

  - Alt 2-3:

    - The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs.

    - FFS: The case where the additional-ROs partially overlap with non-SBFD symbols

  - Alt 2-4:

    - The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs.

In RAN1#117 meeting, agreement on down-selection was reached as below:

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

Based on the above agreements, both Alt 1-1 of Option 1 and Option 2 are supported. But the details of Alt 2-3 and Alt 2-4 of Option 2 needs further discussion in RAN1.

Observation 1: Based on RAN1 agreement, for RACH configuration, both Alt1-1 of Option 1 and Option 2 are supported. For Alt 2-3 and Alt 2-4 of Option 2, details are still FFS in RAN1.

Potential RAN2 impacts

Applicable RACH trigger events

In RAN1#116 meeting, which triggering events should be supported for RACH on SBFD symbol was discussed in [2]. The initial proposal in RAN1 is that for SBFD aware UEs in RRC CONNECTED state, RAN1 aims to support all random access triggering events for random access operation in SBFD symbols, unless specific issues are further identified. But there were no agreements reached since some companies thought this is also related to RAN2.

According to TS38.300 [3], the following RACH triggering events were mentioned:

### 9.2.6 Random Access Procedure The random access procedure is triggered by a number of events: - Initial access from RRC_IDLE; - RRC Connection Re-establishment procedure; - DL or UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure (see clause 18.0) is ongoing, when UL synchronisation status is "non-synchronised"; - UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available; - Handover, except for when RACH-less HO is configured; - SR failure; - Explicit request by RRC upon synchronous reconfiguration; - RRC Connection Resume procedure from RRC_INACTIVE; - To establish time alignment for a primary or a secondary TAG; - Request for Other SI (see clause 7.3); - Beam failure recovery; - Consistent UL LBT failure on SpCell; - SDT in RRC_INACTIVE (see clause 18); - Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning; - Early UL synchronization with an LTM candidate cell; - RACH-based LTM cell switch.

From RAN2 perspective, allowing SBFD-aware to use RACH resources in SBFD symbol can reduce the RACH latency and improve RACH capacity. There is no motivation to prohibit SBFD-aware UE to use RACH resource on SBFD symbol for any of the above RACH trigger events.

Proposal 1: For all the random access trigger events, SBFD-aware UE can use the RACH resources on SBFD symbols.

Whether SBFD should be regarded as a feature?

Considering many features (such as RedCap, SDT, CovEnh and Slicing) need early identification, feature/feature combination was introduced in Rel-17 RACH indication and partitioning. In order to support early identification, the following two solutions were introduced in Rel-17 RACH indication and partitioning:

- Solution 1: Shared RACH resources, preamble partitioning is defined based on feature/feature combination.

- Solution 2: Separate RACH resources (additional RACH) for feature/feature combination.

For Alt1-1 of Option 1, shared RACH resources are used. The ROs in non-SBFD symbols are shared by Rel-19 SBFD-aware UEs and legacy UEs.  If SBFD is set as a feature similar as Rel-17 RedCap, SDT and etc, there are two disadvantages:

- Preamble partitioning is needed for SBFD feature/feature combination at least in non-SBFD symbols. In order to reduce the impact to legacy UE, some CFRA preambles should be re-allocated to SBFD feature/feature combinations. It is doubtable whether there is enough CFRA preambles can be re-allocated.

- According to the current spec, preamble partitioning is the same for one RACH configuration. If preamble partitioning is used in non-SBFD symbols, preamble partitioning is also used in SBFD symbols, which will result in resource waste in SBFD symbols.

For Option 2, separate RACH configuration can be configured for SBFD UEs. But there is no conclusion whether legacy ROs can also be used by SBFD UE.

- If SBFD UE is allowed to use legacy RO and SBFD works as a feature, similar as the analysis in Alt 1-1 of Option 1, for legacy RACH configuration, it needs further preamble partitioning.

- If SBFD UE is not allowed to use legacy RO, since separate RO is used, it is unnecessary to regard SBFD as a feature since early identification is already supported by using separate RA resources.

Based on the above analysis, in order to reduce the impact on legacy preamble allocation and improve the resource efficiency in SBFD symbols, it is prefer not to treat SBFD as a feature similar as Rel-17 RedCap, SDT and etc.

Proposal 2: SBFD is not introduced as a feature (similar as RedCap, SDT and etc.) for random access.

Proposal 3: If Proposal 2 is agreed, send LS to RAN1 to inform that SBFD is not introduced as a feature.

Impact on the RA procedure

In legacy release, the RA resource selection procedure is as below:

Figure-1 Legacy RA resource selection procedure

In the following, we will discuss whether the legacy RA procedure needs to be enhanced. Considering for Alt1-1 of Option 1, there is only one RACH configuration, hence, whether the legacy RA procedure needs to be enhanced mainly depends on Option 2.

For Option 2, there are two alternatives:

- Alt 2-3：The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs;

- Alt 2-4：The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs.

The key difference between Alt2-3 and Alt 2-4 is whether the additional ROs configured in non-SBFD symbol by the additional RACH configuration is valid or not. Considering RACH resources are configured to be within UL usable PRBs, which may be in the middle of UL. If the ROs configured by additional RACH configuration in non-SBFD symbols are valid for SBFD-aware UEs, UL resource fragmentation issue happens in full UL symbols. Further, due to the separate SSB-to-RO mapping for legacy RACH configuration and additional RACH configuration, different SSBs may map to the ROs in the same symbol which requires different Rx beams at gNB side. Hence, from our perspective, if down-selection is performed, Alt 2-3 is preferred compared with Alt 2-4. Hence, our analysis focus on Alt2-3.

For Alt 2-3, if RO in legacy RACH configuration is allowed to be used in non-SBFD symbol, there will be two sets of RACH resources. However, according to the description in sublause 5.1.1 of TS38.321, it mentioned that “When a Random Access procedure is initiated, UE selects a set of Random Access resources as specified in clause 5.1.1b”. Hence, it should further discuss how to treat the two sets of RACH resources. There are two solutions:

- Solution 1：For each RA attempt, RO can be selected based on both the additional RACH configuration (if configured) and the legacy RACH configuration.

In this solution, it should allow UE to select two sets of resources in subclause 5.1.1 and 5.1.1b of TS38.321, and the other steps can follow the legacy RA procedure.

- Solution 2：For each RA attempt, RO can be selected from only one RACH configuration.

In this solution, for SBFD-aware UE, it can prioritize to select the RO configured by the additional RACH configurations. And when the fallback condition is met (e.g., reached the maximum RA retry number on SBFD symbols), the RO in legacy RACH configuration can be selected.

Proposal 4:  With the introduction of SBFD, RAN2 to discuss the following two RA procedures for Option 2:

- Solution 1：For each RA attempt, RO can be selected based on both the additional RACH configuration and the legacy RACH configuration.

- Solution 2：For each RA attempt, RO can be selected from only one RACH configuration. For SBFD-aware UE, it can prioritize to select the RO configured by the additional RACH configuration, and when fallback is needed, it can further select the RO configured by the legacy RACH configuration.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: For all the random access trigger events, SBFD-aware UE can use the RACH resources on SBFD symbols.

Proposal 2: SBFD is not introduced as a feature (similar as RedCap, SDT and etc.) for random access.

Proposal 3: If Proposal 2 is agreed, send LS to RAN1 to inform that SBFD is not introduced as a feature.

Proposal 4:  With the introduction of SBFD, RAN2 to discuss the following two RA procedures for Option 2:

- Solution 1：For each RA attempt, RO can be selected based on both the additional RACH configuration and the legacy RACH configuration.

- Solution 2：For each RA attempt, RO can be selected from only one RACH configuration. For SBFD-aware UE, it can prioritize to select the RO configured by the additional RACH configuration, and when fallback is needed, it can further select the RO configured by the legacy RACH configuration.

Reference

- RP-241614	 Evolution of NR duplex operation: Sub-band full duplex     Huawei, HiSilicon

- R1-2401676	  Summary#1 on SBFD random access operation Moderator (CMCC)

- TS38.300  NR; NR and NG-RAN Overall Description; Stage 2; (Release-18)