3GPP TSG-RAN WG2 Meeting #127                                                                   R2-2406487																						xxx

Maastricht, Holland, Aug 19th – 23rd, 2024

Source:	CATT

Title:	Discussion on other aspects for SBFD

Agenda Item:	8.11.3

Document for:	Discussion

1. Introduction

Evolution of NR duplex operation: Sub-band full duplex (SBFD) will be enabled in Rel-19. The revised WID in RP-241614[1] was approved in RAN#104 in June, 2024.

The objectives which are related with RAN2 except random access can be found as below:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier:

  - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

  - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

  - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2]

    - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon

    - Enhancement on resource allocation in frequency domain in SBFD symbols, including

    - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols,

    - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation

    - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

  - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

    - UL resource muting for PUSCH[RAN1, RAN2, RAN4]

    - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework,[RAN1, RAN2, RAN4]

According to the progress of RAN1, it seems it’s better to start the discussion on SBFD subband indication at the first meeting in RAN2 while the other items can be postponed to discuss for the next meeting. We will focus on the solution on SBFD subband indication within a TDD carrier in this contribution.

2. Discussion

The SBFD subband indication is required in WID [1]:

•	Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

•	Indication of time location of SBFD subbands in SIB is not precluded

•	Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

•	Indication of frequency domain location of SBFD subbands in SIB is not precluded

RAN1 has discussed the topic in RAN1#116, #116bis and #117 and reached some agreements as below:

RAN1#116

Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

RAN1#116bis

Agreement For cell-specific configuration of frequency locations of SBFD subbands, - Option 1: Cell-specific frequency locations of SBFD subbands are separately configured for each SCS configuration in SCS-SpecificCarrierList. - For each SCS configuration, the reference starting PRB is the PRB determined by the SCS configuration and offsetToCarrier corresponding to this subcarrier spacing.

RAN1#117

Agreement

For RRC connected mode UEs, SBFD subband time period:

- When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

Agreement

For cell-specific indication of SBFD subband frequency location, adopt the following option.

- Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

RAN2 may start from the signaling of time and frequency location for SBFD subband indication although there is no clear design on parameters of SBFD subband indication in RAN1 yet. When there is clear RRC parameters design on SBFD subband indication, RAN2 will capture the parameters into the signalling container.

Now RAN2 may focus on the signalling and design the container of the time and frequency location for SBFD subband indication. So we proposed that:

Proposal 1-1: Support cell-specific configuration of SBFD subband time and frequency location via SIB1.

Proposal 1-2: Support cell-specific configuration of SBFD subband time and frequency location via RRC configuration and/or reconfiguration as legacy.

Proposal 2: The detailed parameters (e.g. slot and symbols) will be defined by RAN1.

3. Conclusion

Based on the analysis in section 2, the proposals are summarized as following:

SBFD subband indication

Proposal 1-1: Support cell-specific configuration of SBFD subband time and frequency location via SIB1.

Proposal 1-2: Support cell-specific configuration of SBFD subband time and frequency location via RRC configuration and/or reconfiguration as legacy.

Proposal 2: The detailed parameters (e.g. slot and symbols) will be defined by RAN1.

4. References

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)  Huawei

- 3GPP TR 38.858 Study on Evolution of NR Duplex Operation (Release 18) V2.0.0 (2023-12)

- RAN1 Chair Notes WG1 #116

- RAN1 Chair Notes WG1 #116bis

- RAN1 Chair Notes WG1 #117

- R1-2405423	Summary #2 of SBFD TX/RX/measurement procedures	Moderator (CATT)

- R1-2405641	Summary #4 of CLI handling	Moderator (Huawei)