3GPP TSG-RAN WG2 Meeting #127	R2-2406489

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	7.20.1

Source:	Samsung

Title:	Remaining issues on PHR for mTRP STx2P

WID/SID:	NR_MIMO_evo_DL_UL-Core

Document for:	Discussion and Decision

# Introduction

In this paper, we continue discussion on the remaining issues on PHR for mTRP STx2P, including the RAN1 LS reply (R1-2405619) and how to report one type1 PH for DC case.

# Discussion

## RAN1 LS reply on type 3 PH

RAN1 has send a LS reply (R1-2405619) regarding the type 3 PH in PHR for mTRP, as follows.

Response to RAN2 about Enhanced Multiple Entry PHR for multiple TRP MAC CE for Rel-17 mTRP PUSCH repetition: Question a: Whether UE can provide one type 3 PH value instead of two type 1 PH values for a serving cell that is configured with mTRP PUSCH repetition? Answer on Question a: According to current RAN1 specification, for a serving cell configured with mTRP PUSCH repetition and configured with a single UL carrier, the UE cannot provide one type 3 PH value instead of two type 1 PH values for the serving cell. For a serving cell configured with mTRP PUSCH repetition and configured with two UL carriers, RAN1 has no consensus on whether/how current RAN1 specification can support the UE to report one type 3 PH value instead of two type 1 PH values. Question b: If answer to Question a is yes, in which case will the UE report the type 3 PH value for this serving cell, and in which case will the UE report two Type 1 PH values for this serving cell. Response to RAN2 about Enhanced Multiple Entry PHR for multiple TRP STx2P MAC CE for Rel-18 STx2P: Question c: Whether UE can provide one type 3 PH value with one Pcmax instead of two type 1 PH values with two Pcmax for a serving cell that is configured with multipanelSchemeSDM or multipanelSchemeSFN? Answer on Question c: No Question d: If answer to Question c is yes, in which case will the UE provide type 3 PH value with one Pcmax for this serving cell, in which case will the UE provides two type 1 PH values with two Pcmax for this serving cell.

According to this reply,

- for a serving cell that is configured with mTRP PUSCH repetition, reporting type 3 PH is not supported in the current RAN1 specification;

- for a serving cell that is configured with multipanelSchemeSDM or multipanelSchemeSFN, type 3 PH is not reported.

Consequently, in the MAC PHR procedure we should remove the procedural text of obtaining type 3 PH for serving cell configured with mTRP PUSCH repetition or multipanelSchemeSDM or multipanelSchemeSFN, as follows [1].

… 4> if this MAC entity is configured with twoPHRMode: 5> if this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN and the MAC entity this Serving Cell belongs to is configured with twoPHRMode: 6> obtain two values of the Type 1 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell. 5> else if this Serving Cell is configured with multiple TRP PUSCH repetition (i.e., not configured with multipanelSchemeSDM or multipanelSchemeSFN) and the MAC entity this Serving Cell belongs to is configured with twoPHRMode: 6> obtain two values of the Type 1 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell. 5> else: 6> obtain the value of the Type 1 or Type 3 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell and clause 5.1.1.2 of TS 36.213 [17] for E-UTRA Serving Cell. 4> else (i.e. this MAC entity is not configured with twoPHRMode): 5> if this Serving Cell is configured with multiple TRP PUSCH repetition or multipanelSchemeSDM or multipanelSchemeSFN and if the MAC entity this Serving Cell belongs to is configured with twoPHRMode: 6> if there is at least one real PUSCH transmission at the slot where the PHR MAC CE is transmitted: 7> obtain the value of the Type 1 power headroom of the first real transmission of the corresponding uplink carrier as specified in clause 7.7 of TS 38.213[6] for NR Serving Cell. 6> else if there is no real PUSCH transmission at the slot where the PHR MAC CE is transmitted: 7> obtain the value of the type 1 power headroom of the reference PUSCH transmission associated with the SRS-ResourceSet with a lower SRS-resourceSetID or for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213[6] for NR Serving Cell. 5> else: 6> obtain the value of the Type 1 or Type 3 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell and clause 5.1.1.2 of TS 36.213 [17] for E-UTRA Serving Cell. …

Proposal 1: In PHR procedure, type 3 PH is not reported for serving cell configured with multiple TRP PUSCH repetition or multipanelSchemeSDM or multipanelSchemeSFN. Remove type 3 PH at two places in the above MAC procedural text.

## PHR for the case twoPHRmode not configured

In the last meeting, for Rel-18 PHR for STx2P the following agreement has reached.

- When the MAC entity transmitting the PHR is not configured with twoPHRmode, for Rel-18 multi-panel PHR, apply current Rel-17 procedure that report one type 1 PH with possible wording changes in the current steps (e.g., clarify/modify “first transmission” considering simultaneous tx for multi panel case, etc)

However, for this agreement we didn’t make any changes in TS 38.321 due to the issue found in the post meeting CR review. The issue is that it is not sure how to clarify “first transmission” in simultaneous multi-panel PUSCH transmissions. One interpretation could be the PUSCH associated to the first TCI-state, but RAN1 has to be informed and specify corresponding UE behaviour in RAN1 specification. Some companies suggested to send LS to ask RAN1.

As a way forward, we suggest to continue the discussion from RAN2 perspective and ask RAN1 whether they have any concern on our assumption by sending a LS.

To be aligned with Rel-17 mTRP PUSCH repetition where one type 1 PH is obtained for the first real transmission if there is at least one real PUSCH transmission, we propose the following solution for mTRP STx2P.

Proposal 2-1: RAN2 to discuss for mTRP STx2P, if twoPHRmode is not configured,

- if there is at least a real PUSCH transmission,

  - if the first TCI state is applied for a real PUSCH transmission, UE obtains the type 1 PH and the Pcmax associated with the first TCI state;

  - else UE obtains the type 1 PH and the Pcmax associated with the second TCI state.

- else if there is no real PUSCH transmission, UE obtains the type 1 PH and the Pcmax associated with the first TCI state.

Proposal 2-2: RAN2 sends a LS to RAN1 asking if there is any concern on P2-1.

# Conclusion

Based on the above discussion, we have the following proposals.

Proposal 1: In PHR procedure, type 3 PH is not reported for serving cell configured with multiple TRP PUSCH repetition or multipanelSchemeSDM or multipanelSchemeSFN. Remove type 3 PH at two places in the above MAC procedural text.

Proposal 2-1: RAN2 to discuss for mTRP STx2P, for mTRP STx2P, if twoPHRmode is not configured,

- if there is at least a real PUSCH transmission,

  - if the first TCI state is applied for a real PUSCH transmission, UE obtains the type 1 PH and the Pcmax associated with the first TCI state;

  - else UE obtains the type 1 PH and the Pcmax associated with the second TCI state.

- else if there is no real PUSCH transmission, UE obtains the type 1 PH and the Pcmax associated with the first TCI state.

Proposal 2-2: RAN2 sends a LS to RAN1 asking if there is any concern on P2-1.

# References

[1] 3GPP TS 38.321

[2] 3GPP TS 38.213