3GPP TSG-RAN WG2 Meeting #127	R2-2406490

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.8.2

Source:	Samsung

Title:	Discussion on Downlink Coverage Enhancement

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

For Rel-19 NR NTN, the objective of DL coverage enhancement has been updated in the last RAN plenary meeting.

Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

In this contribution, we continue to share our views on the potential enhancements from RAN2 perspective.

# Discussion

## Cell barring

According to the updated WID, SSB periodicity extension for Rel-19 NR NTN would be considered for DL coverage enhancement. However, such enhancement would cause backward compatibility issue, for which RAN1 is required to analyze.

In the last meeting, RAN2 has also agreed to further discuss the impact to legacy UEs. In our view, if DL coverage enhancement (including potential SSB periodicity extension) has any impacts to legacy UE’s operation, to ensure normal operation for legacy UE it can be isolated in legacy cells. According to the current NTN cell barring procedure, by using the legacy NTN bar bit UE can be barred from accessing a cell operating with such enhancement.

Proposal 1: The existing NTN bar bit can be used to bar legacy UE from accessing a cell operating with DL coverage enhancement.

For a cell operating with DL coverage enhancement, the existing bar bit is used to bar legacy UEs, and a new bar bit is needed to indicate the barring for a UE supporting DL coverage enhancement. UE supporting DL coverage enhancement reads the new bar bit to determine if an NTN cell is barred or not.

Proposal 2: Consider to introduce a new bar bit indicate the barring for UE supporting DL coverage enhancement.

## DTX/DRX

Based on the WID, Rel-19 DL coverage enhancement considers the scenario of beam hopping where only a limited number of satellite beams are illuminated at a certain time due to power sharing. Beams of different states (e.g., active beam, inactive beam, etc.) are considered in terms of time and coverage area. In the last meeting, we have sent a LS to RAN1 asking whether different NR beams in a cell can be in the same state or different states at a certain time.

For power saving purpose, it is be beneficial to inform UE the beam pattern and specify UE behavior for different beam states. If all beams in a cell are in the same state at any time, cell-level DTX/DRX pattern can be considered with Rel-18 NES cell DTX/DRX as a baseline. If beams in a cell can be in different states at any time (as shown in Figure below), UEs under different beam states can perform different operations.

In this scenario, beam-level DTX/DRX pattern can be considered. UE can be configured with multiple DTX/DRX patterns and dynamically activated with a pattern by NW indication.

Proposal 3: If beams in a cell are in the same state at any time, cell DTX/DRX pattern can be considered, with Rel-18 NES cell DTX/DRX as a baseline.

Proposal 4: If beams in a cell can be in different states at any time, consider to support configuration of multiple DTX/DRX patterns and dynamical activation by NW indication.

# Conclusion

In this contribution we discussed issues related to...

Proposal 1: The existing NTN bar bit can be used to bar legacy UE from accessing a cell operating with DL coverage enhancement.

Proposal 2: Consider to introduce a new bar bit indicate the barring for UE supporting DL coverage enhancement.

Proposal 3: If beams in a cell are in the same state at any time, cell DTX/DRX pattern can be considered, with Rel-18 NES cell DTX/DRX as a baseline.

Proposal 4: If beams in a cell can be in different states at any time, consider to support configuration of multiple DTX/DRX patterns and dynamical activation by NW indication.

# Reference

RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

3GPP, TS 38.331 v18.2.0; Radio Resource Control (RRC); Protocol specification.