3GPP TSG-RAN WG2 Meeting #127	R2-2406491

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.8.4

Source:	Samsung

Title:	Discussion on MBS Broadcast Service Area information

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

For Rel-19 NTN, one objective is support MBS broadcast service for smaller areas in an NTN cell. The detailed justification and objective in WID are provided as follows.

MBS feature provides an important add-value for NR NTN system, leveraging the large coverage of the NTN compared to TN. Terrestrial MBS features are equally available for NR NTN in the 5G specifications, but for some cases the intended service area is expected to be smaller than the coverage of a Uu cell, some enhancements need to be done to notify the service area of a Broadcast service. Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In this contribution, we continue to share our views on the signaling of broadcast service area information and UE behavior regarding the information.

# Discussion

## Format of intended service area

In RAN2#125bis, we have made an agreement regarding the geographic format of the intended service area.

- At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered):

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

So far for various distance/location-based enhancements in NTN, e.g., (conditional-) event D1/D2, distance-based measurement initiation for cell reselection, TN coverage area information, the format of a reference location (circle center) with a radius/threshold has been used to describing a geographic area for both (quasi)-earth-fixed and earth-moving cells. Specifically, for earth-moving cell, the reference time for the reference location is provided additionally for UE to derive the real-time reference location based on satellite ephemeris.

As we have agreed in the last meeting that both (quasi)-earth-fixed and earth-moving cells are supported for MBS broadcast service, the format of a circle center with a radius can be easily reused to indicate a fixed geographic area for a MBS intended service area in a (quasi)-earth-fixed cell, and for a geographic area in an earth-moving cell a reference time associated to the circle center can be provided additionally for UE to derive the real-time center location based on the corresponding satellite ephemeris.

Proposal 1: The format of a circle center with a radius is used to indicate a fixed geographic area for an MBS intended service area.

Proposal 2: The format of a circle center with a radius and a reference time is used to indicate a moving geographic area for an MBS intended service area. UE derives the real-time center location based on the corresponding satellite ephemeris.

## Signalling of intended service area

So far, we have several agreements regarding the signaling of intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

A list of geographic areas can be provided in SIB20, with each area identified by an area ID. Further, the mapping between area ID and MBS session ID can be provided in MBSBroadcastConfiguration message in MCCH. This allows multiple area IDs mapping to one MBS session ID and multiple MBS session IDs mapping to one area ID. This also allows separate updates on the geographic area information and on the mapping information, considering the geographic area information may be static. The geographic area information in SIB20 can be updated via the SIB modification procedure, and the mapping information in MBSBroadcastConfiguration message can be updated via the MCCH modification procedure.

Proposal 3: A list of geographic areas with each area identified by an area ID is included in SIB20.

Proposal 4: The legacy SIB modification procedure is applied to update the geographic area information in SIB20.

Proposal 5: The mapping between area ID and MBS session ID is included in MBSBroadcastConfiguration message in MCCH, where multiple area IDs can map to an MBS session ID.

Proposal 6: The legacy MCCH modification procedure is applied to update the mapping between area ID and MBS session ID.

## MRB establishment/release

In the last meeting, we have made the following agreement for UE outside intended area.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

In our understanding, this means preventing UE from receiving MBS broadcast service outside the intended area is not a RAN2 issue or intention in the scenario that the intended area is smaller than the physically broadcasting area. Means of prevention can be supported by higher layer, e.g., by restrictions in application layer.

According to the WID, we interpret that the objective is to design signaling of informing intended service area so that UE outside the intended area does not expect the MBS broadcast service intended to it. From RAN2 perspective, we can specify recommended UE behavior on establishing/releasing MRBs when UE is outside the intended area.

We have discussed UE behavior of MRB establishment/release in the last meeting.

- Come back next meeting to check whether a UE may not (or should not) establish MRB(s) associated with a service limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area).

- Come back next meeting to check whether a UE may (or should) release its established MRB(s) associated with a service limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area).

As a recommended UE behaviour, when UE is outside the intended area, it should not establish MRB(s) for the MBS session associated to the intended area.

Proposal 7: For an MBS broadcast service intended for a certain area, UE should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area.

In the current specification, UE may perform MRB establishment/release at certain circumstances to start/stop receiving an MBS session, as UE may or may not interested in the associated MBS broadcast service.

#### 5.9.3.2 Initiation The UE applies the broadcast MRB establishment procedure to start receiving an MBS session of an MBS broadcast service it is interested in. The procedure may be initiated e.g. upon start of the MBS session, upon entering a cell providing an MBS broadcast service the UE is interested in, upon becoming interested in the ongoing MBS broadcast service, upon removal of the UE capability limitations inhibiting reception of the ongoing MBS broadcast service UE is interested in. The UE applies the broadcast MRB release procedure to stop receiving a session of an MBS broadcast service. The procedure may be initiated e.g. upon stop of the MBS session, upon leaving the cell broadcasting the MBS service the UE is interested in, upon losing interest in the MBS service, when capability limitations start inhibiting reception of the concerned service.

Following the same principle, we can specify behaviour for UE inside the intended service area. For an MBS broadcast service intended for a certain area, based on its interest UE may establish MRB(s) to start receiving the MBS session when it is inside the intended area. On the other hand, when UE is leaving the intended area, we cannot specify the exact timing UE releases the MRB(s). Similar to the existing specification, we can specify UE may release MRB(s) to stop receiving the MBS session when leaving the intended area.

Proposal 8: For an MBS broadcast service intended for a certain area, broadcast MRB establishment procedure may be initiated when UE is inside the intended area; broadcast MRB release procedure may be initiated when UE leaves the intended area.

# Conclusion

In this contribution we discussed issues related to...

Proposal 1: The format of a circle center with a radius is used to indicate a fixed geographic area for an MBS intended service area.

Proposal 2: The format of a circle center with a radius and a reference time is used to indicate a moving geographic area for an MBS intended service area. UE derives the real-time center location based on the corresponding satellite ephemeris.

Proposal 3: A list of geographic areas with each area identified by an area ID is included in SIB20.

Proposal 5: The legacy SIB modification procedure is applied to update the geographic area information in SIB20.

Proposal 4: The mapping between area ID and MBS session ID is included in MBSBroadcastConfiguration message in MCCH, where multiple area IDs can map to an MBS session ID.

Proposal 6: The legacy MCCH modification procedure is applied to update the mapping between area ID and MBS session ID.

Proposal 7: For an MBS broadcast service intended for a certain area, UE should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area.

Proposal 8: For an MBS broadcast service intended for a certain area, broadcast MRB establishment procedure may be initiated when UE is inside the intended area; broadcast MRB release procedure may be initiated when UE leaves the intended area.

# Reference

RP-240775, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

3GPP, TS 38.331 v18.0.0; Radio Resource Control (RRC); Protocol specification.