3GPP TSG-RAN WG2 Meeting #127	 R2-2406495

Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	8.4.2

Source: 	NEC

Title:  	LP-WUS procedure in RRC_IDLE INACTIVE

Document for:	Discussion and Decision

- 1 Introduction

In RAN2#126 meeting, the following agreements are made [1]:

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions.

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

- -	LP-SS configuration

- -	LP-WUS configuration

- -	Entry/exit condition for LP-WUS monitoring (FFS if it is always configured)

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

From RAN2 perspective, no new procedure is introduced for SI reception/updates.

In this contribution, we further discuss the detailed design about LP-WUS applied in RRC_IDLE/INACTIVE based on the current agreement.

- 2 Discussion

- 2.1 LP-WUS configuration

The concern of activation/deactivation of LP-WUS monitoring is basically from the LP-WUS monitoring performance, principle should be as long as the RRC_IDLE/INACTIVE UE is able to decode LP-WUS successfully, LP-WUS monitoring should be activated for power saving reason, otherwise, LP-WUS monitoring should be deactivated. Regarding this, the signal quality which significantly impacts on decoding performance is a key metric to determine whether the UE should monitor LP-WUS or not. It is noted that in WID, one of objective is the target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3, which means the same coverage as normal cell is assumed. However even though we try to aim at the same coverage, the risk of LP-WUS monitoring is higher than PDCCH monitoring in normal RRC_IDLE/INACTIVE due to the fact that the LR is a simple modulator. Therefore it is safe to only allow UE who has good signal quality to activate LP-WUS monitoring, which means that the specification must support signal quality based entry/exit condition for LP-WUS monitoring.

Proposal-1: Signal quality based entry/exit condition for LP-WUS monitoring must be supported.

Furthermore, we believe that there could be some cases to also restrict UE to perform LP-WUS monitoring besides entry/exit condition, e.g., UL transmission, other feature (e.g., SDT/MBS) is ongoing during RRC_IDLE/INACTIVE, or latency requirement, etc. If these exceptional cases are existing, the UE is not allowed to enter LP-WUS monitoring mode or is not allowed to evaluate entry/exit condition. In a word, entry/exit condition could be the general one, but more exceptional cases can be FFS.

Proposal-2: In addition to entry/exit condition, some exception cases which may impact on whether the UE can enter LP-WUS monitoring mode should be considered.

Even though the system information is promising to provide LP-WUS configuration including entry/exit condition, but one question is that whether a unified entry/exit condition broadcasted by system information is suitable for all UEs in one cell considering different UE may have different LR capability, therefore a UE specified entry/exit condition of LP-WUS monitoring by dedicated RRC signalling can also be considered as an optional feature, here, since this is intended for RRC_IDLE/INACTIVE UE, RRCRelease is more suitable.

Proposal-3: Support RRCRelease to optionally carry UE-specific entry/exit condition for LP-WUS monitoring, if it is absent, the UE follows the entry/exit condition broadcasted by system information.

- 2.2 Subgrouping

To save time budget, the subgrouping mechanism which is already completely designed for PEI can also be as baseline for LP-WUS subgrouping. Here more details need to be discussed, that is the association between LP-WUS subgrouping and PEI subgrouping. Since the UE may monitor both LP-WUS and PEI, whether false alarm could be further reduced should be considered, otherwise, it is useless to monitor both LP-WUS and PEI. Here the principle should be the UEs belonging to the same LP-WUS/PEI subgroup should not be assigned to the same PEI/LP-WUS subgroup.

There are 3 types for the association:

Type-1: Only CN based subgrouping is used for both LP-WUS and PEI

Type-2: Both CN based and UE-ID based subgrouping are used, i.e., UE-ID based for LP-WUS and CN based for PEI, vice versa

Type-3: Only UE-ID based subgrouping is used for both LP-WUS and PEI

As long as the UEs belonging to the same LP-WUS/PEI subgroup are assigned to different PEI/LP-WUS subgroup, the false alarm could be further reduced. When CN based subgrouping is involved, e.g., type-1 and type-2, the network can, based on implementation, make a smart decision on differentiating LP-WUS and PEI subgrouping ID among UEs belonging to the same LP-WUS/PEI subgroup. Question for type-2 is that how CN is aware of UE-ID based subgrouping information, for example, if UE-1 and UE-2 are using UE-ID based subgrouping for LP-WUS, when CN assign PEI subgroup, does it know UE-1 and UE-2 will be assigned for the same one LP-WUS subgroup or not? It could be FFS.

Proposal-4a: If CN based subgrouping is used for both LP-WUS and PEI, it is up to NW implementation to differentiate CN based subgroup ID between UEs belonging to the same LP-WUS/PEI subgroup.

Proposal-4b: If both CN based and UE-ID based subgrouping are used, it is up to NW implementation to differentiate CN based subgroup ID between UEs belonging to the same LP-WUS/PEI subgroup, FFS how CN is aware of UE-ID based subgrouping information.

For only UE-ID based subgrouping, it is promising to design LP-WUS subgrouping formula similar as PEI. Again question is that how to make sure that the UEs belonging to the same LP-WUS/PEI subgroup are assigned to different PEI/LP-WUS subgroup. In fact, if different UE-ID based subgrouping number is configured between LP-WUS and PEI, according to the formula, the group of UEs belonging to the same one LP-WUS/PEI subgroup will be assigned to different PEI/LP-WUS subgroup by mod different subgroupsNumForUEID, which further reduce false alarm issue. Note that for this solution (i.e., different subgroupsNumForUEID configuration), although the false alarm gain is not so perfect compared with other orthogonality-like solution, it is very simple to be achieved as long as the NW broadcast different UE-ID based subgroup number between LP-WUS and PEI. Therefore, it is a promising solution, but instead of saying this is totally NW implementation, we would rather make some NOTEs for guidance since it is not easy to let future reader understand how to further reduce false alarm issue when the UE is monitoring both signal. For example, the NOTE could be “the UE is not required to monitor both LP-WUS and PEI if the UE-ID based subgroup number for LP-WUS and PEI broadcasted from system information is same”.

Proposal-4c: If UE-ID based subgrouping is used for both LP-WUS and PEI, it is up to NW implementation to broadcast different UE-ID based subgroup number for both LP-WUS and PEI. And a NOTE is needed, e.g., “the UE is not required to monitor both LP-WUS and PEI if the UE-ID based subgroup number for LP-WUS and PEI broadcasted from system information is same”.

- 2.3 SI/PWS reception

In last meeting, it has been agreed that “From RAN2 perspective, no new procedure is introduced for SI reception/updates”, which means if there is short message sent from NW, the NW would firstly use LP-WUS to trigger UE to wake up, then the UE receive paging short message and proceed system information modification as legacy way. Basically this is same mechanism as PEI solution, but there is some difference, i.e., MR and LR transition is involved.

Based on the current achieved agreement, if the UE receive LP-WUS indication and turn on its MR, realising this paging is for short message (e.g., SI update or PWS reception). What is UE behaviour after SI/PWS reception? For normal UE, the UE will keep monitoring PEI/PO, but for LP-WUS capable UE, since there is no UE specific paging indicated, after reception of SI/PWS, keeping MR turned on is unnecessary, the LP-WUS monitoring UE should go back to LP-WUS monitoring mode again for power saving reason, i.e., turn off MR and keep using LR to monitoring LP-WUS.

Proposal-5: After SI acquisition / PWS reception during MR-ON, the LP-WUS monitoring UE should go back to LP-WUS monitoring mode, i.e., turn off MR and keep using LR to monitoring LP-WUS.

- 2.4 Cell reselection

Regarding mobility (i.e., cell reselection), it is expected that cell reselection can be supported for a LP-WUS capable UE during LP-WUS monitoring mode, since measurement results for both serving cell and neighbour cell can be derived during LP-WUS monitoring mode. And one of the reasons why mobility during LP-WUS monitoring mode should be supported is that we are expecting the UE could monitor LP-WUS as much as possible among cells as long as there is no paging/data/signal arrival even though the cell has already been changed, staying LP-WUS monitoring mode as much as possible is beneficial for UE power saving.

Fig.1 Cell reselection for LP-WUS capable UE

One example for intra frequency is that in legacy, the UE perform cell reselection based on the best cell. However if the best cell is not supporting LP-WUS feature meanwhile some other cells which support LP-WUS feature are also good enough for camping, by legacy way, the LP-WUS capable UE need to exist from LPWUS monitoring mode and camp on the best cell, which is not friendly for power saving. Therefore some solutions could be considered to let UE perform cell reselection to select one cell which supports LP-WUS feature as much as possible, for example, neighbour cell information can be provided by serving cell, which can inform UE of the information about which cell is providing LP-WUS configuration, i.e., LP-WUS supporting cell. Then the UE can, based on the neighbour cell information, optimize its cell reselection decision, of which details is FFS.

Proposal-6: RAN2 is suggested to consider optimization on cell reselection, e.g., introducing neighbour cell information to facilitate LP-WUS monitoring UE to camp on LP-WUS supporting cell as much as possible.

- 3 Conclusions

Proposal-1: Signal quality based entry/exit condition for LP-WUS monitoring must be supported.

Proposal-2: In addition to entry/exit condition, some exception cases which may impact on whether the UE can enter LP-WUS monitoring mode should be considered.

Proposal-3: Support RRCRelease to optionally carry UE-specific entry/exit condition for LP-WUS monitoring, if it is absent, the UE follows the entry/exit condition broadcasted by system information.

Proposal-4a: If CN based subgrouping is used for both LP-WUS and PEI, it is up to NW implementation to differentiate CN based subgroup ID between UEs belonging to the same LP-WUS/PEI subgroup.

Proposal-4b: If both CN based and UE-ID based subgrouping are used, it is up to NW implementation to differentiate CN based subgroup ID between UEs belonging to the same LP-WUS/PEI subgroup, FFS how CN is aware of UE-ID based subgrouping information.

Proposal-4c: If UE-ID based subgrouping is used for both LP-WUS and PEI, it is up to NW implementation to broadcast different UE-ID based subgroup number for both LP-WUS and PEI. And a NOTE is needed, e.g., “the UE is not required to monitor both LP-WUS and PEI if the UE-ID based subgroup number for LP-WUS and PEI broadcasted from system information is same”.

Proposal 5: After SI acquisition / PWS reception during MR-ON, the LP-WUS monitoring UE should go back to LP-WUS monitoring mode, i.e., turn off MR and keep using LR to monitoring LP-WUS.

Proposal 6: RAN2 is suggested to consider optimization on cell reselection so that LP-WUS monitoring UE can select LP-WUS capable cell with higher priority.

- 4 References

[1] R2-2405702, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS