3GPP TSG-RAN WG2 Meeting #127	 R2- 2406496

Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	8.4.3

Source: 	NEC

Title:  	LP-WUS RRM measurement

Document for:	Discussion and Decision

- 1 Introduction

In RAN2#126 meeting, the agreements are made as the following [1]:

For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise.

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

In this contribution we further discuss measurement offloading/relaxation during LP-WUS monitoring mode.

- 2 Discussion

- 2.1 Scope of measurement offloading/relaxation

Before discuss the details about offloading and relaxation, one issue is raised that whether the further relaxed criterion and measurement including serving cell and neighbour cell will be introduced also for normal UE. In our understanding, all offloading/relaxation requirement is discussed based on LR is equipped, it is not clear that whether some impact could be caused for normal UE to perform a further relaxation without LR equipment. In fact, the exiting relaxation mechanism specified in TS 38.304 including measurement rule and measurement relaxation has already shown the power saving gain, no need to further enhance it at least in the later 5G phase. For serving cell measurement relaxation, since it is so important to impact on UE performance, e.g., cell reselection, some feature including SDT, MBS, etc, power saving is not the key issue, it is not preferred to discuss it. Therefore, only LP-WUS capable UE is applicable to LP-WUS specific measurement offloading/relaxation, i.e., it is not applicable to normal UE.

Proposal-1: Only LP-WUS capable UE is applicable to LP-WUS specific measurement offloading/relaxation, i.e., it is not applicable to normal UE.

- 2.2 General measurement framework

In last RAN2#126 meeting, it was discussed whether RRM relaxation by non-LPWUS-capable UE is also included in the WI scope or not. Some companies including us assumed that this is not included, as the WI is specific to UEs supporting LP-WUS/WUR. Some other companies did not have the same understanding. There were also some comments on RAN4 status and thus we checked the RAN4 status after the last meeting.

In last RAN4#111 meeting [2], the agreement about use case for measurement offloading/relaxation is given as the following:

Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |

RAN4 to further discuss case #2 to #4:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

Based on RAN4 agreement we can see that as long as the UE is in LP-WUS monitoring mode, no matter the case is for fully offloading or relaxation, the UE always need to use LR to perform measurement (e.g., LP-SS measurement by OOK-based LR or SSB measurement by OFDM-based LR), furthermore, due to LR complexity, the LR should only perform measurement on serving cell and not support to perform measurement on neighbour cell.

Proposal-2: RAN2 to confirm that non-LPWUS-capable UE is not the target of this WI, e.g. the RRM measurement relaxation is applicable only for LPWUS-capable UE.

Proposal-3: RAN2 to confirm that as long as the UE is in LP-WUS monitoring mode, the UE always need to use LR to perform measurement on serving cell.

For fully offloading case, it means no any measurement is performed by MR during LP-WUS monitoring mode, i.e., the MR is fully turned off. And for relaxed case, at least one of the MR serving cell measurement or MR neighbour cell measurement should be performed but in a relaxed way. However in our understanding, for relaxed case, since the MR anyway has already turned on to perform measurement, the efficient way is to perform both serving cell and neighbour cell measurement in a same requirement (e.g., same measurement period), it is not reasonable to only perform any one of them during MR-ON. Therefore, relaxed case a and b can be excluded. And we RAN2 can further discuss fully offloading and relaxed case b unless more conclusion is made by RAN4.

Proposal-4: RAN2 is suggested to further consider fully offloading case (no any measurement by MR) and relaxed case b (both relaxed serving cell and neighbour cell measurement by MR) identified in RAN4.

- 2.3 Measurement relaxation/offloading criterion

According to TS 38.304 and 38.133, there is already specified measurement rule and relaxation as shown below:

General principle of legacy measurement rule for cell re-selection

- If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ:

- the UE may not perform intra-frequency measurements;

- If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ:

- the UE may not to perform measurements of NR inter-frequency cells of equal or lower priority;

- the UE shall search for inter-frequency of higher priority at least every Thigher_priority_search ([60 * Num of higher priority frequency] seconds).

General principle of legacy measurement relaxation

If the UE has not stopped measurement based on measurement rule, then we go with the following:

- if one of the low-mobility and not-at-cell-edge criterion is fulfilled:

- the UE may choose to perform relaxed measurements according to relaxation factor K1 (3);

- if both low-mobility and not-at-cell-edge criterion are fulfilled:

- the UE is not required to meet the measurement requirement for 1 hour.

NOTE: higher priority frequency measurement relaxation further depends on whether Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ.

Back to LP-WUS mechanism, the intention is to design a further measurement relaxation or offloading on top of the legacy measurement mechanism. Baseline principle is that as long as the UE enters LP-WUS monitoring mode, the UE should not turn on the MR frequently for measurement reason.

Serving cell measurement by MR during LP-WUS monitoring mode

For serving cell measurement during LP-WUS monitoring mode, since the LR anyway need to perform serving cell measurement, the UE is aware of the signal situation to make decision on whether to entry/exit LP-WUS monitoring mode, the reason to still perform serving cell measurement by MR during LP-WUS monitoring mode is to make sure mobility enhancement, e.g., use MR measurement result to evaluate cell reselection, or to compare MR serving measurement result with LR serving measurement result for robust reason. However the MR should perform serving cell measurement in a relaxed way, otherwise, frequent transition between MR-ON and MR-OFF could overkill the power saving gain. Therefore it is suggested that as long as the UE enters LP-WUS monitoring mode, the UE shall perform serving cell measurement by MR in a relaxed way (that is so called serving cell partially offloading). Furthermore, if the UE signal quality is good enough, since serving cell measurement by LR is anyway needed, it is possible to fully offload serving cell measurement from MR to LR.

Neighbor cell measurement by MR during LP-WUS monitoring mode

For neighbor cell measurement during LP-WUS monitoring mode, since LR is not able to perform neighbor cell measurement, the MR is required to turn on to perform neighbor cell measurement for mobility reason, but again, it should be in a relaxed way and it is preferred that the measurement requirement, especially measurement period should be same as that of serving cell measurement by MR to ensure the power efficiency (i.e., perform both serving and neighbor cell measurement by MR during once transition). Furthermore, if the UE signal quality is good enough, then the UE is not required to evaluate neighbor cell for mobility reason as it is not likely to reselect cell.

Baseline criterion

Fig.1 measurement relaxation/offloading framework

First question is that how to coordinate the legacy measurement mechanism with the LP-WUS specific measurement mechanism. Generally speaking, the legacy rule can be referred but the threshold value needs to be more stringent. Otherwise it is strange that the legacy measurement relaxation criterion is not fulfilled but the UE has already begun to perform some relaxed measurement.

For relaxed case, to make things simple, it is straightforward to consider the threshold of entry condition as measurement relaxation condition, once the UE enter LP-WUS monitoring mode, the UE need to perform both serving cell and neighbor cell measurement by MR in a same measurement requirement (i.e., measurement period). Note that in this case, as shown in Fig.1, the UE has already fulfilled not-at-cell-edge criterion, it is natural to directly perform measurement relaxation by MR. As for the specific measurement factor, it could be up to RAN4.

For fully offloading case, currently based on legacy, the UE will not perform neighbor cell measurement (except from higher priority frequency) when Srxlev > SIntraSearchP and Squal > SIntraSearchQ / Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, it is natural that the UE need to perform offloading when this measurement rule is also fulfilled, which is to say, there should be another e.g., cell center criterion related to signal quality as shown in Fig.1 and it should be more stringent than measurement rule and evaluated by LR. Only in this situation, it is safe to fully offload MR measurement (i.e., no any serving cell and neighbor cell measurement by MR) meanwhile the LR serving measurement is still going on.

As for more details about this measurement framework, it can further study whether low-mobility or stationary criterion should be involved and the design of higher priority frequency can be done after we have some consensus on basic framework.

Proposal-5: RAN2 is suggested to consider the following measurement framework:

For relaxed case, the UE performs relaxed measurement on both serving cell and neighbor cell by MR in a same measurement requirement when the UE fulfills the entry condition, the entry condition can be evaluated either by MR or LR;

For fully offloading case, the UE stops measurements on both serving cell and neighbor cell when the UE fulfills cell center criterion (i.e., signal quality is good enough more than legacy measurement rule), this criterion is evaluated by LR;

FFS whether low-mobility/stationary criterion is needed

FFS higher priority frequency measurement rule

- 3 Conclusions

Proposal-1: Only LP-WUS capable UE is applicable to LP-WUS specific measurement offloading/relaxation, i.e., it is not applicable to normal UE.

Proposal-2: RAN2 to confirm that non-LPWUS-capable UE is not the target of this WI, e.g. the RRM measurement relaxation is applicable only for LPWUS-capable UE.

Proposal-3: RAN2 to confirm that as long as the UE is in LP-WUS monitoring mode, the UE always need to use LR to perform measurement on serving cell.

Proposal-4: RAN2 is suggested to further consider fully offloading case (no any measurement by MR) and relaxed case b (both relaxed serving cell and neighbour cell measurement by MR) identified in RAN4.

Proposal-5: RAN2 is suggested to consider the following measurement framework:

For relaxed case, the UE performs relaxed measurement on both serving cell and neighbor cell by MR in a same measurement requirement when the UE fulfills the entry condition, the entry condition can be evaluated either by MR or LR;

For fully offloading case, the UE stops measurements on both serving cell and neighbor cell when the UE fulfills cell center criterion (i.e., signal quality is good enough more than legacy measurement rule), this criterion is evaluated by LR;

FFS whether low-mobility/stationary criterion is needed

FFS higher priority frequency measurement rule

- 4 References

[1] R2-2405702, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

[2] RAN4 #111 RRM session meeting report