3GPP TSG-RAN WG2 Meeting #127	 R2- 2406497

Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	8.4.4

Source: 	NEC

Title:  	LP-WUS procedure in RRC_CONNECTED

Document for:	Discussion and Decision

- 1 Introduction

In RAN2#126 meeting, the following agreements are made:

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1.

For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1.

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In this contribution, we further discuss the general design for LP-WUS in RRC_CONNECTED based on the current agreement.

- 2 Discussion

- 2.1 Background

The candidate solution for LP-WUS applied in RRC_CONNECTED is identified in RAN1 as shown below:

- For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring:

  - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

    - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

      - This option may replace DCP functionality

    - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

      - PDCCH monitoring possibly irrespective of drx-onDurationTimer

        - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

          - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX

        - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

    - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

- Combination of options in Case 1 should be considered.

First of all, RAN1 has precluded the case of LP-WUS without C-DRX, as it could impact more functionality such as RRM measurement requirement or any other possible transmission during C-DRX active window if no C-DRX timer would be running. Therefore, only the case of LP-WUS with C-DRX can be considered.

Proposal-1: RAN2 to confirm that the LP-WUS is always configured with C-DRX in RRC_CONNECTED.

For option 1-1, it has been agreed that legacy DCP and Option 1-1 is not configured simultaneously for a UE. Therefore, it means the legacy DCP can be replaced by LP-WUS, the UE will monitor a lower power consumption signal to control C-DRX active time but the same mechanism is reused. No latency but power saving is improved by this way. Since this is a fundamental mechanism, option 1-1 can be supported as a starting point. Furthermore, to improve both latency and power saving, further solution 1-2 and 1-3 can be considered on the top of option 1-1.

Proposal-2: RAN2 to confirm that option 1-1 is supported and further discuss option 1-2 and option 1-3.

- 2.2 General framework for LP-WUS in RRC_CONNECTED

To our understanding, it is promising to support option 1-1, but furthermore, the option 1-1 is the simplest one as shown in Fig.1, and the power saving gain is only from replacement from DCP to LP-WUS, no any latency optimization. In fact by introducing LP-WUS in some proper ways, not only power consumption is reduced but also latency optimization can be achieved. Therefore, only considering option 1-1 is not good enough in terms of LP-WUS applied in RRC_CONNECTED.

Fig.1 Option 1-1, DCP-like solution

For option 1-2, there are two interpretations:

Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

Fig.2 Option 1-2-1, in addition to LP-WUS triggered timer, DRX onD timer starts as legacy

Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

Fig.2 Option 1-2-2, DRX onD timer will not start as legacy

To our understanding, at least from RAN2 perspective, option 1-2-2 looks a little strange that when C-DRX is configured, drx-onDurationTimer will not trigger PDCCH monitoring, but only LP-WUS can trigger PDCCH monitoring within a new active timer, which means the basic principle of C-DRX mechanism change dramatically. To some extent it is equivalent to LP-WUS without C-DRX which is already precluded by RAN1. The goal for option 1-2-2 (compared with option 1-2-1) is to further reduce UE power consumption since there is no need to use drx-onDurationTimer for PDCCH monitoring. However, even though we go for option 1-2-1, it doesn’t necessarily mean every drx-onDurationTimer will be started, in fact, drx-onDurationTimer could also be conditionally started by LP-WUS triggering. That is to say, besides LP-WUS to trigger PDCCH monitoring within a new active timer, the drx-onDurationTimer should also be controlled by LP-WUS.

The key question is turned to how to combine option 1-2-1 with option 1-1. To our understanding, some LP-WUS will trigger a new active timer, and some LP-WUS will trigger drx-onDurationTimer. Do we need two types of LP-WUS occasion configuration (i.e., part of LP-WUS is offset specific to drx-onDurationTimer, part of LP-WUS is located anywhere outside of C-DRX active time)? We think this makes things complicated and not needed. In fact we can just provide only one type of LP-WUS occasion configuration, and the periodicity of C-DRX cycle may be integer multiple of periodicity of LP-WUS. Then the closet LP-WUS to next drx-onDurationTimer will be used to trigger drx-onDurationTimer and other LP-WUS is used to trigger new active timer. Details about what is closet LP-WUS could be FFS. Details about other C-DRX timer, e.g., drx-InactivityTimer could be FFS. The whole picture is shown as Fig.3.

Note that since the combination of LP-WUS and DCP usage is precluded by RAN1 regarding option 1-1, therefore, the combination of LP-WUS and DCP for option 1-2 can also be precluded as it is not clear for the gain but make things complicated.

Fig.3 option 1-2-1 combined with option 1-1

Proposal-3: Support option 1-2-1 in a combination with option 1-1, i.e., LP-WUS can be used to trigger new active timer and legacy drx-onDurationTimer, detailed triggering FFS.

- 3 Conclusions

Proposal-1: RAN2 to confirm that the LP-WUS is always configured with C-DRX in RRC_CONNECTED.

Proposal-2: RAN2 to confirm that option 1-1 is supported and further discuss option 1-2 and option 1-3.

Proposal-3: Support option 1-2-1 in a combination with option 1-1, i.e., LP-WUS can be used to trigger new active timer and legacy drx-onDurationTimer, detailed triggering FFS.

- 4 References