3GPP TSG-RAN WG2 Meeting #127	 R2- 2406498

Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	8.1.3

Source: 	NEC

Title:  	AIML MDT-based data collection for NW-side model

Document for:	Discussion and Decision

- 1 Introduction

In RAN2#125b meeting, the following agreements are made:

Agreements

1	For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches

2	We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

3	RAN2 supports enhancements to MDT for data collection framework for training.  FSS Whether to enhance logged or immediate MDT

In RAN2#126 meeting, the following agreements are made:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

Based on the agreements, immediate MDT mechanism is selected for both gNB-centric and OAM-centric data collection and in this contribution we will further discuss the detailed design. Furthermore whether L1 signaling is also supported for data collection for NW-side model training is up to RAN1 decision. Here we only focus on using L3 signaling to design data collection for NW-side model training.

- 2 Discussion

- 2.1 General requirement for immediate MDT

For immediate MDT, the OAM can collect data with the UE involvement (e.g., measurement and report) in RRC_CONNECTED. And the immediate MDT configuration for UE measurements is based on existing RRM measurement procedure for configuration and reporting with some extensions for MDT record, i.e., location/sensor/BT/WLAN/PDCP delay information. That is to say, the framework of measConfig is reused, when the report configuration indicate the UE to include MDT record, once the measurement report is initiated either by periodical or event-triggered type, the UE will include the available MDT record into measurement report and sent it to gNB, then the gNB forward these information into OAM.

Based on immediate MDT mechanism, even though the termination of immediate MDT is OAM, the collected training data is firstly transferred to gNB in non-transparent way then the gNB forward data to OAM. Therefore, gNB can also be designed to handle with these training data transferred by immediate MDT. No matter it is gNB-centric or OAM-centric data collection, immediate MDT framework can be used as data collection framework for both.

Proposal-1: RAN2 to confirm that immediate MDT framework is used for both gNB-centric and OAM-centric data collection for NW-side model training.

Following is to discuss what enhancement is needed for data collection based immediate MDT.

For the general framework of immediate MDT, in TR 38.843, it is already said that the analysis/selection of the data collection frameworks should focus on the RRC CONNECTED state for both data generation and reporting. Considering the AI/ML model (except for positioning in RRC_INACTIVE) will be operated during RRC_CONNECTED, and most of use cases are to consider optimizing the UE performance during RRC_CONNECTED, it is suggested that data generation and report should be happened in RRC_CONNECTED at least for this release.

Currently, immediate MDT is used to collect location/sensor/BT/WLAN/PDCP delay information, if AI/ML data collection is involved, specific measurement content related to data collection should also be included. For example, if MDT is used for AI/ML beam management, the network should enable UE to collect and include multiple L1 beam measurement results with or without different time instance into MDT report (i.e., MeasurementReport).

Furthermore, since data collection procedure requires the UE to collect a large amount of training data in quite a long duration, a storage space should be considered, e.g., a new UE variable/storage for data collection to let UE be able to store training data until next measurement report. It is noted that current immediate MDT framework does not support data log/storage.

For report type, currently immediate MDT has supported periodical and event-triggered report, but for data collection reason, the current event-triggered report type may not be enough, specific report type is needed. For example, considering model should be operated under a specific additional condition, therefore the UE just needs to initiate training data report based on model additional condition, e.g., data collection initiation for low-UE-speed BM-Case2 (i.e., temporal) model based on UE velocity, or another example is that the maximum collected date volume should be introduced to control the data log/storage, once the storage is full, the UE initiate training data report, more details about report type can be studied case by case.

Proposal-2: For the enhanced immediate MDT framework, the following requirements are required:

- Data generation and report should be happened in RRC_CONNECTED;

- New measurement content related to data collection should be introduced into MDT report (i.e., MeasurementReport), e.g., multiple L1-beam measurement results with/without different time instance;

- The UE log/storage for storing collected data is needed, e.g., new UE variable.

- New specific report type related to data collection should be introduced, e.g., specific model additional condition, the max volume of stored data.

- 2.2 limitation on data collection for NW-side model training

Immediate MDT report is to reuse the existing measurement report mechanism using SRB1, since data collection for NW-side model training is a long term procedure, which means MeasurementReport will be sent frequently in a quite long time. Obviously it could impact on UE uplink transmission efficiency. Therefore, it is not wise to let UE report training data without any limitation. Some enhancements can be considered, for example, the UE could log multiple measurement results and report them in one single report based on an event, or another example is that a maximum volume can be considered, once the reported training data reach to the maximum volume, the UE should stop/release this time of data collection procedure. More methods can be FFS.

Proposal-3: RAN2 is suggested to consider some limitations on data collection for NW-side model training.

- 3 Conclusions

Proposal-1: RAN2 to confirm that immediate MDT framework is used for both gNB-centric and OAM-centric data collection for NW-side model training.

Proposal-2: For the enhanced immediate MDT framework, the following requirements are required:

- Data generation and report should be happened in RRC_CONNECTED;

- New measurement content related to data collection should be introduced into MDT report (i.e., MeasurementReport), e.g., multiple L1-beam measurement results with/without different time instance;

- The UE log/storage for storing collected data is needed, e.g., new UE variable.

- New specific report type related to data collection should be introduced, e.g., specific model additional condition, the max volume of stored data.

Proposal-3: RAN2 is suggested to consider some limitations on data collection for NW-side model training.

- 4 References