3GPP TSG-RAN WG2 Meeting #127	 R2- 2406499

Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	8.3.2.2

Source: 	NEC

Title:  	AIML mobility RRM measurement prediction

Document for:	Discussion and Decision

- 1 Introduction

In RAN2#126 meeting, for RRM measurement prediction, RAN2 mainly focus on simulation design, and made lots of agreements related to simulation, but besides simulation design, in this contribution, we provide some general aspects/consideration for RRM measurement prediction.

- 2 Discussion

- 2.1 Cell level and Beam level measurement prediction type

In RAN2#125b meeting, the following agreement is made for cell level measurement prediction:

For cell level measurement prediction model, at least consider the following cases:

Case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

Case 2: To directly predict cell level results based on cell level results.

Case 3: To directly predict cell level results based on beam level results

Fig.1 measurement model

The agreed use cases can facilitate the AI model design, but more details are still needed. Since the AI model is for L3 mobility management, the general understanding is that predicted cell level result indicated here is L3 cell level result (i.e., C) for all cases. Furthermore according to TS 38.300 measurement model (Fig.1), for case 1, only L1 beam level result can be used as model input to inference L1 beam level result. For case 2, either the cell quality after beam consolidation/selection (i.e., B) or L3 cell level result can be used as model input to inference L3 cell level result, note that if the cell quality after beam consolidation/selection (i.e., B) is used as model input, the layer 3 filtering procedure is inside AI inference. For case 3, only L1 beam level result can be used as model input to inference L3 cell level result, note that in this case, the layer 3 filtering procedure is inside AI inference. Furthermore, there is still raw beam (i.e., A) can be considered instead of L1 beam, our understanding is that since layer 1 filtering procedure is UE implementation, it is not clear whether/how we use it, therefore, it is preferred that consider L1 beam level result as model input. Based on the analysis we can modify the agreement as the following:

Proposal-1: modify the cell level measurement prediction case as following:

For cell level measurement prediction model, at least consider the following cases:

Case 1: To predict L1 beam level results, then generate L3 cell level results based on the predicted L1 beam level results;

Case 2: To directly predict L3 cell level results based on L3 cell level results or the cell quality after beam consolidation/selection (i.e., B);

Case 3: To directly predict L3 cell level results based on L1 beam level results.

NOTE: for case 2 where the cell quality after beam consolidation/selection (i.e., B) is model input, and for case 3, the layer 3 filtering procedure is internal implementation of AI inference;

Addition to L3 cell level measurement prediction, note that in measurement model, there is another direction to directly derive L3 beam level result (i.e., E/F in Fig.1). This is used to facilitate network configuration, for example, to let target cell be aware of the best beam served for the UE based on measurement report, so that the beam selected for random access can be reasonably configured. Regarding this, it is expected that AI model can also be used for L3 beam level prediction, and the same mechanism as L3 cell level prediction can be reused.

Proposal-2: L3 beam level prediction can be considered as following cases:

Case1: To predict L1 beam level results, then generate L3 beam level results based on the predicted L1 beam level results;

Case2: To directly predict L3 beam level results based on L3 beam level results;

Case3: To directly predict L3 beam level results based on L1 beam level results.

NOTE: for case 3, the layer 3 filtering procedure is internal implementation of AI inference.

- 2.2 General framework on scenarios

Based on the discussion in RAN2 so far, basically we have the following scenarios related to cell level prediction:

| Intra-frequency-intra-cell temporal domain case A/B | intra-F-intra-C-T-Case A/B |
| --- | --- |
| Intra-frequency-intra-cell spatial domain | intra-F-intra-C-S |
| Intra-frequency-inter-cell temporal domain case A | intra-F-inter-C-T-Case A |
| Intra-frequent-inter-cell spatial domain | intra-F-inter-C-S |
| Inter-frequency-inter-cell temporal domain case A | inter-F-inter-C-T-Case A |
| Inter-frequency-inter-cell spatial domain (frequency domain) | inter-F-inter-C-S |

Note that for intra-F-inter-C-T and inter-F-inter-C-T, there is no case B, reason is that case B is to use subset (sparse) of measurement instance to predict the whole set (dense) of measurement instance, it should be happened in the same cell, i.e., intra cell case where input is subset of measurement instance of the same cell and output is the whole set of measurement instance of the same cell. Therefore, it is preferred that for intra-F-inter-C-T and inter-F-inter-C-T, only to study case A, i.e., input is measurement results of cell A within observation window to predict measurement results of cell B within prediction window.

As we can see there are so many scenarios, it does not necessarily mean all scenarios should be specified, but in the initial stage we can analyse all the scenarios and based on pros and cons, to make the down selection later. Here the general framework for model input/output in a combination of scenarios and prediction model types are given in the following tables, the intention is to reach an alignment among companies. Note that for convenience, L1 Cell in the following table means the cell quality after beam consolidation/selection (i.e., B).

Proposal-3: RAN2 to confirm the model input and output for different scenarios based on the following table:

| Scenario | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- |
| intra-F-intra-C-T-Case A/B | For case A: Input: L1 intra beam within observation window Output: L1 intra beam within prediction window For case B: Input: subset of measurement instance of L1 intra beam Output: the whole set of measurement instance of L1 intra beam | For case A: Input: L1 or L3 intra cell within observation window Output: L3 intra cell level within prediction window For case B: Input: subset of measurement instance of L1 or L3 intra cell Output: the whole set of measurement instance of L3 intra cell | For case A: Input: L1 intra beam within observation window Output: L3 intra cell within prediction window For case B: Input: subset of measurement instance of L1 intra beam Output: the whole set of measurement instance of L3 intra cell |
| intra-F-intra-C-S | Input: set B of L1 intra beam Output: set A of L1 intra beam | N/A | Input: set B of L1 intra beam Output: L3 intra cell |
| intra-F-inter-C-T-Case A | Input: L1 intra beam (+ inter beam if cluster) within observation window Output: L1 inter beam within prediction window | Input: L1 or L3 intra cell (+ inter cell if cluster) within observation window Output: L3 inter cell level within prediction window | Input: L1 intra beam (+ inter beam if cluster) within observation window Output: L3 inter cell within prediction window |
| intra-F-inter-C-S | Input: L1 intra beam (+ inter beam if cluster) Output: L1 inter beam | Input: L1 or L3 intra cell (+ inter cell if cluster) Output: L3 inter cell | Input: L1 intra beam (+ inter beam if cluster) Output: L3 inter cell |
| inter-F-inter-C-T-Case A | Input: intra frequency L1 beam (+ inter frequency L1 beam if cluster) within observation window Output: inter frequency L1 beam within prediction window | Input: intra frequency L1 or L3 cell (+ inter frequency cell if cluster) within observation window Output: inter frequency L3 cell within prediction window | Input: intra frequency L1 beam (+ inter frequency L1 beam if cluster) within observation window Output: inter frequency L3 cell within prediction window |
| inter-F-inter-C-S | Input: intra frequency L1 beam + (inter frequency L1 beam if cluster) Output: inter frequency L1 beam | Input: intra frequency L1 or L3 cell (+ inter frequency L1 or L3 cell if cluster) Output: inter frequency L3 cell | Output: intra frequency L1 beam + (inter frequency L1 beam if cluster) Output: inter frequency L3 cell |

- 2.3 Other considerations for RRM measurement prediction

In this section, we provide more details regarding AI mobility design.

Normally when the UE is staying in the cell center (i.e., signal quality is good enough) or the UE is in low mobility / stationary status, it is not likely to perform any operations related to mobility, e.g., handover. Since the AI model is deployed for mobility enhancement, there is no need to perform AI inference for such kind of UE.

Fig.2 Conditions on AI mobility model (de)activation

Proposal-4: AI model can be deactivated when the UE signal quality is good enough and/or the UE is in low mobility or stationary status.

For UE-side model, the model inference input should be derived by real measurement based on the measurement resources configured by the NW. If the NW is not aware of model configuration information (e.g., what is model input), it will not be able to reasonably configure the measurement resources for model inference at UE side. For instance, if the NW lacks clarity on which frequencies/cells/beams are required for model inference input, it may be unable to configure measurement configuration for model inference at UE side. Therefore, the UE should provide the NW with preferred measurement configuration related to the frequencies/cells/beams required for model inference input.

Proposal-5: For UE-side model, the UE should provide the NW with preferred measurement configuration related to the frequencies/cells/beams required for model inference input.

- 3 Conclusions

Proposal-1: modify the cell level measurement prediction case as following:

For cell level measurement prediction model, at least consider the following cases:

Case 1: To predict L1 beam level results, then generate L3 cell level results based on the predicted L1 beam level results;

Case 2: To directly predict L3 cell level results based on L3 cell level results or the cell quality after beam consolidation/selection (i.e., B);

Case 3: To directly predict L3 cell level results based on L1 beam level results.

NOTE: for case 2 where the cell quality after beam consolidation/selection (i.e., B) is model input, and for case 3, the layer 3 filtering procedure is internal implementation of AI inference;

Proposal-2: L3 beam level prediction can be considered as following cases:

Case1: To predict L1 beam level results, then generate L3 beam level results based on the predicted L1 beam level results;

Case2: To directly predict L3 beam level results based on L3 beam level results;

Case3: To directly predict L3 beam level results based on L1 beam level results.

NOTE: for case 3, the layer 3 filtering procedure is internal implementation of AI inference.

Proposal-3: RAN2 to confirm the model input and output for different scenarios based on the following table:

| Scenario | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- |
| intra-F-intra-C-T-Case A/B | For case A: Input: L1 intra beam within observation window Output: L1 intra beam within prediction window For case B: Input: subset of measurement instance of L1 intra beam Output: the whole set of measurement instance of L1 intra beam | For case A: Input: L1 or L3 intra cell within observation window Output: L3 intra cell level within prediction window For case B: Input: subset of measurement instance of L1 or L3 intra cell Output: the whole set of measurement instance of L3 intra cell | For case A: Input: L1 intra beam within observation window Output: L3 intra cell within prediction window For case B: Input: subset of measurement instance of L1 intra beam Output: the whole set of measurement instance of L3 intra cell |
| intra-F-intra-C-S | Input: set B of L1 intra beam Output: set A of L1 intra beam | N/A | Input: set B of L1 intra beam Output: L3 intra cell |
| intra-F-inter-C-T-Case A | Input: L1 intra beam (+ inter beam if cluster) within observation window Output: L1 inter beam within prediction window | Input: L1 or L3 intra cell (+ inter cell if cluster) within observation window Output: L3 inter cell level within prediction window | Input: L1 intra beam (+ inter beam if cluster) within observation window Output: L3 inter cell within prediction window |
| intra-F-inter-C-S | Input: L1 intra beam (+ inter beam if cluster) Output: L1 inter beam | Input: L1 or L3 intra cell (+ inter cell if cluster) Output: L3 inter cell | Input: L1 intra beam (+ inter beam if cluster) Output: L3 inter cell |
| inter-F-inter-C-T-Case A | Input: intra frequency L1 beam (+ inter frequency L1 beam if cluster) within observation window Output: inter frequency L1 beam within prediction window | Input: intra frequency L1 or L3 cell (+ inter frequency cell if cluster) within observation window Output: inter frequency L3 cell within prediction window | Input: intra frequency L1 beam (+ inter frequency L1 beam if cluster) within observation window Output: inter frequency L3 cell within prediction window |
| inter-F-inter-C-S | Input: intra frequency L1 beam + (inter frequency L1 beam if cluster) Output: inter frequency L1 beam | Input: intra frequency L1 or L3 cell (+ inter frequency L1 or L3 cell if cluster) Output: inter frequency L3 cell | Output: intra frequency L1 beam + (inter frequency L1 beam if cluster) Output: inter frequency L3 cell |

Proposal-4: AI model can be deactivated when the UE signal quality is good enough and/or the UE is in low mobility or stationary status.

Proposal-5: For UE-side model, the UE should provide the NW with preferred measurement configuration related to the frequencies/cells/beams required for model inference input.

- 4 References