3GPP TSG RAN WG2 #127                                            R2-2406502

Masstricht, Netherlands, Aug 19th –23rd, 2024

Agenda Item:	8.2.4

Source: 	TCL

Title: 	Random Access for Ambient IOT

Document for:	Discussion and decision

Introduction

In Rel-19, a new SID for Ambient IoT was approved in RAN#102 [1]. The objectives of RAN2-led SI are listed as below.

- Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. - RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

In this contribution, we discuss Random access procedure for Ambient IoT.

Discussion

In the last RAN2 meeting, RAN2 discussed 2 step random access and 4 step random access. This paper will discuss the contetion free random access and TDM/FDM random access.

Contention free based random access

In the legacy NR UE system, contention free based random access means the network allocates a random access resources for the UE, so that the UE has a dedicated random access resource, not by contention. So for an Ambient IoT device, we already agreed that slotted-ALOHA is the baseline for Ambient IoT device random access, we assume that this is a contention based random access, for contention free based random access, the reader should allocate random access resources.

Proposal 1: for contention free based random access, the reader allocate the random access resources.

When the reader decides to send paging likely message to a group of Ambient IoT devices, the reader may or may not know the exact number Ambient IoT devices, so the reader may not be able to allocate random access resources for all the devices, in that sense, it is impossible for the reader to provide contention free random access for the Ambient IoT devices. The reader can only allocate random access resource for the determined device, the random access resource should be allocated device by device.

Proposal 2: The reader can only allocate random access resource for the determined device, the random access resource should be allocated device by device.

As RAN2 has already agreed that the slotted-ALOHA is the baseline for random access, we believe that in the same batch of paging, the first round paging message will include a large number of devices. If the reader decides to use contention free based random access, it means that the paging message will include many radio resources for each devices. So we suggest not to use contention free random access in the first round of random access in the same batch of random access.

And in the 2nd and the rest round of random access of the same batch of random access, as there are not that many devices, if the reader still uses contention based random access, it will turn out that the collision may keeps happening and there always some devices will never get accessed to the reader. So the bast way is in the 2nd round and the rest round of random access, the reader can choose contention free random access.

Observation 1: it is not feasible for the reader to choose contention free random access in the first round of random access due to the large number of devices in the first round of random access.

Proposal 3: the reader should not use contention free random access in the first round of random access of the same batch of random access.

Mixed mode for contention free and contention based random access.

In one round of random access, there may be some higher priority devices who need to access the network as soon as possible. On the other hand, the rest of Ambient IoT devices may be lower priority devices and should be access to the reader without guaranteed random access resource. It may be beneficial to consider mixed mode for contention free and contention based random access, as both of these two types of devices can be allowed to the reader in the same round of random access.

Proposal 4: RAN2 is asked to consider Mixed mode for contention free and contention based random access.

TDM and FDM random access

In the last RAN2 meeting, it was agreed slotted-ALOHA based access is studied, which is similar to the RFID inventory procedure. In another word, the radio resource for Ambient IoT device random access is distributed allocated in time domain and in different slot. Basically, this is TDM based random access mechanism, i.e. the Ambient IoT devices respond to the reader one by one, and the reader will process the next Ambient IoT device after the reader completes the process for the previous Ambient IoT device. In legacy NR system, the RACH procedure can be multiplexed in frequency domain which can increase the RACH capacity.

From the RAN2 point of view, FDM is beneficial as more devices will be allowed to access to the reader simultaneously. However, for Ambient IoT device, whether FDM based random access can be studied or not in RAN2 depending on the RAN1 progress, e.g. whether FDMA can be supported for Ambient IoT D2R transmission.

Proposal 5: RAN2 is asked to agree TDM based random access procedure as the baseline. FDM based random access procedure can be beneficial from RAN2 aspect and wait for RAN1 progress.

Conclusion

In this contribution, we discussed the Ambient IoT device paging issue. Then we have the following observations and proposals:

Observation 1: it is not feasible for the reader to choose contention free random access in the first round of random access due to the large number of devices in the first round of random access.

Proposal 1: for contention free based random access, the reader allocate the random access resources, including the radio resource slot.

Proposal 2: The reader can only allocate random access resource for the determined device, the random access resource should be allocated device by device.

Proposal 3: the reader should not use contention free random access in the first round of random access of the same batch of random access.

Proposal 4: RAN2 is asked to consider Mixed mode for contention free and contention based random access.

Proposal 5: RAN2 is asked to agree TDM based random access procedure as the baseline. FDM based random access procedure can be beneficial from RAN2 aspect and wait for RAN1 progress.

Reference

RAN2#125bis chairman notes