3GPP TSG RAN WG2 #127                                                                     R2-2406503

Masstricht, Netherlands, Aug 19th –23rd, 2024

Source:	TCL

Title:	Discussion on NW side data collection

Agenda Item:	8.1.3

Document for:	Discussion and Decision

- Introduction

Data collection is a very important function in the AIML-based LCM, which is used to provide the input data to the different LCM aspects (i.e., model inference, monitoring, and training). RAN2#126 meeting has achieved some agreements about NW-side data collection for beam management use cases [1].

Agreements for beam management 1. For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement. 2. Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model 3. Enhance the immediate MDT framework to support periodical reporting.00 FFS whether and what event-based reporting is supported and FFS on network request reporting

In this contribution, we provide our views on data collection for network side model training in both beam management and positioning use cases, respectively.

- Discussion

2.1 Beam management

In the last RAN2 meeting, it was agreed that for beam management use cases, both the gNB-centric data collection approach and OAM-centric data collection approach should be studied for the NW-side data collection. Specifically, UE uses a RRC message to transmit multiple instances of logged L1 measurement results to gNB over air interface. And it is FSS how to handle case when a single RRC message is not sufficient.

Observation 1: RAN2 assumed that using a RRC message transfers the L1 measurement results.

In the legacy RRM measurement reporting framework, the network configures a UE to perform measurements using RRCResume message or RRCReconfiguration message, which includes measurement objects, reporting configurations, measurement identities, etc. Based on the measurement configuration, UE reports the measurement results to gNB within the MeasurementReport message in a periodical manner or event-trigger manner. The measurement results information per cell and per beam that includes RSRP, RSRQ, SINR, and other related information such as the maximum number of cells and the maximum number beams per cell.  Moreover, the immediate MDT framework is based on the existing RRM measurement procedure including measurement and reporting configuration, which is also applied in the RRC_CONNECTED UE. UE reports its measurements results with some extensions for location information (e.g. GNSS location information) to the gNB by measurement reporting message, then gNB forwards the information to the TCE or OAM. It is noted that the information is not transparent to the gNB.

Based on the above analysis, it can be found that the RRM measurement reporting framework is workable in the air interface for both gNB-centric data collection and OAM-centric data collection. Hence, in order to reduce the workload of RAN2, the legacy RRM measurement reporting framework can be regarded as a starting point for the NW-side data collection.

Observation 2: Immediate MDT framework is based on the legacy RRM measurement procedure. Proposal 1: For the NW-side data collection, it is preferred to consider the legacy RRM measurement reporting framework as a baseline.

However, the legacy RRM measurement reporting framework is used to collect the L3 measurements results, location formation, as well as other information. If it is reused to the NW-side data collection for model training in beam management use cases, the associated L1 measurement results reporting should be supported. That is to say, the new AI/ML data content for L1 beam measurement results needs to be introduced in the legacy RRM measurement configuration.

Proposal 2: New AI/ML data content for L1 beam measurement results should be introduced in RRM measurement configuration.

In the Rel-18 study item, RAN1 has provided examples to roughly illustrate the typical training data size for beam management [2], which can be considered as potential guidance for RAN2 discussion. We summarize it in Table 1.

Table1: The typical data size for model training in beam management

| Use case | Data content for model training | Data size (example) |
| --- | --- | --- |
| Beam management | L1-RSRPs [and beam-IDs] for Set B | - For Set B = 16 as an example, the typical data size would be 67 (hence up to ~100 bits), - For Set A = 128 as an example, the typical data size would be 515 (hence up to ~500 bits). |
|  | L1-RSRPs and/or beam-IDs from Set A is the training data for the NW-side model |  |

Observation 3: For data collection of NW-sided training, the collected data (i.e., L1-RSRP/beam ID) is generated by UE and terminated at the NW, the corresponding typical data size is up to 500 bits (Set A = 128), or 100 bits (Set B = 16).

Based on above mentioned, the L1 beam measurement results can be transmitted from UE to gNB by MeasurementReport RRC message, which maximum size is up to 9kbyte. According to Table1, it is easy to see that a single RRC massage can carry hundreds of instances training data in beam management use cases. Hence, in some situations where the large-scale training data may be not required, for example, retraining or fine-turning, the multiple instances of L1 measurements carried by a single RRC massage may be sufficient. In fact, whether a single RRC message is enough to collect training data depends on the size of the training data, which needs further information and conclusion from RAN1.

Proposal 3:  Whether a single RRC message is sufficient to collect training data depends on the size of  training data, which needs the further conclusion from RAN1.

RAN2 assumes that if the size of training data is larger than the limit of 9kbytes, a single RRC message is unable to satisfy the large data size requirement of model training and cannot achieve a complete model training data collection. One alternative is to use UE to perform multiple reports for the collected training data, while it will increase the signalling overhead and power consumption. Another alternative is suggested that RRM measurement reporting message supports the RRC segmentation. Since in the current NR specification, UL dedicated message segmentation only applies to UECapabilityInformation and MeasurementReportAppLayer, where SRB1 is used at the transfer of segments of UECapabilityInformation and SRB4 is used at the transfer of segments of MeasurementReportAppLayer. Therefore, if using the measurement reporting message to transmit the training data for the NW-side data collection, it is suggested that RRM measurement reporting message needs to support the RRC segmentation, with a payload size of 144kbyte.

Proposal 4:  Suggest that legacy RRM measurement reporting message (i.e., MeasurementReport ) needs to support the RRC segmentation, if needed.

In the existing RRM measurement reporting framework, UE reports the measurements results based on the configuration such as periodical information and event trigger parameters. However, the one-shot measurement results are transmitted at the time of measurement reporting. For example, the UE reports the immediate measurement results once satisfy the trigger condition and defined TTT. Considering it is needed a large amount of training data  for the model training, the data logging for the legacy RRM measurement reporting framework should be supported.

Observation 4: Data logging for the legacy RRM measurement reporting framework should be supported.

Although the RRM measurement reporting already supports event-trigger and periodical reporting, which are not applicable and useful for the data logging. For instance, the current event-trigger reporting about measurement results is used to assist the HO decision, the measurement event conditions seem not applicable to trigger the training data for AIML. Hence, we suggest to consider the new criteria for training data collection, which is used to log the training data in legacy RRM measurement reporting, e.g.:

- Data size is larger than a threshold.

- Collecting time of data collection meets a defined time duration.

Proposal 5: NW should configure the new criteria for training data collection for logging the training data, e.g.,

-   Data size is larger than a threshold.

-   Collecting time of data collection meets a defined time duration.

2.2 Positioning

In this section, we discuss the first priority positioning use cases Case 3a and Case 3b in Section 2.2.1 and 2.2.2, respectively.

Case 3a

The training data can be autonomously collected by gNB or OAM for Case 3a. As for gNB-centric data collection, SRS is configured reusing the legacy RRC message for measurement data collection. The label data (intermediate positioning measurement) can be calculated based on the UE location information, which can be contained in the RRM measurement report associated with the corresponding SRS configuration for data collection to match the measurement data and label data. The SRS configuration can be contained in the RRM measurement report or associated to the RRM measurement report via a measurement ID.

Proposal 6: The RRM measurement report containing UE location information can be associated with the corresponding SRS configuration for label data collection for Case 3a.

The label data can also be generated by LMF based on the UE location information estimated by traditional uplink positioning method. In this way, the NRPPa protocol can be extended to support the transfer of label data. For example, gNB can request the label data from LMF along with the time stamp (e.g., the SRS transmission time) and then LMF transfers the label data corresponding to the SRS measurement data to gNB. The NRPPa protocol extension can be further discussed in RAN3.

Proposal 7: The NRPPa protocol can be extended to support the transfer of label data generated by LMF for Case 3a and it can be further discussed in RAN3.

As for OAM-centric data collection, the Immediate MDT with RAN measurement configuration can be extended. The channel measurement (e.g., DP, PDP, CIR) by RAN should be additionally supported for Immediate MDT so that OAM can collect the measurement data. The measurement collection can be triggered by event or at the end of measurement collection period, and the trigger events can be further discussed. Besides, the UE location information provision is supported in the legacy Immediate MDT so there should be no extension for the label data collection.

Proposal 8: The Immediate MDT with NW-side channel measurements, e.g., DP, PDP, CIR, can be introduced to support OAM-centric training data collection for Case 3a.

Case 3b

The training data can be collected by LMF for Case 3b. LMF collects the measurement data by indicating the SRS configuration for data collection to gNB. Considering that gNB transfers the measurement data rather than intermediate positioning measurements to LMF for training data collection, the NRPPa protocol can be extended by introducing the indication of data collection in order to make gNB aware of the transferred data format. The detailed data collection configuration can be further discussed in RAN3.

Proposal 9: The NRPPa protocol can be extended to support the measurement data transfer between gNB and LMF for Case 3b, which can be further discussed in RAN3.

The label data can be generated by PRU, non-PRU UE and LMF, which is captured as follows [3].

Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed.

If the label data is generated by PRU and LMF, LMF can easily match the measurement data and label data. However, the non-PRU UE may generate the label data via PRS. In this case, the UE may move within the time gap between the PRS reception and SRS transmission, which leads to the mismatch between the measurement data and label data. Hence, the data association should be considered to guarantee the reliability of model training. Intuitively, LMF can request the label data from UE via LPP signaling and then UE sends the label data to LMF to check validity or sends the label data after self-validation along with the assistance information. The assistance information for data association waits for RAN1 discussion.

Proposal 10: The data association procedure should be considered for Case 3b and the assistance information for data association waits for RAN1 discussion.

- Conclusion

We have the following observations and proposals

Observation 1: RAN2 assumed that using a RRC message transfers the L1 measurement results.

Observation 2: Immediate MDT framework is based on the legacy RRM measurement procedure.

Observation 3: For data collection of NW-sided training, the collected data (i.e., L1-RSRP/beam ID) is generated by UE and terminated at the NW, the corresponding typical data size is up to 500 bits (Set A = 128), or 100 bits (Set B = 16).

Observation 4: Data logging for the legacy RRM measurement reporting framework should be supported.

Proposal 1: For the NW-side data collection, it is preferred to consider the legacy RRM measurement reporting framework as a baseline.

Proposal 2: New AI/ML data content for L1 beam measurement results should be introduced in RRM measurement configuration.

Proposal 3:  Whether a single RRC message is sufficient to collect training data depends on the size of  training data, which needs the further conclusion from RAN1.

Proposal 4:  Suggest that legacy RRM measurement reporting message (i.e., MeasurementReport ) needs to support the RRC segmentation, if needed.

Proposal 5: NW should configure the new criteria for training data collection for logging the training data, e.g.,

- Data size is larger than a threshold.

- Collecting time of data collection meets a defined time duration.

Proposal 6: The RRM measurement report containing UE location information can be associated with the corresponding SRS configuration for label data collection for Case 3a.

Proposal 7: The NRPPa protocol can be extended to support the transfer of label data generated by LMF for Case 3a and it can be further discussed in RAN3.

Proposal 8: The Immediate MDT with NW-side channel measurements, e.g., DP, PDP, CIR, can be introduced to support OAM-centric training data collection for Case 3a.

Proposal 9: The NRPPa protocol can be extended to support the measurement data transfer between gNB and LMF for Case 3b, which can be further discussed in RAN3.

Proposal 10: The data association procedure should be considered for Case 3b and the assistance information for data association waits for RAN1 discussion.

- References

- RAN2 #126 Chair Notes

- R1-2308730 Reply LS on Data Collection Requirements and Assumptions

- RAN1 #117 Chair Notes