3GPP TSG-RAN WG2 Meeting #127	R2-2406504

Maastricht, Netherlands, Aug 18th – 23rd, 2024

Agenda item:		7.5.3

Source:				Samsung

Title:					Corrections for PDCP SN Gap Reporting

Document for:		Discussion & Decision

# 1 Introduction

In this paper, we address the needed corrections for PDCP SN gap reporting.

# 2 Discussion

### 2.1 Setting FDC field

In RAN2#126, the PDCP SN gap reporting issue was discussed; but no conclusion could be reached.

R2-2404665 Corrections for PDCP SN Gap Reporting Samsung discussion Rel-18 Proposal 2: PDCP SN gap report is composed with considering the first discarded PDCP SDU and the last discarded PDCP SDU in the bitmap based approach that have not been reported earlier in PDCP SN gap report. Adopt text proposal provided. - Ericsson and LG thinks it is not a problem and it doesn’t need to be explicitly captured. - Vivo thinks the problem is valid but the correction is not.

It can be noted that discard can possibly take place for PDCP SDUs not in sequence due to different timer configurations for the discardTimer and discardTimerLowImportance. In this scenario, how to set FDC field in the PDCP SN gap report should be clarified. We illustrate this with an example.

FIG 1: An illustration for different FDC field settings

FIG. 1 depicts the following example:

- PDCP SDUs A and C have discardTimerLowImportance configured

- PDCP SDUs B and D have discardTimer configured

- At T1: A and C are discarded and signalled in PDCP SN gap report R1. FDC is set as A.

- At T2: B is discarded. PDCP SN gap report is not reported (e.g. UE implementation may avoid too frequent reporting)

- At T3: D is discarded. PDCP SN gap report R2 is sent.

Now, following cases could be considerable about FDC field setting

- Case 1: B, C and D are signalled in PDCP SN gap report R2. FDC is set as B

- Case 2: A, B, C and D are signalled in PDCP SN gap report R2. FDC is set as A

Case 2 has FDC set to a redundant value of A and causes unnecessary handling at receiver. More particularly, concern is that the different implementations can exist and there is a lack of well-specified mechanism. That is, FDC value setting should be well determined in the specification.

Observation 1: Out-of-Sequence discard of PDCP SDUs is possible due to different configurations of discardTimer and discardTimerLowImportance. Further, it is up to UE implementation how to limit the frequency of PDCP SN gap reporting. In any case, how to set the FDC field should be well determined in the specification.

Proposal 1: FDC field is set to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s) since the last transmission of the PDCP SN gap report. Adopt text proposal TP1 provided.

Text Proposal TP1:

### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - the PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by RLC to lower layers. If a PDCP SN gap report is triggered, the transmitting PDCP entity shall: - compile a PDCP SN gap report as indicated below by: - setting the FDC field to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s) since the last transmission of the PDCP SN gap report; - if more than one PDCP SDUs are discarded: - allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first discarded PDCP SDU FDC up to and including the last discarded PDCP SDU, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first; - setting in the discard bitmap field as '0' for all PDCP SDUs that have not been discarded; - setting in the discard bitmap field as '1' for all PDCP SDUs that have been discarded; - submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface. NOTE: It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

### 2.2 Transmission priority

Apparently it is not yet contemplated whether the PDCP SN gap report transmission would be prioritized (e.g., transmitted as the first PDCP PDU for transmission) or be allowed to be transmitted in any order (e.g. PDCP SN gap report may be transmitted after the PDCP data PDUs). We think transmission priority for PDCP SN gap is significant as it ensures the timely transmission of PDCP SN gap report from PDCP and RLC layers, even in the situation when the uplink grant available is limited. Early availability of discard information is vital for the receiver’s efficient operation. Also, note that PDCP Status report is only applicable at PDCP entity re-establishment or PDCP data receovery where PDCP status report is considered as the first PDCP PDU for the transmission. Therefore, there is no conflict for PDCP SN gap report transmission w.r.t. to PDCP status report.

Proposal 2: PDCP SN gap report is submitted to lower layers as the first PDCP PDU for transmission. Adopt text proposal TP2 provided.

Text Proposal TP2:

### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - the PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by RLC to lower layers. If a PDCP SN gap report is triggered, the transmitting PDCP entity shall: - compile a PDCP SN gap report as indicated below by: - setting the FDC field to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s); - if more than one PDCP SDUs are discarded: - allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first discarded PDCP SDU up to and including the last discarded PDCP SDU, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first; - setting in the discard bitmap field as '0' for all PDCP SDUs that have not been discarded; - setting in the discard bitmap field as '1' for all PDCP SDUs that have been discarded; - submit the PDCP SN gap report to lower layers as the first PDCP PDU for transmission as specified in clause 5.2.1 for Uu interface. NOTE: It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

# 3 Conclusion

RAN2 is requested to discuss and agree to the following observations and proposals:

Observation 1: Out-of-Sequence discard of PDCP SDUs is possible due to different configurations of discardTimer and discardTimerLowImportance. Further, it is up to UE implementation how to limit the frequency of PDCP SN gap reporting. In any case, how to set the FDC field should be well determined in the specification.

Proposal 1: FDC field is set to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s) since the last transmission of the PDCP SN gap report. Adopt text proposal TP1 provided.

Proposal 2: PDCP SN gap report is submitted to lower layers as the first PDCP PDU for transmission. Adopt text proposal TP2 provided.