3GPP TSG-RAN WG2 Meeting #127									   R2-2406505

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.2.3

Source: 	NEC

Title:	Ambient-IoT Functionality Aspects

Document for:	Discussion/Decision

- Introduction

The Rel-19 SID for Ambient IoT (also referred to as A-IoT) includes following RAN2-led objectives [1].

- RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

This contribution discusses the functionality aspects based on the RAN2-led objectives and the latest RAN1 and RAN2 agreements.

- Discussion

  - User Plane Functions

    - Resource Allocation/Scheduling

Since RAN1 has agreed to support physical channel of PRDCH and PDRCH for R2D and D2R transmissions, we think resource allocation/scheduling discussions are needed for both PRDCH and PDRCH. Currently, scheduling related information of PRDCH and PDRCH is under RAN1 discussion and so far, no agreements were achieved.

RAN1 FL summary [2] listed some potential scheduling related information for PRDCH (copied below):

- Device’s identification of the end of PRDCH transmission

- chip length indication

- MCS related information/parameter

- TBS related information/parameters

- AIoT Device ID

Among the above candidates, we think the last one, AIoT Device ID is in RAN2 scope and others are in RAN1 scope. Hereafter, we focus on whether AIoT device ID is needed as PRDCH scheduling related information.

PRDCH can be used to carry MAC layer control information, e.g., initial trigger message (A-IoT paging message), Msg.2 in the RA, etc. For Msg.2 in the RA, RAN2#126 agreed to include the ID received in Msg.1 for contention resolution whereas for AIoT paging message, RAN2#126 agreed to study the following four cases:

•	a message containing an ID of a single A-IoT device.

•	a message containing a group ID that maps to multiple A-IoT devices.

•	a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message.

PRDCH can also be used to carry MAC layer data (i.e., higher-layer data from the CN), for example, the content from higher layer for command “write”. According to description in TR 23.700-13 V0.3.0, the command operations allow the consumer to request to perform a read or write device operation for an AIoT device or a group of AIoT devices, therefore we think for PRDCH carrying MAC layer data, AIoT device ID could be included in the PRDCH as a part of scheduling related information. Other information (i.e., MCS, TBS, etc.) is up to RAN1 decision.

As PRDCH scheduling related information, RAN2 assumes A-IoT device ID could be included for MAC layer control information and MAC layer data transmissions. Other scheduling related information, e.g., Device’s identification of the end of PRDCH transmission, chip length indication, MCS, TBS, etc. is up to RAN1 decision.

RAN1 FL summary [2] also listed some potential scheduling related information for PDRCH (copied below):

- Reader’s identification of the end of PRDCH transmission

- chip length indication

- MCS related information/parameter

- TBS related information/parameters

- Time domain resource allocation (TDRA) related information/parameters

- Frequency domain resource allocation (FDRA) related information/parameters

- repetition

- power control

Further, on the scheduling information of PDRCH transmission, RAN1#117 meeting [3] agreed that

RAN1#117 Agreements: - Scheduling information of PDRCH transmission is provided by a corresponding PRDCH.

RAN2 assumes scheduling information of PDRCH transmission is provided by a corresponding PRDCH. Details of scheduling information of PDRCH transmission are up to RAN1 decision.

    - Message Size/Status Indication

During RAN2#126 discussion, some companies commented that message size/status indication from devices to the reader might be beneficial when the buffer size is larger than the allocated TBS.

RAN1#116bis [4] and RAN1#117 [3] meeting agreed following:

RAN1#116 Agreements: For the reader to acquire the end of PDRCH transmission, study at least following options: • Option 1: D2R postamble immediately follows the PDRCH • Option 2: Based on control information RAN1#117 Agreements: - Study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed), and if so for how long after receiving an R2D transmission.

Based on above RAN1 agreements, we observe that a PDRCH transmission duration with sufficient accuracy after receiving an R2D transmission and the end of PDRCH transmission are under RAN1 discussion.

A maximum PDRCH transmission duration with sufficient accuracy after receiving an R2D transmission and the end of PDRCH transmission are under RAN1 discussion.

TBS, as a potential PDRCH scheduling related information can be assumed to be determined based on PDRCH transmission duration and MCS. When the buffer size is larger than the allocated TBS (i.e., the necessary PDRCH transmission duration is longer than the maximum PDRCH transmission duration, devices may not be able to transmit the end of PDRCH which is mainly used to indicate the early PDRCH termination. Then message size/status indicating can be performed in an implicit way, i.e., whether the end of PDRCH is detected by the reader or not. We suggest waiting for further RAN1 progresses and postpone discussions on message size/status indication.

RAN2 postpones discussions on message size/status indication to wait for further RAN1 progresses.

    - Padding

In A-IoT, the D2R transmission is triggered by the reader while there isn’t any buffer status at that time. Thus, it may happened that the grant is larger than the higher layer data available to transmit. In NR, padding is used to solve this problem by reaching the exact grant size in UL. Since there is no need to keep byte alignment for A-IoT, the device can terminate the transmission early according to the exact data size. How to solve it is also related to the following agreement of RAN1#116bis [4] meeting on acquiring the end of PDRCH transmission:

RAN1#116 Agreements: For the reader to acquire the end of PDRCH transmission, study at least following options: • Option 1: D2R postamble immediately follows the PDRCH • Option 2: Based on control information

Either one of the above options can be used for the indication of the early PDRCH termination. So we suggest not to adopt padding function for MAC layer and the exact method for the indication of the early PDRCH termination is up to RAN1.

RAN2 doesn’t support padding function. And the method for the indication of the early PDRCH termination is up to RAN1.

    - Multiplexing

The transmission of signaling and the transmission of data are usually separate. Thus, there is no need to consider the multiplexing of multiple layer2 channels. Moreover, the reader can schedule resources as much as possible. Thus, there is no need to consider the LCP of multiple layer2 channels.

In previous RAN2 meeting, only one logical channel is agreed for A-IoT transmission. However, during RAN2#126 discussion, some companies mentioned that indications on segmentation and harvesting statues from devices to the reader might be useful for the control of devices. These indication may be reported to the reader via MAC layer. Therefore, these indications may be sent together with the higher layer data. But they are not discussed and decided yet. So we propose:

RAN2 postpones the discussion on multiplexing.

  - Control Plane Functions

    - Harvesting-related Information

As per the SID [1], both type A device and type B device are based on the harvesting procedure to enable the backscattering. However, the following factors may cause A-IoT devices to not have enough power to transmit:

- Too far away from the reader, or

- The harvesting signal is blocked.

Therefore, harvesting-related information can be reported to trigger the network to adjust accordingly.

Control plan also supports harvesting-related information reporting procedure.

    - Device variable control

Considering the limited size and complexity required by practical applications for battery-less devices with no energy storage capability or devices with limited energy storage that do not need to be replaced or recharged manually, the persistence of device variables may consider both the energy status of the A-IoT device and the type of the device variables.

From the persistence behavior perspective, three types of device variable control can be considered:

Type 1 Infinite persistent: The variables required to be kept even the A-IoT device is de-energized. For example, the information related to registration with the NW, which are stored in the device memory.

Type 2 Limited persistent: The variables may be refreshed under some circumstances. For example, device maintain those variables under specific temperature range and the energy storage meet the persistence requirements, otherwise, the device will lose the stored value.

Type 3 Non-persistent: A-IoT device shall refresh non-persistent variables when powered, meaning that every time an A-IoT device loses power its non-persistent variables will be lost.

Considering three types of Device variable control behaviors:

Type 1 Infinite persistent: The variables need to be stored even the A-IoT device is de-energized, such as information related to registration with the NW, etc.

Type 2 Limited persistent: The variables may be refreshed under some circumstances. For example, device maintain those variables under specific temperature range and the energy storage meet the persistence requirements, otherwise, the device will lose the stored value.

Type 3 Non-persistent: A-IoT device shall refresh non-persistent variables when powered, meaning that every time an A-IoT device loses power its non-persistent variables will be lost.

As discussed above, we propose:

RAN2 studies the following functions:

Harvesting-related information reporting procedure;

Device variable control procedure.

- Conclusion

Based on the discussion in this document, we have the following observations and proposals:

Observation 1:	A maximum PDRCH transmission duration with sufficient accuracy after receiving an R2D transmission and the end of PDRCH transmission are under RAN1 discussion.

Observation 2:	Control plan also supports harvesting-related information reporting procedure.

Observation 3:	Considering three types of Device variable control behaviors:

−	Type 1 Infinite persistent: The variables need to be stored even the A-IoT device is de-energized, such as information related to registration with the NW, etc.

−	Type 2 Limited persistent: The variables may be refreshed under some circumstances. For example, device maintain those variables under specific temperature range and the energy storage meet the persistence requirements, otherwise, the device will lose the stored value.

−	Type 3 Non-persistent: A-IoT device shall refresh non-persistent variables when powered, meaning that every time an A-IoT device loses power its non-persistent variables will be lost.

Proposal 1:	As PRDCH scheduling related information, RAN2 assumes A-IoT device ID could be included for MAC layer control information and MAC layer data transmissions. Other scheduling related information, e.g., Device’s identification of the end of PRDCH transmission, chip length indication, MCS, TBS, etc. is up to RAN1 decision.

Proposal 2:	RAN2 assumes scheduling information of PDRCH transmission is provided by a corresponding PRDCH. Details of scheduling information of PDRCH transmission are up to RAN1 decision.

Proposal 3:	RAN2 postpones discussions on message size/status indication to wait for further RAN1 progresses.

Proposal 4:	RAN2 doesn’t support padding function. And the method for the indication of the early PDRCH termination is up to RAN1.

Proposal 5:	RAN2 postpones the discussion on multiplexing.

Proposal 6:	RAN2 studies the following functions:

−	Harvesting-related information reporting procedure;

−	Device variable control procedure.

- Reference

- RP-240156, Rel-19 SID of Ambient IoT.

- R1-2405381, FL summary #3 on frame structure and timing aspects for Rel-19 Ambient IoT, Moderator (vivo)

- Chairman Notes, RAN1 #117 meeting.

- Chairman Notes, RAN1 #116bis meeting.

- Chairman Notes, RAN2 #125bis meeting.