3GPP TSG-RAN WG2 Meeting #127									   R2-2406506

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.13.3

Source: 	NEC

Title:	Considerations on Control Plane of Multi-hop Relay

Document for:	Discussion/Decision

- Introduction

The Rel-19 WID for Sidelink Relay includes following objectives related to control plane [1].

1. Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. A. Relay discovery and (re)selection [RAN2] B. Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] C. Impact on SRAP and QoS handling for multi-hop [RAN2] D. Control plane procedures [RAN2, RAN3]

This contribution discusses control plane related issues.

- Discussion

  - RRC Connection for the First Relay UE

The multi-hop indirect path comprises one additional hop relay on top of the Rel-17 U2N relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB).

The first relay UE is connected to the gNB via the last relay UE. Given the U2N relay scheme, the first relay UE is also under the control of the gNB of the last relay UE. Therefore, for the control of the first relay UE, the RRC connection shall be established.

A first relay UE in RRC_IDLE/RRC_INACTIVE may be selected by the remote UE due to not knowing the RRC status of the first relay UE. Then, the first RRC message received from the remote UE will trigger the first relay UE to enter RRC_CONNECTED when the remote UE and the gNB establish the RRC connection.

Given the U2N relay scheme, the first relay UE is under the control of the gNB. And a first relay UE in RRC_IDLE/RRC_INACTIVE can be selected by the remote UE.

The first RRC message, for the RRC connection establishing between the remote UE and the gNB, received from the remote UE will trigger the first relay UE to enter RRC_CONNECTED.

In addition, for the communication (such as notification message) between the remote UE and the last relay UE, the E2E PC5 connection shall be established.

For the communication between the remote UE and the last relay UE, the E2E PC5 connection shall be established.

  - Data forwarding

Based on the existing methods of U2N relay and U2U relay [2], the connection between the remote UE and the gNB can be divided into two parts:

- The U2U-like connection between the remote UE and the last relay UE, and

- The connection between the last relay UE and the gNB serving it.

The connection between the remote UE and the gNB can be divided into two parts: the U2U-like connection between the remote UE and the last relay UE, and the connection between the last relay UE and the gNB.

Therefore, the data forwarding can also be divided into two parts accordingly.

Based on the U2U-like connection, the U2U-like SRAP method [3], such as the local ID pair of the remote UE and the last relay UE, can be used for the data forwarding between the remote UE and the last relay UE. Additionally, the U2N-like SRAP method [3], wherein only the local ID of the remote UE is used, can be used for the data forwarding between the last relay UE and the gNB. For example, the last relay UE will remove the local ID of the remote UE form the SRAP PDU when it forwards the data to the gNB. And in the opposite direction, the last relay UE will add the local ID of the remote UE into the SRAP PDU.

The U2U-like SRAP method, such as the local ID pair of the remote UE and the last relay UE, can be used for the data forwarding between the remote UE and the last relay UE.

The U2N-like SRAP method, wherein only the local ID of the remote UE is used, can be used for the data forwarding between the last relay UE and the gNB.

  - Configurations of SRAP and QoS

Considering the U2N relay, the SRAP configurations, especially those for the first relay UE, are provided by the gNB.

Similarly, the QoS is under the control of the gNB. Thus, the QoS parameters of the first relay UE are also configured by the gNB.

The SRAP configurations, especially those for the first relay UE, are provided by the gNB.

The QoS parameters, especially those for the first relay UE, are configured by the gNB.

- Conclusion

Based on the discussion in this document, we have the following observations and proposals:

Observation 1:	Given the U2N relay scheme, the first relay UE is under the control of the gNB. And a first relay UE in RRC_IDLE/RRC_INACTIVE can be selected by the remote UE.

Observation 2:	The connection between the remote UE and the gNB can be divided into two parts: the U2U-like connection between the remote UE and the last relay UE, and the connection between the last relay UE and the gNB.

Proposal 1:	The first RRC message, for the RRC connection establishing between the remote UE and the gNB, received from the remote UE will trigger the first relay UE to enter RRC_CONNECTED.

Proposal 2:	For the communication between the remote UE and the last relay UE, the E2E PC5 connection shall be established.

Proposal 3:	The U2U-like SRAP method, such as the local ID pair of the remote UE and the last relay UE, can be used for the data forwarding between the remote UE and the last relay UE.

Proposal 4:	The U2N-like SRAP method, wherein only the local ID of the remote UE is used, can be used for the data forwarding between the last relay UE and the gNB.

Proposal 5:	The SRAP configurations, especially those for the first relay UE, are provided by the gNB.

Proposal 6:	The QoS parameters, especially those for the first relay UE, are configured by the gNB.

- Reference

- RP-241609, Rel-19 WID of NR Sidelink Relay.

- TS38.331-i20.

- TS38.351-i20.